## Appendix A: APIs for Accessing Library Functions

From this point and on contains information about utilizing the radio tool source and library. You may stop here if you just want to use the radio tool as it is.

The followings are the API functions used in the Radio Tool library files. You may access the three library files (CC3100Lib.dll, CC3100LibUART.dll, and CC3200Lib.dll) directly without the use of Radio Tool GUI application for your own tool development.

[Collapse] Extended content

### Radio Tool API The API calls are pretty much identical for CC3100 and CC3200, with the exception of SPI connection, which doesn't require a COM port or baud rate like UART does. #### RadioToolOpen (CC3100 SPI) INT32 RadioToolOpen() - Parameters: - Return - 0 if success, and a negative value if failed. #### RadioToolOpen (UART) INT32 RadioToolOpen(UINT8 comPort, UINT32 baudRate) - Parameters: comPort; UART COM port. baudRate; UART baud rate. - Return - 0 if success, and a negative value if failed. #### RadioToolClose INT32 RadioToolClose() - Parameters: - Return - 0 if success, and a negative value if failed. #### RadioStartTX INT32 RadioStartTX(RadioTxMode_e eTxMode, INT8 powerLevel_Tone, Channel_e eChannel, RateIndex_e eRate, RadioPreamble_e ePreamble, RadioDataPattern_e eDataPattern, UINT16 size, UINT32 delay_amount, UINT8 overrideCCA, UINT8 *pDstMac) - Parameters: eTxMode; TX Mode enumerator, see the struct type below. powerLevel_Tone; Power Level enumerator, see the struct type below. eChannel; Channel enumerator, see the struct type below. eRate; Rate enumerator, see the struct type below. ePreamble; Preamble enumerator, see the struct type below. eDataPattern; Data Pattern enumerator, see the struct type below. size; TX packet size, 1400 bytes maximum. delay_amount; delay time in microsecond (mSec), minimum of 50 mSec. overrideCCA; Yes or No. pDstMac; Destination MAC address. - Return - 0 if success, and a negative value if failed. #### RadioStopTX INT32 RadioStopTX(RadioTxMode_e eTxMode) - Parameters: eTxMode; TX Mode enumerator, see the struct type below. - Return - 0 if success, and a negative value if failed. #### RadioStartRX INT32 RadioStartRX(Channel_e eChannel) - Parameters: eChannel; Channel enumerator, see the struct type below. - Return - 0 if success, and a negative value if failed. #### RadioStopRX INT32 RadioStopRX() - Parameters: - Return - 0 if success, and a negative value if failed. #### RadioGetStats INT32 RadioGetStats(UINT32 *validPackets, UINT32 *fcsPackets, UINT32 *addrMMPackets, INT16 *avgRssiMgmt, INT16 *avgRssiOther, UINT16 *pRssiHistogram, UINT16 *pRateHistogram) - Parameters: validPackets; Number of valid packets. fcsPackets; Number of FCS error packets. addrMMPackets; Number of MAC address mismatch packets. avgRssiMgmt; Average RSSI signal strength (management frame). avgRssiOther; Average RSSI signal strength (other frame). pRssiHistogram; An array of 6 elements. RSSI range is from -40dBm to -87dBm with 8dB step.. pRateHistogram; An array of 20 elements. The number of packet occurrences for each rate. Please refer to Rate enumerator for a complete list.. - Return - 0 if success, and a negative value if failed. #### RadioGetMacAddr INT32 RadioGetMacAddr(UINT8 *pMacAddress) - Parameters: pMacAddress; The MAC address read from the device. MAC address is searched in the NVMEM first and if not exists, in the EFUSE.. - Return - 0 if success, and a negative value if failed. #### RadioGetDeviceVersion INT32 RadioGetDeviceVersion(UINT8 *pDevVersion) - Parameters: pDevVersion; The device version. The version includes NWP/MAC/PHY versions, chip ID and ROM version. All can be found under DeviceVersion_s (in Data Structures section). 46 bytes long. In CC3200, the last two padding bytes indicate on-board application version.. - Return - 0 if success, and a negative value if failed. ### Type definition The following definitions are used in the PC-based library functions. #### Primitive types - INT8: signed 8-bit integer - INT16: signed 16-bit integer - INT32: signed 32-bit integer - UINT8 : unsigned 8-bit integer - UINT16: unsigned 16-bit integer - UINT32: unsigned 32-bit integer #### Struct types - TX Mode enumerator typedef enum { RADIO_TX_CONTINUOUS = 1, RADIO_TX_PACKETIZED = 2, RADIO_TX_CW = 3, MAX_RADIO_TX_MODE = 0xff }RadioTxMode_e; - Power Level enumerator The Max power level is 0. The lowest power level is 15 typedef enum { HIGH_GAIN_STEP_0 = 0, HIGH_GAIN_STEP_1 = 1, HIGH_GAIN_STEP_2 = 2, HIGH_GAIN_STEP_3 = 3, MID_GAIN_STEP_0 = 4, MID_GAIN_STEP_1 = 5, MID_GAIN_STEP_2 = 6, MID_GAIN_STEP_3 = 7, MID_GAIN_STEP_4 = 8, MID_GAIN_STEP_5 = 9, MID_GAIN_STEP_6 = 10, MID_GAIN_STEP_7 = 11, MID_GAIN_STEP_8 = 12, MID_GAIN_STEP_9 = 13, MID_GAIN_STEP_10 = 14, MID_GAIN_STEP_11 = 15, MAX_RADIO_POWER_LEVEL = 0xff }RadioPowerLevel_e; - Channel enumerator typedef enum { CHANNEL_1 = 1, CHANNEL_2 = 2, CHANNEL_3 = 3, CHANNEL_4 = 4, CHANNEL_5 = 5, CHANNEL_6 = 6, CHANNEL_7 = 7, CHANNEL_8 = 8, CHANNEL_9 = 9, CHANNEL_10 = 10, CHANNEL_11 = 11, CHANNEL_12 = 12, CHANNEL_11 = 13, MAX_CHANNELS = 0xff }RadioChannel_e; - Rate enumerator typedef enum { RATE_1M = 1, RATE_2M = 2, RATE_5_5M = 3, RATE_11M = 4, RATE_6M = 6, RATE_9M = 7, RATE_12M = 8, RATE_18M = 9, RATE_24M = 10, RATE_36M = 11, RATE_48M = 12, RATE_54M = 13, RATE_MCS_0 = 14, RATE_MCS_1 = 15, RATE_MCS_2 = 16, RATE_MCS_3 = 17, RATE_MCS_4 = 18, RATE_MCS_5 = 19, RATE_MCS_6 = 20, RATE_MCS_7 = 21, MAX_NUM_RATES = 0xFF }RadioRateIndex_e; - Preamble enumerator typedef enum { LONG_PREAMBLE_MODE = 0, SHORT_PREAMBLE_MODE = 1, OFDM_PREAMBLE_MODE = 2, N_MIXED_MODE_PREAMBLE_MODE = 3, GREENFIELD_PREAMBLE_MODE = 4, MAX_NUM_PREAMBLE = 0xff }RadioPreamble_e; - Data Pattern enumerator typedef enum { ALL_0_PATTERN = 0, ALL_1_PATTERN = 1, INCREMENTAL_PATTERN = 2, DECREMENTAL_PATTERN = 3, PN9_PATTERN = 4, PN15_PATTERN = 5, PN23_PATTERN = 6, MAX_NUM_PATTERN = 0xff }RadioDataPattern_e; - Device version structure typedef struct { UNIT16 DeviceRadioToolAppVersion[2] UINT32 ChipId; UINT32 FwVersion[4]; UINT8 PhyVersion[4]; UINT32 NwpVersion[4]; UINT16 RomVersion; UINT16 Padding; }DeviceVersion_s; Note: Chip IDs and corresponding names: Chip ID; Name. 0x00; CC3100R. 0x01; CC3100F. 0x03; CC3100Z. 0x10; CC3200R. 0x11; CC3200F. 0x13; CC3200Z. 0x18; CC3200RS. 0x1B; CC3200ZS.