KeyStone 2

EMIF16 Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial release |

Contents

1	Overview	3

2	Test Algorithms	3

2.1	Data filling Test	3

2.2	Addressing test	3

2.3	Bit walking test	4

3	Test code/project	5

3.1	CCS project	5

3.2	To run the test program on KeyStone 2 EVM	7

3.3	Test configuration	7

3.4	Migrate the test to custom board	8

References	9

Appendix: Typical test output	11

Figures

Figure 1.	Directory structure of test project	6

Figure 2.	define symbols in CCS compiler options to select device and board	8

Tables

Table 1.	Source files of the test codes	6

- Overview

The (EMIF16) is an External Memory InterFace utilizing a 16-bit data bus. The EMIF16 STK can test NAND FLASH and NOR FLASH over EMIF, which is the most common usage of EMIF.

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test Algorithms

Three memory test algorithms are introduced in this section, and the purpose of these test algorithms is discussed.

  - Data filling Test

The pseudocode for data pattern filling test is:

for(memory range under test)

fill the memory with a value;

for(memory range under test)

read back the memory and compare the readback value to the written value

Normally, this test is done several times with different test data value. The common values used for test include 0, 0xFFFFFFFF, 0x55555555, 0xAAAAAAAA.

This test can detect stuck data bit, for example, if

written value = 0, readback value = 0x8,

It indicates bit 3 sticks to 1. If

written value = 0xFFFFFFFF, readback value = 0xFFFFFFFE,

It indicates bit 0 sticks to 0.

  - Addressing test

The pseudocode for addressing test is:

for(memory range under test)

fill each memory unit with its address value;

for(memory range under test)

read back the memory and compare the readback value to the written value

This test can detect stuck bit on address bus, for example, if

written value = 0 at address 0

written value = 1 at address 1

written value = 2 at address 2

written value = 3 at address 3

……

readback value = 2 at address 0

readback value = 3 at address 1

readback value = 2 at address 2

readback value = 3 at address 3

……

For this case, it indicates bit 1 of address bus sticks. So, the contents in address 0 and 2 are same, and the contents in address 1 and 3 are same.

For external memory bus, once we identify the stuck bit. Normally, there are three possible causes:

- PCB issue. Normally, it is soldering issue, for example, one bus bit is connected to power or ground. We can use multimeter to measure the resistance between the bus signal and the power or ground. If the resistance to power or ground closes to 0, it normally indicates soldering issue.

- external memory failure. We can use oscilloscope to monitor the bus, if the bit only sticks when read back, it normally indicates external memory failure.

- memory controller failure. If we exclude above possibilities, then we can swap the devices on a good board and the problematic board, if the problem goes with the K2 device, then it normally indicates it is memory controller failure.

  - Bit walking test

The pseudocode for bit walking test is:

test_value = 1;

for(0 to 15) //for 16-bit bus

{

write test_value to memory;

readback the data and compared to written value;

inversed_test_value = bit inverse of test_value;

write inversed_test_value to memory;

readback the data and compared to written value;

test_value = test_value<<1; 	//bit walking

}

This test is used to identify cross-talk issue. The test intentionally make one bit on the bus or memory cells different from all other bits, thus the negative cross-talk from all other bits is strongest. If there is cross-talk issue, it should be exposed by this test.

For example, if

written value = 0x00000010, readback value = 0

written value = 0xFFFFFFEF, readback value = 0xFFFFFFFF

It indicates the bit 4 is affected by cross-talk issue.

To indentify cross-talk issue on a data bus, this test only need be executed at one address; to identify cross-talk of memory bit cells, this test need be executed for whole memory range under test.

For external memory interface, once we identify this, we should use oscilloscope to check the signal integrity of all related signals to find the source of cross-talk, and then PCB re-layout may be necessary to solve it.

- Test code/project

This section introduces the implementation of these memory tests on KeyStone 2 devices.

  - CCS project

Following figure shows the directory structure of the test project.

- Directory structure of test project

The project files are in “EMIF” folder. There is some commonly used initialization and driver code for EMIF and PLL… in “common” folder. The main source code files are in the “EMIF\src” subfolder. The NAND and NOR FLASH driver is in “EMIF\src\FLASH” subfolder.

The main code are described as below:

- Source files of the test codes

| Function | Source file | Description |
| --- | --- | --- |
| KeyStone_EMIF16_init | \common\ KeyStone_EMIF16_Init.c | EMIF initialization function |
| NOR_FLASH_test | \EMIF\src\ EMIF_NOR_FLASH_test.c | Main control function for NOR FLASH test |
| NAND_FLASH_test | \EMIF\src\ EMIF_NAND_FLASH_test.c | Main control function for NAND FLASH test |
| FLASH_MEM_Test | \EMIF\src\EMIF_FLASH_mem_test.c | Fill FLASH with fixed pattern or address, and then verify them. |
| NOR_erase | \EMIF\src\FLASH\flash_nor.c | Implementation of low level operations on NOR FLASH. |
| NOR_writeBytes |  |  |
| NAND_eraseBlocks | \EMIF\src\FLASH\flash_nand.c | Implementation of low level operations on NAND FLASH. |
| NAND_writePage |  |  |
| NAND_readPage |  |  |

  - To run the test program on KeyStone 2 EVM

The test program can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- Load the program to ARM core 0 of a device.

- run the core, then, you should see output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

Following macro is used to select between NAND FLASH and NOR FLASH test, set “1” for NAND FLASH test; “0” for NOR FLASH test.

/*select between NAND FLASH or NOR FLASH test*/

#define NAND_FLASH_TEST 	1

  - Migrate the test to custom board

These tests are implemented based on TI’s EVM boards. In your real system, the core speed may be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

For FLASH different from the one on TI EVM, you may need change following code in “EMIF_NAND_FLASH_test.c”:

NAND_InfoObj gNandInfo =

{/*configuration for NAND512R3A2DZA6E*/

EMIF_NAND_FLASH_CS,	//CSOffset;

FLASH_BUS_WIDTH_1_BYTE,	//busWidth;

4096,	//numBlocks;

32,		//pagesPerBlock;

512,	//dataBytesPerPage;

16,		//spareBytesPerPage;

1, 		//numOpsPerPage;

512,	//dataBytesPerOp;

16, 	//spareBytesPerOp;

NAND_NO_ECC 	//ECC_mode;

};

For NOR FLASH test, in “EMIF_NOR_FLASH_test.c”:

NOR_InfoObj gNorInfo =

{

CSL_AEMIF_SLV_AS1_EMIF16_CS_DATA+ DEVICE_EMIF_INTER_CE_REGION_SIZE*EMIF_NOR_FLASH_CS,/*flashBase */

FLASH_BUS_WIDTH_2_BYTES,/*busWidth;                   */

FLASH_BUS_WIDTH_2_BYTES,/*chipOperatingWidth;         */

FLASH_BUS_WIDTH_2_BYTES/*maxTotalWidth;              */

};

For different memory devices, you may need change many of the EMIF configurations in “EMIF_main.c” as below:

gNandCeCfg.busWidth= EMIF_BUS_8BIT,

gNandCeCfg.opMode  = NAND_MODE,

gNandCeCfg.strobeMode = SS_STROBE,

gNandCeCfg.waitMode = EMIF_WAIT_NONE,

/*timing configuration for NAND512R3A2DZA6E*/

gNandCeCfg.wrSetup = 1;//CSL_EMIF16_A0CR_WSETUP_RESETVAL,

gNandCeCfg.wrStrobe = 4;//CSL_EMIF16_A0CR_WSTROBE_RESETVAL,

gNandCeCfg.wrHold= 5;//CSL_EMIF16_A0CR_WHOLD_RESETVAL,

gNandCeCfg.rdSetup= 1,//CSL_EMIF16_A0CR_RSETUP_RESETVAL

gNandCeCfg.rdStrobe= 7,//CSL_EMIF16_A0CR_RSTROBE_RESETVAL

gNandCeCfg.rdHold= 2,//CSL_EMIF16_A0CR_RHOLD_RESETVAL

gNandCeCfg.turnAroundCycles= CSL_EMIF16_A0CR_TA_RESETVAL,

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone Architecture External Memory Interface (EMIF16) User Guide (SPRUGZ3A)

- KeyStone II Architecture ARM CorePac User's Guide (SPRUHJ4)

Appendix: Typical test output

Below is the NAND FLASH test output on K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

DDR3A initialization

Initialize DDR data rate = 100.000/1*20/6*4= 1333.3 MTS, bus width = 64 bits.

Initializing NAND flash...

Manufacturer ID = 2ch, Device ID = ach

NAND FLASH size = 2048 x 64 x 4096 = 536870912 Bytes

Save data from 0x1fe00000 to 0x20000000.

Erasing blocks 4080 through 4095

Passed Memory Fill Test from 0x1fe00000 to 0x20000000 with pattern 0x       0

Erasing blocks 4080 through 4095

Passed Memory Fill Test from 0x1fe00000 to 0x20000000 with pattern 0xaaaaaaaa

Erasing blocks 4080 through 4095

Passed Memory Fill Test from 0x1fe00000 to 0x20000000 with pattern 0x55555555

Erasing blocks 4080 through 4095

Passed Memory Address Test from 0x1fe00000 to 0x20000000

Erasing blocks 4080 through 4095

Restore data from 0x1fe00000 to 0x20000000.

FLASH test complete.