KeyStone 2

GPIO Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial Release |

Contents

1	Introduction	3

2	Test code/project	3

2.1	CCS project	3

2.2	To run the test program on KeyStone 2 EVM	4

2.3	Test configuration	4

2.4	Migrate the test to custom board	5

References	6

Appendix: Typical test output	7

Figures

Figure 1.	Directory structure of test project	3

Figure 2.	define symbols in CCS compiler options to select device and board	5

- Introduction

There are 32 or 64 GPIO pins on KeyStone 2 devices. The GPIO STK (Self Test Kit) is designed to test all functions of GPIO pins.

To test the GPIO, the interrupt for all GPIO pins are enabled on both rising edge and falling edge, the ISR (Interrupt Service routine) is implemented to report the GPIO state changing that triggers the interrupt.

An internal loopback test is implemented to test the GPIO internal function without external trigger. The GPIO pin is configured as output in this test, software can toggle the GPIO output register to change the pin state and in turn trigger the interrupt event.

When internal loopback test is enabled, all GPIO pins are configured as output, the GPIO external signal can also be measured on board when software toggle the GPIO output register;

When internal loopback test is disabled, all GPIO pins are configured as input, user can toggle the GPIO pins on board, and check if the GPIO interrupt is triggered as expected.

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test code/project

This section introduces the implementation of these tests on KeyStone 2 devices.

  - CCS project

Following figure shows the directory structure of the test project

- Directory structure of test project

The project files are in “GPIO” folder. There is some commonly used initialization and driver code for interrupt, exception and PLL… in “common” folder. The main source code files are in the “GPIO\src” subfolder. “GPIO_main.c” includes GPIO and interrupts initialization, and test code.

  - To run the test program on KeyStone 2 EVM

The test program can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- load the program to ARM core 0 of a device.

- run the core, then, you should see output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

The GPIO test mode can be changed with following macro at the beginning of “GPIO_main.c”. Set the macro to 1 to enable the loopback test; clear it to 0 to disable the loopback test.

#define GPIO_LOOP_BACK_TEST 	1

  - Migrate the test to custom board

These tests are implemented based on TI’s EVM boards. In your real system the core speed may be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone Architecture General Purpose Input/Output (GPIO)User Guide (SPRUGV1)

- KeyStone II Architecture ARM CorePac User's Guide (SPRUHJ4)

Appendix: Typical test output

Below is the test output on core 0 of K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

GPIO 0 interrupt happens.

GPIO IN_DATA= 0x00000001

GPIO 0 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 1 interrupt happens.

GPIO IN_DATA= 0x00000002

GPIO 1 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 2 interrupt happens.

GPIO IN_DATA= 0x00000004

GPIO 2 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 3 interrupt happens.

GPIO IN_DATA= 0x00000008

GPIO 3 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 4 interrupt happens.

GPIO IN_DATA= 0x00000010

GPIO 4 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 5 interrupt happens.

GPIO IN_DATA= 0x00000020

GPIO 5 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 6 interrupt happens.

GPIO IN_DATA= 0x00000040

GPIO 6 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 7 interrupt happens.

GPIO IN_DATA= 0x00000080

GPIO 7 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 8 interrupt happens.

GPIO IN_DATA= 0x00000100

GPIO 8 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 9 interrupt happens.

GPIO IN_DATA= 0x00000200

GPIO 9 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 10 interrupt happens.

GPIO IN_DATA= 0x00000400

GPIO 10 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 11 interrupt happens.

GPIO IN_DATA= 0x00000800

GPIO 11 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 12 interrupt happens.

GPIO IN_DATA= 0x00001000

GPIO 12 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 13 interrupt happens.

GPIO IN_DATA= 0x00002000

GPIO 13 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 14 interrupt happens.

GPIO IN_DATA= 0x00004000

GPIO 14 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 15 interrupt happens.

GPIO IN_DATA= 0x00008000

GPIO 15 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 16 interrupt happens.

GPIO IN_DATA= 0x00010000

GPIO 16 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 17 interrupt happens.

GPIO IN_DATA= 0x00020000

GPIO 17 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 18 interrupt happens.

GPIO IN_DATA= 0x00040000

GPIO 18 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 19 interrupt happens.

GPIO IN_DATA= 0x00080000

GPIO 19 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 20 interrupt happens.

GPIO IN_DATA= 0x00100000

GPIO 20 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 21 interrupt happens.

GPIO IN_DATA= 0x00200000

GPIO 21 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 22 interrupt happens.

GPIO IN_DATA= 0x00400000

GPIO 22 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 23 interrupt happens.

GPIO IN_DATA= 0x00800000

GPIO 23 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 24 interrupt happens.

GPIO IN_DATA= 0x01000000

GPIO 24 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 25 interrupt happens.

GPIO IN_DATA= 0x02000000

GPIO 25 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 26 interrupt happens.

GPIO IN_DATA= 0x04000000

GPIO 26 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 27 interrupt happens.

GPIO IN_DATA= 0x08000000

GPIO 27 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 28 interrupt happens.

GPIO IN_DATA= 0x10000000

GPIO 28 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 29 interrupt happens.

GPIO IN_DATA= 0x20000000

GPIO 29 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 30 interrupt happens.

GPIO IN_DATA= 0x40000000

GPIO 30 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO 31 interrupt happens.

GPIO IN_DATA= 0x80000000

GPIO 31 interrupt happens.

GPIO IN_DATA= 0x00000000

GPIO Test complete.