KeyStone 2

Memory Performance Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial release |

Contents

1	Overview	3

2	Test methods	4

2.1	Throughput Test	4

2.2	ARM core read/write latency test	4

2.3	DMA overhead test	5

2.4	EDMA test with different ACNT or index	5

3	Test Coverage	6

3.1	Test by different masters	6

3.2	Test all memories in the system	6

3.3	Test through different paths	6

4	Test code/project	6

4.1	CCS project	6

4.2	To run the test program on KeyStone 2 EVM	7

4.3	Test configuration	8

4.4	Migrate the test to custom board	10

References	11

Appendix: Typical test output	12

Figures

Figure 1.	KeyStone 2 SOC memory architecture	3

Figure 2.	Directory structure of test project	7

Figure 3.	define symbols in CCS compiler options to select device and board	8

Tables

Table 1.	Source files of the test codes	7

- Overview

Figure 1 shows the common KeyStone 2 SOC memory architecture.

- KeyStone 2 SOC memory architecture

The memory performance STK tests the performance of all memories in the system, which includes:

- CPU memory copy throughput

- CPU read/write latency

- EDMA memory copy throughput

- EDMA transfer overhead

- EDMA test with different ACNT

- EDMA test with different Index

Different cache/prefetch buffer configuration cases for CPU are also covered.

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test methods

  - Throughput Test

The pseudocode for throughput test is:

flushCache()；

startTime= getCycleCount()；

memoryCopy(number of bytes);

totalTime = getCycleCount()-startTime；

Throughput= (number of bytes)/totalTime；

In these tests, the default memory block size is 128KB.

  - ARM core read/write latency test

The pseudocode for ARM core memory access latency test is:

flushCache()；

startCycle= getCycleCount()；

for(i=0；i< AccessTimes；i++)

{

Access memory at address；

address+= stride；

}

totalCycles = getCycleCount()-startCycle；

Cycles/Access= totalCycles /AccessTimes；

In the test, “Access memory” is implemented with LDRD or STRD instruction in assembly.

  - DMA overhead test

Initial latency is defined as the time between DMA event happen to real data transfer begin. Since initial latency is hard to measure. We measure the sum of the Latency, and the time to transfer smallest element.

The pseudocode for DMA overhead test is:

flushCache()；

startTime= getCycleCount()；

DMA transfer one word;

overhead = getCycleCount()-startTime；

  - EDMA test with different ACNT or index

The ACNT and index affect EDMA throughput dramatically.

The pseudocode for EDMA test with different ACNT is:

flushCache()；

for(different ACNT)

{

startTime= getCycleCount()；

EDMA copy with this ACNT (BCNT=1);

totalTime = getCycleCount()-startTime；

Throughput= ACNT/totalTime；

}

The pseudocode for EDMA index test is:

flushCache()；

for(different index)

{

startTime= getCycleCount()；

EDMA copy with this index;

totalTime = getCycleCount()-startTime；

Throughput= totalBytes/totalTime；

}

- Test Coverage

Memory performance tests should be implemented to cover:

- All memories in the system;

- All ARM cores and DMA transfer controllers which can access memory

- All paths for memory access

  - Test by different masters

As shown in figure 1, there are mainly two types of masters can be used to access memory based on ARM program:

- ARM core, can access all memories

- EDMA, can access all memories

Please note, there are DSP cores in most KeyStone 2 devices, but this document only discuss the tests run on the ARM core. Separate test program is needed to test memory through DSP core.

  - Test all memories in the system

As shown in figure 1, there are mainly two types of memories in the system can be accessed through ARM program, all of them should be covered by memory test:

- Internal MSRAM (Multicore Shared RAM) memory

- External DDR SDRAM

External DDR SDRAM should be configured properly before test.

  - Test through different paths

Since each master has its own bus, same memory may be accessed by different masters from different data paths. For example, ARM core and EDMA access MSRAM through different path.

Same memory may be accessed thought different data path by same master. For example, DDR SDRAM can be accessed by ARM core through cache or directly without cache.

All these different paths should be covered by memory test.

- Test code/project

This section introduces the implementation of these memory tests on KeyStone 2 SOC.

  - CCS project

Following figure shows the directory structure of the test project.

- Directory structure of test project

The project files are in “Memory_Performance” folder. There is some commonly used initialization and driver code for DDR, EDMA and PLL… in “common” folder. The main source code files are in the “Memory_Performance\src” subfolder. Below table describes these source files.

- Source files of the test codes

| Source files | Descriptions |
| --- | --- |
| Mem_Access_CPU_performance.c | Implementation of ARM core memory copy test and ARM core read/write latency test for different memories. |
| CPU_LD_ST_Test.s | Low level implementation of ARM core read/write latency test in assembly. The key API is: Asm_LDRD_Test(srcBuff, uiIndex, uiCount); Asm_STRD_Test(srcBuff, uiIndex, uiCount); |
| CPU_Access_test.c | Optimized implementation of ARM core copy multiple of 8 bytes to maximize the throughput. |
| Mem_Access_Edma_Performance.c | Implementation of EDMA performance tests including EDMA throughput, overhead, ACNT and index test. |
| Mem_Access_performance.c | main function for initialization and call all test cases. |

  - To run the test program on KeyStone 2 EVM

The test program can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- Load the program to ARM core 0 of a device.

- run the core, then, you should see output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

There are multiple macros defined in the source code to configure the test.

Following configuration structures in “Mem_Access_Performance.c” can be modified to set different memories block/type been tested.

/*list of all memory blocks for test.

load store latency test will be done on each blocks.

memory copy test will be done inside each block and between all blocks.*/

Memory_Test_Block mem_test_blocks[]=

{

/*MSMC RAM blocks for test*/

{MSMC_RAM_TEST_BASE_ADDR+0*MSMC_RAM_TEST_BLOCK_SIZE, MSMC_RAM_TEST_BLOCK_SIZE, "MSRAM(strongly ordered)"}

//,{MSMC_RAM_TEST_BASE_ADDR+0*MSMC_RAM_TEST_BLOCK_SIZE, MSMC_RAM_TEST_BLOCK_SIZE, "MSRAM(device type)     "}

,{MSMC_RAM_TEST_BASE_ADDR+1*MSMC_RAM_TEST_BLOCK_SIZE, MSMC_RAM_TEST_BLOCK_SIZE, "MSRAM(normal non-cache)"}

,{MSMC_RAM_TEST_BASE_ADDR+2*MSMC_RAM_TEST_BLOCK_SIZE, MSMC_RAM_TEST_BLOCK_SIZE, "MSRAM(RA_WB_WA,  in sh)"}

//,{MSMC_RAM_TEST_BASE_ADDR+3*MSMC_RAM_TEST_BLOCK_SIZE, MSMC_RAM_TEST_BLOCK_SIZE, "MSRAM(RA_WB_WA, out sh)"}

/*DDR3A blocks for test*/

,{DDR3A_TEST_BASE_ADDR+0*DDR3A_TEST_BLOCK_SIZE, DDR3A_TEST_BLOCK_SIZE, "DDR3A(strongly ordered)"}

//,{DDR3A_TEST_BASE_ADDR+0*DDR3A_TEST_BLOCK_SIZE, DDR3A_TEST_BLOCK_SIZE, "DDR3A(device type)     "}

,{DDR3A_TEST_BASE_ADDR+1*DDR3A_TEST_BLOCK_SIZE, DDR3A_TEST_BLOCK_SIZE, "DDR3A(normal non-cache)"}

,{DDR3A_TEST_BASE_ADDR+2*DDR3A_TEST_BLOCK_SIZE, DDR3A_TEST_BLOCK_SIZE, "DDR3A(RA_WB_WA,  in sh)"}

//,{DDR3A_TEST_BASE_ADDR+3*DDR3A_TEST_BLOCK_SIZE, DDR3A_TEST_BLOCK_SIZE, "DDR3A(RA_WB_WA, out sh)"}

#if (2==CSL_DDR3_PER_CNT)

/*DDR3B blocks for test*/

,{DDR3B_TEST_BASE_ADDR+0*DDR3B_TEST_BLOCK_SIZE, DDR3B_TEST_BLOCK_SIZE, "DDR3B(strongly ordered)"}

//,{DDR3B_TEST_BASE_ADDR+0*DDR3B_TEST_BLOCK_SIZE, DDR3B_TEST_BLOCK_SIZE, "DDR3B(device type)     "}

,{DDR3B_TEST_BASE_ADDR+1*DDR3B_TEST_BLOCK_SIZE, DDR3B_TEST_BLOCK_SIZE, "DDR3B(normal non-cache)"}

,{DDR3B_TEST_BASE_ADDR+2*DDR3B_TEST_BLOCK_SIZE, DDR3B_TEST_BLOCK_SIZE, "DDR3B(RA_WB_WA,  in sh)"}

//,{DDR3B_TEST_BASE_ADDR+3*DDR3B_TEST_BLOCK_SIZE, DDR3B_TEST_BLOCK_SIZE, "DDR3B(RA_WB_WA, out sh)"}

#endif

#ifdef CSL_OSR_DATA 	/*for K2L only*/

/*OSR blocks for test*/

,{OSR_TEST_BASE_ADDR+0*OSR_TEST_BLOCK_SIZE, OSR_TEST_BLOCK_SIZE, "OSR (strongly ordered) "}

//,{OSR_TEST_BASE_ADDR+0*OSR_TEST_BLOCK_SIZE, OSR_TEST_BLOCK_SIZE, "OSR (device type)      "}

,{OSR_TEST_BASE_ADDR+1*OSR_TEST_BLOCK_SIZE, OSR_TEST_BLOCK_SIZE, "OSR (normal non-cache) "}

,{OSR_TEST_BASE_ADDR+2*OSR_TEST_BLOCK_SIZE, OSR_TEST_BLOCK_SIZE, "OSR (RA_WB_WA,  in sh) "}

//,{OSR_TEST_BASE_ADDR+3*OSR_TEST_BLOCK_SIZE, OSR_TEST_BLOCK_SIZE, "OSR (RA_WB_WA, out sh) "}

#endif

};

Following macros at the beginning of “Mem_Access_CPU_performance.c” can be modified to change the test block size.

#define MAX_COPY_SIZE 	(128*1024)

Following macros at the beginning of “Mem_Access_Edma_Performance.c” can be modified to change the EDMA test block size and ACNT size.

#define MAX_COPY_SIZE 	(128*1024)

#define 	A_COUNT 	(16*1024)

  - Migrate the test to custom board

In your real system, the DDR configuration may be changed according to your hardware design in the function K2_DDR3A_config() and K2_DDR3B_config() in “K2_Board_Init.c”.

The size of DDR3 may be changed through following macros in “K2_Board_Init.h”.

#define DDR3A_SIZE_BYTES    (1*1024*1024*1024UL)

#define DDR3B_SIZE_BYTES    (1*1024*1024*1024)

The core speed may be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone II Architecture ARM CorePac User's Guide (SPRUHJ4)

- KeyStone Multicore Shared Memory Controller (MSMC) User's Guide (SRPUGW7)

- KeyStone DDR3 Memory Controller User's Guide (SPRUGV8)

- KeyStone Architecture Enhanced Direct Memory Access (EDMA3) Controller User Guide (SPRUGS5)

Appendix: Typical test output

Below is the test output on K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

DDR3A initialization

Initialize DDR data rate = 100.000/1*16/4*4= 1600.0 MTS, bus width = 64 bits.

DDR3B initialization

Initialize DDR data rate = 100.000/1*16/4*4= 1600.0 MTS, bus width = 64 bits.

83 MB/s, copy 131072 bytes from 0x0c080000(MSRAM(strongly ordered)) to 0x0c0a0000(MSRAM(strongly ordered)), consumes 1874603 cycles

126 MB/s, copy 131072 bytes from 0x0c080000(MSRAM(strongly ordered)) to 0x0c0c0000(MSRAM(normal non-cache)), consumes 1241655 cycles

128 MB/s, copy 131072 bytes from 0x0c080000(MSRAM(strongly ordered)) to 0x0c100000(MSRAM(RA_WB_WA,  in sh)), consumes 1222808 cycles

65 MB/s, copy 131072 bytes from 0x0c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes 2387995 cycles

129 MB/s, copy 131072 bytes from 0x0c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes 1219192 cycles

128 MB/s, copy 131072 bytes from 0x0c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes 1220714 cycles

47 MB/s, copy 131072 bytes from 0x0c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes 3333305 cycles

129 MB/s, copy 131072 bytes from 0x0c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes 1217563 cycles

128 MB/s, copy 131072 bytes from 0x0c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes 1220209 cycles

127 MB/s, copy 131072 bytes from 0x0c0c0000(MSRAM(normal non-cache)) to 0x0c080000(MSRAM(strongly ordered)), consumes 1234093 cycles

147 MB/s, copy 131072 bytes from 0x0c0c0000(MSRAM(normal non-cache)) to 0x0c0e0000(MSRAM(normal non-cache)), consumes 1066230 cycles

147 MB/s, copy 131072 bytes from 0x0c0c0000(MSRAM(normal non-cache)) to 0x0c100000(MSRAM(RA_WB_WA,  in sh)), consumes 1065912 cycles

130 MB/s, copy 131072 bytes from 0x0c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes 1207707 cycles

147 MB/s, copy 131072 bytes from 0x0c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes 1064603 cycles

147 MB/s, copy 131072 bytes from 0x0c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes 1067097 cycles

130 MB/s, copy 131072 bytes from 0x0c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes 1207999 cycles

147 MB/s, copy 131072 bytes from 0x0c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes 1065478 cycles

147 MB/s, copy 131072 bytes from 0x0c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes 1067278 cycles

683 MB/s, copy 131072 bytes from 0x0c100000(MSRAM(RA_WB_WA,  in sh)) to 0x0c080000(MSRAM(strongly ordered)), consumes 230264 cycles

3228 MB/s, copy 131072 bytes from 0x0c100000(MSRAM(RA_WB_WA,  in sh)) to 0x0c0c0000(MSRAM(normal non-cache)), consumes 48723 cycles

4234 MB/s, copy 131072 bytes from 0x0c100000(MSRAM(RA_WB_WA,  in sh)) to 0x0c120000(MSRAM(RA_WB_WA,  in sh)), consumes 37147 cycles

662 MB/s, copy 131072 bytes from 0x0c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes 237477 cycles

1994 MB/s, copy 131072 bytes from 0x0c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes 78870 cycles

4150 MB/s, copy 131072 bytes from 0x0c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes 37894 cycles

654 MB/s, copy 131072 bytes from 0x0c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes 240499 cycles

1230 MB/s, copy 131072 bytes from 0x0c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes 127850 cycles

4024 MB/s, copy 131072 bytes from 0x0c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes 39085 cycles

49 MB/s, copy 131072 bytes from 0x80000000(DDR3A(strongly ordered)) to 0x0c080000(MSRAM(strongly ordered)), consumes 3177755 cycles

62 MB/s, copy 131072 bytes from 0x80000000(DDR3A(strongly ordered)) to 0x0c0c0000(MSRAM(normal non-cache)), consumes 2499236 cycles

61 MB/s, copy 131072 bytes from 0x80000000(DDR3A(strongly ordered)) to 0x0c100000(MSRAM(RA_WB_WA,  in sh)), consumes 2537828 cycles

32 MB/s, copy 131072 bytes from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes 4837829 cycles

43 MB/s, copy 131072 bytes from 0x80000000(DDR3A(strongly ordered)) to 0x80022000(DDR3A(strongly ordered)), consumes 3605072 cycles

44 MB/s, copy 131072 bytes from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes 3497198 cycles

60 MB/s, copy 131072 bytes from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes 2608728 cycles

33 MB/s, copy 131072 bytes from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes 4687484 cycles

62 MB/s, copy 131072 bytes from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes 2500272 cycles

61 MB/s, copy 131072 bytes from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes 2549932 cycles

61 MB/s, copy 131072 bytes from 0x88000000(DDR3A(normal non-cache)) to 0x0c080000(MSRAM(strongly ordered)), consumes 2553104 cycles

65 MB/s, copy 131072 bytes from 0x88000000(DDR3A(normal non-cache)) to 0x0c0c0000(MSRAM(normal non-cache)), consumes 2413996 cycles

64 MB/s, copy 131072 bytes from 0x88000000(DDR3A(normal non-cache)) to 0x0c100000(MSRAM(RA_WB_WA,  in sh)), consumes 2425973 cycles

35 MB/s, copy 131072 bytes from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes 4445066 cycles

52 MB/s, copy 131072 bytes from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes 3022116 cycles

64 MB/s, copy 131072 bytes from 0x88000000(DDR3A(normal non-cache)) to 0x88022000(DDR3A(normal non-cache)), consumes 2424279 cycles

63 MB/s, copy 131072 bytes from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes 2483177 cycles

61 MB/s, copy 131072 bytes from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes 2554537 cycles

65 MB/s, copy 131072 bytes from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes 2416774 cycles

65 MB/s, copy 131072 bytes from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes 2416503 cycles

683 MB/s, copy 131072 bytes from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x0c080000(MSRAM(strongly ordered)), consumes 230208 cycles

3040 MB/s, copy 131072 bytes from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x0c0c0000(MSRAM(normal non-cache)), consumes 51724 cycles

3565 MB/s, copy 131072 bytes from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x0c100000(MSRAM(RA_WB_WA,  in sh)), consumes 44117 cycles

546 MB/s, copy 131072 bytes from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes 288032 cycles

1346 MB/s, copy 131072 bytes from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes 116801 cycles

2328 MB/s, copy 131072 bytes from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes 67554 cycles

2744 MB/s, copy 131072 bytes from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90022000(DDR3A(RA_WB_WA,  in sh)), consumes 57308 cycles

651 MB/s, copy 131072 bytes from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes 241425 cycles

1216 MB/s, copy 131072 bytes from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes 129269 cycles

3338 MB/s, copy 131072 bytes from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes 47108 cycles

37 MB/s, copy 131072 bytes from 0x60000000(DDR3B(strongly ordered)) to 0x0c080000(MSRAM(strongly ordered)), consumes 4148233 cycles

46 MB/s, copy 131072 bytes from 0x60000000(DDR3B(strongly ordered)) to 0x0c0c0000(MSRAM(normal non-cache)), consumes 3412920 cycles

45 MB/s, copy 131072 bytes from 0x60000000(DDR3B(strongly ordered)) to 0x0c100000(MSRAM(RA_WB_WA,  in sh)), consumes 3439386 cycles

33 MB/s, copy 131072 bytes from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes 4690170 cycles

46 MB/s, copy 131072 bytes from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes 3412074 cycles

45 MB/s, copy 131072 bytes from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes 3437413 cycles

23 MB/s, copy 131072 bytes from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes 6674183 cycles

28 MB/s, copy 131072 bytes from 0x60000000(DDR3B(strongly ordered)) to 0x60022000(DDR3B(strongly ordered)), consumes 5481742 cycles

37 MB/s, copy 131072 bytes from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes 4181869 cycles

44 MB/s, copy 131072 bytes from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes 3500741 cycles

45 MB/s, copy 131072 bytes from 0x68000000(DDR3B(normal non-cache)) to 0x0c080000(MSRAM(strongly ordered)), consumes 3488996 cycles

47 MB/s, copy 131072 bytes from 0x68000000(DDR3B(normal non-cache)) to 0x0c0c0000(MSRAM(normal non-cache)), consumes 3336510 cycles

47 MB/s, copy 131072 bytes from 0x68000000(DDR3B(normal non-cache)) to 0x0c100000(MSRAM(RA_WB_WA,  in sh)), consumes 3336542 cycles

45 MB/s, copy 131072 bytes from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes 3487967 cycles

47 MB/s, copy 131072 bytes from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes 3336501 cycles

47 MB/s, copy 131072 bytes from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes 3338174 cycles

30 MB/s, copy 131072 bytes from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes 5141052 cycles

39 MB/s, copy 131072 bytes from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes 3946472 cycles

47 MB/s, copy 131072 bytes from 0x68000000(DDR3B(normal non-cache)) to 0x68022000(DDR3B(normal non-cache)), consumes 3336934 cycles

46 MB/s, copy 131072 bytes from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes 3389687 cycles

676 MB/s, copy 131072 bytes from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x0c080000(MSRAM(strongly ordered)), consumes 232598 cycles

2816 MB/s, copy 131072 bytes from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x0c0c0000(MSRAM(normal non-cache)), consumes 55850 cycles

2810 MB/s, copy 131072 bytes from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x0c100000(MSRAM(RA_WB_WA,  in sh)), consumes 55956 cycles

659 MB/s, copy 131072 bytes from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes 238496 cycles

1858 MB/s, copy 131072 bytes from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes 84649 cycles

2847 MB/s, copy 131072 bytes from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes 55246 cycles

522 MB/s, copy 131072 bytes from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes 301067 cycles

947 MB/s, copy 131072 bytes from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes 165989 cycles

1925 MB/s, copy 131072 bytes from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes 81703 cycles

2411 MB/s, copy 131072 bytes from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70022000(DDR3B(RA_WB_WA,  in sh)), consumes 65215 cycles

Memory load performance test at 0x0c080000(MSRAM(strongly ordered))

Index=   0  Cycles/LDRD= 15.16

Index=   1  Cycles/LDRD= 15.14

Index=   2  Cycles/LDRD= 15.30

Index=   3  Cycles/LDRD= 15.10

Index=   4  Cycles/LDRD= 12.71

Index=   5  Cycles/LDRD= 15.30

Index=   6  Cycles/LDRD= 15.21

Index=   7  Cycles/LDRD= 15.25

Index=   8  Cycles/LDRD= 12.69

Index=  10  Cycles/LDRD= 15.30

Index=  12  Cycles/LDRD= 15.28

Index=  14  Cycles/LDRD= 15.30

Index=  16  Cycles/LDRD= 12.66

Index=  18  Cycles/LDRD= 15.30

Index=  20  Cycles/LDRD= 15.28

Index=  22  Cycles/LDRD= 15.30

Index=  24  Cycles/LDRD= 12.70

Index=  26  Cycles/LDRD= 15.30

Index=  28  Cycles/LDRD= 15.28

Index=  30  Cycles/LDRD= 15.30

Index=  32  Cycles/LDRD= 12.70

Index=  40  Cycles/LDRD= 12.70

Index=  48  Cycles/LDRD= 12.67

Index=  56  Cycles/LDRD= 14.30

Memory store performance test at 0x0c080000(MSRAM(strongly ordered))

Index=   0  Cycles/STRD= 8.28

Index=   1  Cycles/STRD= 8.04

Index=   2  Cycles/STRD= 7.80

Index=   3  Cycles/STRD= 7.55

Index=   4  Cycles/STRD= 7.30

Index=   5  Cycles/STRD= 7.04

Index=   6  Cycles/STRD= 6.79

Index=   7  Cycles/STRD= 6.54

Index=   8  Cycles/STRD= 6.29

Index=  10  Cycles/STRD= 6.30

Index=  12  Cycles/STRD= 6.29

Index=  14  Cycles/STRD= 6.28

Index=  16  Cycles/STRD= 6.28

Index=  18  Cycles/STRD= 6.28

Index=  20  Cycles/STRD= 6.28

Index=  22  Cycles/STRD= 6.28

Index=  24  Cycles/STRD= 6.27

Index=  26  Cycles/STRD= 6.27

Index=  28  Cycles/STRD= 6.28

Index=  30  Cycles/STRD= 6.28

Index=  32  Cycles/STRD= 6.28

Index=  40  Cycles/STRD= 6.27

Index=  48  Cycles/STRD= 6.28

Index=  56  Cycles/STRD= 6.28

Memory load performance test at 0x0c0c0000(MSRAM(normal non-cache))

Index=   0  Cycles/LDRD= 64.64

Index=   1  Cycles/LDRD= 65.36

Index=   2  Cycles/LDRD= 65.23

Index=   3  Cycles/LDRD= 64.97

Index=   4  Cycles/LDRD= 65.05

Index=   5  Cycles/LDRD= 65.08

Index=   6  Cycles/LDRD= 65.25

Index=   7  Cycles/LDRD= 65.55

Index=   8  Cycles/LDRD= 64.64

Index=  10  Cycles/LDRD= 65.09

Index=  12  Cycles/LDRD= 65.14

Index=  14  Cycles/LDRD= 65.32

Index=  16  Cycles/LDRD= 64.64

Index=  18  Cycles/LDRD= 65.27

Index=  20  Cycles/LDRD= 65.25

Index=  22  Cycles/LDRD= 65.51

Index=  24  Cycles/LDRD= 64.48

Index=  26  Cycles/LDRD= 65.35

Index=  28  Cycles/LDRD= 65.17

Index=  30  Cycles/LDRD= 65.41

Index=  32  Cycles/LDRD= 63.95

Index=  40  Cycles/LDRD= 64.32

Index=  48  Cycles/LDRD= 64.85

Index=  56  Cycles/LDRD= 64.13

Memory store performance test at 0x0c0c0000(MSRAM(normal non-cache))

Index=   0  Cycles/STRD= 1.93

Index=   1  Cycles/STRD= 2.83

Index=   2  Cycles/STRD= 5.72

Index=   3  Cycles/STRD= 4.75

Index=   4  Cycles/STRD= 4.75

Index=   5  Cycles/STRD= 4.52

Index=   6  Cycles/STRD= 4.56

Index=   7  Cycles/STRD= 4.53

Index=   8  Cycles/STRD= 4.45

Index=  10  Cycles/STRD= 4.50

Index=  12  Cycles/STRD= 4.65

Index=  14  Cycles/STRD= 4.47

Index=  16  Cycles/STRD= 4.44

Index=  18  Cycles/STRD= 4.48

Index=  20  Cycles/STRD= 4.49

Index=  22  Cycles/STRD= 4.63

Index=  24  Cycles/STRD= 4.62

Index=  26  Cycles/STRD= 4.48

Index=  28  Cycles/STRD= 4.49

Index=  30  Cycles/STRD= 4.61

Index=  32  Cycles/STRD= 4.28

Index=  40  Cycles/STRD= 4.46

Index=  48  Cycles/STRD= 4.45

Index=  56  Cycles/STRD= 4.63

Memory load performance test at 0x0c100000(MSRAM(RA_WB_WA,  in sh))

Index=   0  Cycles/LDRD= 1.33

Index=   1  Cycles/LDRD= 2.53

Index=   2  Cycles/LDRD= 3.22

Index=   3  Cycles/LDRD= 4.61

Index=   4  Cycles/LDRD= 6.11

Index=   5  Cycles/LDRD= 7.98

Index=   6  Cycles/LDRD= 9.10

Index=   7  Cycles/LDRD= 10.39

Index=   8  Cycles/LDRD= 11.30

Index=  10  Cycles/LDRD= 11.76

Index=  12  Cycles/LDRD= 11.76

Index=  14  Cycles/LDRD= 12.39

Index=  16  Cycles/LDRD= 16.48

Index=  18  Cycles/LDRD= 12.68

Index=  20  Cycles/LDRD= 12.72

Index=  22  Cycles/LDRD= 13.40

Index=  24  Cycles/LDRD= 13.09

Index=  26  Cycles/LDRD= 13.11

Index=  28  Cycles/LDRD= 13.03

Index=  30  Cycles/LDRD= 14.95

Index=  32  Cycles/LDRD= 27.00

Index=  40  Cycles/LDRD= 13.30

Index=  48  Cycles/LDRD= 13.64

Index=  56  Cycles/LDRD= 11.93

Memory store performance test at 0x0c100000(MSRAM(RA_WB_WA,  in sh))

Index=   0  Cycles/STRD= 1.38

Index=   1  Cycles/STRD= 1.80

Index=   2  Cycles/STRD= 3.32

Index=   3  Cycles/STRD= 4.32

Index=   4  Cycles/STRD= 6.01

Index=   5  Cycles/STRD= 7.76

Index=   6  Cycles/STRD= 8.93

Index=   7  Cycles/STRD= 10.17

Index=   8  Cycles/STRD= 11.64

Index=  10  Cycles/STRD= 11.83

Index=  12  Cycles/STRD= 11.80

Index=  14  Cycles/STRD= 11.69

Index=  16  Cycles/STRD= 16.80

Index=  18  Cycles/STRD= 11.79

Index=  20  Cycles/STRD= 12.30

Index=  22  Cycles/STRD= 12.51

Index=  24  Cycles/STRD= 13.06

Index=  26  Cycles/STRD= 12.27

Index=  28  Cycles/STRD= 12.63

Index=  30  Cycles/STRD= 12.14

Index=  32  Cycles/STRD= 25.69

Index=  40  Cycles/STRD= 12.71

Index=  48  Cycles/STRD= 14.13

Index=  56  Cycles/STRD= 12.28

Memory load performance test at 0x80000000(DDR3A(strongly ordered))

Index=   0  Cycles/LDRD= 26.71

Index=   1  Cycles/LDRD= 24.62

Index=   2  Cycles/LDRD= 24.60

Index=   3  Cycles/LDRD= 24.64

Index=   4  Cycles/LDRD= 24.68

Index=   5  Cycles/LDRD= 24.60

Index=   6  Cycles/LDRD= 24.68

Index=   7  Cycles/LDRD= 24.67

Index=   8  Cycles/LDRD= 24.61

Index=  10  Cycles/LDRD= 24.67

Index=  12  Cycles/LDRD= 24.70

Index=  14  Cycles/LDRD= 24.73

Index=  16  Cycles/LDRD= 24.68

Index=  18  Cycles/LDRD= 24.80

Index=  20  Cycles/LDRD= 24.78

Index=  22  Cycles/LDRD= 24.72

Index=  24  Cycles/LDRD= 25.01

Index=  26  Cycles/LDRD= 24.83

Index=  28  Cycles/LDRD= 25.10

Index=  30  Cycles/LDRD= 24.81

Index=  32  Cycles/LDRD= 24.99

Index=  40  Cycles/LDRD= 25.22

Index=  48  Cycles/LDRD= 27.25

Index=  56  Cycles/LDRD= 26.04

Index=  64  Cycles/LDRD= 25.29

Index=  96  Cycles/LDRD= 25.32

Index= 128  Cycles/LDRD= 26.23

Index= 192  Cycles/LDRD= 26.23

Index= 256  Cycles/LDRD= 27.50

Index= 384  Cycles/LDRD= 29.64

Index= 512  Cycles/LDRD= 30.53

Index= 768  Cycles/LDRD= 30.59

Index=1024  Cycles/LDRD= 30.60

Index=1536  Cycles/LDRD= 31.17

Index=2048  Cycles/LDRD= 30.62

Index=3072  Cycles/LDRD= 30.59

Index=4096  Cycles/LDRD= 31.95

Index=6144  Cycles/LDRD= 31.09

Index=8192  Cycles/LDRD= 63.43

Index=12288  Cycles/LDRD= 31.95

Index=16384  Cycles/LDRD= 63.43

Index=20480  Cycles/LDRD= 31.95

Index=24576  Cycles/LDRD= 63.44

Index=28672  Cycles/LDRD= 31.95

Memory store performance test at 0x80000000(DDR3A(strongly ordered))

Index=   0  Cycles/STRD= 12.35

Index=   1  Cycles/STRD= 14.42

Index=   2  Cycles/STRD= 14.43

Index=   3  Cycles/STRD= 12.36

Index=   4  Cycles/STRD= 12.36

Index=   5  Cycles/STRD= 12.35

Index=   6  Cycles/STRD= 14.43

Index=   7  Cycles/STRD= 12.35

Index=   8  Cycles/STRD= 12.36

Index=  10  Cycles/STRD= 12.35

Index=  12  Cycles/STRD= 12.36

Index=  14  Cycles/STRD= 12.34

Index=  16  Cycles/STRD= 12.34

Index=  18  Cycles/STRD= 12.34

Index=  20  Cycles/STRD= 14.23

Index=  22  Cycles/STRD= 12.33

Index=  24  Cycles/STRD= 14.52

Index=  26  Cycles/STRD= 12.34

Index=  28  Cycles/STRD= 12.34

Index=  30  Cycles/STRD= 12.34

Index=  32  Cycles/STRD= 12.33

Index=  40  Cycles/STRD= 14.41

Index=  48  Cycles/STRD= 12.34

Index=  56  Cycles/STRD= 12.34

Index=  64  Cycles/STRD= 12.34

Index=  96  Cycles/STRD= 14.23

Index= 128  Cycles/STRD= 12.34

Index= 192  Cycles/STRD= 12.35

Index= 256  Cycles/STRD= 12.33

Index= 384  Cycles/STRD= 12.34

Index= 512  Cycles/STRD= 12.34

Index= 768  Cycles/STRD= 12.33

Index=1024  Cycles/STRD= 12.34

Index=1536  Cycles/STRD= 15.33

Index=2048  Cycles/STRD= 19.78

Index=3072  Cycles/STRD= 14.24

Index=4096  Cycles/STRD= 39.24

Index=6144  Cycles/STRD= 21.85

Index=8192  Cycles/STRD= 79.87

Index=12288  Cycles/STRD= 39.24

Index=16384  Cycles/STRD= 79.73

Index=20480  Cycles/STRD= 39.24

Index=24576  Cycles/STRD= 79.74

Index=28672  Cycles/STRD= 39.36

Memory load performance test at 0x88000000(DDR3A(normal non-cache))

Index=   0  Cycles/LDRD= 143.37

Index=   1  Cycles/LDRD= 145.73

Index=   2  Cycles/LDRD= 145.28

Index=   3  Cycles/LDRD= 145.77

Index=   4  Cycles/LDRD= 145.77

Index=   5  Cycles/LDRD= 146.94

Index=   6  Cycles/LDRD= 145.75

Index=   7  Cycles/LDRD= 145.20

Index=   8  Cycles/LDRD= 144.89

Index=  10  Cycles/LDRD= 145.76

Index=  12  Cycles/LDRD= 145.30

Index=  14  Cycles/LDRD= 149.19

Index=  16  Cycles/LDRD= 147.42

Index=  18  Cycles/LDRD= 145.30

Index=  20  Cycles/LDRD= 145.84

Index=  22  Cycles/LDRD= 145.52

Index=  24  Cycles/LDRD= 147.88

Index=  26  Cycles/LDRD= 145.48

Index=  28  Cycles/LDRD= 145.93

Index=  30  Cycles/LDRD= 147.08

Index=  32  Cycles/LDRD= 143.91

Index=  40  Cycles/LDRD= 144.34

Index=  48  Cycles/LDRD= 144.50

Index=  56  Cycles/LDRD= 146.77

Index=  64  Cycles/LDRD= 148.83

Index=  96  Cycles/LDRD= 149.79

Index= 128  Cycles/LDRD= 145.85

Index= 192  Cycles/LDRD= 152.47

Index= 256  Cycles/LDRD= 153.83

Index= 384  Cycles/LDRD= 157.43

Index= 512  Cycles/LDRD= 159.81

Index= 768  Cycles/LDRD= 168.27

Index=1024  Cycles/LDRD= 181.69

Index=1536  Cycles/LDRD= 177.39

Index=2048  Cycles/LDRD= 183.11

Index=3072  Cycles/LDRD= 179.45

Index=4096  Cycles/LDRD= 181.54

Index=6144  Cycles/LDRD= 181.21

Index=8192  Cycles/LDRD= 187.64

Index=12288  Cycles/LDRD= 179.37

Index=16384  Cycles/LDRD= 182.58

Index=20480  Cycles/LDRD= 183.94

Index=24576  Cycles/LDRD= 179.71

Index=28672  Cycles/LDRD= 179.32

Memory store performance test at 0x88000000(DDR3A(normal non-cache))

Index=   0  Cycles/STRD= 2.21

Index=   1  Cycles/STRD= 4.99

Index=   2  Cycles/STRD= 10.33

Index=   3  Cycles/STRD= 8.63

Index=   4  Cycles/STRD= 7.57

Index=   5  Cycles/STRD= 8.06

Index=   6  Cycles/STRD= 7.41

Index=   7  Cycles/STRD= 7.25

Index=   8  Cycles/STRD= 7.01

Index=  10  Cycles/STRD= 7.00

Index=  12  Cycles/STRD= 7.01

Index=  14  Cycles/STRD= 7.05

Index=  16  Cycles/STRD= 7.04

Index=  18  Cycles/STRD= 7.12

Index=  20  Cycles/STRD= 7.16

Index=  22  Cycles/STRD= 7.21

Index=  24  Cycles/STRD= 7.26

Index=  26  Cycles/STRD= 7.36

Index=  28  Cycles/STRD= 7.38

Index=  30  Cycles/STRD= 7.41

Index=  32  Cycles/STRD= 7.46

Index=  40  Cycles/STRD= 7.78

Index=  48  Cycles/STRD= 7.90

Index=  56  Cycles/STRD= 8.18

Index=  64  Cycles/STRD= 8.68

Index=  96  Cycles/STRD= 8.61

Index= 128  Cycles/STRD= 8.91

Index= 192  Cycles/STRD= 9.34

Index= 256  Cycles/STRD= 9.51

Index= 384  Cycles/STRD= 9.81

Index= 512  Cycles/STRD= 9.93

Index= 768  Cycles/STRD= 10.68

Index=1024  Cycles/STRD= 12.34

Index=1536  Cycles/STRD= 16.71

Index=2048  Cycles/STRD= 20.84

Index=3072  Cycles/STRD= 12.24

Index=4096  Cycles/STRD= 39.37

Index=6144  Cycles/STRD= 20.73

Index=8192  Cycles/STRD= 73.75

Index=12288  Cycles/STRD= 37.30

Index=16384  Cycles/STRD= 75.76

Index=20480  Cycles/STRD= 37.29

Index=24576  Cycles/STRD= 73.87

Index=28672  Cycles/STRD= 37.30

Memory load performance test at 0x90000000(DDR3A(RA_WB_WA,  in sh))

Index=   0  Cycles/LDRD= 1.50

Index=   1  Cycles/LDRD= 3.25

Index=   2  Cycles/LDRD= 5.02

Index=   3  Cycles/LDRD= 6.81

Index=   4  Cycles/LDRD= 8.87

Index=   5  Cycles/LDRD= 12.21

Index=   6  Cycles/LDRD= 12.77

Index=   7  Cycles/LDRD= 15.36

Index=   8  Cycles/LDRD= 16.57

Index=  10  Cycles/LDRD= 17.62

Index=  12  Cycles/LDRD= 17.60

Index=  14  Cycles/LDRD= 19.76

Index=  16  Cycles/LDRD= 20.99

Index=  18  Cycles/LDRD= 19.93

Index=  20  Cycles/LDRD= 18.05

Index=  22  Cycles/LDRD= 19.11

Index=  24  Cycles/LDRD= 19.37

Index=  26  Cycles/LDRD= 20.12

Index=  28  Cycles/LDRD= 20.61

Index=  30  Cycles/LDRD= 21.59

Index=  32  Cycles/LDRD= 29.84

Index=  40  Cycles/LDRD= 20.82

Index=  48  Cycles/LDRD= 24.28

Index=  56  Cycles/LDRD= 25.65

Index=  64  Cycles/LDRD= 26.79

Index=  96  Cycles/LDRD= 26.72

Index= 128  Cycles/LDRD= 27.98

Index= 192  Cycles/LDRD= 27.78

Index= 256  Cycles/LDRD= 29.03

Index= 384  Cycles/LDRD= 29.06

Index= 512  Cycles/LDRD= 40.76

Index= 768  Cycles/LDRD= 32.01

Index=1024  Cycles/LDRD= 69.26

Index=1536  Cycles/LDRD= 40.15

Index=2048  Cycles/LDRD= 100.73

Index=3072  Cycles/LDRD= 60.31

Index=4096  Cycles/LDRD= 100.45

Index=6144  Cycles/LDRD= 100.28

Index=8192  Cycles/LDRD= 108.44

Index=12288  Cycles/LDRD= 104.65

Index=16384  Cycles/LDRD= 106.13

Index=20480  Cycles/LDRD= 100.46

Index=24576  Cycles/LDRD= 105.97

Index=28672  Cycles/LDRD= 101.08

Memory store performance test at 0x90000000(DDR3A(RA_WB_WA,  in sh))

Index=   0  Cycles/STRD= 1.39

Index=   1  Cycles/STRD= 1.98

Index=   2  Cycles/STRD= 4.46

Index=   3  Cycles/STRD= 6.71

Index=   4  Cycles/STRD= 8.95

Index=   5  Cycles/STRD= 11.67

Index=   6  Cycles/STRD= 13.26

Index=   7  Cycles/STRD= 19.96

Index=   8  Cycles/STRD= 17.43

Index=  10  Cycles/STRD= 17.15

Index=  12  Cycles/STRD= 17.90

Index=  14  Cycles/STRD= 17.83

Index=  16  Cycles/STRD= 21.36

Index=  18  Cycles/STRD= 18.83

Index=  20  Cycles/STRD= 19.36

Index=  22  Cycles/STRD= 21.62

Index=  24  Cycles/STRD= 19.94

Index=  26  Cycles/STRD= 21.03

Index=  28  Cycles/STRD= 20.36

Index=  30  Cycles/STRD= 19.89

Index=  32  Cycles/STRD= 29.45

Index=  40  Cycles/STRD= 21.56

Index=  48  Cycles/STRD= 26.99

Index=  56  Cycles/STRD= 26.78

Index=  64  Cycles/STRD= 27.08

Index=  96  Cycles/STRD= 27.83

Index= 128  Cycles/STRD= 28.33

Index= 192  Cycles/STRD= 28.02

Index= 256  Cycles/STRD= 34.56

Index= 384  Cycles/STRD= 30.85

Index= 512  Cycles/STRD= 47.31

Index= 768  Cycles/STRD= 36.46

Index=1024  Cycles/STRD= 59.71

Index=1536  Cycles/STRD= 56.34

Index=2048  Cycles/STRD= 116.96

Index=3072  Cycles/STRD= 71.38

Index=4096  Cycles/STRD= 128.61

Index=6144  Cycles/STRD= 113.45

Index=8192  Cycles/STRD= 153.88

Index=12288  Cycles/STRD= 127.11

Index=16384  Cycles/STRD= 158.20

Index=20480  Cycles/STRD= 126.72

Index=24576  Cycles/STRD= 152.42

Index=28672  Cycles/STRD= 128.38

Memory load performance test at 0x60000000(DDR3B(strongly ordered))

Index=   0  Cycles/LDRD= 33.20

Index=   1  Cycles/LDRD= 33.18

Index=   2  Cycles/LDRD= 33.09

Index=   3  Cycles/LDRD= 33.11

Index=   4  Cycles/LDRD= 33.10

Index=   5  Cycles/LDRD= 33.18

Index=   6  Cycles/LDRD= 33.15

Index=   7  Cycles/LDRD= 33.16

Index=   8  Cycles/LDRD= 33.18

Index=  10  Cycles/LDRD= 33.18

Index=  12  Cycles/LDRD= 33.14

Index=  14  Cycles/LDRD= 33.15

Index=  16  Cycles/LDRD= 33.18

Index=  18  Cycles/LDRD= 33.23

Index=  20  Cycles/LDRD= 33.21

Index=  22  Cycles/LDRD= 33.25

Index=  24  Cycles/LDRD= 33.69

Index=  26  Cycles/LDRD= 33.22

Index=  28  Cycles/LDRD= 33.50

Index=  30  Cycles/LDRD= 33.23

Index=  32  Cycles/LDRD= 33.38

Index=  40  Cycles/LDRD= 33.40

Index=  48  Cycles/LDRD= 33.51

Index=  56  Cycles/LDRD= 34.40

Index=  64  Cycles/LDRD= 33.76

Index=  96  Cycles/LDRD= 33.78

Index= 128  Cycles/LDRD= 34.78

Index= 192  Cycles/LDRD= 34.54

Index= 256  Cycles/LDRD= 36.05

Index= 384  Cycles/LDRD= 36.03

Index= 512  Cycles/LDRD= 39.57

Index= 768  Cycles/LDRD= 39.01

Index=1024  Cycles/LDRD= 39.12

Index=1536  Cycles/LDRD= 39.65

Index=2048  Cycles/LDRD= 39.11

Index=3072  Cycles/LDRD= 39.12

Index=4096  Cycles/LDRD= 39.76

Index=6144  Cycles/LDRD= 39.18

Index=8192  Cycles/LDRD= 60.54

Index=12288  Cycles/LDRD= 39.15

Index=16384  Cycles/LDRD= 60.54

Index=20480  Cycles/LDRD= 39.09

Index=24576  Cycles/LDRD= 60.54

Index=28672  Cycles/LDRD= 39.13

Memory store performance test at 0x60000000(DDR3B(strongly ordered))

Index=   0  Cycles/STRD= 12.88

Index=   1  Cycles/STRD= 12.89

Index=   2  Cycles/STRD= 12.90

Index=   3  Cycles/STRD= 12.91

Index=   4  Cycles/STRD= 12.92

Index=   5  Cycles/STRD= 12.95

Index=   6  Cycles/STRD= 12.97

Index=   7  Cycles/STRD= 13.00

Index=   8  Cycles/STRD= 12.96

Index=  10  Cycles/STRD= 13.02

Index=  12  Cycles/STRD= 13.07

Index=  14  Cycles/STRD= 13.10

Index=  16  Cycles/STRD= 13.09

Index=  18  Cycles/STRD= 13.18

Index=  20  Cycles/STRD= 13.24

Index=  22  Cycles/STRD= 13.26

Index=  24  Cycles/STRD= 13.74

Index=  26  Cycles/STRD= 13.40

Index=  28  Cycles/STRD= 13.50

Index=  30  Cycles/STRD= 13.61

Index=  32  Cycles/STRD= 13.59

Index=  40  Cycles/STRD= 13.92

Index=  48  Cycles/STRD= 14.15

Index=  56  Cycles/STRD= 14.46

Index=  64  Cycles/STRD= 14.62

Index=  96  Cycles/STRD= 15.83

Index= 128  Cycles/STRD= 16.34

Index= 192  Cycles/STRD= 16.17

Index= 256  Cycles/STRD= 16.77

Index= 384  Cycles/STRD= 16.65

Index= 512  Cycles/STRD= 16.87

Index= 768  Cycles/STRD= 17.08

Index=1024  Cycles/STRD= 17.36

Index=1536  Cycles/STRD= 17.23

Index=2048  Cycles/STRD= 20.17

Index=3072  Cycles/STRD= 17.27

Index=4096  Cycles/STRD= 39.33

Index=6144  Cycles/STRD= 19.88

Index=8192  Cycles/STRD= 80.78

Index=12288  Cycles/STRD= 39.33

Index=16384  Cycles/STRD= 80.78

Index=20480  Cycles/STRD= 39.33

Index=24576  Cycles/STRD= 80.77

Index=28672  Cycles/STRD= 39.33

Memory load performance test at 0x68000000(DDR3B(normal non-cache))

Index=   0  Cycles/LDRD= 200.87

Index=   1  Cycles/LDRD= 201.59

Index=   2  Cycles/LDRD= 201.49

Index=   3  Cycles/LDRD= 202.39

Index=   4  Cycles/LDRD= 205.70

Index=   5  Cycles/LDRD= 201.49

Index=   6  Cycles/LDRD= 201.45

Index=   7  Cycles/LDRD= 201.58

Index=   8  Cycles/LDRD= 200.86

Index=  10  Cycles/LDRD= 201.82

Index=  12  Cycles/LDRD= 201.44

Index=  14  Cycles/LDRD= 201.56

Index=  16  Cycles/LDRD= 200.90

Index=  18  Cycles/LDRD= 201.75

Index=  20  Cycles/LDRD= 201.56

Index=  22  Cycles/LDRD= 201.57

Index=  24  Cycles/LDRD= 201.85

Index=  26  Cycles/LDRD= 201.60

Index=  28  Cycles/LDRD= 201.53

Index=  30  Cycles/LDRD= 201.88

Index=  32  Cycles/LDRD= 201.52

Index=  40  Cycles/LDRD= 202.05

Index=  48  Cycles/LDRD= 202.05

Index=  56  Cycles/LDRD= 202.21

Index=  64  Cycles/LDRD= 201.40

Index=  96  Cycles/LDRD= 201.82

Index= 128  Cycles/LDRD= 201.91

Index= 192  Cycles/LDRD= 204.06

Index= 256  Cycles/LDRD= 207.42

Index= 384  Cycles/LDRD= 211.67

Index= 512  Cycles/LDRD= 216.25

Index= 768  Cycles/LDRD= 225.91

Index=1024  Cycles/LDRD= 233.29

Index=1536  Cycles/LDRD= 233.24

Index=2048  Cycles/LDRD= 236.64

Index=3072  Cycles/LDRD= 233.44

Index=4096  Cycles/LDRD= 237.29

Index=6144  Cycles/LDRD= 235.98

Index=8192  Cycles/LDRD= 238.43

Index=12288  Cycles/LDRD= 237.83

Index=16384  Cycles/LDRD= 238.70

Index=20480  Cycles/LDRD= 238.14

Index=24576  Cycles/LDRD= 238.39

Index=28672  Cycles/LDRD= 237.55

Memory store performance test at 0x68000000(DDR3B(normal non-cache))

Index=   0  Cycles/STRD= 3.37

Index=   1  Cycles/STRD= 7.73

Index=   2  Cycles/STRD= 14.66

Index=   3  Cycles/STRD= 13.97

Index=   4  Cycles/STRD= 12.17

Index=   5  Cycles/STRD= 12.50

Index=   6  Cycles/STRD= 11.93

Index=   7  Cycles/STRD= 11.97

Index=   8  Cycles/STRD= 11.40

Index=  10  Cycles/STRD= 11.57

Index=  12  Cycles/STRD= 11.40

Index=  14  Cycles/STRD= 11.39

Index=  16  Cycles/STRD= 11.39

Index=  18  Cycles/STRD= 11.53

Index=  20  Cycles/STRD= 11.43

Index=  22  Cycles/STRD= 11.58

Index=  24  Cycles/STRD= 11.44

Index=  26  Cycles/STRD= 11.54

Index=  28  Cycles/STRD= 11.64

Index=  30  Cycles/STRD= 11.92

Index=  32  Cycles/STRD= 11.53

Index=  40  Cycles/STRD= 11.84

Index=  48  Cycles/STRD= 11.78

Index=  56  Cycles/STRD= 11.82

Index=  64  Cycles/STRD= 12.23

Index=  96  Cycles/STRD= 13.09

Index= 128  Cycles/STRD= 12.53

Index= 192  Cycles/STRD= 13.68

Index= 256  Cycles/STRD= 14.09

Index= 384  Cycles/STRD= 14.48

Index= 512  Cycles/STRD= 14.21

Index= 768  Cycles/STRD= 15.17

Index=1024  Cycles/STRD= 15.88

Index=1536  Cycles/STRD= 16.96

Index=2048  Cycles/STRD= 21.63

Index=3072  Cycles/STRD= 15.79

Index=4096  Cycles/STRD= 37.62

Index=6144  Cycles/STRD= 21.28

Index=8192  Cycles/STRD= 74.90

Index=12288  Cycles/STRD= 37.61

Index=16384  Cycles/STRD= 74.91

Index=20480  Cycles/STRD= 37.61

Index=24576  Cycles/STRD= 74.90

Index=28672  Cycles/STRD= 37.61

Memory load performance test at 0x70000000(DDR3B(RA_WB_WA,  in sh))

Index=   0  Cycles/LDRD= 1.60

Index=   1  Cycles/LDRD= 4.26

Index=   2  Cycles/LDRD= 6.39

Index=   3  Cycles/LDRD= 8.39

Index=   4  Cycles/LDRD= 11.51

Index=   5  Cycles/LDRD= 14.55

Index=   6  Cycles/LDRD= 15.88

Index=   7  Cycles/LDRD= 18.93

Index=   8  Cycles/LDRD= 21.10

Index=  10  Cycles/LDRD= 21.75

Index=  12  Cycles/LDRD= 22.22

Index=  14  Cycles/LDRD= 22.36

Index=  16  Cycles/LDRD= 24.06

Index=  18  Cycles/LDRD= 23.65

Index=  20  Cycles/LDRD= 23.06

Index=  22  Cycles/LDRD= 23.81

Index=  24  Cycles/LDRD= 25.55

Index=  26  Cycles/LDRD= 25.46

Index=  28  Cycles/LDRD= 25.54

Index=  30  Cycles/LDRD= 26.30

Index=  32  Cycles/LDRD= 33.33

Index=  40  Cycles/LDRD= 25.88

Index=  48  Cycles/LDRD= 34.06

Index=  56  Cycles/LDRD= 34.30

Index=  64  Cycles/LDRD= 35.14

Index=  96  Cycles/LDRD= 35.26

Index= 128  Cycles/LDRD= 35.97

Index= 192  Cycles/LDRD= 36.24

Index= 256  Cycles/LDRD= 37.41

Index= 384  Cycles/LDRD= 37.25

Index= 512  Cycles/LDRD= 50.39

Index= 768  Cycles/LDRD= 40.47

Index=1024  Cycles/LDRD= 74.18

Index=1536  Cycles/LDRD= 48.38

Index=2048  Cycles/LDRD= 134.68

Index=3072  Cycles/LDRD= 82.80

Index=4096  Cycles/LDRD= 127.68

Index=6144  Cycles/LDRD= 133.62

Index=8192  Cycles/LDRD= 134.58

Index=12288  Cycles/LDRD= 137.20

Index=16384  Cycles/LDRD= 135.23

Index=20480  Cycles/LDRD= 127.06

Index=24576  Cycles/LDRD= 135.38

Index=28672  Cycles/LDRD= 127.09

Memory store performance test at 0x70000000(DDR3B(RA_WB_WA,  in sh))

Index=   0  Cycles/STRD= 1.42

Index=   1  Cycles/STRD= 2.21

Index=   2  Cycles/STRD= 5.98

Index=   3  Cycles/STRD= 8.53

Index=   4  Cycles/STRD= 11.39

Index=   5  Cycles/STRD= 17.46

Index=   6  Cycles/STRD= 17.93

Index=   7  Cycles/STRD= 23.47

Index=   8  Cycles/STRD= 24.62

Index=  10  Cycles/STRD= 23.35

Index=  12  Cycles/STRD= 24.31

Index=  14  Cycles/STRD= 23.94

Index=  16  Cycles/STRD= 27.07

Index=  18  Cycles/STRD= 24.58

Index=  20  Cycles/STRD= 25.21

Index=  22  Cycles/STRD= 26.09

Index=  24  Cycles/STRD= 27.02

Index=  26  Cycles/STRD= 26.07

Index=  28  Cycles/STRD= 28.13

Index=  30  Cycles/STRD= 28.00

Index=  32  Cycles/STRD= 33.83

Index=  40  Cycles/STRD= 30.37

Index=  48  Cycles/STRD= 35.35

Index=  56  Cycles/STRD= 35.46

Index=  64  Cycles/STRD= 37.11

Index=  96  Cycles/STRD= 37.31

Index= 128  Cycles/STRD= 38.12

Index= 192  Cycles/STRD= 38.59

Index= 256  Cycles/STRD= 43.06

Index= 384  Cycles/STRD= 39.38

Index= 512  Cycles/STRD= 64.05

Index= 768  Cycles/STRD= 45.48

Index=1024  Cycles/STRD= 103.04

Index=1536  Cycles/STRD= 66.19

Index=2048  Cycles/STRD= 145.11

Index=3072  Cycles/STRD= 95.58

Index=4096  Cycles/STRD= 158.33

Index=6144  Cycles/STRD= 145.02

Index=8192  Cycles/STRD= 183.32

Index=12288  Cycles/STRD= 157.11

Index=16384  Cycles/STRD= 184.38

Index=20480  Cycles/STRD= 157.78

Index=24576  Cycles/STRD= 181.51

Index=28672  Cycles/STRD= 160.85

Throughput test with EDMA0 TC0

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  15749 cycles, achieve throughput  9987 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  15885 cycles, achieve throughput  9901 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  15763 cycles, achieve throughput  9978 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  16380 cycles, achieve throughput  9602 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  16372 cycles, achieve throughput  9607 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  17312 cycles, achieve throughput  9085 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  14926 cycles, achieve throughput 10537 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  14921 cycles, achieve throughput 10541 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14923 cycles, achieve throughput 10539 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  15712 cycles, achieve throughput 10010 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  15752 cycles, achieve throughput  9985 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  15664 cycles, achieve throughput 10041 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  17287 cycles, achieve throughput  9098 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  16339 cycles, achieve throughput  9626 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  16426 cycles, achieve throughput  9575 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  14937 cycles, achieve throughput 10529 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  14919 cycles, achieve throughput 10542 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14929 cycles, achieve throughput 10535 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  15658 cycles, achieve throughput 10045 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  15659 cycles, achieve throughput 10044 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  15751 cycles, achieve throughput  9985 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  16483 cycles, achieve throughput  9542 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  16451 cycles, achieve throughput  9560 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  16392 cycles, achieve throughput  9595 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  14810 cycles, achieve throughput 10620 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  14933 cycles, achieve throughput 10532 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14938 cycles, achieve throughput 10529 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  21672 cycles, achieve throughput  7257 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  20467 cycles, achieve throughput  7684 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  21011 cycles, achieve throughput  7485 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  47287 cycles, achieve throughput  3326 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  46809 cycles, achieve throughput  3360 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  46816 cycles, achieve throughput  3359 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  20484 cycles, achieve throughput  7678 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  20936 cycles, achieve throughput  7512 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  20790 cycles, achieve throughput  7565 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  20763 cycles, achieve throughput  7575 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  21454 cycles, achieve throughput  7331 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  20578 cycles, achieve throughput  7643 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  47147 cycles, achieve throughput  3336 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  47092 cycles, achieve throughput  3339 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  46925 cycles, achieve throughput  3351 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  21675 cycles, achieve throughput  7256 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  20584 cycles, achieve throughput  7641 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  20938 cycles, achieve throughput  7512 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  22125 cycles, achieve throughput  7108 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  21203 cycles, achieve throughput  7418 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  21559 cycles, achieve throughput  7295 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  47812 cycles, achieve throughput  3289 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  47992 cycles, achieve throughput  3277 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  47600 cycles, achieve throughput  3304 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  22234 cycles, achieve throughput  7074 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  21368 cycles, achieve throughput  7360 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  21340 cycles, achieve throughput  7370 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  18799 cycles, achieve throughput  8366 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  18795 cycles, achieve throughput  8368 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  18811 cycles, achieve throughput  8361 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  19694 cycles, achieve throughput  7986 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  19228 cycles, achieve throughput  8180 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  18896 cycles, achieve throughput  8323 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  41734 cycles, achieve throughput  3768 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  41733 cycles, achieve throughput  3768 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  41299 cycles, achieve throughput  3808 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  18801 cycles, achieve throughput  8365 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  18806 cycles, achieve throughput  8363 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  18824 cycles, achieve throughput  8355 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  18906 cycles, achieve throughput  8319 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  18840 cycles, achieve throughput  8348 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  18794 cycles, achieve throughput  8368 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  41398 cycles, achieve throughput  3799 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  40957 cycles, achieve throughput  3840 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  41287 cycles, achieve throughput  3809 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  19985 cycles, achieve throughput  7870 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  20077 cycles, achieve throughput  7834 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  20292 cycles, achieve throughput  7751 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  20058 cycles, achieve throughput  7841 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  19981 cycles, achieve throughput  7871 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  20014 cycles, achieve throughput  7858 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  42631 cycles, achieve throughput  3689 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  42623 cycles, achieve throughput  3690 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  42623 cycles, achieve throughput  3690 MB/s

Throughput test with EDMA0 TC1

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  15687 cycles, achieve throughput 10026 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  15722 cycles, achieve throughput 10004 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  15798 cycles, achieve throughput  9956 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  17365 cycles, achieve throughput  9057 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  16410 cycles, achieve throughput  9584 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  17474 cycles, achieve throughput  9001 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  14941 cycles, achieve throughput 10527 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  14924 cycles, achieve throughput 10539 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14819 cycles, achieve throughput 10613 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  15694 cycles, achieve throughput 10022 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  15811 cycles, achieve throughput  9947 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  15779 cycles, achieve throughput  9968 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  16349 cycles, achieve throughput  9620 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  16429 cycles, achieve throughput  9573 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  16809 cycles, achieve throughput  9357 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  14917 cycles, achieve throughput 10544 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  14906 cycles, achieve throughput 10551 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14822 cycles, achieve throughput 10611 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  15699 cycles, achieve throughput 10018 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  15717 cycles, achieve throughput 10007 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  15717 cycles, achieve throughput 10007 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  16487 cycles, achieve throughput  9540 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  16287 cycles, achieve throughput  9657 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  16496 cycles, achieve throughput  9534 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  14802 cycles, achieve throughput 10626 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  14941 cycles, achieve throughput 10527 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14920 cycles, achieve throughput 10541 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  21563 cycles, achieve throughput  7294 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  20560 cycles, achieve throughput  7650 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  20557 cycles, achieve throughput  7651 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  47471 cycles, achieve throughput  3313 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  48134 cycles, achieve throughput  3267 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  48313 cycles, achieve throughput  3255 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  20822 cycles, achieve throughput  7553 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  22150 cycles, achieve throughput  7100 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  20698 cycles, achieve throughput  7599 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  20610 cycles, achieve throughput  7631 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  20625 cycles, achieve throughput  7626 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  21643 cycles, achieve throughput  7267 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  46884 cycles, achieve throughput  3354 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  47043 cycles, achieve throughput  3343 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  47361 cycles, achieve throughput  3321 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  20802 cycles, achieve throughput  7561 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  20577 cycles, achieve throughput  7643 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  21056 cycles, achieve throughput  7469 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  21249 cycles, achieve throughput  7402 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  21229 cycles, achieve throughput  7409 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  20934 cycles, achieve throughput  7513 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  48108 cycles, achieve throughput  3269 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  49345 cycles, achieve throughput  3187 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  47593 cycles, achieve throughput  3304 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  21146 cycles, achieve throughput  7438 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  20922 cycles, achieve throughput  7517 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  21676 cycles, achieve throughput  7256 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  18798 cycles, achieve throughput  8367 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  18821 cycles, achieve throughput  8356 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  18810 cycles, achieve throughput  8361 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  18823 cycles, achieve throughput  8356 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  19784 cycles, achieve throughput  7950 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  18917 cycles, achieve throughput  8314 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  40954 cycles, achieve throughput  3840 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  40956 cycles, achieve throughput  3840 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  41398 cycles, achieve throughput  3799 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  18800 cycles, achieve throughput  8366 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  18891 cycles, achieve throughput  8325 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  18831 cycles, achieve throughput  8352 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  18915 cycles, achieve throughput  8315 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  18894 cycles, achieve throughput  8324 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  18889 cycles, achieve throughput  8326 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  40953 cycles, achieve throughput  3840 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  41289 cycles, achieve throughput  3809 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  41618 cycles, achieve throughput  3779 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  19951 cycles, achieve throughput  7883 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  20287 cycles, achieve throughput  7753 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  19987 cycles, achieve throughput  7869 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  20311 cycles, achieve throughput  7743 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  19966 cycles, achieve throughput  7877 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  19999 cycles, achieve throughput  7864 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  42177 cycles, achieve throughput  3729 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  42630 cycles, achieve throughput  3689 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  42853 cycles, achieve throughput  3670 MB/s

Throughput test with EDMA1 TC0

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  25382 cycles, achieve throughput  6196 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  25354 cycles, achieve throughput  6203 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25345 cycles, achieve throughput  6205 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  25510 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  25448 cycles, achieve throughput  6180 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25638 cycles, achieve throughput  6134 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  25365 cycles, achieve throughput  6200 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25270 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  25370 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  25364 cycles, achieve throughput  6201 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26238 cycles, achieve throughput  5994 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  25426 cycles, achieve throughput  6186 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  25516 cycles, achieve throughput  6164 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25446 cycles, achieve throughput  6181 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  25365 cycles, achieve throughput  6200 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25291 cycles, achieve throughput  6219 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  25364 cycles, achieve throughput  6201 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  26118 cycles, achieve throughput  6022 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  25370 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25348 cycles, achieve throughput  6205 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  25433 cycles, achieve throughput  6184 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  25440 cycles, achieve throughput  6182 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25512 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  28304 cycles, achieve throughput  5557 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  29569 cycles, achieve throughput  5319 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28380 cycles, achieve throughput  5542 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  60109 cycles, achieve throughput  2616 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  60876 cycles, achieve throughput  2583 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  60210 cycles, achieve throughput  2612 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  29559 cycles, achieve throughput  5321 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  28670 cycles, achieve throughput  5486 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  28562 cycles, achieve throughput  5506 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  29455 cycles, achieve throughput  5339 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28350 cycles, achieve throughput  5548 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  29541 cycles, achieve throughput  5324 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  59920 cycles, achieve throughput  2624 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  60440 cycles, achieve throughput  2602 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  61366 cycles, achieve throughput  2563 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  28662 cycles, achieve throughput  5487 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  28540 cycles, achieve throughput  5511 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  28565 cycles, achieve throughput  5506 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  29187 cycles, achieve throughput  5388 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  30671 cycles, achieve throughput  5128 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  29146 cycles, achieve throughput  5396 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  60470 cycles, achieve throughput  2601 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  61155 cycles, achieve throughput  2571 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  60281 cycles, achieve throughput  2609 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  29584 cycles, achieve throughput  5316 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  29380 cycles, achieve throughput  5353 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  29366 cycles, achieve throughput  5356 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  26590 cycles, achieve throughput  5915 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26901 cycles, achieve throughput  5846 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26820 cycles, achieve throughput  5864 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  26835 cycles, achieve throughput  5861 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  26931 cycles, achieve throughput  5840 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  27666 cycles, achieve throughput  5685 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  51854 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  51855 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  51851 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  26600 cycles, achieve throughput  5913 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26506 cycles, achieve throughput  5933 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26833 cycles, achieve throughput  5861 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  26921 cycles, achieve throughput  5842 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  26495 cycles, achieve throughput  5936 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26910 cycles, achieve throughput  5844 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  51849 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  51848 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  51855 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  28144 cycles, achieve throughput  5588 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28165 cycles, achieve throughput  5584 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  27834 cycles, achieve throughput  5650 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  28699 cycles, achieve throughput  5480 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  27476 cycles, achieve throughput  5724 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  28136 cycles, achieve throughput  5590 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  53411 cycles, achieve throughput  2944 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  53524 cycles, achieve throughput  2938 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  53298 cycles, achieve throughput  2951 MB/s

Throughput test with EDMA1 TC1

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25365 cycles, achieve throughput  6200 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25270 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  25367 cycles, achieve throughput  6200 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  26224 cycles, achieve throughput  5997 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25379 cycles, achieve throughput  6197 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  25768 cycles, achieve throughput  6103 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  25297 cycles, achieve throughput  6217 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25617 cycles, achieve throughput  6139 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  25270 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  25365 cycles, achieve throughput  6200 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  25362 cycles, achieve throughput  6201 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  25369 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26092 cycles, achieve throughput  6028 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  25519 cycles, achieve throughput  6163 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  25531 cycles, achieve throughput  6160 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25647 cycles, achieve throughput  6132 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  25291 cycles, achieve throughput  6219 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25270 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  25364 cycles, achieve throughput  6201 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  25369 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  25351 cycles, achieve throughput  6204 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26157 cycles, achieve throughput  6013 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  25439 cycles, achieve throughput  6182 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  25735 cycles, achieve throughput  6111 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25512 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  28360 cycles, achieve throughput  5546 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28317 cycles, achieve throughput  5554 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28316 cycles, achieve throughput  5554 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  59700 cycles, achieve throughput  2634 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  59677 cycles, achieve throughput  2635 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  60777 cycles, achieve throughput  2587 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  28575 cycles, achieve throughput  5504 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  28672 cycles, achieve throughput  5485 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  28580 cycles, achieve throughput  5503 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  28335 cycles, achieve throughput  5550 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  29370 cycles, achieve throughput  5355 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  29478 cycles, achieve throughput  5335 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  61381 cycles, achieve throughput  2562 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  59557 cycles, achieve throughput  2640 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  59979 cycles, achieve throughput  2622 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  28651 cycles, achieve throughput  5489 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  29899 cycles, achieve throughput  5260 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  28909 cycles, achieve throughput  5440 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  29110 cycles, achieve throughput  5403 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28964 cycles, achieve throughput  5430 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28904 cycles, achieve throughput  5441 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  60786 cycles, achieve throughput  2587 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  63129 cycles, achieve throughput  2491 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  62251 cycles, achieve throughput  2526 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  31013 cycles, achieve throughput  5071 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  29357 cycles, achieve throughput  5357 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  29568 cycles, achieve throughput  5319 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  26862 cycles, achieve throughput  5855 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26816 cycles, achieve throughput  5865 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26497 cycles, achieve throughput  5936 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  26565 cycles, achieve throughput  5920 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  26897 cycles, achieve throughput  5847 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26494 cycles, achieve throughput  5936 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  52183 cycles, achieve throughput  3014 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  52181 cycles, achieve throughput  3014 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  51849 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  26842 cycles, achieve throughput  5859 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26833 cycles, achieve throughput  5861 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26838 cycles, achieve throughput  5860 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  26797 cycles, achieve throughput  5869 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  27803 cycles, achieve throughput  5657 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  27463 cycles, achieve throughput  5727 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  52407 cycles, achieve throughput  3001 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  52189 cycles, achieve throughput  3013 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  52069 cycles, achieve throughput  3020 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  28139 cycles, achieve throughput  5589 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  27836 cycles, achieve throughput  5650 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28134 cycles, achieve throughput  5590 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  28810 cycles, achieve throughput  5459 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  28711 cycles, achieve throughput  5478 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  28803 cycles, achieve throughput  5460 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  53412 cycles, achieve throughput  2944 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  53409 cycles, achieve throughput  2944 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  53413 cycles, achieve throughput  2944 MB/s

Throughput test with EDMA1 TC2

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25270 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  25371 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  25372 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25575 cycles, achieve throughput  6150 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  25438 cycles, achieve throughput  6183 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  25326 cycles, achieve throughput  6210 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25520 cycles, achieve throughput  6163 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  25270 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25365 cycles, achieve throughput  6200 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  25337 cycles, achieve throughput  6207 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  25377 cycles, achieve throughput  6197 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26025 cycles, achieve throughput  6043 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  25510 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  25304 cycles, achieve throughput  6215 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25656 cycles, achieve throughput  6130 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  25372 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25267 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  26039 cycles, achieve throughput  6040 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  25349 cycles, achieve throughput  6204 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25356 cycles, achieve throughput  6203 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  25515 cycles, achieve throughput  6164 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  25514 cycles, achieve throughput  6164 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25515 cycles, achieve throughput  6164 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  28217 cycles, achieve throughput  5574 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28421 cycles, achieve throughput  5534 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  29343 cycles, achieve throughput  5360 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  59855 cycles, achieve throughput  2627 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  61357 cycles, achieve throughput  2563 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  61711 cycles, achieve throughput  2548 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  28555 cycles, achieve throughput  5508 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  28797 cycles, achieve throughput  5461 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  29803 cycles, achieve throughput  5277 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  29478 cycles, achieve throughput  5335 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  29434 cycles, achieve throughput  5343 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28337 cycles, achieve throughput  5550 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  59695 cycles, achieve throughput  2634 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  61546 cycles, achieve throughput  2555 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  61264 cycles, achieve throughput  2567 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  28551 cycles, achieve throughput  5508 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  28673 cycles, achieve throughput  5485 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  28799 cycles, achieve throughput  5461 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  30242 cycles, achieve throughput  5200 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28876 cycles, achieve throughput  5446 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  29101 cycles, achieve throughput  5404 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  60649 cycles, achieve throughput  2593 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  62781 cycles, achieve throughput  2505 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  62678 cycles, achieve throughput  2509 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  29248 cycles, achieve throughput  5377 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  29455 cycles, achieve throughput  5339 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  29427 cycles, achieve throughput  5344 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  26616 cycles, achieve throughput  5909 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26846 cycles, achieve throughput  5858 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26806 cycles, achieve throughput  5867 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  26506 cycles, achieve throughput  5933 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  26804 cycles, achieve throughput  5868 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26914 cycles, achieve throughput  5844 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  51850 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  52186 cycles, achieve throughput  3013 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  52184 cycles, achieve throughput  3014 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  26509 cycles, achieve throughput  5933 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26847 cycles, achieve throughput  5858 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26911 cycles, achieve throughput  5844 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  26769 cycles, achieve throughput  5875 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  26921 cycles, achieve throughput  5842 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26822 cycles, achieve throughput  5864 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  52196 cycles, achieve throughput  3013 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  51856 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  52069 cycles, achieve throughput  3020 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  28152 cycles, achieve throughput  5587 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28040 cycles, achieve throughput  5609 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  27836 cycles, achieve throughput  5650 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  27801 cycles, achieve throughput  5657 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  28168 cycles, achieve throughput  5583 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  27482 cycles, achieve throughput  5723 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  53079 cycles, achieve throughput  2963 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  53739 cycles, achieve throughput  2926 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  53529 cycles, achieve throughput  2938 MB/s

Throughput test with EDMA1 TC3

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25322 cycles, achieve throughput  6211 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25323 cycles, achieve throughput  6211 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  25399 cycles, achieve throughput  6192 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  26150 cycles, achieve throughput  6014 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26258 cycles, achieve throughput  5990 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  25514 cycles, achieve throughput  6164 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  25434 cycles, achieve throughput  6184 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25512 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  25267 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  25267 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25364 cycles, achieve throughput  6201 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  25351 cycles, achieve throughput  6204 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  26247 cycles, achieve throughput  5992 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26281 cycles, achieve throughput  5984 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  25505 cycles, achieve throughput  6166 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  25321 cycles, achieve throughput  6211 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25648 cycles, achieve throughput  6132 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25290 cycles, achieve throughput  6219 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  25378 cycles, achieve throughput  6197 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  26122 cycles, achieve throughput  6021 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25369 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  25429 cycles, achieve throughput  6185 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  25515 cycles, achieve throughput  6164 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25633 cycles, achieve throughput  6136 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  28143 cycles, achieve throughput  5588 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  29342 cycles, achieve throughput  5360 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28306 cycles, achieve throughput  5556 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  61217 cycles, achieve throughput  2569 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  59319 cycles, achieve throughput  2651 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  60063 cycles, achieve throughput  2618 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  28457 cycles, achieve throughput  5527 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  28554 cycles, achieve throughput  5508 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  28691 cycles, achieve throughput  5482 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  28131 cycles, achieve throughput  5591 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  29447 cycles, achieve throughput  5341 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28339 cycles, achieve throughput  5550 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  60982 cycles, achieve throughput  2579 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  61305 cycles, achieve throughput  2565 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  59896 cycles, achieve throughput  2625 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  29685 cycles, achieve throughput  5298 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  29683 cycles, achieve throughput  5298 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  29773 cycles, achieve throughput  5282 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  28917 cycles, achieve throughput  5439 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  29818 cycles, achieve throughput  5274 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  30221 cycles, achieve throughput  5204 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  63128 cycles, achieve throughput  2491 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  60977 cycles, achieve throughput  2579 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  60466 cycles, achieve throughput  2601 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  29355 cycles, achieve throughput  5358 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  30688 cycles, achieve throughput  5125 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  29330 cycles, achieve throughput  5362 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  26814 cycles, achieve throughput  5865 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26833 cycles, achieve throughput  5861 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26498 cycles, achieve throughput  5935 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  26918 cycles, achieve throughput  5843 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  26839 cycles, achieve throughput  5860 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26934 cycles, achieve throughput  5839 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  51847 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  52299 cycles, achieve throughput  3007 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  52294 cycles, achieve throughput  3007 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  26748 cycles, achieve throughput  5880 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26824 cycles, achieve throughput  5863 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26909 cycles, achieve throughput  5845 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  27941 cycles, achieve throughput  5629 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  26598 cycles, achieve throughput  5913 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  27464 cycles, achieve throughput  5727 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  51853 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  51849 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  52082 cycles, achieve throughput  3019 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  27804 cycles, achieve throughput  5656 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  27939 cycles, achieve throughput  5629 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28159 cycles, achieve throughput  5585 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  29007 cycles, achieve throughput  5422 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  28282 cycles, achieve throughput  5561 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  28139 cycles, achieve throughput  5589 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  53299 cycles, achieve throughput  2951 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  53411 cycles, achieve throughput  2944 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  53406 cycles, achieve throughput  2945 MB/s

Throughput test with EDMA2 TC0

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25323 cycles, achieve throughput  6211 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  25370 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  25342 cycles, achieve throughput  6206 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25355 cycles, achieve throughput  6203 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  25775 cycles, achieve throughput  6102 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  25511 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25659 cycles, achieve throughput  6129 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  25270 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  25349 cycles, achieve throughput  6204 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  25361 cycles, achieve throughput  6201 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26025 cycles, achieve throughput  6043 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  25441 cycles, achieve throughput  6182 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  25515 cycles, achieve throughput  6164 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25443 cycles, achieve throughput  6181 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25290 cycles, achieve throughput  6219 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  25378 cycles, achieve throughput  6197 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  25372 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25395 cycles, achieve throughput  6193 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  25511 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  25519 cycles, achieve throughput  6163 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25442 cycles, achieve throughput  6182 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  28318 cycles, achieve throughput  5554 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28350 cycles, achieve throughput  5548 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28357 cycles, achieve throughput  5546 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  60794 cycles, achieve throughput  2587 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  60133 cycles, achieve throughput  2615 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  60044 cycles, achieve throughput  2619 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  30041 cycles, achieve throughput  5235 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  28559 cycles, achieve throughput  5507 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  29667 cycles, achieve throughput  5301 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  29431 cycles, achieve throughput  5344 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28336 cycles, achieve throughput  5550 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28559 cycles, achieve throughput  5507 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  60990 cycles, achieve throughput  2578 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  60016 cycles, achieve throughput  2620 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  59988 cycles, achieve throughput  2621 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  28572 cycles, achieve throughput  5504 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  29909 cycles, achieve throughput  5258 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  28555 cycles, achieve throughput  5508 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  31229 cycles, achieve throughput  5036 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  29209 cycles, achieve throughput  5384 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  30353 cycles, achieve throughput  5181 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  61092 cycles, achieve throughput  2574 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  60780 cycles, achieve throughput  2587 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  60637 cycles, achieve throughput  2593 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  29795 cycles, achieve throughput  5278 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  29126 cycles, achieve throughput  5400 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  30433 cycles, achieve throughput  5168 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  26853 cycles, achieve throughput  5857 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26820 cycles, achieve throughput  5864 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26508 cycles, achieve throughput  5933 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  26917 cycles, achieve throughput  5843 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  27815 cycles, achieve throughput  5654 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26572 cycles, achieve throughput  5919 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  51740 cycles, achieve throughput  3039 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  52301 cycles, achieve throughput  3007 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  51858 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  26797 cycles, achieve throughput  5869 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26718 cycles, achieve throughput  5886 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26579 cycles, achieve throughput  5917 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  26931 cycles, achieve throughput  5840 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  26592 cycles, achieve throughput  5914 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  27599 cycles, achieve throughput  5698 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  51855 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  51745 cycles, achieve throughput  3039 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  52294 cycles, achieve throughput  3007 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  28156 cycles, achieve throughput  5586 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28133 cycles, achieve throughput  5590 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28164 cycles, achieve throughput  5584 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  28137 cycles, achieve throughput  5590 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  28139 cycles, achieve throughput  5589 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  29260 cycles, achieve throughput  5375 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  53078 cycles, achieve throughput  2963 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  52739 cycles, achieve throughput  2982 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  53307 cycles, achieve throughput  2950 MB/s

Throughput test with EDMA2 TC1

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  25361 cycles, achieve throughput  6201 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25323 cycles, achieve throughput  6211 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  25371 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  25334 cycles, achieve throughput  6208 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25365 cycles, achieve throughput  6200 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  25518 cycles, achieve throughput  6163 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  25294 cycles, achieve throughput  6218 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25440 cycles, achieve throughput  6182 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  25389 cycles, achieve throughput  6195 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  25350 cycles, achieve throughput  6204 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25370 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  25441 cycles, achieve throughput  6182 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  25659 cycles, achieve throughput  6129 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25516 cycles, achieve throughput  6164 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  25344 cycles, achieve throughput  6206 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  25374 cycles, achieve throughput  6198 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25460 cycles, achieve throughput  6177 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  25618 cycles, achieve throughput  6139 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  25776 cycles, achieve throughput  6102 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25448 cycles, achieve throughput  6180 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  28339 cycles, achieve throughput  5550 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28349 cycles, achieve throughput  5548 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28348 cycles, achieve throughput  5548 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  59759 cycles, achieve throughput  2632 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  60002 cycles, achieve throughput  2621 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  59846 cycles, achieve throughput  2628 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  28562 cycles, achieve throughput  5506 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  28669 cycles, achieve throughput  5486 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  29775 cycles, achieve throughput  5282 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  29559 cycles, achieve throughput  5321 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28379 cycles, achieve throughput  5542 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28337 cycles, achieve throughput  5550 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  60028 cycles, achieve throughput  2620 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  59883 cycles, achieve throughput  2626 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  61926 cycles, achieve throughput  2539 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  28673 cycles, achieve throughput  5485 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  28577 cycles, achieve throughput  5503 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  28908 cycles, achieve throughput  5440 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  29102 cycles, achieve throughput  5404 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  29930 cycles, achieve throughput  5255 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  29908 cycles, achieve throughput  5259 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  61132 cycles, achieve throughput  2572 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  60952 cycles, achieve throughput  2580 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  60792 cycles, achieve throughput  2587 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  29143 cycles, achieve throughput  5397 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  29366 cycles, achieve throughput  5356 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  30566 cycles, achieve throughput  5145 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  26485 cycles, achieve throughput  5938 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26610 cycles, achieve throughput  5910 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26840 cycles, achieve throughput  5860 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  26918 cycles, achieve throughput  5843 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  26574 cycles, achieve throughput  5918 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26488 cycles, achieve throughput  5938 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  51964 cycles, achieve throughput  3026 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  51855 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  51852 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  26819 cycles, achieve throughput  5864 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26842 cycles, achieve throughput  5859 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26859 cycles, achieve throughput  5856 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  26589 cycles, achieve throughput  5915 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  26845 cycles, achieve throughput  5859 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26927 cycles, achieve throughput  5841 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  52191 cycles, achieve throughput  3013 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  52079 cycles, achieve throughput  3020 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  52305 cycles, achieve throughput  3007 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  28140 cycles, achieve throughput  5589 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28154 cycles, achieve throughput  5586 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28160 cycles, achieve throughput  5585 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  28467 cycles, achieve throughput  5525 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  28355 cycles, achieve throughput  5547 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  28697 cycles, achieve throughput  5480 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  53194 cycles, achieve throughput  2956 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  53407 cycles, achieve throughput  2945 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  53293 cycles, achieve throughput  2951 MB/s

Throughput test with EDMA2 TC2

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25348 cycles, achieve throughput  6205 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25323 cycles, achieve throughput  6211 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  25375 cycles, achieve throughput  6198 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  25576 cycles, achieve throughput  6149 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25346 cycles, achieve throughput  6205 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  25507 cycles, achieve throughput  6166 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  25512 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25659 cycles, achieve throughput  6129 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  25366 cycles, achieve throughput  6200 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  25267 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  25369 cycles, achieve throughput  6199 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  25374 cycles, achieve throughput  6198 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26143 cycles, achieve throughput  6016 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  25508 cycles, achieve throughput  6166 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  25512 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25534 cycles, achieve throughput  6159 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  25267 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  25377 cycles, achieve throughput  6197 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  26135 cycles, achieve throughput  6018 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25376 cycles, achieve throughput  6198 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  25511 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  25516 cycles, achieve throughput  6164 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25432 cycles, achieve throughput  6184 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  28330 cycles, achieve throughput  5551 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28346 cycles, achieve throughput  5548 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  29420 cycles, achieve throughput  5346 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  60031 cycles, achieve throughput  2620 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  60006 cycles, achieve throughput  2621 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  61587 cycles, achieve throughput  2553 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  29788 cycles, achieve throughput  5280 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  28585 cycles, achieve throughput  5502 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  28564 cycles, achieve throughput  5506 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  29586 cycles, achieve throughput  5316 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28128 cycles, achieve throughput  5591 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28338 cycles, achieve throughput  5550 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  59810 cycles, achieve throughput  2629 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  60031 cycles, achieve throughput  2620 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  60334 cycles, achieve throughput  2606 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  28592 cycles, achieve throughput  5501 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  29690 cycles, achieve throughput  5297 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  29806 cycles, achieve throughput  5277 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  29085 cycles, achieve throughput  5407 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  29118 cycles, achieve throughput  5401 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  29084 cycles, achieve throughput  5408 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  61248 cycles, achieve throughput  2568 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  60624 cycles, achieve throughput  2594 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  60337 cycles, achieve throughput  2606 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  29360 cycles, achieve throughput  5357 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  30354 cycles, achieve throughput  5181 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  29474 cycles, achieve throughput  5336 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  26492 cycles, achieve throughput  5937 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26491 cycles, achieve throughput  5937 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26505 cycles, achieve throughput  5934 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  27703 cycles, achieve throughput  5677 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  26803 cycles, achieve throughput  5868 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26822 cycles, achieve throughput  5864 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  52184 cycles, achieve throughput  3014 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  51851 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  51858 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  26910 cycles, achieve throughput  5844 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26808 cycles, achieve throughput  5867 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26498 cycles, achieve throughput  5935 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  26907 cycles, achieve throughput  5845 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  26924 cycles, achieve throughput  5841 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26902 cycles, achieve throughput  5846 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  51847 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  52309 cycles, achieve throughput  3006 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  52188 cycles, achieve throughput  3013 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  27808 cycles, achieve throughput  5656 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  27818 cycles, achieve throughput  5654 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28113 cycles, achieve throughput  5594 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  28144 cycles, achieve throughput  5588 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  28895 cycles, achieve throughput  5443 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  29811 cycles, achieve throughput  5276 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  53296 cycles, achieve throughput  2951 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  52965 cycles, achieve throughput  2969 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  53070 cycles, achieve throughput  2963 MB/s

Throughput test with EDMA2 TC3

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25306 cycles, achieve throughput  6215 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25267 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  25384 cycles, achieve throughput  6196 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  25387 cycles, achieve throughput  6195 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25375 cycles, achieve throughput  6198 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  25439 cycles, achieve throughput  6182 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  25656 cycles, achieve throughput  6130 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25665 cycles, achieve throughput  6128 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  25366 cycles, achieve throughput  6200 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  25268 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25267 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  26148 cycles, achieve throughput  6015 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  25354 cycles, achieve throughput  6203 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25367 cycles, achieve throughput  6200 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  25514 cycles, achieve throughput  6164 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  25439 cycles, achieve throughput  6182 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25518 cycles, achieve throughput  6163 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  25269 cycles, achieve throughput  6224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25290 cycles, achieve throughput  6219 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  25351 cycles, achieve throughput  6204 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  25568 cycles, achieve throughput  6151 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  25373 cycles, achieve throughput  6198 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25336 cycles, achieve throughput  6208 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  25518 cycles, achieve throughput  6163 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  25511 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25654 cycles, achieve throughput  6131 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  28410 cycles, achieve throughput  5536 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28378 cycles, achieve throughput  5542 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28122 cycles, achieve throughput  5593 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  61193 cycles, achieve throughput  2570 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  61005 cycles, achieve throughput  2578 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  60048 cycles, achieve throughput  2619 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  28593 cycles, achieve throughput  5500 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  28446 cycles, achieve throughput  5529 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  28793 cycles, achieve throughput  5462 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  29251 cycles, achieve throughput  5377 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28358 cycles, achieve throughput  5546 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28335 cycles, achieve throughput  5550 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  60853 cycles, achieve throughput  2584 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  60323 cycles, achieve throughput  2607 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  60573 cycles, achieve throughput  2596 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  28589 cycles, achieve throughput  5501 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  29984 cycles, achieve throughput  5245 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  28796 cycles, achieve throughput  5462 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  29465 cycles, achieve throughput  5338 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  30227 cycles, achieve throughput  5203 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  30103 cycles, achieve throughput  5224 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  61899 cycles, achieve throughput  2541 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  61511 cycles, achieve throughput  2557 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  60671 cycles, achieve throughput  2592 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  30539 cycles, achieve throughput  5150 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  29457 cycles, achieve throughput  5339 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  29493 cycles, achieve throughput  5333 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  26589 cycles, achieve throughput  5915 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26812 cycles, achieve throughput  5866 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26586 cycles, achieve throughput  5916 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  26703 cycles, achieve throughput  5890 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  26712 cycles, achieve throughput  5888 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26488 cycles, achieve throughput  5938 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  51745 cycles, achieve throughput  3039 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  51851 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  52072 cycles, achieve throughput  3020 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  26832 cycles, achieve throughput  5861 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  26494 cycles, achieve throughput  5936 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  26584 cycles, achieve throughput  5916 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  27578 cycles, achieve throughput  5703 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  26500 cycles, achieve throughput  5935 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26575 cycles, achieve throughput  5918 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  52192 cycles, achieve throughput  3013 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  51962 cycles, achieve throughput  3026 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  51849 cycles, achieve throughput  3033 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  28152 cycles, achieve throughput  5587 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28161 cycles, achieve throughput  5585 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28160 cycles, achieve throughput  5585 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  28584 cycles, achieve throughput  5502 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  27802 cycles, achieve throughput  5657 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  27834 cycles, achieve throughput  5650 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  53418 cycles, achieve throughput  2944 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  53409 cycles, achieve throughput  2944 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  53406 cycles, achieve throughput  2945 MB/s

Throughput test with EDMA3 TC0

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  25256 cycles, achieve throughput  6227 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25320 cycles, achieve throughput  6211 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25316 cycles, achieve throughput  6212 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  25663 cycles, achieve throughput  6128 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  25543 cycles, achieve throughput  6157 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26624 cycles, achieve throughput  5907 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  25889 cycles, achieve throughput  6075 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  25564 cycles, achieve throughput  6152 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25676 cycles, achieve throughput  6125 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  25274 cycles, achieve throughput  6223 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  25263 cycles, achieve throughput  6225 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25273 cycles, achieve throughput  6223 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  25505 cycles, achieve throughput  6166 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  25529 cycles, achieve throughput  6161 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25717 cycles, achieve throughput  6116 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  25342 cycles, achieve throughput  6206 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  25562 cycles, achieve throughput  6153 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25891 cycles, achieve throughput  6074 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  25265 cycles, achieve throughput  6225 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25275 cycles, achieve throughput  6223 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  25274 cycles, achieve throughput  6223 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  25523 cycles, achieve throughput  6162 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  25504 cycles, achieve throughput  6167 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25748 cycles, achieve throughput  6108 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  25568 cycles, achieve throughput  6151 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  25564 cycles, achieve throughput  6152 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25900 cycles, achieve throughput  6072 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  29933 cycles, achieve throughput  5254 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  30412 cycles, achieve throughput  5171 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  29959 cycles, achieve throughput  5250 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  61095 cycles, achieve throughput  2574 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  60716 cycles, achieve throughput  2590 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  61006 cycles, achieve throughput  2578 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  30383 cycles, achieve throughput  5176 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  30486 cycles, achieve throughput  5159 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  31738 cycles, achieve throughput  4955 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  30132 cycles, achieve throughput  5219 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  31192 cycles, achieve throughput  5042 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  30155 cycles, achieve throughput  5215 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  61059 cycles, achieve throughput  2575 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  61002 cycles, achieve throughput  2578 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  61072 cycles, achieve throughput  2575 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  30388 cycles, achieve throughput  5175 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  30859 cycles, achieve throughput  5096 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  30632 cycles, achieve throughput  5134 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  30711 cycles, achieve throughput  5121 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  30626 cycles, achieve throughput  5135 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  30704 cycles, achieve throughput  5122 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  61235 cycles, achieve throughput  2568 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  63301 cycles, achieve throughput  2484 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  61085 cycles, achieve throughput  2574 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  31419 cycles, achieve throughput  5006 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  31221 cycles, achieve throughput  5037 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  31174 cycles, achieve throughput  5045 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  27216 cycles, achieve throughput  5779 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  27576 cycles, achieve throughput  5703 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  27520 cycles, achieve throughput  5715 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  27481 cycles, achieve throughput  5723 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  27851 cycles, achieve throughput  5647 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  28947 cycles, achieve throughput  5433 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  51520 cycles, achieve throughput  3052 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  51191 cycles, achieve throughput  3072 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  51526 cycles, achieve throughput  3052 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  27220 cycles, achieve throughput  5778 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  27207 cycles, achieve throughput  5781 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  27525 cycles, achieve throughput  5714 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  27498 cycles, achieve throughput  5719 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  27514 cycles, achieve throughput  5716 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  27790 cycles, achieve throughput  5659 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  51183 cycles, achieve throughput  3073 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  51185 cycles, achieve throughput  3072 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  51520 cycles, achieve throughput  3052 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  28636 cycles, achieve throughput  5492 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28755 cycles, achieve throughput  5469 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28750 cycles, achieve throughput  5470 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  28718 cycles, achieve throughput  5476 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  29053 cycles, achieve throughput  5413 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  29855 cycles, achieve throughput  5268 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  52750 cycles, achieve throughput  2981 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  52073 cycles, achieve throughput  3020 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  52629 cycles, achieve throughput  2988 MB/s

Throughput test with EDMA3 TC1

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  25271 cycles, achieve throughput  6223 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25324 cycles, achieve throughput  6210 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25282 cycles, achieve throughput  6221 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  25509 cycles, achieve throughput  6165 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  25508 cycles, achieve throughput  6166 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25499 cycles, achieve throughput  6168 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  25665 cycles, achieve throughput  6128 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  25672 cycles, achieve throughput  6126 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25896 cycles, achieve throughput  6073 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  25373 cycles, achieve throughput  6198 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  25281 cycles, achieve throughput  6221 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  25280 cycles, achieve throughput  6221 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  25507 cycles, achieve throughput  6166 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  25487 cycles, achieve throughput  6171 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  25738 cycles, achieve throughput  6111 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  25893 cycles, achieve throughput  6074 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  25664 cycles, achieve throughput  6128 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25671 cycles, achieve throughput  6127 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  25373 cycles, achieve throughput  6198 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  25288 cycles, achieve throughput  6219 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  25282 cycles, achieve throughput  6221 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  26654 cycles, achieve throughput  5901 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  25522 cycles, achieve throughput  6162 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  26294 cycles, achieve throughput  5981 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  25454 cycles, achieve throughput  6179 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  25783 cycles, achieve throughput  6100 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  25665 cycles, achieve throughput  6128 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  29908 cycles, achieve throughput  5259 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  29929 cycles, achieve throughput  5255 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  29885 cycles, achieve throughput  5263 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  61459 cycles, achieve throughput  2559 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  62399 cycles, achieve throughput  2520 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  62638 cycles, achieve throughput  2511 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  32861 cycles, achieve throughput  4786 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  31479 cycles, achieve throughput  4996 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  30656 cycles, achieve throughput  5130 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  29918 cycles, achieve throughput  5257 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  30090 cycles, achieve throughput  5227 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  29870 cycles, achieve throughput  5265 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  62402 cycles, achieve throughput  2520 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  61350 cycles, achieve throughput  2563 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  62522 cycles, achieve throughput  2515 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  31737 cycles, achieve throughput  4955 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  31975 cycles, achieve throughput  4919 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  30630 cycles, achieve throughput  5135 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  30896 cycles, achieve throughput  5090 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  31811 cycles, achieve throughput  4944 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  30662 cycles, achieve throughput  5129 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  62115 cycles, achieve throughput  2532 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  61798 cycles, achieve throughput  2545 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  64177 cycles, achieve throughput  2450 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  31474 cycles, achieve throughput  4997 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  32303 cycles, achieve throughput  4869 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  32623 cycles, achieve throughput  4821 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  27170 cycles, achieve throughput  5788 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  27537 cycles, achieve throughput  5711 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  27185 cycles, achieve throughput  5785 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  27646 cycles, achieve throughput  5689 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  27530 cycles, achieve throughput  5713 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  28940 cycles, achieve throughput  5434 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  51633 cycles, achieve throughput  3046 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  50850 cycles, achieve throughput  3093 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  51194 cycles, achieve throughput  3072 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  27222 cycles, achieve throughput  5777 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  27164 cycles, achieve throughput  5790 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  27427 cycles, achieve throughput  5734 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  28985 cycles, achieve throughput  5426 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  27867 cycles, achieve throughput  5644 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  27955 cycles, achieve throughput  5626 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  51189 cycles, achieve throughput  3072 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  51188 cycles, achieve throughput  3072 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  51628 cycles, achieve throughput  3046 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  28773 cycles, achieve throughput  5466 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  28756 cycles, achieve throughput  5469 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  28815 cycles, achieve throughput  5458 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  29418 cycles, achieve throughput  5346 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  29077 cycles, achieve throughput  5409 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  29852 cycles, achieve throughput  5268 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  52071 cycles, achieve throughput  3020 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  52627 cycles, achieve throughput  2988 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  52409 cycles, achieve throughput  3001 MB/s

Throughput test with EDMA4 TC0

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  15779 cycles, achieve throughput  9968 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  15746 cycles, achieve throughput  9988 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  15727 cycles, achieve throughput 10001 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  16425 cycles, achieve throughput  9576 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  17497 cycles, achieve throughput  8989 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  16415 cycles, achieve throughput  9581 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  14851 cycles, achieve throughput 10590 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  14817 cycles, achieve throughput 10615 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14829 cycles, achieve throughput 10606 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  15672 cycles, achieve throughput 10036 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  15700 cycles, achieve throughput 10018 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  15689 cycles, achieve throughput 10025 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  16323 cycles, achieve throughput  9635 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  16428 cycles, achieve throughput  9574 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  16946 cycles, achieve throughput  9281 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  14924 cycles, achieve throughput 10539 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  14914 cycles, achieve throughput 10546 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14914 cycles, achieve throughput 10546 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  15666 cycles, achieve throughput 10039 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  15720 cycles, achieve throughput 10005 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  15697 cycles, achieve throughput 10020 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  16379 cycles, achieve throughput  9602 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  16368 cycles, achieve throughput  9609 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  16407 cycles, achieve throughput  9586 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  14922 cycles, achieve throughput 10540 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  14934 cycles, achieve throughput 10532 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14941 cycles, achieve throughput 10527 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  21773 cycles, achieve throughput  7223 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  20563 cycles, achieve throughput  7649 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  20533 cycles, achieve throughput  7660 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  47254 cycles, achieve throughput  3328 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  47559 cycles, achieve throughput  3307 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  48305 cycles, achieve throughput  3256 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  21810 cycles, achieve throughput  7211 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  20568 cycles, achieve throughput  7647 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  20707 cycles, achieve throughput  7595 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  21676 cycles, achieve throughput  7256 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  20561 cycles, achieve throughput  7649 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  20542 cycles, achieve throughput  7656 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  48008 cycles, achieve throughput  3276 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  47143 cycles, achieve throughput  3336 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  48425 cycles, achieve throughput  3248 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  20924 cycles, achieve throughput  7517 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  20700 cycles, achieve throughput  7598 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  20700 cycles, achieve throughput  7598 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  21143 cycles, achieve throughput  7439 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  21119 cycles, achieve throughput  7447 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  21171 cycles, achieve throughput  7429 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  47808 cycles, achieve throughput  3289 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  49064 cycles, achieve throughput  3205 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  47516 cycles, achieve throughput  3310 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  21354 cycles, achieve throughput  7365 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  21391 cycles, achieve throughput  7352 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  21147 cycles, achieve throughput  7437 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  18818 cycles, achieve throughput  8358 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  18817 cycles, achieve throughput  8358 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  18848 cycles, achieve throughput  8344 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  18810 cycles, achieve throughput  8361 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  20094 cycles, achieve throughput  7827 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  18906 cycles, achieve throughput  8319 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  41513 cycles, achieve throughput  3788 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  41511 cycles, achieve throughput  3789 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  41296 cycles, achieve throughput  3808 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  18829 cycles, achieve throughput  8353 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  18826 cycles, achieve throughput  8354 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  18826 cycles, achieve throughput  8354 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  18909 cycles, achieve throughput  8318 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  18886 cycles, achieve throughput  8328 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  20359 cycles, achieve throughput  7725 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  41292 cycles, achieve throughput  3809 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  41627 cycles, achieve throughput  3778 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  41188 cycles, achieve throughput  3818 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  19956 cycles, achieve throughput  7881 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  20070 cycles, achieve throughput  7836 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  20330 cycles, achieve throughput  7736 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  20038 cycles, achieve throughput  7849 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  20047 cycles, achieve throughput  7845 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  20115 cycles, achieve throughput  7819 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  42631 cycles, achieve throughput  3689 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  42627 cycles, achieve throughput  3689 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  42409 cycles, achieve throughput  3708 MB/s

Throughput test with EDMA4 TC1

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0a0000(MSRAM(strongly ordered)), consumes  15720 cycles, achieve throughput 10005 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  15721 cycles, achieve throughput 10004 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  15728 cycles, achieve throughput 10000 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  16425 cycles, achieve throughput  9576 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  17369 cycles, achieve throughput  9055 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  16423 cycles, achieve throughput  9577 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  14931 cycles, achieve throughput 10534 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  14930 cycles, achieve throughput 10534 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c080000(MSRAM(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14924 cycles, achieve throughput 10539 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  15699 cycles, achieve throughput 10018 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c0e0000(MSRAM(normal non-cache)), consumes  15657 cycles, achieve throughput 10045 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  15718 cycles, achieve throughput 10006 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  16839 cycles, achieve throughput  9340 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  16460 cycles, achieve throughput  9555 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  17201 cycles, achieve throughput  9144 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  14933 cycles, achieve throughput 10532 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  14919 cycles, achieve throughput 10542 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c0c0000(MSRAM(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14920 cycles, achieve throughput 10541 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  15722 cycles, achieve throughput 10004 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  15661 cycles, achieve throughput 10043 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x c120000(MSRAM(RA_WB_WA,  in sh)), consumes  15754 cycles, achieve throughput  9983 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  16396 cycles, achieve throughput  9592 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  16252 cycles, achieve throughput  9677 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  16323 cycles, achieve throughput  9635 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  14927 cycles, achieve throughput 10537 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  15030 cycles, achieve throughput 10464 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x c100000(MSRAM(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  14936 cycles, achieve throughput 10530 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  20569 cycles, achieve throughput  7646 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  20777 cycles, achieve throughput  7570 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  20618 cycles, achieve throughput  7628 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x80020000(DDR3A(strongly ordered)), consumes  49710 cycles, achieve throughput  3164 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  47882 cycles, achieve throughput  3284 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  48633 cycles, achieve throughput  3234 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x60000000(DDR3B(strongly ordered)), consumes  20796 cycles, achieve throughput  7563 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  21702 cycles, achieve throughput  7247 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x80000000(DDR3A(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  21705 cycles, achieve throughput  7246 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  20859 cycles, achieve throughput  7540 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  20894 cycles, achieve throughput  7527 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  20570 cycles, achieve throughput  7646 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  49335 cycles, achieve throughput  3188 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x88020000(DDR3A(normal non-cache)), consumes  46966 cycles, achieve throughput  3348 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  47249 cycles, achieve throughput  3328 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  20562 cycles, achieve throughput  7649 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x68000000(DDR3B(normal non-cache)), consumes  20706 cycles, achieve throughput  7596 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x88000000(DDR3A(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  20593 cycles, achieve throughput  7637 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  21318 cycles, achieve throughput  7378 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  21926 cycles, achieve throughput  7173 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  21334 cycles, achieve throughput  7372 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  47883 cycles, achieve throughput  3284 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  49844 cycles, achieve throughput  3155 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x90020000(DDR3A(RA_WB_WA,  in sh)), consumes  49079 cycles, achieve throughput  3204 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  21379 cycles, achieve throughput  7357 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  21118 cycles, achieve throughput  7447 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x90000000(DDR3A(RA_WB_WA,  in sh)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  21716 cycles, achieve throughput  7242 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c080000(MSRAM(strongly ordered)), consumes  18830 cycles, achieve throughput  8352 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  18789 cycles, achieve throughput  8371 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  18810 cycles, achieve throughput  8361 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x80000000(DDR3A(strongly ordered)), consumes  18915 cycles, achieve throughput  8315 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x88000000(DDR3A(normal non-cache)), consumes  18844 cycles, achieve throughput  8346 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  19886 cycles, achieve throughput  7909 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x60020000(DDR3B(strongly ordered)), consumes  41740 cycles, achieve throughput  3768 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x68000000(DDR3B(normal non-cache)), consumes  41290 cycles, achieve throughput  3809 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x60000000(DDR3B(strongly ordered)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  41729 cycles, achieve throughput  3769 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c080000(MSRAM(strongly ordered)), consumes  18799 cycles, achieve throughput  8366 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  18803 cycles, achieve throughput  8364 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  18848 cycles, achieve throughput  8344 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x80000000(DDR3A(strongly ordered)), consumes  18842 cycles, achieve throughput  8347 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x88000000(DDR3A(normal non-cache)), consumes  18871 cycles, achieve throughput  8334 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  18795 cycles, achieve throughput  8368 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x60000000(DDR3B(strongly ordered)), consumes  41177 cycles, achieve throughput  3819 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x68020000(DDR3B(normal non-cache)), consumes  41183 cycles, achieve throughput  3819 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x68000000(DDR3B(normal non-cache)) to 0x70000000(DDR3B(RA_WB_WA,  in sh)), consumes  41844 cycles, achieve throughput  3758 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c080000(MSRAM(strongly ordered)), consumes  19638 cycles, achieve throughput  8009 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c0c0000(MSRAM(normal non-cache)), consumes  19942 cycles, achieve throughput  7887 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x c100000(MSRAM(RA_WB_WA,  in sh)), consumes  19741 cycles, achieve throughput  7967 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x80000000(DDR3A(strongly ordered)), consumes  20027 cycles, achieve throughput  7853 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x88000000(DDR3A(normal non-cache)), consumes  20058 cycles, achieve throughput  7841 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x90000000(DDR3A(RA_WB_WA,  in sh)), consumes  20030 cycles, achieve throughput  7852 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x60000000(DDR3B(strongly ordered)), consumes  42618 cycles, achieve throughput  3690 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x68000000(DDR3B(normal non-cache)), consumes  42618 cycles, achieve throughput  3690 MB/s

transfer    8 * 16384 Bytes with index=16384 from 0x70000000(DDR3B(RA_WB_WA,  in sh)) to 0x70020000(DDR3B(RA_WB_WA,  in sh)), consumes  42401 cycles, achieve throughput  3709 MB/s

EDMA ACNT test with EDMA0 TC0

transfer    1 *     1 Bytes with index=    1 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    601 cycles, achieve throughput     1 MB/s

transfer    1 *     2 Bytes with index=    2 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    593 cycles, achieve throughput     4 MB/s

transfer    1 *     3 Bytes with index=    3 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput     6 MB/s

transfer    1 *     4 Bytes with index=    4 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    592 cycles, achieve throughput     8 MB/s

transfer    1 *     5 Bytes with index=    5 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    593 cycles, achieve throughput    10 MB/s

transfer    1 *     6 Bytes with index=    6 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput    12 MB/s

transfer    1 *     7 Bytes with index=    7 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput    14 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput    16 MB/s

transfer    1 *     9 Bytes with index=    9 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput    18 MB/s

transfer    1 *    10 Bytes with index=   10 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput    20 MB/s

transfer    1 *    11 Bytes with index=   11 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput    22 MB/s

transfer    1 *    12 Bytes with index=   12 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    594 cycles, achieve throughput    24 MB/s

transfer    1 *    13 Bytes with index=   13 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    592 cycles, achieve throughput    26 MB/s

transfer    1 *    14 Bytes with index=   14 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput    28 MB/s

transfer    1 *    15 Bytes with index=   15 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput    30 MB/s

transfer    1 *    16 Bytes with index=   16 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput    32 MB/s

transfer    1 *    32 Bytes with index=   32 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    586 cycles, achieve throughput    65 MB/s

transfer    1 *    48 Bytes with index=   48 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    592 cycles, achieve throughput    97 MB/s

transfer    1 *    64 Bytes with index=   64 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput   129 MB/s

transfer    1 *    80 Bytes with index=   80 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput   162 MB/s

transfer    1 *    96 Bytes with index=   96 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput   195 MB/s

transfer    1 *   112 Bytes with index=  112 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    592 cycles, achieve throughput   227 MB/s

transfer    1 *   128 Bytes with index=  128 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput   260 MB/s

transfer    1 *   144 Bytes with index=  144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput   292 MB/s

transfer    1 *   160 Bytes with index=  160 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput   325 MB/s

transfer    1 *   176 Bytes with index=  176 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput   358 MB/s

transfer    1 *   192 Bytes with index=  192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput   390 MB/s

transfer    1 *   208 Bytes with index=  208 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput   423 MB/s

transfer    1 *   224 Bytes with index=  224 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    588 cycles, achieve throughput   457 MB/s

transfer    1 *   240 Bytes with index=  240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput   488 MB/s

transfer    1 *   256 Bytes with index=  256 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    593 cycles, achieve throughput   518 MB/s

transfer    1 *   288 Bytes with index=  288 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput   585 MB/s

transfer    1 *   320 Bytes with index=  320 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput   649 MB/s

transfer    1 *   352 Bytes with index=  352 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput   717 MB/s

transfer    1 *   384 Bytes with index=  384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput   781 MB/s

transfer    1 *   416 Bytes with index=  416 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput   847 MB/s

transfer    1 *   448 Bytes with index=  448 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput   911 MB/s

transfer    1 *   480 Bytes with index=  480 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput   977 MB/s

transfer    1 *   512 Bytes with index=  512 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    592 cycles, achieve throughput  1037 MB/s

transfer    1 *   576 Bytes with index=  576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    705 cycles, achieve throughput   980 MB/s

transfer    1 *   640 Bytes with index=  640 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput  1097 MB/s

transfer    1 *   704 Bytes with index=  704 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput  1203 MB/s

transfer    1 *   768 Bytes with index=  768 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput  1312 MB/s

transfer    1 *   832 Bytes with index=  832 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput  1422 MB/s

transfer    1 *   896 Bytes with index=  896 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1590 cycles, achieve throughput   676 MB/s

transfer    1 *   960 Bytes with index=  960 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput  1645 MB/s

transfer    1 *  1024 Bytes with index= 1024 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    699 cycles, achieve throughput  1757 MB/s

transfer    1 *  1152 Bytes with index= 1152 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput  1974 MB/s

transfer    1 *  1280 Bytes with index= 1280 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput  2194 MB/s

transfer    1 *  1408 Bytes with index= 1408 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  2410 MB/s

transfer    1 *  1536 Bytes with index= 1536 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    813 cycles, achieve throughput  2267 MB/s

transfer    1 *  1664 Bytes with index= 1664 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    813 cycles, achieve throughput  2456 MB/s

transfer    1 *  1792 Bytes with index= 1792 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    813 cycles, achieve throughput  2645 MB/s

transfer    1 *  1920 Bytes with index= 1920 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1596 cycles, achieve throughput  1443 MB/s

transfer    1 *  2048 Bytes with index= 2048 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    811 cycles, achieve throughput  3030 MB/s

transfer    1 *  2304 Bytes with index= 2304 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    816 cycles, achieve throughput  3388 MB/s

transfer    1 *  2560 Bytes with index= 2560 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    928 cycles, achieve throughput  3310 MB/s

transfer    1 *  2816 Bytes with index= 2816 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    925 cycles, achieve throughput  3653 MB/s

transfer    1 *  3072 Bytes with index= 3072 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    927 cycles, achieve throughput  3976 MB/s

transfer    1 *  3328 Bytes with index= 3328 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    925 cycles, achieve throughput  4317 MB/s

transfer    1 *  3584 Bytes with index= 3584 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1036 cycles, achieve throughput  4151 MB/s

transfer    1 *  3840 Bytes with index= 3840 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1034 cycles, achieve throughput  4456 MB/s

transfer    1 *  4096 Bytes with index= 4096 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1036 cycles, achieve throughput  4744 MB/s

transfer    1 *  4608 Bytes with index= 4608 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1159 cycles, achieve throughput  4771 MB/s

transfer    1 *  5120 Bytes with index= 5120 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1155 cycles, achieve throughput  5319 MB/s

transfer    1 *  5632 Bytes with index= 5632 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1268 cycles, achieve throughput  5329 MB/s

transfer    1 *  6144 Bytes with index= 6144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1269 cycles, achieve throughput  5809 MB/s

transfer    1 *  6656 Bytes with index= 6656 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1390 cycles, achieve throughput  5746 MB/s

transfer    1 *  7168 Bytes with index= 7168 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1387 cycles, achieve throughput  6201 MB/s

transfer    1 *  7680 Bytes with index= 7680 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1502 cycles, achieve throughput  6135 MB/s

transfer    1 *  8192 Bytes with index= 8192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1520 cycles, achieve throughput  6467 MB/s

transfer    1 *  9216 Bytes with index= 9216 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1632 cycles, achieve throughput  6776 MB/s

transfer    1 * 10240 Bytes with index=10240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1751 cycles, achieve throughput  7017 MB/s

transfer    1 * 11264 Bytes with index=11264 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1944 cycles, achieve throughput  6953 MB/s

transfer    1 * 12288 Bytes with index=12288 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2073 cycles, achieve throughput  7113 MB/s

transfer    1 * 13312 Bytes with index=13312 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2185 cycles, achieve throughput  7310 MB/s

transfer    1 * 14336 Bytes with index=14336 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2314 cycles, achieve throughput  7434 MB/s

transfer    1 * 15360 Bytes with index=15360 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2493 cycles, achieve throughput  7393 MB/s

transfer    1 * 16384 Bytes with index=16384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2528 cycles, achieve throughput  7777 MB/s

transfer    1 * 17408 Bytes with index=17408 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2648 cycles, achieve throughput  7888 MB/s

transfer    1 * 18432 Bytes with index=18432 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2793 cycles, achieve throughput  7919 MB/s

transfer    1 * 19456 Bytes with index=19456 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2880 cycles, achieve throughput  8106 MB/s

transfer    1 * 20480 Bytes with index=20480 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3008 cycles, achieve throughput  8170 MB/s

transfer    1 * 21504 Bytes with index=21504 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3206 cycles, achieve throughput  8048 MB/s

transfer    1 * 22528 Bytes with index=22528 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3334 cycles, achieve throughput  8108 MB/s

transfer    1 * 23552 Bytes with index=23552 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3428 cycles, achieve throughput  8244 MB/s

transfer    1 * 24576 Bytes with index=24576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3545 cycles, achieve throughput  8319 MB/s

EDMA ACNT test with EDMA0 TC0

transfer    1 *     1 Bytes with index=    1 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    590 cycles, achieve throughput     2 MB/s

transfer    1 *     2 Bytes with index=    2 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    595 cycles, achieve throughput     4 MB/s

transfer    1 *     3 Bytes with index=    3 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    589 cycles, achieve throughput     6 MB/s

transfer    1 *     4 Bytes with index=    4 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    589 cycles, achieve throughput     8 MB/s

transfer    1 *     5 Bytes with index=    5 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    593 cycles, achieve throughput    10 MB/s

transfer    1 *     6 Bytes with index=    6 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    588 cycles, achieve throughput    12 MB/s

transfer    1 *     7 Bytes with index=    7 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    590 cycles, achieve throughput    14 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     9 Bytes with index=    9 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    591 cycles, achieve throughput    18 MB/s

transfer    1 *    10 Bytes with index=   10 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    591 cycles, achieve throughput    20 MB/s

transfer    1 *    11 Bytes with index=   11 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    590 cycles, achieve throughput    22 MB/s

transfer    1 *    12 Bytes with index=   12 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    589 cycles, achieve throughput    24 MB/s

transfer    1 *    13 Bytes with index=   13 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    592 cycles, achieve throughput    26 MB/s

transfer    1 *    14 Bytes with index=   14 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    590 cycles, achieve throughput    28 MB/s

transfer    1 *    15 Bytes with index=   15 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    592 cycles, achieve throughput    30 MB/s

transfer    1 *    16 Bytes with index=   16 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    590 cycles, achieve throughput    32 MB/s

transfer    1 *    32 Bytes with index=   32 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    590 cycles, achieve throughput    65 MB/s

transfer    1 *    48 Bytes with index=   48 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    592 cycles, achieve throughput    97 MB/s

transfer    1 *    64 Bytes with index=   64 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    589 cycles, achieve throughput   130 MB/s

transfer    1 *    80 Bytes with index=   80 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    586 cycles, achieve throughput   163 MB/s

transfer    1 *    96 Bytes with index=   96 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    589 cycles, achieve throughput   195 MB/s

transfer    1 *   112 Bytes with index=  112 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    589 cycles, achieve throughput   228 MB/s

transfer    1 *   128 Bytes with index=  128 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1589 cycles, achieve throughput    96 MB/s

transfer    1 *   144 Bytes with index=  144 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    590 cycles, achieve throughput   292 MB/s

transfer    1 *   160 Bytes with index=  160 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    592 cycles, achieve throughput   324 MB/s

transfer    1 *   176 Bytes with index=  176 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    591 cycles, achieve throughput   357 MB/s

transfer    1 *   192 Bytes with index=  192 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    592 cycles, achieve throughput   389 MB/s

transfer    1 *   208 Bytes with index=  208 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    593 cycles, achieve throughput   420 MB/s

transfer    1 *   224 Bytes with index=  224 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    589 cycles, achieve throughput   456 MB/s

transfer    1 *   240 Bytes with index=  240 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    704 cycles, achieve throughput   409 MB/s

transfer    1 *   256 Bytes with index=  256 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    702 cycles, achieve throughput   437 MB/s

transfer    1 *   288 Bytes with index=  288 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    588 cycles, achieve throughput   587 MB/s

transfer    1 *   320 Bytes with index=  320 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    592 cycles, achieve throughput   648 MB/s

transfer    1 *   352 Bytes with index=  352 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    704 cycles, achieve throughput   600 MB/s

transfer    1 *   384 Bytes with index=  384 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    704 cycles, achieve throughput   654 MB/s

transfer    1 *   416 Bytes with index=  416 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    814 cycles, achieve throughput   613 MB/s

transfer    1 *   448 Bytes with index=  448 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    705 cycles, achieve throughput   762 MB/s

transfer    1 *   480 Bytes with index=  480 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    703 cycles, achieve throughput   819 MB/s

transfer    1 *   512 Bytes with index=  512 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    700 cycles, achieve throughput   877 MB/s

transfer    1 *   576 Bytes with index=  576 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    702 cycles, achieve throughput   984 MB/s

transfer    1 *   640 Bytes with index=  640 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    702 cycles, achieve throughput  1094 MB/s

transfer    1 *   704 Bytes with index=  704 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    702 cycles, achieve throughput  1203 MB/s

transfer    1 *   768 Bytes with index=  768 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    703 cycles, achieve throughput  1310 MB/s

transfer    1 *   832 Bytes with index=  832 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    701 cycles, achieve throughput  1424 MB/s

transfer    1 *   896 Bytes with index=  896 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    707 cycles, achieve throughput  1520 MB/s

transfer    1 *   960 Bytes with index=  960 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    697 cycles, achieve throughput  1652 MB/s

transfer    1 *  1024 Bytes with index= 1024 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    704 cycles, achieve throughput  1745 MB/s

transfer    1 *  1152 Bytes with index= 1152 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    814 cycles, achieve throughput  1698 MB/s

transfer    1 *  1280 Bytes with index= 1280 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    813 cycles, achieve throughput  1889 MB/s

transfer    1 *  1408 Bytes with index= 1408 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    811 cycles, achieve throughput  2083 MB/s

transfer    1 *  1536 Bytes with index= 1536 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    814 cycles, achieve throughput  2264 MB/s

transfer    1 *  1664 Bytes with index= 1664 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    812 cycles, achieve throughput  2459 MB/s

transfer    1 *  1792 Bytes with index= 1792 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    923 cycles, achieve throughput  2329 MB/s

transfer    1 *  1920 Bytes with index= 1920 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    816 cycles, achieve throughput  2823 MB/s

transfer    1 *  2048 Bytes with index= 2048 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1041 cycles, achieve throughput  2360 MB/s

transfer    1 *  2304 Bytes with index= 2304 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    927 cycles, achieve throughput  2982 MB/s

transfer    1 *  2560 Bytes with index= 2560 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes    924 cycles, achieve throughput  3324 MB/s

transfer    1 *  2816 Bytes with index= 2816 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1045 cycles, achieve throughput  3233 MB/s

transfer    1 *  3072 Bytes with index= 3072 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1043 cycles, achieve throughput  3534 MB/s

transfer    1 *  3328 Bytes with index= 3328 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1151 cycles, achieve throughput  3469 MB/s

transfer    1 *  3584 Bytes with index= 3584 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1148 cycles, achieve throughput  3746 MB/s

transfer    1 *  3840 Bytes with index= 3840 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1148 cycles, achieve throughput  4013 MB/s

transfer    1 *  4096 Bytes with index= 4096 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1374 cycles, achieve throughput  3577 MB/s

transfer    1 *  4608 Bytes with index= 4608 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1262 cycles, achieve throughput  4381 MB/s

transfer    1 *  5120 Bytes with index= 5120 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1375 cycles, achieve throughput  4468 MB/s

transfer    1 *  5632 Bytes with index= 5632 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1481 cycles, achieve throughput  4563 MB/s

transfer    1 *  6144 Bytes with index= 6144 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1486 cycles, achieve throughput  4961 MB/s

transfer    1 *  6656 Bytes with index= 6656 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1600 cycles, achieve throughput  4992 MB/s

transfer    1 *  7168 Bytes with index= 7168 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1596 cycles, achieve throughput  5389 MB/s

transfer    1 *  7680 Bytes with index= 7680 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1711 cycles, achieve throughput  5386 MB/s

transfer    1 *  8192 Bytes with index= 8192 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1817 cycles, achieve throughput  5410 MB/s

transfer    1 *  9216 Bytes with index= 9216 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   1933 cycles, achieve throughput  5721 MB/s

transfer    1 * 10240 Bytes with index=10240 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   2059 cycles, achieve throughput  5967 MB/s

transfer    1 * 11264 Bytes with index=11264 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   2269 cycles, achieve throughput  5957 MB/s

transfer    1 * 12288 Bytes with index=12288 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   2383 cycles, achieve throughput  6187 MB/s

transfer    1 * 13312 Bytes with index=13312 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   2607 cycles, achieve throughput  6127 MB/s

transfer    1 * 14336 Bytes with index=14336 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   2720 cycles, achieve throughput  6324 MB/s

transfer    1 * 15360 Bytes with index=15360 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   2838 cycles, achieve throughput  6494 MB/s

transfer    1 * 16384 Bytes with index=16384 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   3044 cycles, achieve throughput  6458 MB/s

transfer    1 * 17408 Bytes with index=17408 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   3166 cycles, achieve throughput  6598 MB/s

transfer    1 * 18432 Bytes with index=18432 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   3300 cycles, achieve throughput  6702 MB/s

transfer    1 * 19456 Bytes with index=19456 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   3500 cycles, achieve throughput  6670 MB/s

transfer    1 * 20480 Bytes with index=20480 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   4731 cycles, achieve throughput  5194 MB/s

transfer    1 * 21504 Bytes with index=21504 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   3831 cycles, achieve throughput  6735 MB/s

transfer    1 * 22528 Bytes with index=22528 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   3944 cycles, achieve throughput  6854 MB/s

transfer    1 * 23552 Bytes with index=23552 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   4085 cycles, achieve throughput  6918 MB/s

transfer    1 * 24576 Bytes with index=24576 from 0x80000000(DDR3A) to 0x c080000(MSMC RAM), consumes   4284 cycles, achieve throughput  6884 MB/s

EDMA ACNTxBCNT test with EDMA0 TC0

transfer    1 *     1 Bytes with index=    1 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput     2 MB/s

transfer    1 *     2 Bytes with index=    2 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput     4 MB/s

transfer    1 *     3 Bytes with index=    3 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    594 cycles, achieve throughput     6 MB/s

transfer    1 *     4 Bytes with index=    4 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput     8 MB/s

transfer    1 *     5 Bytes with index=    5 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    610 cycles, achieve throughput     9 MB/s

transfer    1 *     6 Bytes with index=    6 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    616 cycles, achieve throughput    11 MB/s

transfer    1 *     7 Bytes with index=    7 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    627 cycles, achieve throughput    13 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    598 cycles, achieve throughput    16 MB/s

transfer    1 *     9 Bytes with index=    9 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    597 cycles, achieve throughput    18 MB/s

transfer    1 *    10 Bytes with index=   10 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    614 cycles, achieve throughput    19 MB/s

transfer    1 *    11 Bytes with index=   11 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    595 cycles, achieve throughput    22 MB/s

transfer    1 *    12 Bytes with index=   12 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    596 cycles, achieve throughput    24 MB/s

transfer    1 *    13 Bytes with index=   13 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    610 cycles, achieve throughput    25 MB/s

transfer    1 *    14 Bytes with index=   14 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    615 cycles, achieve throughput    27 MB/s

transfer    1 *    15 Bytes with index=   15 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    613 cycles, achieve throughput    29 MB/s

transfer    1 *    16 Bytes with index=   16 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    616 cycles, achieve throughput    31 MB/s

transfer    1 *    32 Bytes with index=   32 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    712 cycles, achieve throughput    53 MB/s

transfer    1 *    48 Bytes with index=   48 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    595 cycles, achieve throughput    96 MB/s

transfer    1 *    64 Bytes with index=   64 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    596 cycles, achieve throughput   128 MB/s

transfer    1 *    80 Bytes with index=   80 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    614 cycles, achieve throughput   156 MB/s

transfer    1 *    96 Bytes with index=   96 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    611 cycles, achieve throughput   188 MB/s

transfer    1 *   112 Bytes with index=  112 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    612 cycles, achieve throughput   219 MB/s

transfer    1 *   128 Bytes with index=  128 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    617 cycles, achieve throughput   248 MB/s

transfer    1 *   144 Bytes with index=  144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1525 cycles, achieve throughput   113 MB/s

transfer    1 *   160 Bytes with index=  160 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    597 cycles, achieve throughput   321 MB/s

transfer    1 *   176 Bytes with index=  176 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    613 cycles, achieve throughput   344 MB/s

transfer    1 *   192 Bytes with index=  192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    614 cycles, achieve throughput   375 MB/s

transfer    1 *   208 Bytes with index=  208 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    614 cycles, achieve throughput   406 MB/s

transfer    1 *   224 Bytes with index=  224 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    615 cycles, achieve throughput   437 MB/s

transfer    1 *   240 Bytes with index=  240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    614 cycles, achieve throughput   469 MB/s

transfer    1 *   256 Bytes with index=  256 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    613 cycles, achieve throughput   501 MB/s

transfer    1 *   288 Bytes with index=  288 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    618 cycles, achieve throughput   559 MB/s

transfer    1 *   320 Bytes with index=  320 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    614 cycles, achieve throughput   625 MB/s

transfer    1 *   352 Bytes with index=  352 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    614 cycles, achieve throughput   687 MB/s

transfer    1 *   384 Bytes with index=  384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    613 cycles, achieve throughput   751 MB/s

transfer    1 *   416 Bytes with index=  416 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    616 cycles, achieve throughput   810 MB/s

transfer    1 *   448 Bytes with index=  448 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    613 cycles, achieve throughput   876 MB/s

transfer    1 *   480 Bytes with index=  480 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    615 cycles, achieve throughput   936 MB/s

transfer    1 *   512 Bytes with index=  512 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    614 cycles, achieve throughput  1000 MB/s

transfer    1 *   576 Bytes with index=  576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    617 cycles, achieve throughput  1120 MB/s

transfer    1 *   640 Bytes with index=  640 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    620 cycles, achieve throughput  1238 MB/s

transfer    1 *   704 Bytes with index=  704 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    615 cycles, achieve throughput  1373 MB/s

transfer    1 *   768 Bytes with index=  768 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    617 cycles, achieve throughput  1493 MB/s

transfer    1 *   832 Bytes with index=  832 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    714 cycles, achieve throughput  1398 MB/s

transfer    1 *   896 Bytes with index=  896 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    698 cycles, achieve throughput  1540 MB/s

transfer    1 *   960 Bytes with index=  960 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  1643 MB/s

transfer    1 *  1024 Bytes with index= 1024 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    704 cycles, achieve throughput  1745 MB/s

transfer    1 *  1152 Bytes with index= 1152 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    709 cycles, achieve throughput  1949 MB/s

transfer    1 *  1280 Bytes with index= 1280 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    748 cycles, achieve throughput  2053 MB/s

transfer    1 *  1408 Bytes with index= 1408 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput  2406 MB/s

transfer    1 *  1536 Bytes with index= 1536 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput  2633 MB/s

transfer    1 *  1664 Bytes with index= 1664 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    813 cycles, achieve throughput  2456 MB/s

transfer    1 *  1792 Bytes with index= 1792 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    812 cycles, achieve throughput  2648 MB/s

transfer    1 *  1920 Bytes with index= 1920 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    817 cycles, achieve throughput  2820 MB/s

transfer    1 *  2048 Bytes with index= 2048 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    813 cycles, achieve throughput  3022 MB/s

transfer    1 *  2304 Bytes with index= 2304 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    818 cycles, achieve throughput  3379 MB/s

transfer    1 *  2560 Bytes with index= 2560 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    933 cycles, achieve throughput  3292 MB/s

transfer    1 *  2816 Bytes with index= 2816 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    953 cycles, achieve throughput  3545 MB/s

transfer    1 *  3072 Bytes with index= 3072 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    961 cycles, achieve throughput  3836 MB/s

transfer    1 *  3328 Bytes with index= 3328 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    952 cycles, achieve throughput  4194 MB/s

transfer    1 *  3584 Bytes with index= 3584 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1063 cycles, achieve throughput  4045 MB/s

transfer    1 *  3840 Bytes with index= 3840 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1062 cycles, achieve throughput  4338 MB/s

transfer    1 *  4096 Bytes with index= 4096 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1041 cycles, achieve throughput  4721 MB/s

transfer    1 *  4608 Bytes with index= 4608 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1169 cycles, achieve throughput  4730 MB/s

transfer    1 *  5120 Bytes with index= 5120 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1172 cycles, achieve throughput  5242 MB/s

transfer    1 *  5632 Bytes with index= 5632 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1281 cycles, achieve throughput  5275 MB/s

transfer    1 *  6144 Bytes with index= 6144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1414 cycles, achieve throughput  5214 MB/s

transfer    1 *  6656 Bytes with index= 6656 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2426 cycles, achieve throughput  3292 MB/s

transfer    1 *  7168 Bytes with index= 7168 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1523 cycles, achieve throughput  5647 MB/s

transfer    1 *  7680 Bytes with index= 7680 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1491 cycles, achieve throughput  6181 MB/s

transfer    1 *  8192 Bytes with index= 8192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1603 cycles, achieve throughput  6132 MB/s

transfer    1 *  9216 Bytes with index= 9216 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1632 cycles, achieve throughput  6776 MB/s

transfer    1 * 10240 Bytes with index=10240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1840 cycles, achieve throughput  6678 MB/s

transfer    1 * 11264 Bytes with index=11264 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1963 cycles, achieve throughput  6885 MB/s

transfer    1 * 12288 Bytes with index=12288 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2074 cycles, achieve throughput  7109 MB/s

transfer    1 * 13312 Bytes with index=13312 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2171 cycles, achieve throughput  7358 MB/s

transfer    1 * 14336 Bytes with index=14336 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2310 cycles, achieve throughput  7447 MB/s

transfer    1 * 15360 Bytes with index=15360 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2406 cycles, achieve throughput  7660 MB/s

transfer    1 * 16384 Bytes with index=16384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2623 cycles, achieve throughput  7495 MB/s

transfer    1 * 17408 Bytes with index=17408 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2648 cycles, achieve throughput  7888 MB/s

transfer    1 * 18432 Bytes with index=18432 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2769 cycles, achieve throughput  7987 MB/s

transfer    1 * 19456 Bytes with index=19456 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2918 cycles, achieve throughput  8001 MB/s

transfer    1 * 20480 Bytes with index=20480 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2988 cycles, achieve throughput  8224 MB/s

transfer    1 * 21504 Bytes with index=21504 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3129 cycles, achieve throughput  8246 MB/s

transfer    1 * 22528 Bytes with index=22528 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3310 cycles, achieve throughput  8167 MB/s

transfer    1 * 23552 Bytes with index=23552 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3351 cycles, achieve throughput  8434 MB/s

transfer    1 * 24576 Bytes with index=24576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3572 cycles, achieve throughput  8256 MB/s

transfer    2 *     1 Bytes with index=    1 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput     4 MB/s

transfer    2 *     2 Bytes with index=    2 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    592 cycles, achieve throughput     8 MB/s

transfer    2 *     3 Bytes with index=    3 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    588 cycles, achieve throughput    12 MB/s

transfer    2 *     4 Bytes with index=    4 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput    16 MB/s

transfer    2 *     5 Bytes with index=    5 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput    20 MB/s

transfer    2 *     6 Bytes with index=    6 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    588 cycles, achieve throughput    24 MB/s

transfer    2 *     7 Bytes with index=    7 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput    28 MB/s

transfer    2 *     8 Bytes with index=    8 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput    32 MB/s

transfer    2 *     9 Bytes with index=    9 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    588 cycles, achieve throughput    36 MB/s

transfer    2 *    10 Bytes with index=   10 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput    40 MB/s

transfer    2 *    11 Bytes with index=   11 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput    44 MB/s

transfer    2 *    12 Bytes with index=   12 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput    48 MB/s

transfer    2 *    13 Bytes with index=   13 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput    52 MB/s

transfer    2 *    14 Bytes with index=   14 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput    56 MB/s

transfer    2 *    15 Bytes with index=   15 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput    61 MB/s

transfer    2 *    16 Bytes with index=   16 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput    64 MB/s

transfer    2 *    32 Bytes with index=   32 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput   130 MB/s

transfer    2 *    48 Bytes with index=   48 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    596 cycles, achieve throughput   193 MB/s

transfer    2 *    64 Bytes with index=   64 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput   259 MB/s

transfer    2 *    80 Bytes with index=   80 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput   273 MB/s

transfer    2 *    96 Bytes with index=   96 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput   328 MB/s

transfer    2 *   112 Bytes with index=  112 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput   383 MB/s

transfer    2 *   128 Bytes with index=  128 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    592 cycles, achieve throughput   518 MB/s

transfer    2 *   144 Bytes with index=  144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput   491 MB/s

transfer    2 *   160 Bytes with index=  160 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    812 cycles, achieve throughput   472 MB/s

transfer    2 *   176 Bytes with index=  176 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1034 cycles, achieve throughput   408 MB/s

transfer    2 *   192 Bytes with index=  192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput   782 MB/s

transfer    2 *   208 Bytes with index=  208 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    704 cycles, achieve throughput   709 MB/s

transfer    2 *   224 Bytes with index=  224 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    704 cycles, achieve throughput   763 MB/s

transfer    2 *   240 Bytes with index=  240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput   821 MB/s

transfer    2 *   256 Bytes with index=  256 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput  1043 MB/s

transfer    2 *   288 Bytes with index=  288 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    812 cycles, achieve throughput   851 MB/s

transfer    2 *   320 Bytes with index=  320 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    704 cycles, achieve throughput  1090 MB/s

transfer    2 *   352 Bytes with index=  352 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput  1206 MB/s

transfer    2 *   384 Bytes with index=  384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput  1310 MB/s

transfer    2 *   416 Bytes with index=  416 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  1424 MB/s

transfer    2 *   448 Bytes with index=  448 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  1533 MB/s

transfer    2 *   480 Bytes with index=  480 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    705 cycles, achieve throughput  1634 MB/s

transfer    2 *   512 Bytes with index=  512 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput  1747 MB/s

transfer    2 *   576 Bytes with index=  576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput  1969 MB/s

transfer    2 *   640 Bytes with index=  640 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput  2188 MB/s

transfer    2 *   704 Bytes with index=  704 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  2410 MB/s

transfer    2 *   768 Bytes with index=  768 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput  2625 MB/s

transfer    2 *   832 Bytes with index=  832 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    812 cycles, achieve throughput  2459 MB/s

transfer    2 *   896 Bytes with index=  896 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    813 cycles, achieve throughput  2645 MB/s

transfer    2 *   960 Bytes with index=  960 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    812 cycles, achieve throughput  2837 MB/s

transfer    2 *  1024 Bytes with index= 1024 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    815 cycles, achieve throughput  3015 MB/s

transfer    2 *  1152 Bytes with index= 1152 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    812 cycles, achieve throughput  3404 MB/s

transfer    2 *  1280 Bytes with index= 1280 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    929 cycles, achieve throughput  3306 MB/s

transfer    2 *  1408 Bytes with index= 1408 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    919 cycles, achieve throughput  3677 MB/s

transfer    2 *  1536 Bytes with index= 1536 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    923 cycles, achieve throughput  3993 MB/s

transfer    2 *  1664 Bytes with index= 1664 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1034 cycles, achieve throughput  3862 MB/s

transfer    2 *  1792 Bytes with index= 1792 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1151 cycles, achieve throughput  3736 MB/s

transfer    2 *  1920 Bytes with index= 1920 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1030 cycles, achieve throughput  4473 MB/s

transfer    2 *  2048 Bytes with index= 2048 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1035 cycles, achieve throughput  4748 MB/s

transfer    2 *  2304 Bytes with index= 2304 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1150 cycles, achieve throughput  4808 MB/s

transfer    2 *  2560 Bytes with index= 2560 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1149 cycles, achieve throughput  5347 MB/s

transfer    2 *  2816 Bytes with index= 2816 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1269 cycles, achieve throughput  5325 MB/s

transfer    2 *  3072 Bytes with index= 3072 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1376 cycles, achieve throughput  5358 MB/s

transfer    2 *  3328 Bytes with index= 3328 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1393 cycles, achieve throughput  5733 MB/s

transfer    2 *  3584 Bytes with index= 3584 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1387 cycles, achieve throughput  6201 MB/s

transfer    2 *  3840 Bytes with index= 3840 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1492 cycles, achieve throughput  6176 MB/s

transfer    2 *  4096 Bytes with index= 4096 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1602 cycles, achieve throughput  6136 MB/s

transfer    2 *  4608 Bytes with index= 4608 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1718 cycles, achieve throughput  6437 MB/s

transfer    2 *  5120 Bytes with index= 5120 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1837 cycles, achieve throughput  6689 MB/s

transfer    2 *  5632 Bytes with index= 5632 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1951 cycles, achieve throughput  6928 MB/s

transfer    2 *  6144 Bytes with index= 6144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2066 cycles, achieve throughput  7137 MB/s

transfer    2 *  6656 Bytes with index= 6656 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2164 cycles, achieve throughput  7381 MB/s

transfer    2 *  7168 Bytes with index= 7168 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2293 cycles, achieve throughput  7502 MB/s

transfer    2 *  7680 Bytes with index= 7680 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2442 cycles, achieve throughput  7547 MB/s

transfer    2 *  8192 Bytes with index= 8192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2520 cycles, achieve throughput  7801 MB/s

transfer    2 *  9216 Bytes with index= 9216 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2854 cycles, achieve throughput  7749 MB/s

transfer    2 * 10240 Bytes with index=10240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3091 cycles, achieve throughput  7950 MB/s

transfer    2 * 11264 Bytes with index=11264 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3302 cycles, achieve throughput  8187 MB/s

transfer    2 * 12288 Bytes with index=12288 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3561 cycles, achieve throughput  8281 MB/s

transfer    2 * 13312 Bytes with index=13312 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3751 cycles, achieve throughput  8517 MB/s

transfer    2 * 14336 Bytes with index=14336 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   4000 cycles, achieve throughput  8601 MB/s

transfer    2 * 15360 Bytes with index=15360 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   4263 cycles, achieve throughput  8647 MB/s

transfer    2 * 16384 Bytes with index=16384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   4566 cycles, achieve throughput  8611 MB/s

transfer    2 * 17408 Bytes with index=17408 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   4801 cycles, achieve throughput  8702 MB/s

transfer    2 * 18432 Bytes with index=18432 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   5000 cycles, achieve throughput  8847 MB/s

transfer    2 * 19456 Bytes with index=19456 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   5232 cycles, achieve throughput  8924 MB/s

transfer    2 * 20480 Bytes with index=20480 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   5500 cycles, achieve throughput  8936 MB/s

transfer    2 * 21504 Bytes with index=21504 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   5801 cycles, achieve throughput  8896 MB/s

transfer    2 * 22528 Bytes with index=22528 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   6057 cycles, achieve throughput  8926 MB/s

transfer    2 * 23552 Bytes with index=23552 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   6287 cycles, achieve throughput  8990 MB/s

transfer    2 * 24576 Bytes with index=24576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   6437 cycles, achieve throughput  9163 MB/s

transfer    3 *     1 Bytes with index=    1 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput     6 MB/s

transfer    3 *     2 Bytes with index=    2 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput    12 MB/s

transfer    3 *     3 Bytes with index=    3 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput    15 MB/s

transfer    3 *     4 Bytes with index=    4 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    593 cycles, achieve throughput    24 MB/s

transfer    3 *     5 Bytes with index=    5 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput    25 MB/s

transfer    3 *     6 Bytes with index=    6 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput    30 MB/s

transfer    3 *     7 Bytes with index=    7 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput    35 MB/s

transfer    3 *     8 Bytes with index=    8 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    593 cycles, achieve throughput    48 MB/s

transfer    3 *     9 Bytes with index=    9 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput    46 MB/s

transfer    3 *    10 Bytes with index=   10 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput    51 MB/s

transfer    3 *    11 Bytes with index=   11 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    704 cycles, achieve throughput    56 MB/s

transfer    3 *    12 Bytes with index=   12 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput    61 MB/s

transfer    3 *    13 Bytes with index=   13 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput    66 MB/s

transfer    3 *    14 Bytes with index=   14 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput    72 MB/s

transfer    3 *    15 Bytes with index=   15 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    699 cycles, achieve throughput    77 MB/s

transfer    3 *    16 Bytes with index=   16 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput    97 MB/s

transfer    3 *    32 Bytes with index=   32 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    594 cycles, achieve throughput   193 MB/s

transfer    3 *    48 Bytes with index=   48 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput   245 MB/s

transfer    3 *    64 Bytes with index=   64 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput   390 MB/s

transfer    3 *    80 Bytes with index=   80 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput   411 MB/s

transfer    3 *    96 Bytes with index=   96 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    705 cycles, achieve throughput   490 MB/s

transfer    3 *   112 Bytes with index=  112 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput   575 MB/s

transfer    3 *   128 Bytes with index=  128 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    592 cycles, achieve throughput   778 MB/s

transfer    3 *   144 Bytes with index=  144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput   738 MB/s

transfer    3 *   160 Bytes with index=  160 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput   819 MB/s

transfer    3 *   176 Bytes with index=  176 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput   903 MB/s

transfer    3 *   192 Bytes with index=  192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    705 cycles, achieve throughput   980 MB/s

transfer    3 *   208 Bytes with index=  208 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  1068 MB/s

transfer    3 *   224 Bytes with index=  224 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  1150 MB/s

transfer    3 *   240 Bytes with index=  240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput  1230 MB/s

transfer    3 *   256 Bytes with index=  256 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput  1310 MB/s

transfer    3 *   288 Bytes with index=  288 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput  1474 MB/s

transfer    3 *   320 Bytes with index=  320 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  1643 MB/s

transfer    3 *   352 Bytes with index=  352 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    705 cycles, achieve throughput  1797 MB/s

transfer    3 *   384 Bytes with index=  384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    698 cycles, achieve throughput  1980 MB/s

transfer    3 *   416 Bytes with index=  416 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    816 cycles, achieve throughput  1835 MB/s

transfer    3 *   448 Bytes with index=  448 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  2300 MB/s

transfer    3 *   480 Bytes with index=  480 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    821 cycles, achieve throughput  2104 MB/s

transfer    3 *   512 Bytes with index=  512 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    813 cycles, achieve throughput  2267 MB/s

transfer    3 *   576 Bytes with index=  576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    813 cycles, achieve throughput  2550 MB/s

transfer    3 *   640 Bytes with index=  640 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    814 cycles, achieve throughput  2830 MB/s

transfer    3 *   704 Bytes with index=  704 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    812 cycles, achieve throughput  3121 MB/s

transfer    3 *   768 Bytes with index=  768 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    812 cycles, achieve throughput  3404 MB/s

transfer    3 *   832 Bytes with index=  832 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    926 cycles, achieve throughput  3234 MB/s

transfer    3 *   896 Bytes with index=  896 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    926 cycles, achieve throughput  3483 MB/s

transfer    3 *   960 Bytes with index=  960 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    930 cycles, achieve throughput  3716 MB/s

transfer    3 *  1024 Bytes with index= 1024 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    922 cycles, achieve throughput  3998 MB/s

transfer    3 *  1152 Bytes with index= 1152 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1374 cycles, achieve throughput  3018 MB/s

transfer    3 *  1280 Bytes with index= 1280 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1043 cycles, achieve throughput  4418 MB/s

transfer    3 *  1408 Bytes with index= 1408 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1149 cycles, achieve throughput  4411 MB/s

transfer    3 *  1536 Bytes with index= 1536 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1150 cycles, achieve throughput  4808 MB/s

transfer    3 *  1664 Bytes with index= 1664 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1149 cycles, achieve throughput  5213 MB/s

transfer    3 *  1792 Bytes with index= 1792 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1265 cycles, achieve throughput  5099 MB/s

transfer    3 *  1920 Bytes with index= 1920 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1264 cycles, achieve throughput  5468 MB/s

transfer    3 *  2048 Bytes with index= 2048 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1268 cycles, achieve throughput  5814 MB/s

transfer    3 *  2304 Bytes with index= 2304 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1378 cycles, achieve throughput  6019 MB/s

transfer    3 *  2560 Bytes with index= 2560 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1501 cycles, achieve throughput  6139 MB/s

transfer    3 *  2816 Bytes with index= 2816 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1599 cycles, achieve throughput  6339 MB/s

transfer    3 *  3072 Bytes with index= 3072 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1715 cycles, achieve throughput  6448 MB/s

transfer    3 *  3328 Bytes with index= 3328 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1836 cycles, achieve throughput  6525 MB/s

transfer    3 *  3584 Bytes with index= 3584 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1840 cycles, achieve throughput  7012 MB/s

transfer    3 *  3840 Bytes with index= 3840 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1947 cycles, achieve throughput  7100 MB/s

transfer    3 *  4096 Bytes with index= 4096 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2070 cycles, achieve throughput  7123 MB/s

transfer    3 *  4608 Bytes with index= 4608 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2268 cycles, achieve throughput  7314 MB/s

transfer    3 *  5120 Bytes with index= 5120 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2504 cycles, achieve throughput  7361 MB/s

transfer    3 *  5632 Bytes with index= 5632 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2626 cycles, achieve throughput  7720 MB/s

transfer    3 *  6144 Bytes with index= 6144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2749 cycles, achieve throughput  8045 MB/s

transfer    3 *  6656 Bytes with index= 6656 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   4052 cycles, achieve throughput  5913 MB/s

transfer    3 *  7168 Bytes with index= 7168 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3199 cycles, achieve throughput  8066 MB/s

transfer    3 *  7680 Bytes with index= 7680 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3345 cycles, achieve throughput  8265 MB/s

transfer    3 *  8192 Bytes with index= 8192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3467 cycles, achieve throughput  8506 MB/s

transfer    3 *  9216 Bytes with index= 9216 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3932 cycles, achieve throughput  8437 MB/s

transfer    3 * 10240 Bytes with index=10240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   4247 cycles, achieve throughput  8680 MB/s

transfer    3 * 11264 Bytes with index=11264 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   4598 cycles, achieve throughput  8819 MB/s

transfer    3 * 12288 Bytes with index=12288 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   5071 cycles, achieve throughput  8723 MB/s

transfer    3 * 13312 Bytes with index=13312 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   5309 cycles, achieve throughput  9026 MB/s

transfer    3 * 14336 Bytes with index=14336 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   5933 cycles, achieve throughput  8698 MB/s

transfer    3 * 15360 Bytes with index=15360 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   6158 cycles, achieve throughput  8979 MB/s

transfer    3 * 16384 Bytes with index=16384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   6437 cycles, achieve throughput  9163 MB/s

transfer    3 * 17408 Bytes with index=17408 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   6814 cycles, achieve throughput  9197 MB/s

transfer    3 * 18432 Bytes with index=18432 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   7295 cycles, achieve throughput  9095 MB/s

transfer    3 * 19456 Bytes with index=19456 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   7560 cycles, achieve throughput  9264 MB/s

transfer    3 * 20480 Bytes with index=20480 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   7982 cycles, achieve throughput  9236 MB/s

transfer    3 * 21504 Bytes with index=21504 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   8328 cycles, achieve throughput  9295 MB/s

transfer    3 * 22528 Bytes with index=22528 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   8742 cycles, achieve throughput  9277 MB/s

transfer    3 * 23552 Bytes with index=23552 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   9086 cycles, achieve throughput  9331 MB/s

transfer    3 * 24576 Bytes with index=24576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   9438 cycles, achieve throughput  9374 MB/s

transfer    4 *     1 Bytes with index=    1 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput     8 MB/s

transfer    4 *     2 Bytes with index=    2 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    593 cycles, achieve throughput    16 MB/s

transfer    4 *     3 Bytes with index=    3 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput    20 MB/s

transfer    4 *     4 Bytes with index=    4 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    591 cycles, achieve throughput    32 MB/s

transfer    4 *     5 Bytes with index=    5 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    699 cycles, achieve throughput    34 MB/s

transfer    4 *     6 Bytes with index=    6 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput    41 MB/s

transfer    4 *     7 Bytes with index=    7 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    707 cycles, achieve throughput    47 MB/s

transfer    4 *     8 Bytes with index=    8 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput    65 MB/s

transfer    4 *     9 Bytes with index=    9 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    923 cycles, achieve throughput    46 MB/s

transfer    4 *    10 Bytes with index=   10 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    705 cycles, achieve throughput    68 MB/s

transfer    4 *    11 Bytes with index=   11 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput    75 MB/s

transfer    4 *    12 Bytes with index=   12 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    925 cycles, achieve throughput    62 MB/s

transfer    4 *    13 Bytes with index=   13 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput    88 MB/s

transfer    4 *    14 Bytes with index=   14 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput    96 MB/s

transfer    4 *    15 Bytes with index=   15 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput   102 MB/s

transfer    4 *    16 Bytes with index=   16 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    812 cycles, achieve throughput    94 MB/s

transfer    4 *    32 Bytes with index=   32 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    593 cycles, achieve throughput   259 MB/s

transfer    4 *    48 Bytes with index=   48 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput   328 MB/s

transfer    4 *    64 Bytes with index=   64 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    589 cycles, achieve throughput   521 MB/s

transfer    4 *    80 Bytes with index=   80 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput   547 MB/s

transfer    4 *    96 Bytes with index=   96 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput   658 MB/s

transfer    4 *   112 Bytes with index=  112 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    702 cycles, achieve throughput   765 MB/s

transfer    4 *   128 Bytes with index=  128 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    590 cycles, achieve throughput  1041 MB/s

transfer    4 *   144 Bytes with index=  144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    813 cycles, achieve throughput   850 MB/s

transfer    4 *   160 Bytes with index=  160 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput  1092 MB/s

transfer    4 *   176 Bytes with index=  176 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    817 cycles, achieve throughput  1034 MB/s

transfer    4 *   192 Bytes with index=  192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    703 cycles, achieve throughput  1310 MB/s

transfer    4 *   208 Bytes with index=  208 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    813 cycles, achieve throughput  1228 MB/s

transfer    4 *   224 Bytes with index=  224 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    814 cycles, achieve throughput  1320 MB/s

transfer    4 *   240 Bytes with index=  240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    811 cycles, achieve throughput  1420 MB/s

transfer    4 *   256 Bytes with index=  256 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    700 cycles, achieve throughput  1755 MB/s

transfer    4 *   288 Bytes with index=  288 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    815 cycles, achieve throughput  1696 MB/s

transfer    4 *   320 Bytes with index=  320 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  2191 MB/s

transfer    4 *   352 Bytes with index=  352 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    818 cycles, achieve throughput  2065 MB/s

transfer    4 *   384 Bytes with index=  384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  2629 MB/s

transfer    4 *   416 Bytes with index=  416 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    820 cycles, achieve throughput  2435 MB/s

transfer    4 *   448 Bytes with index=  448 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    815 cycles, achieve throughput  2638 MB/s

transfer    4 *   480 Bytes with index=  480 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    934 cycles, achieve throughput  2466 MB/s

transfer    4 *   512 Bytes with index=  512 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1037 cycles, achieve throughput  2369 MB/s

transfer    4 *   576 Bytes with index=  576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    814 cycles, achieve throughput  3396 MB/s

transfer    4 *   640 Bytes with index=  640 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    924 cycles, achieve throughput  3324 MB/s

transfer    4 *   704 Bytes with index=  704 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    929 cycles, achieve throughput  3637 MB/s

transfer    4 *   768 Bytes with index=  768 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    924 cycles, achieve throughput  3989 MB/s

transfer    4 *   832 Bytes with index=  832 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    933 cycles, achieve throughput  4280 MB/s

transfer    4 *   896 Bytes with index=  896 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1042 cycles, achieve throughput  4127 MB/s

transfer    4 *   960 Bytes with index=  960 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1035 cycles, achieve throughput  4452 MB/s

transfer    4 *  1024 Bytes with index= 1024 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1033 cycles, achieve throughput  4758 MB/s

transfer    4 *  1152 Bytes with index= 1152 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1173 cycles, achieve throughput  4714 MB/s

transfer    4 *  1280 Bytes with index= 1280 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1158 cycles, achieve throughput  5305 MB/s

transfer    4 *  1408 Bytes with index= 1408 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1267 cycles, achieve throughput  5334 MB/s

transfer    4 *  1536 Bytes with index= 1536 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1372 cycles, achieve throughput  5373 MB/s

transfer    4 *  1664 Bytes with index= 1664 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1390 cycles, achieve throughput  5746 MB/s

transfer    4 *  1792 Bytes with index= 1792 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1495 cycles, achieve throughput  5753 MB/s

transfer    4 *  1920 Bytes with index= 1920 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1484 cycles, achieve throughput  6210 MB/s

transfer    4 *  2048 Bytes with index= 2048 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1602 cycles, achieve throughput  6136 MB/s

transfer    4 *  2304 Bytes with index= 2304 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1711 cycles, achieve throughput  6463 MB/s

transfer    4 *  2560 Bytes with index= 2560 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1819 cycles, achieve throughput  6755 MB/s

transfer    4 *  2816 Bytes with index= 2816 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1947 cycles, achieve throughput  6942 MB/s

transfer    4 *  3072 Bytes with index= 3072 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2054 cycles, achieve throughput  7178 MB/s

transfer    4 *  3328 Bytes with index= 3328 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2170 cycles, achieve throughput  7361 MB/s

transfer    4 *  3584 Bytes with index= 3584 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2292 cycles, achieve throughput  7505 MB/s

transfer    4 *  3840 Bytes with index= 3840 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2407 cycles, achieve throughput  7657 MB/s

transfer    4 *  4096 Bytes with index= 4096 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2522 cycles, achieve throughput  7795 MB/s

transfer    4 *  4608 Bytes with index= 4608 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2851 cycles, achieve throughput  7758 MB/s

transfer    4 *  5120 Bytes with index= 5120 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3065 cycles, achieve throughput  8018 MB/s

transfer    4 *  5632 Bytes with index= 5632 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3300 cycles, achieve throughput  8192 MB/s

transfer    4 *  6144 Bytes with index= 6144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3622 cycles, achieve throughput  8142 MB/s

transfer    4 *  6656 Bytes with index= 6656 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   3754 cycles, achieve throughput  8510 MB/s

transfer    4 *  7168 Bytes with index= 7168 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   4038 cycles, achieve throughput  8520 MB/s

transfer    4 *  7680 Bytes with index= 7680 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   4229 cycles, achieve throughput  8716 MB/s

transfer    4 *  8192 Bytes with index= 8192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   4631 cycles, achieve throughput  8490 MB/s

transfer    4 *  9216 Bytes with index= 9216 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   5022 cycles, achieve throughput  8808 MB/s

transfer    4 * 10240 Bytes with index=10240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   5513 cycles, achieve throughput  8915 MB/s

transfer    4 * 11264 Bytes with index=11264 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   6073 cycles, achieve throughput  8902 MB/s

transfer    4 * 12288 Bytes with index=12288 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   6557 cycles, achieve throughput  8995 MB/s

transfer    4 * 13312 Bytes with index=13312 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   7070 cycles, achieve throughput  9037 MB/s

transfer    4 * 14336 Bytes with index=14336 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   7433 cycles, achieve throughput  9257 MB/s

transfer    4 * 15360 Bytes with index=15360 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   7985 cycles, achieve throughput  9233 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   8504 cycles, achieve throughput  9247 MB/s

transfer    4 * 17408 Bytes with index=17408 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   9015 cycles, achieve throughput  9268 MB/s

transfer    4 * 18432 Bytes with index=18432 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   9419 cycles, achieve throughput  9393 MB/s

transfer    4 * 19456 Bytes with index=19456 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   9977 cycles, achieve throughput  9360 MB/s

transfer    4 * 20480 Bytes with index=20480 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  10535 cycles, achieve throughput  9331 MB/s

transfer    4 * 21504 Bytes with index=21504 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  10875 cycles, achieve throughput  9491 MB/s

transfer    4 * 22528 Bytes with index=22528 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  11441 cycles, achieve throughput  9451 MB/s

transfer    4 * 23552 Bytes with index=23552 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  11928 cycles, achieve throughput  9477 MB/s

transfer    4 * 24576 Bytes with index=24576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  12448 cycles, achieve throughput  9476 MB/s

EDMA Index test with EDMA0 TC0

transfer 1024 *     1 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32613 cycles, achieve throughput    37 MB/s

transfer 1024 *     2 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32306 cycles, achieve throughput    76 MB/s

transfer 1024 *     3 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33197 cycles, achieve throughput   111 MB/s

transfer 1024 *     4 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32085 cycles, achieve throughput   153 MB/s

transfer 1024 *     5 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33198 cycles, achieve throughput   185 MB/s

transfer 1024 *     6 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33088 cycles, achieve throughput   222 MB/s

transfer 1024 *     7 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32054 cycles, achieve throughput   268 MB/s

transfer 1024 *     8 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32054 cycles, achieve throughput   306 MB/s

transfer 1024 *     9 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31832 cycles, achieve throughput   347 MB/s

transfer 1024 *    10 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31832 cycles, achieve throughput   386 MB/s

transfer 1024 *    11 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33057 cycles, achieve throughput   408 MB/s

transfer 1024 *    12 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31990 cycles, achieve throughput   460 MB/s

transfer 1024 *    13 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33091 cycles, achieve throughput   482 MB/s

transfer 1024 *    14 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31751 cycles, achieve throughput   541 MB/s

transfer 1024 *    15 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31973 cycles, achieve throughput   576 MB/s

transfer 1024 *    16 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31982 cycles, achieve throughput   614 MB/s

transfer 1024 *    32 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31972 cycles, achieve throughput  1229 MB/s

transfer 1024 *    48 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33318 cycles, achieve throughput  1770 MB/s

transfer 1024 *    64 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  35674 cycles, achieve throughput  2204 MB/s

transfer 1024 *    80 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33437 cycles, achieve throughput  2939 MB/s

transfer 1024 *    96 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  34428 cycles, achieve throughput  3426 MB/s

transfer 1024 *   112 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  34487 cycles, achieve throughput  3990 MB/s

transfer 1024 *   128 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  35789 cycles, achieve throughput  4394 MB/s

transfer 1024 *   144 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  34466 cycles, achieve throughput  5133 MB/s

transfer 1024 *   160 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  34178 cycles, achieve throughput  5752 MB/s

transfer 1024 *   176 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  34428 cycles, achieve throughput  6281 MB/s

transfer 1024 *   192 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  34195 cycles, achieve throughput  6899 MB/s

transfer 1024 *   208 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  37343 cycles, achieve throughput  6844 MB/s

transfer 1024 *   224 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  35757 cycles, achieve throughput  7697 MB/s

transfer 1024 *   240 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  36390 cycles, achieve throughput  8104 MB/s

transfer 1024 *   256 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  36783 cycles, achieve throughput  8552 MB/s

transfer 1024 *   320 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  42134 cycles, achieve throughput  9332 MB/s

transfer 1024 *   384 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  48631 cycles, achieve throughput  9702 MB/s

transfer 1024 *   448 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  60292 cycles, achieve throughput  9130 MB/s

transfer 1024 *   512 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  70004 cycles, achieve throughput  8987 MB/s

transfer 1024 *   768 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 102464 cycles, achieve throughput  9210 MB/s

transfer 1024 *  1024 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 137334 cycles, achieve throughput  9162 MB/s

transfer  682 *  1536 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 135480 cycles, achieve throughput  9278 MB/s

transfer  512 *  2048 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 133136 cycles, achieve throughput  9451 MB/s

transfer  409 *  2560 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 133045 cycles, achieve throughput  9443 MB/s

transfer  341 *  3072 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 131460 cycles, achieve throughput  9562 MB/s

transfer  292 *  3584 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132952 cycles, achieve throughput  9445 MB/s

transfer  256 *  4096 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132253 cycles, achieve throughput  9514 MB/s

transfer  227 *  4608 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 134526 cycles, achieve throughput  9330 MB/s

transfer  204 *  5120 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 131756 cycles, achieve throughput  9512 MB/s

transfer  186 *  5632 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 133602 cycles, achieve throughput  9409 MB/s

transfer  170 *  6144 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 133436 cycles, achieve throughput  9393 MB/s

transfer  157 *  6656 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 133187 cycles, achieve throughput  9415 MB/s

transfer  146 *  7168 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132697 cycles, achieve throughput  9463 MB/s

transfer  136 *  7680 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132155 cycles, achieve throughput  9484 MB/s

transfer  128 *  8192 Bytes with index=    0 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132564 cycles, achieve throughput  9491 MB/s

transfer 1024 *     1 Bytes with index=    1 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    701 cycles, achieve throughput  1752 MB/s

transfer 1024 *     2 Bytes with index=    2 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes    814 cycles, achieve throughput  3019 MB/s

transfer 1024 *     3 Bytes with index=    3 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32218 cycles, achieve throughput   114 MB/s

transfer 1024 *     4 Bytes with index=    4 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1053 cycles, achieve throughput  4667 MB/s

transfer 1024 *     5 Bytes with index=    5 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33447 cycles, achieve throughput   183 MB/s

transfer 1024 *     6 Bytes with index=    6 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31879 cycles, achieve throughput   231 MB/s

transfer 1024 *     7 Bytes with index=    7 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33684 cycles, achieve throughput   255 MB/s

transfer 1024 *     8 Bytes with index=    8 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   1601 cycles, achieve throughput  6140 MB/s

transfer 1024 *     9 Bytes with index=    9 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33904 cycles, achieve throughput   326 MB/s

transfer 1024 *    10 Bytes with index=   10 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32436 cycles, achieve throughput   378 MB/s

transfer 1024 *    11 Bytes with index=   11 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32993 cycles, achieve throughput   409 MB/s

transfer 1024 *    12 Bytes with index=   12 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31790 cycles, achieve throughput   463 MB/s

transfer 1024 *    13 Bytes with index=   13 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33459 cycles, achieve throughput   477 MB/s

transfer 1024 *    14 Bytes with index=   14 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  34998 cycles, achieve throughput   491 MB/s

transfer 1024 *    15 Bytes with index=   15 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33468 cycles, achieve throughput   550 MB/s

transfer 1024 *    16 Bytes with index=   16 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   2522 cycles, achieve throughput  7795 MB/s

transfer 1024 *    32 Bytes with index=   32 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   4541 cycles, achieve throughput  8659 MB/s

transfer 1024 *    48 Bytes with index=   48 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  34229 cycles, achieve throughput  1723 MB/s

transfer 1024 *    64 Bytes with index=   64 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes   8517 cycles, achieve throughput  9233 MB/s

transfer 1024 *    80 Bytes with index=   80 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  37350 cycles, achieve throughput  2631 MB/s

transfer 1024 *    96 Bytes with index=   96 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  30282 cycles, achieve throughput  3895 MB/s

transfer 1024 *   112 Bytes with index=  112 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  42162 cycles, achieve throughput  3264 MB/s

transfer 1024 *   128 Bytes with index=  128 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  17411 cycles, achieve throughput  9033 MB/s

transfer 1024 *   144 Bytes with index=  144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  49174 cycles, achieve throughput  3598 MB/s

transfer 1024 *   160 Bytes with index=  160 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  43120 cycles, achieve throughput  4559 MB/s

transfer 1024 *   176 Bytes with index=  176 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  53258 cycles, achieve throughput  4060 MB/s

transfer 1024 *   192 Bytes with index=  192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  28505 cycles, achieve throughput  8276 MB/s

transfer 1024 *   208 Bytes with index=  208 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  59160 cycles, achieve throughput  4320 MB/s

transfer 1024 *   224 Bytes with index=  224 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  51852 cycles, achieve throughput  5308 MB/s

transfer 1024 *   240 Bytes with index=  240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  64143 cycles, achieve throughput  4597 MB/s

transfer 1024 *   256 Bytes with index=  256 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  35735 cycles, achieve throughput  8802 MB/s

transfer 1024 *   320 Bytes with index=  320 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  43952 cycles, achieve throughput  8946 MB/s

transfer 1024 *   384 Bytes with index=  384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  52583 cycles, achieve throughput  8973 MB/s

transfer 1024 *   448 Bytes with index=  448 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  58483 cycles, achieve throughput  9413 MB/s

transfer 1024 *   512 Bytes with index=  512 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  67947 cycles, achieve throughput  9259 MB/s

transfer 1024 *   768 Bytes with index=  768 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 101096 cycles, achieve throughput  9334 MB/s

transfer 1024 *  1024 Bytes with index= 1024 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 135288 cycles, achieve throughput  9300 MB/s

transfer  682 *  1536 Bytes with index= 1536 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 133744 cycles, achieve throughput  9399 MB/s

transfer  512 *  2048 Bytes with index= 2048 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 134197 cycles, achieve throughput  9376 MB/s

transfer  409 *  2560 Bytes with index= 2560 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132954 cycles, achieve throughput  9450 MB/s

transfer  341 *  3072 Bytes with index= 3072 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132140 cycles, achieve throughput  9513 MB/s

transfer  292 *  3584 Bytes with index= 3584 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132750 cycles, achieve throughput  9460 MB/s

transfer  256 *  4096 Bytes with index= 4096 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132052 cycles, achieve throughput  9528 MB/s

transfer  227 *  4608 Bytes with index= 4608 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132347 cycles, achieve throughput  9484 MB/s

transfer  204 *  5120 Bytes with index= 5120 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 131768 cycles, achieve throughput  9511 MB/s

transfer  186 *  5632 Bytes with index= 5632 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132986 cycles, achieve throughput  9452 MB/s

transfer  170 *  6144 Bytes with index= 6144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 131194 cycles, achieve throughput  9553 MB/s

transfer  157 *  6656 Bytes with index= 6656 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132532 cycles, achieve throughput  9461 MB/s

transfer  146 *  7168 Bytes with index= 7168 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 131715 cycles, achieve throughput  9534 MB/s

transfer  136 *  7680 Bytes with index= 7680 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 133150 cycles, achieve throughput  9413 MB/s

transfer  128 *  8192 Bytes with index= 8192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 131620 cycles, achieve throughput  9560 MB/s

transfer 1024 *     1 Bytes with index=    2 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32772 cycles, achieve throughput    37 MB/s

transfer 1024 *     2 Bytes with index=    3 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31560 cycles, achieve throughput    77 MB/s

transfer 1024 *     3 Bytes with index=    4 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  30556 cycles, achieve throughput   120 MB/s

transfer 1024 *     4 Bytes with index=    5 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31788 cycles, achieve throughput   154 MB/s

transfer 1024 *     5 Bytes with index=    6 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31774 cycles, achieve throughput   193 MB/s

transfer 1024 *     6 Bytes with index=    7 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33264 cycles, achieve throughput   221 MB/s

transfer 1024 *     7 Bytes with index=    8 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  28993 cycles, achieve throughput   296 MB/s

transfer 1024 *     8 Bytes with index=    9 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32327 cycles, achieve throughput   304 MB/s

transfer 1024 *     9 Bytes with index=   10 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32351 cycles, achieve throughput   341 MB/s

transfer 1024 *    10 Bytes with index=   11 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33549 cycles, achieve throughput   366 MB/s

transfer 1024 *    11 Bytes with index=   12 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32908 cycles, achieve throughput   410 MB/s

transfer 1024 *    12 Bytes with index=   13 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33911 cycles, achieve throughput   434 MB/s

transfer 1024 *    13 Bytes with index=   14 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  34217 cycles, achieve throughput   466 MB/s

transfer 1024 *    14 Bytes with index=   15 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32904 cycles, achieve throughput   522 MB/s

transfer 1024 *    15 Bytes with index=   16 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  27113 cycles, achieve throughput   679 MB/s

transfer 1024 *    16 Bytes with index=   17 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  34356 cycles, achieve throughput   572 MB/s

transfer 1024 *    32 Bytes with index=   33 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  35428 cycles, achieve throughput  1109 MB/s

transfer 1024 *    48 Bytes with index=   49 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  42655 cycles, achieve throughput  1382 MB/s

transfer 1024 *    64 Bytes with index=   65 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  43868 cycles, achieve throughput  1792 MB/s

transfer 1024 *    80 Bytes with index=   81 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  46705 cycles, achieve throughput  2104 MB/s

transfer 1024 *    96 Bytes with index=   97 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  49619 cycles, achieve throughput  2377 MB/s

transfer 1024 *   112 Bytes with index=  113 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  50277 cycles, achieve throughput  2737 MB/s

transfer 1024 *   128 Bytes with index=  129 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  52772 cycles, achieve throughput  2980 MB/s

transfer 1024 *   144 Bytes with index=  145 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  53185 cycles, achieve throughput  3327 MB/s

transfer 1024 *   160 Bytes with index=  161 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  57503 cycles, achieve throughput  3419 MB/s

transfer 1024 *   176 Bytes with index=  177 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  59281 cycles, achieve throughput  3648 MB/s

transfer 1024 *   192 Bytes with index=  193 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  62873 cycles, achieve throughput  3752 MB/s

transfer 1024 *   208 Bytes with index=  209 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  63117 cycles, achieve throughput  4049 MB/s

transfer 1024 *   224 Bytes with index=  225 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  68571 cycles, achieve throughput  4014 MB/s

transfer 1024 *   240 Bytes with index=  241 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  69816 cycles, achieve throughput  4224 MB/s

transfer 1024 *   256 Bytes with index=  257 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  76392 cycles, achieve throughput  4117 MB/s

transfer 1024 *   320 Bytes with index=  321 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  85659 cycles, achieve throughput  4590 MB/s

transfer 1024 *   384 Bytes with index=  385 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  90465 cycles, achieve throughput  5215 MB/s

transfer 1024 *   448 Bytes with index=  449 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 103868 cycles, achieve throughput  5300 MB/s

transfer 1024 *   512 Bytes with index=  513 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 109261 cycles, achieve throughput  5758 MB/s

transfer 1024 *   768 Bytes with index=  769 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 134626 cycles, achieve throughput  7009 MB/s

transfer 1023 *  1024 Bytes with index= 1025 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 171931 cycles, achieve throughput  7311 MB/s

transfer  682 *  1536 Bytes with index= 1537 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 165091 cycles, achieve throughput  7614 MB/s

transfer  511 *  2048 Bytes with index= 2049 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 162277 cycles, achieve throughput  7738 MB/s

transfer  409 *  2560 Bytes with index= 2561 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 158564 cycles, achieve throughput  7923 MB/s

transfer  341 *  3072 Bytes with index= 3073 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 159746 cycles, achieve throughput  7869 MB/s

transfer  292 *  3584 Bytes with index= 3585 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 159852 cycles, achieve throughput  7856 MB/s

transfer  255 *  4096 Bytes with index= 4097 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 164462 cycles, achieve throughput  7621 MB/s

transfer  227 *  4608 Bytes with index= 4609 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 162805 cycles, achieve throughput  7709 MB/s

transfer  204 *  5120 Bytes with index= 5121 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 159505 cycles, achieve throughput  7857 MB/s

transfer  186 *  5632 Bytes with index= 5633 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 162562 cycles, achieve throughput  7732 MB/s

transfer  170 *  6144 Bytes with index= 6145 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 161769 cycles, achieve throughput  7747 MB/s

transfer  157 *  6656 Bytes with index= 6657 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 160295 cycles, achieve throughput  7823 MB/s

transfer  146 *  7168 Bytes with index= 7169 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 161087 cycles, achieve throughput  7795 MB/s

transfer  136 *  7680 Bytes with index= 7681 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 159028 cycles, achieve throughput  7881 MB/s

transfer  127 *  8192 Bytes with index= 8193 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 163473 cycles, achieve throughput  7637 MB/s

transfer 1024 *     1 Bytes with index=    5 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  30538 cycles, achieve throughput    40 MB/s

transfer 1024 *     2 Bytes with index=    6 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  29856 cycles, achieve throughput    82 MB/s

transfer 1024 *     3 Bytes with index=    7 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31663 cycles, achieve throughput   116 MB/s

transfer 1024 *     4 Bytes with index=    8 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  30004 cycles, achieve throughput   163 MB/s

transfer 1024 *     5 Bytes with index=    9 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  30899 cycles, achieve throughput   198 MB/s

transfer 1024 *     6 Bytes with index=   10 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  30139 cycles, achieve throughput   244 MB/s

transfer 1024 *     7 Bytes with index=   11 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  30883 cycles, achieve throughput   278 MB/s

transfer 1024 *     8 Bytes with index=   12 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  29754 cycles, achieve throughput   330 MB/s

transfer 1024 *     9 Bytes with index=   13 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31115 cycles, achieve throughput   355 MB/s

transfer 1024 *    10 Bytes with index=   14 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  30561 cycles, achieve throughput   402 MB/s

transfer 1024 *    11 Bytes with index=   15 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31347 cycles, achieve throughput   431 MB/s

transfer 1024 *    12 Bytes with index=   16 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  25880 cycles, achieve throughput   569 MB/s

transfer 1024 *    13 Bytes with index=   17 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31789 cycles, achieve throughput   502 MB/s

transfer 1024 *    14 Bytes with index=   18 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  32222 cycles, achieve throughput   533 MB/s

transfer 1024 *    15 Bytes with index=   19 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  31916 cycles, achieve throughput   577 MB/s

transfer 1024 *    16 Bytes with index=   20 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  30553 cycles, achieve throughput   643 MB/s

transfer 1024 *    32 Bytes with index=   36 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  34701 cycles, achieve throughput  1133 MB/s

transfer 1024 *    48 Bytes with index=   52 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  37930 cycles, achieve throughput  1555 MB/s

transfer 1024 *    64 Bytes with index=   68 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  41885 cycles, achieve throughput  1877 MB/s

transfer 1024 *    80 Bytes with index=   84 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  42672 cycles, achieve throughput  2303 MB/s

transfer 1024 *    96 Bytes with index=  100 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  45434 cycles, achieve throughput  2596 MB/s

transfer 1024 *   112 Bytes with index=  116 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  48155 cycles, achieve throughput  2857 MB/s

transfer 1024 *   128 Bytes with index=  132 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  48538 cycles, achieve throughput  3240 MB/s

transfer 1024 *   144 Bytes with index=  148 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  51823 cycles, achieve throughput  3414 MB/s

transfer 1024 *   160 Bytes with index=  164 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  53663 cycles, achieve throughput  3663 MB/s

transfer 1024 *   176 Bytes with index=  180 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  59761 cycles, achieve throughput  3618 MB/s

transfer 1024 *   192 Bytes with index=  196 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  59995 cycles, achieve throughput  3932 MB/s

transfer 1024 *   208 Bytes with index=  212 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  61094 cycles, achieve throughput  4183 MB/s

transfer 1024 *   224 Bytes with index=  228 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  64687 cycles, achieve throughput  4255 MB/s

transfer 1024 *   240 Bytes with index=  244 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  67394 cycles, achieve throughput  4375 MB/s

transfer 1024 *   256 Bytes with index=  260 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  71895 cycles, achieve throughput  4375 MB/s

transfer 1024 *   320 Bytes with index=  324 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  81174 cycles, achieve throughput  4844 MB/s

transfer 1024 *   384 Bytes with index=  388 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  86353 cycles, achieve throughput  5464 MB/s

transfer 1024 *   448 Bytes with index=  452 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  99464 cycles, achieve throughput  5534 MB/s

transfer 1024 *   512 Bytes with index=  516 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 106696 cycles, achieve throughput  5896 MB/s

transfer 1024 *   768 Bytes with index=  772 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 130497 cycles, achieve throughput  7231 MB/s

transfer 1020 *  1024 Bytes with index= 1028 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 170091 cycles, achieve throughput  7368 MB/s

transfer  680 *  1536 Bytes with index= 1540 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 160866 cycles, achieve throughput  7791 MB/s

transfer  511 *  2048 Bytes with index= 2052 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 155852 cycles, achieve throughput  8057 MB/s

transfer  408 *  2560 Bytes with index= 2564 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 155367 cycles, achieve throughput  8067 MB/s

transfer  340 *  3072 Bytes with index= 3076 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 154440 cycles, achieve throughput  8115 MB/s

transfer  292 *  3584 Bytes with index= 3588 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 157449 cycles, achieve throughput  7976 MB/s

transfer  255 *  4096 Bytes with index= 4100 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 155124 cycles, achieve throughput  8079 MB/s

transfer  227 *  4608 Bytes with index= 4612 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 157587 cycles, achieve throughput  7965 MB/s

transfer  204 *  5120 Bytes with index= 5124 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 158244 cycles, achieve throughput  7920 MB/s

transfer  186 *  5632 Bytes with index= 5636 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 157266 cycles, achieve throughput  7993 MB/s

transfer  170 *  6144 Bytes with index= 6148 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 156690 cycles, achieve throughput  7999 MB/s

transfer  157 *  6656 Bytes with index= 6660 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 157398 cycles, achieve throughput  7967 MB/s

transfer  146 *  7168 Bytes with index= 7172 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 157218 cycles, achieve throughput  7987 MB/s

transfer  136 *  7680 Bytes with index= 7684 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 158818 cycles, achieve throughput  7891 MB/s

transfer  127 *  8192 Bytes with index= 8196 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 156877 cycles, achieve throughput  7958 MB/s

transfer 1024 *     1 Bytes with index=   17 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  25885 cycles, achieve throughput    47 MB/s

transfer 1024 *     2 Bytes with index=   18 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  25240 cycles, achieve throughput    97 MB/s

transfer 1024 *     3 Bytes with index=   19 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  26689 cycles, achieve throughput   138 MB/s

transfer 1024 *     4 Bytes with index=   20 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  25217 cycles, achieve throughput   194 MB/s

transfer 1024 *     5 Bytes with index=   21 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  26881 cycles, achieve throughput   228 MB/s

transfer 1024 *     6 Bytes with index=   22 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  25250 cycles, achieve throughput   291 MB/s

transfer 1024 *     7 Bytes with index=   23 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  25909 cycles, achieve throughput   331 MB/s

transfer 1024 *     8 Bytes with index=   24 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  23352 cycles, achieve throughput   420 MB/s

transfer 1024 *     9 Bytes with index=   25 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  26565 cycles, achieve throughput   416 MB/s

transfer 1024 *    10 Bytes with index=   26 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  26577 cycles, achieve throughput   462 MB/s

transfer 1024 *    11 Bytes with index=   27 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  27586 cycles, achieve throughput   489 MB/s

transfer 1024 *    12 Bytes with index=   28 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  27130 cycles, achieve throughput   543 MB/s

transfer 1024 *    13 Bytes with index=   29 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  29121 cycles, achieve throughput   548 MB/s

transfer 1024 *    14 Bytes with index=   30 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  28806 cycles, achieve throughput   597 MB/s

transfer 1024 *    15 Bytes with index=   31 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  28610 cycles, achieve throughput   644 MB/s

transfer 1024 *    16 Bytes with index=   32 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  22108 cycles, achieve throughput   889 MB/s

transfer 1024 *    32 Bytes with index=   48 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  25231 cycles, achieve throughput  1558 MB/s

transfer 1024 *    48 Bytes with index=   64 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  16888 cycles, achieve throughput  3492 MB/s

transfer 1024 *    64 Bytes with index=   80 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  29407 cycles, achieve throughput  2674 MB/s

transfer 1024 *    80 Bytes with index=   96 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  30265 cycles, achieve throughput  3248 MB/s

transfer 1024 *    96 Bytes with index=  112 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  37455 cycles, achieve throughput  3149 MB/s

transfer 1024 *   112 Bytes with index=  128 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  23132 cycles, achieve throughput  5949 MB/s

transfer 1024 *   128 Bytes with index=  144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  40403 cycles, achieve throughput  3892 MB/s

transfer 1024 *   144 Bytes with index=  160 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  44327 cycles, achieve throughput  3991 MB/s

transfer 1024 *   160 Bytes with index=  176 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  44464 cycles, achieve throughput  4421 MB/s

transfer 1024 *   176 Bytes with index=  192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  28520 cycles, achieve throughput  7583 MB/s

transfer 1024 *   192 Bytes with index=  208 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  50558 cycles, achieve throughput  4666 MB/s

transfer 1024 *   208 Bytes with index=  224 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  53889 cycles, achieve throughput  4742 MB/s

transfer 1024 *   224 Bytes with index=  240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  52017 cycles, achieve throughput  5291 MB/s

transfer 1024 *   240 Bytes with index=  256 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  36008 cycles, achieve throughput  8190 MB/s

transfer 1024 *   256 Bytes with index=  272 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  55182 cycles, achieve throughput  5700 MB/s

transfer 1024 *   320 Bytes with index=  336 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  63705 cycles, achieve throughput  6172 MB/s

transfer 1024 *   384 Bytes with index=  400 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  72625 cycles, achieve throughput  6497 MB/s

transfer 1024 *   448 Bytes with index=  464 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  81423 cycles, achieve throughput  6761 MB/s

transfer 1024 *   512 Bytes with index=  528 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  89430 cycles, achieve throughput  7035 MB/s

transfer 1024 *   768 Bytes with index=  784 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 121050 cycles, achieve throughput  7796 MB/s

transfer 1008 *  1024 Bytes with index= 1040 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 152409 cycles, achieve throughput  8127 MB/s

transfer  675 *  1536 Bytes with index= 1552 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 147601 cycles, achieve throughput  8429 MB/s

transfer  508 *  2048 Bytes with index= 2064 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 143599 cycles, achieve throughput  8694 MB/s

transfer  407 *  2560 Bytes with index= 2576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 145171 cycles, achieve throughput  8612 MB/s

transfer  339 *  3072 Bytes with index= 3088 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 145602 cycles, achieve throughput  8582 MB/s

transfer  291 *  3584 Bytes with index= 3600 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 145154 cycles, achieve throughput  8622 MB/s

transfer  255 *  4096 Bytes with index= 4112 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 144827 cycles, achieve throughput  8654 MB/s

transfer  226 *  4608 Bytes with index= 4624 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 146028 cycles, achieve throughput  8557 MB/s

transfer  204 *  5120 Bytes with index= 5136 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 144615 cycles, achieve throughput  8666 MB/s

transfer  185 *  5632 Bytes with index= 5648 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 146576 cycles, achieve throughput  8530 MB/s

transfer  170 *  6144 Bytes with index= 6160 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 145305 cycles, achieve throughput  8625 MB/s

transfer  157 *  6656 Bytes with index= 6672 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 145345 cycles, achieve throughput  8627 MB/s

transfer  145 *  7168 Bytes with index= 7184 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 145546 cycles, achieve throughput  8569 MB/s

transfer  136 *  7680 Bytes with index= 7696 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 146950 cycles, achieve throughput  8529 MB/s

transfer  127 *  8192 Bytes with index= 8208 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 146234 cycles, achieve throughput  8537 MB/s

transfer 1024 *     1 Bytes with index=   65 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  13033 cycles, achieve throughput    94 MB/s

transfer 1024 *     2 Bytes with index=   66 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  12948 cycles, achieve throughput   189 MB/s

transfer 1024 *     3 Bytes with index=   67 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  13160 cycles, achieve throughput   280 MB/s

transfer 1024 *     4 Bytes with index=   68 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  12875 cycles, achieve throughput   381 MB/s

transfer 1024 *     5 Bytes with index=   69 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  13269 cycles, achieve throughput   463 MB/s

transfer 1024 *     6 Bytes with index=   70 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  13251 cycles, achieve throughput   556 MB/s

transfer 1024 *     7 Bytes with index=   71 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  13432 cycles, achieve throughput   640 MB/s

transfer 1024 *     8 Bytes with index=   72 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  12892 cycles, achieve throughput   762 MB/s

transfer 1024 *     9 Bytes with index=   73 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  13696 cycles, achieve throughput   807 MB/s

transfer 1024 *    10 Bytes with index=   74 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  13687 cycles, achieve throughput   897 MB/s

transfer 1024 *    11 Bytes with index=   75 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  13873 cycles, achieve throughput   974 MB/s

transfer 1024 *    12 Bytes with index=   76 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  13623 cycles, achieve throughput  1082 MB/s

transfer 1024 *    13 Bytes with index=   77 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  14083 cycles, achieve throughput  1134 MB/s

transfer 1024 *    14 Bytes with index=   78 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  14086 cycles, achieve throughput  1221 MB/s

transfer 1024 *    15 Bytes with index=   79 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  14221 cycles, achieve throughput  1296 MB/s

transfer 1024 *    16 Bytes with index=   80 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  12927 cycles, achieve throughput  1520 MB/s

transfer 1024 *    32 Bytes with index=   96 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  12826 cycles, achieve throughput  3065 MB/s

transfer 1024 *    48 Bytes with index=  112 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  15992 cycles, achieve throughput  3688 MB/s

transfer 1024 *    64 Bytes with index=  128 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  15939 cycles, achieve throughput  4934 MB/s

transfer 1024 *    80 Bytes with index=  144 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  19957 cycles, achieve throughput  4925 MB/s

transfer 1024 *    96 Bytes with index=  160 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  20070 cycles, achieve throughput  5877 MB/s

transfer 1024 *   112 Bytes with index=  176 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  22177 cycles, achieve throughput  6205 MB/s

transfer 1024 *   128 Bytes with index=  192 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  22133 cycles, achieve throughput  7106 MB/s

transfer 1024 *   144 Bytes with index=  208 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  27044 cycles, achieve throughput  6542 MB/s

transfer 1024 *   160 Bytes with index=  224 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  27967 cycles, achieve throughput  7029 MB/s

transfer 1024 *   176 Bytes with index=  240 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  30786 cycles, achieve throughput  7024 MB/s

transfer 1024 *   192 Bytes with index=  256 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  29367 cycles, achieve throughput  8033 MB/s

transfer 1024 *   208 Bytes with index=  272 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  33845 cycles, achieve throughput  7551 MB/s

transfer 1024 *   224 Bytes with index=  288 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  35130 cycles, achieve throughput  7835 MB/s

transfer 1024 *   240 Bytes with index=  304 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  39631 cycles, achieve throughput  7441 MB/s

transfer 1024 *   256 Bytes with index=  320 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  35093 cycles, achieve throughput  8963 MB/s

transfer 1024 *   320 Bytes with index=  384 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  41995 cycles, achieve throughput  9363 MB/s

transfer 1024 *   384 Bytes with index=  448 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  50570 cycles, achieve throughput  9330 MB/s

transfer 1024 *   448 Bytes with index=  512 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  60016 cycles, achieve throughput  9172 MB/s

transfer 1024 *   512 Bytes with index=  576 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes  67127 cycles, achieve throughput  9372 MB/s

transfer 1024 *   768 Bytes with index=  832 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 102094 cycles, achieve throughput  9243 MB/s

transfer  963 *  1024 Bytes with index= 1088 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 127185 cycles, achieve throughput  9304 MB/s

transfer  655 *  1536 Bytes with index= 1600 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 128335 cycles, achieve throughput  9407 MB/s

transfer  496 *  2048 Bytes with index= 2112 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132566 cycles, achieve throughput  9195 MB/s

transfer  399 *  2560 Bytes with index= 2624 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 129504 cycles, achieve throughput  9464 MB/s

transfer  334 *  3072 Bytes with index= 3136 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 130068 cycles, achieve throughput  9466 MB/s

transfer  287 *  3584 Bytes with index= 3648 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 129395 cycles, achieve throughput  9539 MB/s

transfer  252 *  4096 Bytes with index= 4160 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 129082 cycles, achieve throughput  9595 MB/s

transfer  224 *  4608 Bytes with index= 4672 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 133268 cycles, achieve throughput  9294 MB/s

transfer  202 *  5120 Bytes with index= 5184 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132811 cycles, achieve throughput  9344 MB/s

transfer  184 *  5632 Bytes with index= 5696 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132619 cycles, achieve throughput  9376 MB/s

transfer  168 *  6144 Bytes with index= 6208 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132017 cycles, achieve throughput  9382 MB/s

transfer  156 *  6656 Bytes with index= 6720 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 132475 cycles, achieve throughput  9405 MB/s

transfer  144 *  7168 Bytes with index= 7232 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 130026 cycles, achieve throughput  9526 MB/s

transfer  135 *  7680 Bytes with index= 7744 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 134634 cycles, achieve throughput  9241 MB/s

transfer  127 *  8192 Bytes with index= 8256 from 0x c080000(MSMC RAM) to 0x80000000(DDR3A), consumes 133371 cycles, achieve throughput  9360 MB/s

EDMA test complete

Test complete