KeyStone 2

PCIE Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial Release |

Contents

1	Introduction	3

2	Test Methods	3

2.1	Integrity Test	3

2.1.1	Data filling Test	3

2.1.2	Addressing test	4

2.2	PCIE Performance Test	4

2.2.1	PCIE Latency Test	5

2.2.2	PCIE Throughput Test	5

2.3	PCIE error reporting and checking	5

2.4	PCIE initialization process	6

3	Test cases	7

3.1	Data transfer by different masters	7

3.2	Test through different data path	7

3.3	Test different memories in the system	8

3.3.1	Test from RC to EP	8

3.3.2	Test from EP to RC	11

3.3.3	PHY loopback test	12

4	Test code/project	13

4.1	CCS project	13

4.2	To run the test program on KeyStone 2 EVM	14

4.3	Test configuration	14

4.4	Migrate the test to custom board	17

References	17

Appendix: Typical test output	18

Figures

Figure 1.	PHY Loopback Data Path	8

Figure 2.	Example of PCIE mapping different memories of EP to RC	9

Figure 3.	Mapping of test buffers in STK	10

Figure 4.	Example of PCIE data transfer from EP to RC	11

Figure 5.	Example of PCIE MSI interrupt from EP to RC	12

Figure 6.	Directory structure of test project	13

Figure 7.	define symbols in CCS compiler options to select device and board	15

Tables

Table 1.	Example of memory map between RC and EP	9

Table 2.	Mapping of test buffers in STK	10

Table 3.	Example of memory map of RC to EP	11

Table 4.	Example of memory map in PHY loopback mode	12

Table 5.	Source files of the test codes	13

- Introduction

The PCI Express (PCIe) module is a multi-lane I/O interconnect providing low pin count, high reliability, and high-speed data transfer at rates of up to 5.0 Gbps per lane per direction, for serial links on backplanes and printed wiring boards. It is a 3rd Generation I/O Interconnect technology succeeding ISA and PCI bus that is designed to be used as a general-purpose serial I/O interconnect in multiple market segments, including desktop, mobile, server, storage and embedded communications.

The PCIe STK test covers below test cases and work modes:

- ARM works in RC mode or EP mode respectively

- Remote access or PHY loopback transfer mode

- Access PCIE with ARM core or EDMA

- Access different memories through PCIE

- EP generate MSI interrupt to RC through PCIE

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test Methods

  - Integrity Test

Two memory test algorithms are used to test the memory access through PCIE, and the purpose of these test algorithms is discussed below.

    - Data filling Test

The pseudocode for data pattern filling test is:

for(memory range under test)

fill the memory with a value;

for(memory range under test)

read back the memory and compare the readback value to the written value

Normally, this test is done several times with different test data value. The common values used for test include 0, 0xFFFFFFFF, 0x55555555, 0xAAAAAAAA.

This test can detect stuck data bit, for example, if

written value = 0, readback value = 0x8,

It indicates bit 3 sticks to 1. If

written value = 0xFFFFFFFF, readback value = 0xFFFFFFFE,

It indicates bit 0 sticks to 0.

If this test fails, the possible reasons include PCIE physical layer error, target memory error. Trying internal loopback test against test between two devices may isolate the reason.

    - Addressing test

The pseudo code for addressing test is:

for(memory range under test)

fill each memory unit with its address value;

for(memory range under test)

read back the memory and compare the readback value to the written value

This test can detect stuck bit on address bus, for example, if

written value = 0 at address 0

written value = 1 at address 1

written value = 2 at address 2

written value = 3 at address 3

……

readback value = 2 at address 0

readback value = 3 at address 1

readback value = 2 at address 2

readback value = 3 at address 3

……

For this case, it indicates bit 1 of address bus sticks. So, the contents in address 0 and 2 are same, and the contents in address 1 and 3 are same.

If the data filling test passes, while this test fails, the possible reasons include PCIE address translation error, the PCIE buffer error or target memory error. Trying internal loopback test against test between two devices may isolate the reason.

  - PCIE Performance Test

The performance of PCIE is also measured. If the test shows performance degradation, normally we should check clock speed related configuration.

    - PCIE Latency Test

The pseudo codes for measuring the latency of ARM core access a memory unit through PCIE are like following:

flushCache();

startCycle= getCycleCount();

for(i=0; i< accessTimes; i++)

{

Access Memory at address;

address+= stride;

}

cycles = getCycleCount()-startCycle;

cycles/Access= cycles/accessTimes;

The latency of the interrupt generated by PCIE is also measured with following pseudo code:

……

startCCNT= getCycleCount();

manually trigger a MSI interrupt;

wait for the queue pending interrupt ;

delay= intCCNT - startCCNT;

……

void PCIE_ISR()	//PCIE Interrupt Service Routine

{

intCCNT= getCycleCount(); 	//save the Cycle Count when the interrupt happens

……

}

This test is done with internal loopback mode only.

    - PCIE Throughput Test

The throughput is measured for different memory copy cases with ARM core or EDMA. The throughput is measured by taking total bytes copied and dividing it by the time it used.

  - PCIE error reporting and checking

PCIE detects error during transfer and reports them through some error/status registers.

The PCIE error can also trigger interrupt to ARM core. Interrupt Service Routine is also implemented to print/report the error information.

  - PCIE initialization process

The PCIE initialization process in the STK is implemented to match real PCIE network initialization as much as possible.

Following is the basic flow for RC and EP remote loopback test:

EP initializes BAR mask and properties, which indicates the PCI address space request to RC. In the STK, one BAR is initialized for prefetchable space, one BAR is initialized for non-prefetchable space.

EP initializes MSI Capabilities Register, which indicates the number of interrupt vectors request to RC.

EP starts physical link training, and waits for link training completion…

RC initializes the base and limit of PCIE address space.

RC starts physical link training, and waits for link training completion …

RC reads BAR mask and properties from EP, allocates PCI address space accordingly, writes the PCI start address to the BAR register of EP, and then RC configures its own outbound memory regions to map the PCI address space to its own local access window.

EP polls the BAR register value, once it gets valid PCIE address set by RC, EP configures its own inbound memory regions to map the PCI address space to its own local address space.

RC reads the MSI capabilities register of EP, allocates interrupt vectors accordingly, writes start index and number of the allocated interrupt vectors to EP MSI registers.

Following is the basic flow for RC Phy loopback test:

RC (acting EP) initializes BAR mask and properties, which indicates the PCI address space request to RC. In the STK, one BAR is initialized for prefetchable space in PHY loopback test

RC (acting EP) initializes MSI Capabilities Register through internal access bus directly, which indicates the number of interrupt vectors request to RC.

RC forces physical link training skipped, as it is only PHY internal loopback and no PCle training set in the lane

RC initializes the base and limit of PCIE address space through memory access method directly

RC reads BAR mask and properties by PHY loopback mode, allocates PCI address space accordingly, writes the PCI start address to the BAR register of acting EP, and then RC configures its own outbound memory regions to map the PCI address space to its own local access window.

RC (acting EP) polls the BAR register value, once it gets valid PCIE address set by RC through PHY loopback mode, it configures its own inbound memory regions to map the PCI address space to its own local address space.

RC reads the MSI capabilities register through PHY loopback method, allocates interrupt vectors accordingly, writes start index and number of the allocated interrupt vectors to RC MSI registers.

- Test cases

PCIE Tests should be implemented to cover:

- Internal loopback and test between two devices

- Access PCIE with ARM core or EDMA

- Access different memories through PCIE

- MSI interrupt test in internal loopback and remote mode

  - Data transfer by different masters

The test cases executed with ARM core including:

- Throughput test

- Latency of Memory access through PCIE

- PCIE Interrupt latency test

The test cases executed with EDMA including:

- Throughput test

- overhead of EDMA transfer through PCIE

  - Test through different data path

Internal PHY loopback test is done with the TX data loopback in Serdes module. The internal loopback test can be used to check the function of PCIE modules inside the KeyStone SOC. If internal loopback test failed, normally, we should check the PCIE related powers and clocks.

- PHY Loopback Data Path

For the test between two devices, device 1's memory are mapped to device 0 through PCIE, so, device 0 accesses device 1 through the PCIE memory window just like access it’s own memories.

If internal loopback test passes, while the test between two devices fails, normally, we should check the signal integrity on board, and measure the eye-diagram.

  - Test different memories in the system

All memories of K2 SOC can be accessed through PCIE. The PCIE memory map is very flexible. Following sections discuss the memory map configuration for different test cases.

    - Test from RC to EP

In this test, device 1’s memory are mapped to device 0’s PCIE memory window from 0x50000000 to 0x5FFFFFFF, the device 0 can access all memory types in device 1 including DSP core LL2 (Local L2), MSRAM (Multicore Shared RAM) and DDR just like access device 0’s local memory. Following figure shows an example.

- Example of PCIE mapping different memories of EP to RC

The memory map in this example is summarized in following table.

- Example of memory map between RC and EP

| RC device PCIE data space | EP device memory space | Size |
| --- | --- | --- |
| 0x50800000 (To EP DDR) | 0x80000000 (DDR3A) | 64MB |
| 0x54800000 (To EP MSRAM) | 0x0C100000 (MSRAM) | 1MB |
| 0x54900000 (To EP core1 L2) | 0x11820000 (core1 L2) | 256KB |

Following figure shows the mapping of the test buffers used in the STK.

- Mapping of test buffers in STK

The memory map in this example is summarized in following table.

- Mapping of test buffers in STK

| Virtual buffer at RC side | Test buffer at EP side | Size |
| --- | --- | --- |
| 0x50800000 (To EP DDR3A) | 0x80000000 (DDR3A) | 16MB |
| 0x51800000 (To EP DDR3A) | 0x81000000 (DDR3A) | 16MB |
| 0x54800000 (To EP MSRAM) | 0x0C100000 (MSRAM) | 256KB |
| 0x54840000 (To EP MSRAM) | 0x0C140000 (MSRAM) | 256KB |
| 0x54900000 (To EP core1 L2) | 0x11820000 (core1 L2) | 128KB |
| 0x54920000 (To EP core1 L2) | 0x11840000 (core1 L2) | 128KB |

With this configuration, when device 0 core0 or EDMA accesses address 0x54900000, it actually accesses the LL2 memory of device 1,starting from 0x11820000. When device 0 core 0 or EDMA access address 0x54800000, it actually accesses the 0x0C100000 in share memory of device 1; When device 0 core 0 or EDMA access address 0x50800000, it actually accesses the 0x80000000 in DDR3 memory of device 1.

For internal loopback test, the memory map configuration is same, the only difference is that, device 0 actually access its own local memory through PCIE memory window.

    - Test from EP to RC

      - Test PCIE data transfer from EP to RC

According to PCIE protocol, in RC, if the incoming TLP address is outside of the range defined by the three Base/Limit register sets (for non-prefetchable memory, prefetchable memory, and I/O), and then that TLP is accepted. That is PCIE engine doesn’t do address translation and just use the inbound address from PCIE bus. Table 3 and Figure 4 is an example of the address info in this case:

- Example of memory map of RC to EP

| RC device memory space | EP PCIE data space |
| --- | --- |
| 0x90120000(DDR3A) | 0x51120000 |

- Example of PCIE data transfer from EP to RC

      - EP to RC MSI interrupt test

EP MSI interrupts are generated by a PCIe 32-bit memory write transaction from EP PCIE window to MSI_IRQ register in RC. To generate MSI interrupts, following steps need to be taken:

1. Ensure that the MSI support has been enabled in the device (Set MSI_EN bit in MSI Capabilities Register (MSI_CAP).)

2. EP Reads the value of the MSI Data Register in the local PCIe configuration space and determines the number of MSI vectors allocated from RC, generates the MSI_DATA which will be used to write to MSI_IRQ

3. EP issues a memory write transaction to the RC MSI_IRQ register’s mapping address in EP’s outbound window. In RC, the access to MSI_IRQ goes through BAR0 with default mapping.

- Example of PCIE MSI interrupt from EP to RC

    - PHY loopback test

This mode can be used to perform TLP loopback in single Keystone2 device. The memory partition is shown as following table.

- Example of memory map in PHY loopback mode

| RC PCIE data space | RC memory address |
| --- | --- |
| 0x51800000 | 0x81000000 (DDR3) |
| 0x54800000 | 0x0C100000 (MSRAM) |
| 0x54900000 | 0x11820000 (DSP core1 L2) |

- Test code/project

This section introduces the implementation of these PCIE tests on KeyStone 2 devices.

  - CCS project

Following figure shows the directory structure of the test project.

- Directory structure of test project

The project files are in “PCIE” folder. There is some commonly used initialization and driver code for PCIE, DDR, EDMA, Serdes and PLL… in “common” folder. The main source code files are in the “PCIE\src” subfolder. Below table describes these source files.

- Source files of the test codes

| Source files | Descriptions |
| --- | --- |
| K2_common | Initialization for EDMA and PLL. The key API include: void KeyStone_main_PLL_init (float ref_clock_MHz, unsigned int multiplier, unsigned int divisor); void EDMA_init (); |
| K2_DDR_Init | Low level DDR initialization. The key API is: void K2_DDR_Init(DDR3_Config * DDR_cfg); |
| K2_PCIE_Init | Low level PCIE initialization. The key API is: void K2_PCIE_Init(PCIE_Config * PCIE_cfg); |
| SerDes | Low level Serdes initialization. They are copied from pdk_keystone2_3_01_01_04. |
| PCIE_Test | main function of the tests, PCIE configuration, integrity test and interrupt latency test. |
| PCIE_CPU_performance | PCIE throughput test with ARM core; test latency of memory access through PCIE with ARM core. |
| PCIE_EDMA_performance | PCIE throughput test with EDMA; test overhead for EDMA transfer through PCIE |
| PCIE_Intc | Interrupts configuration and ISR for PCIE |

  - To run the test program on KeyStone 2 EVM

The test program can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- For loopback test, Load the program to ARM core 0 of a device. For test between two devices, load the program to core 0 of first device(RC), and load the program to core 1 of the second device(EP).

- For loopback test, run core 0; for test between two devices, run the core 1 of the second device firstly and then run the core 0 of the first device.

- See output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

There are multiple macros defined in the source code to configure the test.

Following variable in the “PCIE_Test.c” is used to switch between the internal loopback test and the two devices test. The value of “PCIE_PHY_LOOPBACK” means internal loopback test, and “PCIE_LOOPBACK_DISABLE” means test between two devices.

PCIE_Loopback_Mode loop_mode= PCIE_LOOPBACK_DISABLE;//PCIE_LOOPBACK_DISABLE;

Following variable in the “PCIE_Test.c” is used to set the memory access address width. The value of” PCIE_ADDRESS_32_BITS” means PCIE access through 32bit memory width, and “PCIE_ADDRESS_64_BITS” means PCIE access through 64bit memory width

PCIE_Address_Width address_width= PCIE_ADDRESS_32_BITS;// PCIE_ADDRESS_64_BITS

The base address of prefetch region and non-prefetch region be accessed through PCIE are defined with following macros in “PCIE_Test.c”.

#define PCIE_PREFETCH_BASE_ADDRESS 	0x80000000

#define PCIE_NONFETCH_BASE_ADDRESS 	0x10000000

The target memories be accessed through PCIE is configured with following configuration structure in “PCIE_Test.c”.

PCIE_Memory_Region memory_regions[]=

{

{0x90000000, 32*1024*1024}, //DDR3A

#ifdef DDRB_TEST

{0x70000000, 32*1024*1024},  //DDR3B

#else

{0xA0000000, 32*1024*1024},  //DDR3A

#endif

{0x0C100000, 1*1024*1024}, 	//MSRAM

{0x11820000, 256*1024} 	//LL2

};

If above configuration structure is modified, following configuration structure should be modified accordingly.

PCIERemoteTestAddress PcieRemoteTestAddr=

{

PCIE_OUTBOUND_DATA_WINDOW+0x0000000,//DDRA_SRC_ADDR;

PCIE_OUTBOUND_DATA_WINDOW+0x1000000,//DDRA_DST_ADDR;

#ifdef DDRB_TEST

PCIE_OUTBOUND_DATA_WINDOW+0x2000000,//DDRB_SRC_ADDR;

PCIE_OUTBOUND_DATA_WINDOW+0x3000000,//DDRB_DST_ADDR;

#else

0,//DDRB_SRC_ADDR;

0,//DDRB_DST_ADDR;

#endif

PCIE_OUTBOUND_DATA_WINDOW+0x4000000,//MSRAM_SRC_ADDR;

PCIE_OUTBOUND_DATA_WINDOW+0x4040000,//MSRAM_DST_ADDR;

PCIE_OUTBOUND_DATA_WINDOW+0x4100000,//LL2_SRC_ADDR;

PCIE_OUTBOUND_DATA_WINDOW+0x4120000,//LL2_DST_ADDR;

};

  - Migrate the test to custom board

These tests are implemented based on TI’s EVM boards. In your real system, the DDR configuration may be changed according to your hardware design in the function K2_DDR3A_config() and K2_DDR3B_config() in “K2_Board_Init.c”.

The core speed may be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

The PCIE Serdes reference clock speed may be changed through following macro in “K2_Board_Init.h”:

#define PCIE_SERDES_REF_CLK         CSL_SERDES_REF_CLOCK_100M

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone Architecture PCIE User Guide (sprugs6)

- KeyStone Architecture Enhanced Direct Memory Access (EDMA3) Controller User Guide (SPRUGS5)

- KeyStone II Architecture ARM CorePac User's Guide (SPRUHJ4)

Appendix: Typical test output

Below is the test output on K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

DDR3A initialization

Initialize DDR data rate = 100.000/1*20/6*4= 1333.3 MTS, bus width = 64 bits.

PCIE PHY loopback mode at 5.0GHz.

PCIE link 0 start link training...

PCIE link 0 training is finished.

PCIE memory test passed at address 0x51800000

PCIE memory test passed at address 0x54840000

PCIE memory test passed at address 0x54920000

PCIE MSI8 interrupt generated!

PCIE interrupt latency is 1885 cycles

#########RC starts core access performance test!#########

204 MB/s, copy  65536 bytes from 0x0c080000( local MSRAM) to 0x54920000(  remote LL2), consumes 385311 cycles

30 MB/s, copy  65536 bytes from 0x54920000(  remote LL2) to 0x0c080000( local MSRAM), consumes 2569611 cycles

27 MB/s, copy  65536 bytes from 0x54920000(  remote LL2) to 0x54930000(  remote LL2), consumes 2823768 cycles

203 MB/s, copy  65536 bytes from 0x0c080000( local MSRAM) to 0x54840000(remote MSRAM), consumes 385799 cycles

29 MB/s, copy  65536 bytes from 0x54840000(remote MSRAM) to 0x0c080000( local MSRAM), consumes 2678788 cycles

26 MB/s, copy  65536 bytes from 0x54840000(remote MSRAM) to 0x54850000(remote MSRAM), consumes 2936019 cycles

203 MB/s, copy  65536 bytes from 0x0c080000( local MSRAM) to 0x51800000(remote DDR3A), consumes 386400 cycles

27 MB/s, copy  65536 bytes from 0x51800000(remote DDR3A) to 0x0c080000( local MSRAM), consumes 2837688 cycles

24 MB/s, copy  65536 bytes from 0x51800000(remote DDR3A) to 0x51810000(remote DDR3A), consumes 3170943 cycles

Memory load performance test at 0x54920000(  remote LL2)

Index=   0  Cycles/LDRD= 175.52

Index=   1  Cycles/LDRD= 175.20

Index=   2  Cycles/LDRD= 175.12

Index=   3  Cycles/LDRD= 175.12

Index=   4  Cycles/LDRD= 175.22

Index=   5  Cycles/LDRD= 175.14

Index=   6  Cycles/LDRD= 175.22

Index=   7  Cycles/LDRD= 175.14

Index=   8  Cycles/LDRD= 175.08

Index=  10  Cycles/LDRD= 175.24

Index=  12  Cycles/LDRD= 175.21

Index=  14  Cycles/LDRD= 175.09

Index=  16  Cycles/LDRD= 175.20

Index=  18  Cycles/LDRD= 175.20

Index=  20  Cycles/LDRD= 175.19

Index=  22  Cycles/LDRD= 175.17

Index=  24  Cycles/LDRD= 175.18

Index=  26  Cycles/LDRD= 175.21

Index=  28  Cycles/LDRD= 175.29

Index=  30  Cycles/LDRD= 175.22

Index=  32  Cycles/LDRD= 175.14

Index=  40  Cycles/LDRD= 175.14

Index=  48  Cycles/LDRD= 175.23

Index=  56  Cycles/LDRD= 175.12

Memory store performance test at 0x54920000(  remote LL2)

Index=   0  Cycles/STRD= 92.80

Index=   1  Cycles/STRD= 92.73

Index=   2  Cycles/STRD= 93.20

Index=   3  Cycles/STRD= 92.91

Index=   4  Cycles/STRD= 93.18

Index=   5  Cycles/STRD= 92.85

Index=   6  Cycles/STRD= 92.90

Index=   7  Cycles/STRD= 92.86

Index=   8  Cycles/STRD= 92.86

Index=  10  Cycles/STRD= 93.11

Index=  12  Cycles/STRD= 92.85

Index=  14  Cycles/STRD= 93.64

Index=  16  Cycles/STRD= 93.02

Index=  18  Cycles/STRD= 93.19

Index=  20  Cycles/STRD= 93.31

Index=  22  Cycles/STRD= 93.82

Index=  24  Cycles/STRD= 93.57

Index=  26  Cycles/STRD= 92.87

Index=  28  Cycles/STRD= 92.97

Index=  30  Cycles/STRD= 92.97

Index=  32  Cycles/STRD= 93.07

Index=  40  Cycles/STRD= 92.83

Index=  48  Cycles/STRD= 92.90

Index=  56  Cycles/STRD= 93.06

Memory load performance test at 0x54840000(remote MSRAM)

Index=   0  Cycles/LDRD= 184.42

Index=   1  Cycles/LDRD= 184.53

Index=   2  Cycles/LDRD= 184.57

Index=   3  Cycles/LDRD= 184.50

Index=   4  Cycles/LDRD= 184.86

Index=   5  Cycles/LDRD= 184.43

Index=   6  Cycles/LDRD= 184.59

Index=   7  Cycles/LDRD= 184.44

Index=   8  Cycles/LDRD= 184.28

Index=  10  Cycles/LDRD= 184.54

Index=  12  Cycles/LDRD= 184.60

Index=  14  Cycles/LDRD= 184.50

Index=  16  Cycles/LDRD= 184.24

Index=  18  Cycles/LDRD= 184.48

Index=  20  Cycles/LDRD= 184.72

Index=  22  Cycles/LDRD= 184.58

Index=  24  Cycles/LDRD= 184.27

Index=  26  Cycles/LDRD= 184.62

Index=  28  Cycles/LDRD= 184.68

Index=  30  Cycles/LDRD= 184.55

Index=  32  Cycles/LDRD= 184.39

Index=  40  Cycles/LDRD= 184.54

Index=  48  Cycles/LDRD= 184.48

Index=  56  Cycles/LDRD= 184.50

Index=  64  Cycles/LDRD= 184.61

Index=  96  Cycles/LDRD= 184.73

Index= 128  Cycles/LDRD= 184.95

Index= 192  Cycles/LDRD= 185.07

Memory store performance test at 0x54840000(remote MSRAM)

Index=   0  Cycles/STRD= 93.09

Index=   1  Cycles/STRD= 92.87

Index=   2  Cycles/STRD= 92.95

Index=   3  Cycles/STRD= 93.04

Index=   4  Cycles/STRD= 92.85

Index=   5  Cycles/STRD= 92.96

Index=   6  Cycles/STRD= 92.86

Index=   7  Cycles/STRD= 92.89

Index=   8  Cycles/STRD= 92.95

Index=  10  Cycles/STRD= 92.88

Index=  12  Cycles/STRD= 93.50

Index=  14  Cycles/STRD= 92.76

Index=  16  Cycles/STRD= 93.58

Index=  18  Cycles/STRD= 93.16

Index=  20  Cycles/STRD= 92.88

Index=  22  Cycles/STRD= 93.15

Index=  24  Cycles/STRD= 93.21

Index=  26  Cycles/STRD= 93.42

Index=  28  Cycles/STRD= 93.02

Index=  30  Cycles/STRD= 92.64

Index=  32  Cycles/STRD= 93.21

Index=  40  Cycles/STRD= 92.91

Index=  48  Cycles/STRD= 93.37

Index=  56  Cycles/STRD= 93.58

Index=  64  Cycles/STRD= 93.12

Index=  96  Cycles/STRD= 93.00

Index= 128  Cycles/STRD= 93.16

Index= 192  Cycles/STRD= 93.50

Memory load performance test at 0x51800000(remote DDR3A)

Index=   0  Cycles/LDRD= 196.94

Index=   1  Cycles/LDRD= 198.28

Index=   2  Cycles/LDRD= 201.32

Index=   3  Cycles/LDRD= 199.58

Index=   4  Cycles/LDRD= 198.04

Index=   5  Cycles/LDRD= 195.52

Index=   6  Cycles/LDRD= 196.97

Index=   7  Cycles/LDRD= 200.31

Index=   8  Cycles/LDRD= 197.43

Index=  10  Cycles/LDRD= 195.54

Index=  12  Cycles/LDRD= 199.99

Index=  14  Cycles/LDRD= 198.83

Index=  16  Cycles/LDRD= 198.85

Index=  18  Cycles/LDRD= 196.57

Index=  20  Cycles/LDRD= 200.89

Index=  22  Cycles/LDRD= 196.91

Index=  24  Cycles/LDRD= 197.86

Index=  26  Cycles/LDRD= 198.05

Index=  28  Cycles/LDRD= 199.83

Index=  30  Cycles/LDRD= 197.77

Index=  32  Cycles/LDRD= 198.89

Index=  40  Cycles/LDRD= 199.02

Index=  48  Cycles/LDRD= 198.23

Index=  56  Cycles/LDRD= 199.51

Index=  64  Cycles/LDRD= 199.16

Index=  96  Cycles/LDRD= 196.90

Index= 128  Cycles/LDRD= 202.13

Index= 192  Cycles/LDRD= 197.35

Index= 256  Cycles/LDRD= 202.50

Index= 384  Cycles/LDRD= 199.81

Index= 512  Cycles/LDRD= 204.42

Index= 768  Cycles/LDRD= 206.31

Index=1024  Cycles/LDRD= 204.15

Index=1536  Cycles/LDRD= 203.03

Index=2048  Cycles/LDRD= 205.93

Index=3072  Cycles/LDRD= 203.68

Memory store performance test at 0x51800000(remote DDR3A)

Index=   0  Cycles/STRD= 95.73

Index=   1  Cycles/STRD= 94.45

Index=   2  Cycles/STRD= 93.40

Index=   3  Cycles/STRD= 96.10

Index=   4  Cycles/STRD= 94.83

Index=   5  Cycles/STRD= 95.02

Index=   6  Cycles/STRD= 95.58

Index=   7  Cycles/STRD= 96.42

Index=   8  Cycles/STRD= 94.09

Index=  10  Cycles/STRD= 93.64

Index=  12  Cycles/STRD= 93.44

Index=  14  Cycles/STRD= 95.65

Index=  16  Cycles/STRD= 94.43

Index=  18  Cycles/STRD= 94.56

Index=  20  Cycles/STRD= 96.04

Index=  22  Cycles/STRD= 94.61

Index=  24  Cycles/STRD= 93.72

Index=  26  Cycles/STRD= 94.60

Index=  28  Cycles/STRD= 94.88

Index=  30  Cycles/STRD= 94.27

Index=  32  Cycles/STRD= 96.78

Index=  40  Cycles/STRD= 95.35

Index=  48  Cycles/STRD= 93.34

Index=  56  Cycles/STRD= 95.84

Index=  64  Cycles/STRD= 95.48

Index=  96  Cycles/STRD= 95.31

Index= 128  Cycles/STRD= 97.65

Index= 192  Cycles/STRD= 95.78

Index= 256  Cycles/STRD= 95.03

Index= 384  Cycles/STRD= 94.50

Index= 512  Cycles/STRD= 97.29

Index= 768  Cycles/STRD= 97.30

Index=1024  Cycles/STRD= 94.36

Index=1536  Cycles/STRD= 94.78

Index=2048  Cycles/STRD= 95.40

Index=3072  Cycles/STRD= 94.70

#########RC starts EDMA performance test!#########

Overhead test with EDMA CC0 TC0

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    593 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    589 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1586 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1586 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1702 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1480 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1480 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1589 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC0 TC0

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 138685 cycles, achieve throughput   567 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 139824 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 140243 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108772 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108796 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 111779 cycles, achieve throughput   703 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 140243 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 140192 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 139572 cycles, achieve throughput   563 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108661 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108699 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 112556 cycles, achieve throughput   698 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 139797 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 139524 cycles, achieve throughput   563 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 140795 cycles, achieve throughput   558 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108666 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 108824 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 112006 cycles, achieve throughput   702 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 207732 cycles, achieve throughput   378 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207401 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 208057 cycles, achieve throughput   377 MB/s

Overhead test with EDMA CC0 TC1

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    591 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    589 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1587 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1590 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    585 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    585 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1479 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   2032 cycles, achieve throughput     4 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1588 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC0 TC1

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 138795 cycles, achieve throughput   566 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 140159 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 140013 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108672 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108714 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 111994 cycles, achieve throughput   702 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 140014 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 140514 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 138356 cycles, achieve throughput   568 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108662 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108703 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 112112 cycles, achieve throughput   701 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 140901 cycles, achieve throughput   558 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 140419 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 140572 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108666 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 108819 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 113002 cycles, achieve throughput   695 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 207730 cycles, achieve throughput   378 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207394 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 208172 cycles, achieve throughput   377 MB/s

Overhead test with EDMA CC1 TC0

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    481 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    477 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    477 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1366 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1588 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    585 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    480 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    474 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1479 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1589 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1588 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1701 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1365 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1586 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC1 TC0

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 140351 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 138845 cycles, achieve throughput   566 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 140028 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108664 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108735 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 110994 cycles, achieve throughput   708 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 140353 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 139966 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 140246 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108663 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108722 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 110773 cycles, achieve throughput   709 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 140575 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 140409 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 140572 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108779 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 108836 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 112670 cycles, achieve throughput   697 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 209284 cycles, achieve throughput   375 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207297 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 207963 cycles, achieve throughput   378 MB/s

Overhead test with EDMA CC1 TC1

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    480 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    476 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    477 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1365 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1591 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    476 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    475 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1586 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    591 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1592 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1700 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1366 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1479 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1588 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC1 TC1

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 140244 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 140496 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 139469 cycles, achieve throughput   563 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108668 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108707 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 111557 cycles, achieve throughput   704 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 140478 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 139278 cycles, achieve throughput   564 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 140480 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108690 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108718 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 112664 cycles, achieve throughput   698 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 140009 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 140004 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 140133 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108770 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 109690 cycles, achieve throughput   716 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 113000 cycles, achieve throughput   695 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 207613 cycles, achieve throughput   378 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207281 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 208065 cycles, achieve throughput   377 MB/s

Overhead test with EDMA CC1 TC2

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    488 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    476 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    475 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1369 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1588 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    474 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    475 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1588 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1480 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1587 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1701 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1367 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1591 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC1 TC2

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 140244 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 140425 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 140476 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108655 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108730 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 111221 cycles, achieve throughput   707 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 140685 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 140064 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 139905 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108668 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108837 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 111989 cycles, achieve throughput   702 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 139797 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 139395 cycles, achieve throughput   564 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 140569 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108675 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 108747 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 113336 cycles, achieve throughput   693 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 207729 cycles, achieve throughput   378 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207265 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 208070 cycles, achieve throughput   377 MB/s

Overhead test with EDMA CC1 TC3

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    479 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    478 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    475 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1368 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1588 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    474 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    474 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1697 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    590 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1481 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1588 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1700 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1367 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1589 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC1 TC3

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 139465 cycles, achieve throughput   563 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 139952 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 139899 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108659 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108729 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 112329 cycles, achieve throughput   700 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 140017 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 139745 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 139797 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108659 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108694 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 111442 cycles, achieve throughput   705 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 140359 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 139884 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 140573 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108774 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 108724 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 113900 cycles, achieve throughput   690 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 207730 cycles, achieve throughput   378 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207262 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 208171 cycles, achieve throughput   377 MB/s

Overhead test with EDMA CC2 TC0

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    478 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    475 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    475 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1365 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1701 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    477 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    477 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1587 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    585 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1589 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1701 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1365 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1587 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC2 TC0

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 140016 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 140318 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 140237 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108664 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108727 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 111889 cycles, achieve throughput   702 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 140471 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 139743 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 140796 cycles, achieve throughput   558 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108666 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108818 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 112114 cycles, achieve throughput   701 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 140348 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 139838 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 139901 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108771 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 108841 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 111894 cycles, achieve throughput   702 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 207628 cycles, achieve throughput   378 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207383 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 208401 cycles, achieve throughput   377 MB/s

Overhead test with EDMA CC2 TC1

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    480 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    475 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    478 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1366 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1587 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    585 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    476 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    477 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1590 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1474 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1586 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   2477 cycles, achieve throughput     3 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1364 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1589 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC2 TC1

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 139689 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 138528 cycles, achieve throughput   567 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 140469 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108664 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108706 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 111663 cycles, achieve throughput   704 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 140457 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 140736 cycles, achieve throughput   558 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 139455 cycles, achieve throughput   563 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108658 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108829 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 111440 cycles, achieve throughput   705 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 140463 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 139627 cycles, achieve throughput   563 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 140790 cycles, achieve throughput   558 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108667 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 108731 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 113235 cycles, achieve throughput   694 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 209179 cycles, achieve throughput   375 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207282 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 207852 cycles, achieve throughput   378 MB/s

Overhead test with EDMA CC2 TC2

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    476 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    477 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    477 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1365 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1588 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    476 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    474 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1586 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    589 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    593 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   2365 cycles, achieve throughput     4 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1705 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1366 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1587 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC2 TC2

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 140687 cycles, achieve throughput   558 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 140068 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 140129 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108664 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108739 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 111555 cycles, achieve throughput   704 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 140124 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 140527 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 139794 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108782 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108737 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 111002 cycles, achieve throughput   708 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 140908 cycles, achieve throughput   558 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 139314 cycles, achieve throughput   564 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 140241 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108775 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 108735 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 113001 cycles, achieve throughput   695 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 207620 cycles, achieve throughput   378 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207296 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 207861 cycles, achieve throughput   378 MB/s

Overhead test with EDMA CC2 TC3

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    482 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    478 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    478 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1366 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1590 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    475 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    475 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1587 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1589 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1697 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1364 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1586 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC2 TC3

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 140246 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 140073 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 140463 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108662 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108707 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 112224 cycles, achieve throughput   700 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 140575 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 140309 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 139347 cycles, achieve throughput   564 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108669 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108729 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 111451 cycles, achieve throughput   705 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 140353 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 140885 cycles, achieve throughput   558 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 140350 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108783 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 108931 cycles, achieve throughput   721 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 111559 cycles, achieve throughput   704 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 207734 cycles, achieve throughput   378 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207332 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 207953 cycles, achieve throughput   378 MB/s

Overhead test with EDMA CC3 TC0

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    479 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    475 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    476 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1367 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1588 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    476 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    476 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1589 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1699 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1365 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1588 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC3 TC0

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 115887 cycles, achieve throughput   678 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 116017 cycles, achieve throughput   677 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 116107 cycles, achieve throughput   677 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 149912 cycles, achieve throughput   524 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 155048 cycles, achieve throughput   507 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 168477 cycles, achieve throughput   466 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 116003 cycles, achieve throughput   677 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 115852 cycles, achieve throughput   678 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 116669 cycles, achieve throughput   674 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 149914 cycles, achieve throughput   524 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 155178 cycles, achieve throughput   506 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 168257 cycles, achieve throughput   467 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 116115 cycles, achieve throughput   677 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 116123 cycles, achieve throughput   677 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 117018 cycles, achieve throughput   672 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 150147 cycles, achieve throughput   523 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 155970 cycles, achieve throughput   504 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 174942 cycles, achieve throughput   449 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 247420 cycles, achieve throughput   317 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 247916 cycles, achieve throughput   317 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 250423 cycles, achieve throughput   314 MB/s

Overhead test with EDMA CC3 TC1

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    484 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    476 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    477 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1479 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1589 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1479 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1590 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    593 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1588 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1699 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1588 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC3 TC1

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 115997 cycles, achieve throughput   677 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 115906 cycles, achieve throughput   678 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 116781 cycles, achieve throughput   673 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 150038 cycles, achieve throughput   524 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 155162 cycles, achieve throughput   506 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 166605 cycles, achieve throughput   472 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 115997 cycles, achieve throughput   677 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 115974 cycles, achieve throughput   678 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 116778 cycles, achieve throughput   673 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 150028 cycles, achieve throughput   524 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 155176 cycles, achieve throughput   506 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 170040 cycles, achieve throughput   462 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 116340 cycles, achieve throughput   675 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 116030 cycles, achieve throughput   677 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 116892 cycles, achieve throughput   672 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 150026 cycles, achieve throughput   524 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 155198 cycles, achieve throughput   506 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 173707 cycles, achieve throughput   452 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 247437 cycles, achieve throughput   317 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 247934 cycles, achieve throughput   317 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 251976 cycles, achieve throughput   312 MB/s

Overhead test with EDMA CC4 TC0

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    591 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1480 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1586 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    585 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1479 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1587 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    591 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1474 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1705 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1809 cycles, achieve throughput     5 MB/s

Throughput test with EDMA CC4 TC0

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 139794 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 140082 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 140034 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108778 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108712 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 110992 cycles, achieve throughput   708 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 139574 cycles, achieve throughput   563 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 140608 cycles, achieve throughput   559 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 139462 cycles, achieve throughput   563 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108660 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108726 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 111440 cycles, achieve throughput   705 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 140804 cycles, achieve throughput   558 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 140304 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 140243 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108671 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 108739 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 113333 cycles, achieve throughput   693 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 207614 cycles, achieve throughput   378 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207297 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 207959 cycles, achieve throughput   378 MB/s

Overhead test with EDMA CC4 TC1

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1477 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1587 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1479 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1588 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes    586 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1475 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1478 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1697 cycles, achieve throughput     5 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes   1480 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x54800000(remote MSRAM) to 0x54840000(local MSRAM ), consumes   1476 cycles, achieve throughput     6 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes   1593 cycles, achieve throughput     6 MB/s

Throughput test with EDMA CC4 TC1

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54920000(remote LL2  ), consumes 140019 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x54840000(remote MSRAM), consumes 139956 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x51800000(remote DDRA ), consumes 139917 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x10840000(local LL2   ), consumes 108774 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x10840000(local LL2   ), consumes 108718 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x10840000(local LL2   ), consumes 110782 cycles, achieve throughput   709 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54920000(remote LL2  ), consumes 139908 cycles, achieve throughput   562 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x54840000(remote MSRAM), consumes 140275 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x51800000(remote DDRA ), consumes 140241 cycles, achieve throughput   560 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes 108664 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes 108730 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes 111220 cycles, achieve throughput   707 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54920000(remote LL2  ), consumes 140151 cycles, achieve throughput   561 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x54840000(remote MSRAM), consumes 140839 cycles, achieve throughput   558 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x51800000(remote DDRA ), consumes 139574 cycles, achieve throughput   563 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x88100000(local DDR   ), consumes 108664 cycles, achieve throughput   723 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x88100000(local DDR   ), consumes 108821 cycles, achieve throughput   722 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x88100000(local DDR   ), consumes 112111 cycles, achieve throughput   701 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54900000(remote LL2  ) to 0x54920000(remote LL2  ), consumes 207728 cycles, achieve throughput   378 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x54800000(remote MSRAM) to 0x54840000(remote MSRAM), consumes 207386 cycles, achieve throughput   379 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x50800000(remote DDRA ) to 0x51800000(remote DDRA ), consumes 208066 cycles, achieve throughput   377 MB/s

EDMA test complete

PCIE test complete.