KeyStone 2

SPI Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial release |

Contents

1	Overview of KeyStone II SPI	3

2	Test mode and test cases	3

2.1	SPI Loopback Test	4

2.2	SPI EDMA Test	5

2.3	SPI FLASH Test	5

2.3.1	Data filling Test	5

2.3.2	Addressing Test	5

3	Test project configuration	6

3.1	CCS project	6

3.2	To run the test program on KeyStone EVM	8

3.3	Test configuration	8

3.4	Migrate the test to custom board	9

References	10

Appendix: Typical test output	11

Figures

Figure 1.	KeyStone II SPI functional block diagram	3

Figure 2.	KeyStone II SPI internal loopback	4

Figure 3.	Directory structure of SPI project	7

Figure 4.	define symbols in CCS compiler options to select device and board	9

Tables

Table 1.	Source files of the SPI codes	7

- Overview of KeyStone II SPI

Figure 1 shows the functional block diagram for SPI. The SPI is a high-speed synchronous serial input/output port that allows a serial bit stream of programmed length (2 to 16 bits) to be shifted into and out of the device at a programmed bit-transfer rate. The SPI is normally used for communication between the device and external peripherals, such as FLASH.

- KeyStone II SPI functional block diagram

Keystone SPI has the following features:

- Programmable SPI clock frequency range

- Programmable character length(2 to 16 bits)

- Programmable clock phase(delay or no delay)

- Programmable clock polarity(high or low)

- Interrupt capability

- DMA support

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test mode and test cases

The SPI test program covers following test case:

- internal loopback

- EDMA loopback

- external FLASH (N25Qxxx FLASH)

The basic test method is to fill destination memory with fixed pattern (0, 0xFF, 0x55, 0xAA) or address, and then read back to verify it.

  - SPI Loopback Test

The internal loopback test can be used to check the transmit path and receive path including the transmit and receive buffers in the Keystone2 SOC. In loopback mode, the SPICLK, SPISOMI, SPISIMO pins should be set as functional pins, therefore the SPISIMO pin is internally connected to the SPISOMI pin. The transmit data is looped back as receive data and is stored in the receive buffer.

- KeyStone II SPI internal loopback

If internal loopback test passes, while external test fails, then, it normally means internal modules work well, the problem should on external signals or the other device connected to SPI. Normally, we should check the signal integrity on PCB.

  - SPI EDMA Test

EDMA can be used to avoid CPU overhead in handling the SPI traffic. The SPI module has two DMA synchronization event outputs for receive (REVT) and transmit (XEVT), allowing DMA transfers to be triggered by SPI read receive and write transmit events. The SPI module enables DMA requests by enabling the DMA request enable (DMAREQEN) bit in the SPI interrupt register (SPIINT0). When a character is to be transmitted the SPI module signals the DMA via the XEVT signal. The DMA controller then transfers the data from the source buffer into the SPI transmit data register (SPIDAT1). When a character is received, the SPI module signals the DMA via the REVT signal. The DMA controller then reads the data from the SPI receive buffer register (SPIBUF) and transfers it to a destination buffer for ready access.

In the SPI EDMA test, the SPI is set in loopback mode. EDMA is used to send data from TX buffer to SPI, and receive data from SPI to RX buffer, and then, the data in TX and RX buffer is compared. The total transfer time is recorded and the throughput is calculated by taking total bytes transferred and dividing it by the time it used.

  - SPI FLASH Test

SPI FLASH or FLASH is the most common device connected to SPI, the STK SPI FLASH test is designed for N25Qxxx, it should be compatible with most of other SPI FLASH.

Two algorithms are used for SPI FLASH test.

    - Data filling Test

The pseudocode for SPI data pattern filling test is:

for(memory range under test)

fill the memory with a value;

for(memory range under test)

read back the memory and compare the readback value to the written value

The data filling is tested with different data pattern. The common values used for test include 0, 0xFFFFFFFF, 0x12345678, 0xAAAAAAAA. This test can detect stuck data bit, for example, if written value = 0, readback value = 0x8, It indicates bit 3 sticks to 1. If written value = 0xFFFFFFFF, readback value = 0xFFFFFFFE, It indicates bit 0 sticks to 0.

    - Addressing Test

The pseudocode for addressing test is:

for(memory range under test)

fill each memory unit with its address value;

for(memory range under test)

read back the memory and compare the readback value to the written value

This test can detect stuck bit on address bus, for example, if

written value = 0 at address 0

written value = 1 at address 1

written value = 2 at address 2

written value = 3 at address 3

……

readback value = 2 at address 0

readback value = 3 at address 1

readback value = 2 at address 2

readback value = 3 at address 3

……

For this case, it indicates bit 1 of address bus sticks. So, the contents in address 0 and 2 are same, and the contents in address 1 and 3 are same.

- Test project configuration

This section introduces the implementation of SPI test on KeyStone II devices.

  - CCS project

The folders of the test project are as below:

- Directory structure of SPI project

The project files are in “SPI” folder. There is some commonly used initialization and driver code for EDMA and PLL… in “common” folder. The main source code files are in the “SPI\src” subfolder. Below table describes these source files.

- Source files of the SPI codes

| Source files | Descriptions |
| --- | --- |
| K2_SPI_Init_drv.c | SPI initialization and SPI Tx/Rx transfer function. |
| SPI_EDMA_Test.c | this source code implements below functionality: - EDMA initialization - EDMA transfer parameter configuration - Transmission data verification |
| SPI_loopback_test.c | Internal loopback test, which transfers data from source buffer in K2 device to destination buffer also in the same device through SPI. |
| SPI_FLASH_drv.c | Implementation of low level operations on SPI FLASH. |
| SPI_FLASH_Test.c | Fill SPI FLASH with fixed pattern or address, and then verify them. |
| SPI_main.c | High level configuration and control for SPI loopback/EDMA/FLASH test. |

  - To run the test program on KeyStone EVM

The project can be run on all K2 EVM.

The steps to run the test cases on EVM board are:

- Extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- Load the program to ARM core 0 of a device.

- Run the core, you should see output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

Loopback, EDMA or FLASH test can be configured to be tested or not through the following macros. The value of “1” means enable the test, and “0” means disable the test.

#define SPI_LOOPBACK_TEST 	1

#define SPI_FLASH_TEST 		1

#define SPI_EDMA_TEST           		1

  - Migrate the test to custom board

This SPI example is implemented based on TI’s EVM boards. In your real system, the SPI FLASH configuration may be changed according to your hardware design in “K2_Board_Init.h”.

#define SPI_FLASH_SIZE_BYTES	(16*1024*1024)

#define SPI_FLASH_SPEED_KHZ	54000

For SPI devices different from FLASH, you may need change many of the SPI configurations in SPI_Data_Format configuration structure, following is the example for the SPI FLASH test in “SPI_FLASH_drv.c”:

/*data format for FLASH test*/

SPI_Data_Format FLASHDataFormat =

{

/*.delayBetweenTrans_ns=*/0,

/*.ShifDirection       =*/SPI_MSB_SHIFT_FIRST,

/*.disable_CS_timing   =*/0,

/*.clockPolarity       =*/SPI_CLOCK_LOW_INACTIVE,

/*.clockPhase          =*/1,

/*.clockSpeedKHz       =*/ SPI_FLASH_SPEED_KHZ,

/*.wordLength          =*/8

};

The core speed may be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone Architecture Serial Peripheral Interface User Guide (SPRUGP2)

Appendix: Typical test output

Below is the test output on K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

DDR3A initialization

Initialize DDR data rate = 100.000/1*20/6*4= 1333.3 MTS, bus width = 64 bits.

SPI port 0 internal loopback test at 66MHz...

SPI loopback test passed with data pattern 0x0. Throughput= 30Mbps

SPI loopback test passed with data pattern 0xffff. Throughput= 30Mbps

SPI loopback test passed with data pattern 0x5555. Throughput= 30Mbps

SPI0 interrupt test: manually generate RX overrun error...

SPI0 receive buffer overrun interrupt happened at 42082233

SPI port 1 internal loopback test at 66MHz...

SPI loopback test passed with data pattern 0x0. Throughput= 30Mbps

SPI loopback test passed with data pattern 0xffff. Throughput= 30Mbps

SPI loopback test passed with data pattern 0x5555. Throughput= 30Mbps

SPI1 interrupt test: manually generate RX overrun error...

SPI1 receive buffer overrun interrupt happened at 195537569

SPI port 2 internal loopback test at 66MHz...

SPI loopback test passed with data pattern 0x0. Throughput= 30Mbps

SPI loopback test passed with data pattern 0xffff. Throughput= 30Mbps

SPI loopback test passed with data pattern 0x5555. Throughput= 30Mbps

SPI2 interrupt test: manually generate RX overrun error...

SPI2 receive buffer overrun interrupt happened at 348993931

SPI port 0 EDMA test at 66MHz...

SPI0 EDMA loopback test passed. Throughput= 46Mbps

SPI port 1 EDMA test at 66MHz...

SPI1 EDMA loopback test passed. Throughput= 46Mbps

SPI port 2 EDMA test at 66MHz...

SPI2 EDMA loopback test passed. Throughput= 46Mbps

SPI port 0 NOR_FLASH test at 54MHz...

Read SPI NOR FLASH ID = 0x20 0xbb 0x18

Save data from 0xf80000 to 0xfc0000.

Passed SPI_NOR_FLASH Fill Test from 0xf80000 to 0xfc0000 with pattern 0x       0

Passed SPI_NOR_FLASH Fill Test from 0xf80000 to 0xfc0000 with pattern 0xaaaaaaaa

Passed SPI_NOR_FLASH Address Test from 0xf80000 to 0xfc0000

Restore data from 0xf80000 to 0xfc0000.

Save data from 0xfc0000 to 0x1000000.

Passed SPI_NOR_FLASH Fill Test from 0xfc0000 to 0x1000000 with pattern 0x       0

Passed SPI_NOR_FLASH Fill Test from 0xfc0000 to 0x1000000 with pattern 0xaaaaaaaa

Passed SPI_NOR_FLASH Address Test from 0xfc0000 to 0x1000000

Restore data from 0xfc0000 to 0x1000000.

SPI test complete at 34334900813 cycles.