KeyStone 2

Timer Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial Release |

Contents

1	Introduction	3

1.1	Generate square wave with Timer and EDMA	3

2	Test code/project	4

2.1	CCS project	4

2.2	To run the test program on KeyStone 2 EVM	5

2.3	Test configuration	5

2.4	Migrate the test to custom board	7

References	7

Appendix: Typical test output	8

Figures

Figure 1.	Generate square waves with Timer + EDMA	3

Figure 2.	Directory structure of test project	4

Figure 3.	define symbols in CCS compiler options to select device and board	6

Tables

Table 1.	Source files of the test codes	4

- Introduction

Timer on KeyStone 2 devices is very flexible. The Timer STK (Self Test Kit) is designed to verify four main features of Timer:

- generate one-shot pulse and interrupt

- generate continual clock and interrupts

- generate continual square wave and interrupts

- watch-dog

The pulse, clocks are output on TIMO0 pin in these tests. The square waves are output on TIMO1. The difference between square wave and clock is the duty cycle of clock is always 50%.

Interrupt Service Routine is setup to monitor and verify if the interrupts are generated as expected.

Watch-dog timer is setup up, and be serviced for sometimes, and then stop servicing the watch-dog timer and let it trigger exception, a pulse is also generated on TIMO0.

Exception service routine is setup to monitor and verify if the exception is triggered by watch-dog timer as expected.

The number of the timer for square wave test is 8; the number of the timer for other tests is 16+N, where N is the ARM core number, because only timer 16+N can be used as watch dog timer for core N.

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

  - Generate square wave with Timer and EDMA

Period reload function of timer plus mode and EDMA is used to generate the square waves, following figure shows the setup:

- Generate square waves with Timer + EDMA

Please note, there are some limitation on K2 between EDMA and Timer:

- Only some EDMA TCs can access timer register;

- Only some timers can trigger some EDMA CCs;

So, in the test, Timer 8 combined with EDMA CC1 channel 22 is used.

In plus mode, the

The key initialization parameters of timer include:

- PLUSEN= 1 (enable plus features)

- ENAMODE= CONT_RELOAD (continuously period reload);

- TIMMODE= DUAL_UNCHAINED (dual 32-bit timers unchained mode);

- PRDLO = low period value;

- RELLO = high period value;

- Test code/project

This section introduces the implementation of these tests on KeyStone 2 devices.

  - CCS project

Following figure shows the directory structure of the test project

- Directory structure of test project

The project files are in “Timer” folder. There is some commonly used initialization and driver code for Timer, interrupt, exception and PLL… in “common” folder. The main source code files are in the “Timer\src” subfolder. Below table describes these source files.

- Source files of the test codes

| Function | Source file | Description |
| --- | --- | --- |
| Timer64_Init | \common\ K2_common.c | configure timer |
| KeyStone_Exception_cfg |  | Exception configuration |
| FIQ_Handler, IRQ_Handler |  | Interrupt/Exception Service routine |
| KeyStone_main_PLL_init, KeyStone_PLL_init |  | Main PLL and ARM core PLL configuration |
| generate_pulse_by_timer | \Timer\src\ timer_main.c | use local timer to generate a pulse on TIMO0, an interrupt is generated as well |
| generate_clocks_by_timer |  | use local timer to generate a clock on TIMO0, interrupts are generated as well |
| generate_waves_by_timer |  | use timer 8 to generate square waves on TIMO1, interrupts are generated as well. EDMA is used to reload period register |
| watchdog_timer_test |  | set local timer as watch dog timer, service the watch-dog for sometimes, and then stop servicing watch-dog, let it trigger NMI |
| Timer_Interrupts_Init |  | Initialize interrupt of timer |
| Timer_ISR |  | Interrupt service routine for timer |

  - To run the test program on KeyStone 2 EVM

The test program can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- load the program to any ARM core of a device.

- run the core, then, you should see output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

The period or delay of the timer can be changed with following macro at the beginning of “timer_main.c”.

/*delay (in millisecond) before the timer generate pulse*/

#define PULSE_DELAY_MS 			2000

/*period (in millisecond) of the clock generated by timer*/

#define CLOCK_PERIOD_MS 		    1

/*number of the clocks generated in this test*/

#define NUM_CLOCKS_GENERATED 	500

/*period (in millisecond) of the waveform generated by timer*/

#define WAVE_PERIOD_MS 		1

/*Duty cycle of the waveform (percentage of low period)*/

#define WAVE_LOW_PERCENT 		66

/*number of the waves generated in this test*/

#define NUM_WAVES_GENERATED 	500

/*period (in millisecond) of the watch-dog timer*/

#define WATCH_DOG_PERIOD_MS 	    2000

The period or delay are defined in millisecond with above macro, they will be converted and configured to the PRD (period) register of timer.

Please note, the timer runs at (main core clock)/6, the conversion between millisecond and timer PRD value is:

PRD = (xxx_MS/1000)*MAIN_CLK_HZ/6

= xxx_MS*(MAIN_CLK_HZ/1000)/6

If you only want to run one or two of the four test cases, just comment one or two of the four function calls in main function:

generate_pulse_by_timer();

generate_clocks_by_timer();

generate_waves_by_timer();

watchdog_timer_test();

  - Migrate the test to custom board

These tests are implemented based on TI’s EVM boards. In your real system, the core speed may be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone Architecture Timer64 User Guide (SPRUGV5)

- KeyStone II Architecture ARM CorePac User's Guide (SPRUHJ4)

Appendix: Typical test output

Below is the test output on K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

a pulse will be generated on TIMO0...

a pulse was generated on TIMO0

clocks will be generated on TIMO0...

clocks were generated on TIMO0

Square waves will be generated on TIMO0...

Square waves were generated on TIMO0

start watch-dog timer...

service watch-dog 1 times, at time counter = 20

service watch-dog 2 times, at time counter = 41471515

service watch-dog 3 times, at time counter = 40012113

service watch-dog 4 times, at time counter = 41536967

service watch-dog 5 times, at time counter = 41584600

service watch-dog 6 times, at time counter = 40370709

service watch-dog 7 times, at time counter = 41608429

service watch-dog 8 times, at time counter = 41346981

service watch-dog 9 times, at time counter = 41198330

service watch-dog 10 times, at time counter = 40607732

!!!stop servicing watch-dog, it will timeout and trigger exception after 3000 ms...

Watch dog timer for ARM core 0 expired at program address 0xc0076a4!