KeyStone 2

USIM Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 11, 2015 | Brighton Feng | Initial release |

Contents

1	Overview	3

2	Test Methods and test cases	3

2.1	Get USIM ATR (Answer To Reset)	3

2.2	Read USIM ICCID (IC Card ID)	3

3	Test Project	4

3.1	CCS Project	4

3.2	To Run the Test Program on KeyStone 2 EVM	5

3.3	Test Configurations	5

3.4	Migrate the Project to Customs’ Boards	7

References	7

Appendix: Typical test output	8

Figures

Figure 1.	Directory structure of test projects	4

Figure 2.	define symbols in CCS compiler options to select device or board	6

Tables

Table 1.	Source files of the test codes	4

- Overview

The USIM interface on KeyStone2 devices supports both mobile phone SIM card and EMV card. The USIM STK only supports mobile phone SIM card (asynchronous with the T = 0 protocol) for now.

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test Methods and test cases

  - Get USIM ATR (Answer To Reset)

When the SIM card is activated (released from RESET), SIM card should send ATR string to controller, STK will output error information if no ATR is received by controller. If the controller receives ATR, the STK parsers some basic information in the ATR including:

- coding convention, direct or inverse;

- Fi/Di. Normally, Fi= 512; Di is multiple of 8;

- extra Guard Time integer (N). Normally, it is 0.

- Length of ATR, should not larger than 33.

This information can be used to determine if the ATR is correct.

Only the RX part of the USIM interface is tested with this case.

  - Read USIM ICCID (IC Card ID)

Each SIM card is internationally identified by its integrated circuit card identifier (ICCID). ICCID is stored in the SIM card and is also printed on the SIM card body.

The STK reads and outputs the ICCID, user can compare the ICCID read by STK and the ICCID printed on the SIM card to determine if the test passed.

The ICCID is saved in the SIM card as a file. A typical sequence to read it is:

- Send SELECT command;

- Receive response;

- Send file ID of ICCID;

- Receive response;

- Send GET_RESPONSE command;

- Receive file properties including file size;

- Send READ_BINARY command;

- Receive data including ICCID.

Both TX and RX parts of the USIM interface are tested with this case.

- Test Project

This section introduces the implementation of the USIM tests on KeyStone 2 devices.

  - CCS Project

Following figure shows the directory structure of the test projects

- Directory structure of test projects

The project files are in USIM folder. There is some commonly used initialization and driver code for USIM and PLL… in common folder. The main source code files are in the USIM->src subfolder. Below table describes these source files.

- Source files of the test codes

| Source files | Descriptions |
| --- | --- |
| USIM_main.c | Implementation of main USIM tests, the key APIs include: void USIM_config(); void USIM_test(); |
| K2_USIM_Init_drv.c | Implementation of USIM initialization and driver level APIs. The key APIs are: void K2_USIM_Init(USIM_Config * cfg); void K2_USIM_activate_card(USIM_FIFO_Config * fifo_cfg); void K2_USIM_read_ICCID(); Uint32 K2_USIM_read(Uint8 * Rx_Buf, Uint32 uiBufLength); void K2_USIM_transmit(Uint8 * data, Uint32 uiLength); Uint32 K2_USIM_IRQ_handler(); |
| USIM_Interrupt.c | Implementation of USIM interrupt initialization and ISR: void K2_USIM_ ISR(); void K2_USIM_Interrupts_Init(); |

  - To Run the Test Program on KeyStone 2 EVM

The test program can be run on K2H, K2K and K2L EVM.

The steps to run the test program on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder);

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- Insert SIM card into the card connector on EVM.

- Select proper voltage for the card through jumper (CN6 on the K2L EVM, CN21 on K2H/K EVM). By default it is 1.8V. Some old SIM card may only work with 3V, most new card work with both 1.8V and 3V. If you are not sure, please start with 1.8V, if it does not work, then change to 3V.

- load the program to ARM core 0.

- Run the core 0, then, you should see output in console window.

Refer to appendix of this document to see the typical test output.

  - Test Configurations

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2L

- define symbols in CCS compiler options to select device or board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2L_EVM

The USIM configurations are configured in function “USIM_config” which defined in “USIM_main.c” as below. You can modify the parameters in the code.

usim_fifo_cfg.rx_FIFO_trigger_thresh= 64;

usim_fifo_cfg.tx_FIFO_trigger_thresh= 15;

usim_cfg.sim_clk_div= USIM_CLK_DIV_10_MASK;

usim_cfg.CGT= 12; 	//in number of ETU, >=12

usim_cfg.CWT= 8203; 	//in number of ETU, >=11

usim_cfg.BWT= 15371; 	//in number of ETU, >=12

usim_cfg.WAITI= 9; //WWT= (WAITI+1)*960*ETU, WI*D=CONFWAITI+1.

usim_cfg.maxParityRetries= 3;

usim_cfg.parErrLevel= 1;

usim_cfg.interruptEnableMask=  	//value for IRQENABLE register

CSL_USIMOCP_IRQENABLE_TS_ERR_EN_MASK

|CSL_USIMOCP_IRQENABLE_TOC_EN_MASK

|CSL_USIMOCP_IRQENABLE_NATR_EN_MASK

  - Migrate the Project to Customs’ Boards

These tests are implemented based on TI’s EVM boards. In your real system, the core speed may be changed through following macros in “K2_Board_Init.h” in common folder:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- USIM (Universal Subscriber Identity Module) User Guide (SPRUHD2)

- 3GPP TS 31.101: UICC-terminal interface; Physical and logical characteristics

- ETSI TS 102 221: Smart Cards; UICC-Terminal interface; Physical and logical characteristics

- 3GPP TS 31.102: Characteristics of the Universal Subscriber Identity Module (USIM) application

- http://en.wikipedia.org/wiki/Answer_to_reset

Appendix: Typical test output

Below is the test output on K2L EVM for tests not including external loopback.

JTAG ID= 0x0b9a702f. This is a K2L device, version variant = 0

DEVSTAT= 0x0e000ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0400500a, 0x0c00ea81, 0x00000000, 0x5f260020

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 100.00MHz/1x12 = 1200MHz

Initialize ARM clock = 100.00MHz/1x12 = 1200MHz

Get 16 bytes in ATR: 0x3b 0x7b 0x94 0x00 0x00 0x80 0x65 0x35 0x02 0xff 0x86 0x53 0x83 0x00 0x00 0x00

TS= 0x3b, direct coding convention.

TA= 0x94, Fi= 512, Di= 8

TB= 0x00

TC, extra Guard Time integer (N) = 0

number of Ti in ATR = 11.

Select ICCID file...

Transmit 5 bytes: a0 a4 00 00 02

Received 1 bytes: a4

Transmit 2 bytes: 2f e2

Received 2 bytes: 9f 0f

Transmit 5 bytes: a0 c0 00 00 0f

Received 18 bytes: c0 00 00 00 0a 2f e2 04 00 04 f0 44 01 02 00 00 90 00

Read ICCID...

Transmit 5 bytes: a0 b0 00 00 0a

Received 13 bytes: b0 98 68 30 01 07 57 25 53 74 39 90 00

ICCID = 89860310707552354793

USIM test complete.