Traces:

| Trace | Start | Step | Stop | IF Bandwidth | Detector | Dwell time | Attenuation | Transducer | LISN | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scan 1 | 30.0 MHz | 50.0 kHz | 300.0 MHz | 120kHz | QP | 1.0 s | Auto | TDF1, TDF2, TDF3 | None- |  |

Transducers:

TDF1: cdne20db

TDF2: Cable MC 38377 - GAUSS Instruments (DC - 3GHz)

TDF3: Testec BNC cable

Limit lines:

CISPR15ED9: CISPR 15 ED9 QP,CDNE, 30MHz-300MHz

Scan1: 30.0 MHz, 50.0 kHz, 300.0 MHz; IF:120kHz, 1.0 s QP, Att AutodB

| f | Mag [dBµV] | Limit | Diff | Name | Comment |
| --- | --- | --- | --- | --- | --- |
| 79.888 MHz | 46.39 | 55.87 | 9.47 | CISPR15ED9 |  |
| 137.927 MHz | 51.03 | 54.00 | 2.97 | CISPR15ED9 |  |
| 148.164 MHz | 54.33 | 54.00 | -0.33 | CISPR15ED9 | Failed |
| 149.267 MHz | 54.50 | 54.00 | -0.50 | CISPR15ED9 | Failed |
| 149.424 MHz | 54.58 | 54.00 | -0.58 | CISPR15ED9 | Failed |
| 150.684 MHz | 54.66 | 54.00 | -0.66 | CISPR15ED9 | Failed |
| 151.157 MHz | 54.69 | 54.00 | -0.69 | CISPR15ED9 | Failed |
| 152.574 MHz | 54.65 | 54.00 | -0.65 | CISPR15ED9 | Failed |
| 153.047 MHz | 54.57 | 54.00 | -0.57 | CISPR15ED9 | Failed |
| 153.834 MHz | 54.49 | 54.00 | -0.49 | CISPR15ED9 | Failed |
| 154.543 MHz | 54.36 | 54.00 | -0.36 | CISPR15ED9 | Failed |
| 155.409 MHz | 54.21 | 54.00 | -0.21 | CISPR15ED9 | Failed |
| 194.548 MHz | 39.59 | 54.00 | 14.41 | CISPR15ED9 |  |
| 266.408 MHz | 34.42 | 51.88 | 17.46 | CISPR15ED9 |  |