Traces:

| Trace | Start | Step | Stop | IF Bandwidth | Detector | Dwell time | Attenuation | Transducer | LISN | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scan 1 | 30.0 MHz | 50.0 kHz | 300.0 MHz | 120kHz | QP | 1.0 s | Auto | TDF1, TDF2, TDF3 | None- |  |

Transducers:

TDF1: cdne20db

TDF2: Cable MC 38377 - GAUSS Instruments (DC - 3GHz)

TDF3: Testec BNC cable

Limit lines:

CISPR15ED9: CISPR 15 ED9 QP,CDNE, 30MHz-300MHz

Scan1: 30.0 MHz, 50.0 kHz, 300.0 MHz; IF:120kHz, 1.0 s QP, Att AutodB

| f | Mag [dBµV] | Limit | Diff | Name | Comment |
| --- | --- | --- | --- | --- | --- |
| 80.006 MHz | 48.47 | 55.85 | 7.38 | CISPR15ED9 |  |
| 93.236 MHz | 48.27 | 54.58 | 6.31 | CISPR15ED9 |  |
| 143.243 MHz | 54.06 | 54.00 | -0.06 | CISPR15ED9 | Failed |
| 143.636 MHz | 54.21 | 54.00 | -0.21 | CISPR15ED9 | Failed |
| 143.794 MHz | 54.24 | 54.00 | -0.24 | CISPR15ED9 | Failed |
| 143.951 MHz | 54.29 | 54.00 | -0.29 | CISPR15ED9 | Failed |
| 143.991 MHz | 54.36 | 54.00 | -0.36 | CISPR15ED9 | Failed |
| 144.266 MHz | 54.28 | 54.00 | -0.28 | CISPR15ED9 | Failed |
| 144.463 MHz | 54.20 | 54.00 | -0.20 | CISPR15ED9 | Failed |
| 144.581 MHz | 54.19 | 54.00 | -0.19 | CISPR15ED9 | Failed |
| 144.739 MHz | 54.15 | 54.00 | -0.15 | CISPR15ED9 | Failed |
| 144.896 MHz | 54.11 | 54.00 | -0.11 | CISPR15ED9 | Failed |
| 194.588 MHz | 38.94 | 54.00 | 15.06 | CISPR15ED9 |  |
| 282.236 MHz | 33.75 | 51.45 | 17.71 | CISPR15ED9 |  |