Traces:

| Trace | Start | Step | Stop | IF Bandwidth | Detector | Dwell time | Attenuation | Transducer | LISN | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scan 1 | 30.0 MHz | 50.0 kHz | 300.0 MHz | 120kHz | QP | 1.0 s | Auto | TDF1, TDF2, TDF3 | None- |  |

Transducers:

TDF1: cdne20db

TDF2: Cable MC 38377 - GAUSS Instruments (DC - 3GHz)

TDF3: Testec BNC cable

Limit lines:

CISPR15ED9: CISPR 15 ED9 QP,CDNE, 30MHz-300MHz

Scan1: 30.0 MHz, 50.0 kHz, 300.0 MHz; IF:120kHz, 1.0 s QP, Att AutodB

| f | Mag [dBµV] | Limit | Diff | Name | Comment |
| --- | --- | --- | --- | --- | --- |
| 79.849 MHz | 48.65 | 55.87 | 7.22 | CISPR15ED9 |  |
| 137.927 MHz | 50.83 | 54.00 | 3.17 | CISPR15ED9 |  |
| 141.392 MHz | 54.64 | 54.00 | -0.64 | CISPR15ED9 | Failed |
| 142.140 MHz | 55.09 | 54.00 | -1.09 | CISPR15ED9 | Failed |
| 143.046 MHz | 55.32 | 54.00 | -1.32 | CISPR15ED9 | Failed |
| 143.676 MHz | 55.42 | 54.00 | -1.42 | CISPR15ED9 | Failed |
| 143.991 MHz | 55.42 | 54.00 | -1.42 | CISPR15ED9 | Failed |
| 144.778 MHz | 55.01 | 54.00 | -1.01 | CISPR15ED9 | Failed |
| 145.526 MHz | 54.50 | 54.00 | -0.50 | CISPR15ED9 | Failed |
| 147.062 MHz | 54.24 | 54.00 | -0.24 | CISPR15ED9 | Failed |
| 147.377 MHz | 54.25 | 54.00 | -0.25 | CISPR15ED9 | Failed |
| 148.007 MHz | 54.16 | 54.00 | -0.16 | CISPR15ED9 | Failed |
| 194.666 MHz | 39.30 | 54.00 | 14.70 | CISPR15ED9 |  |
| 282.236 MHz | 33.49 | 51.45 | 17.97 | CISPR15ED9 |  |