Traces:

| Trace | Start | Step | Stop | IF Bandwidth | Detector | Dwell time | Attenuation | Transducer | LISN | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scan 1 | 30.0 MHz | 50.0 kHz | 300.0 MHz | 120kHz | QP | 1.0 s | Auto | TDF1, TDF2, TDF3 | None- |  |

Transducers:

TDF1: cdne20db

TDF2: Cable MC 38377 - GAUSS Instruments (DC - 3GHz)

TDF3: Testec BNC cable

Limit lines:

CISPR15ED9: CISPR 15 ED9 QP,CDNE, 30MHz-300MHz

Scan1: 30.0 MHz, 50.0 kHz, 300.0 MHz; IF:120kHz, 1.0 s QP, Att AutodB

| f | Mag [dBµV] | Limit | Diff | Name | Comment |
| --- | --- | --- | --- | --- | --- |
| 79.927 MHz | 45.49 | 55.86 | 10.38 | CISPR15ED9 |  |
| 137.966 MHz | 47.86 | 54.00 | 6.14 | CISPR15ED9 |  |
| 168.521 MHz | 54.21 | 54.00 | -0.21 | CISPR15ED9 | Failed |
| 168.836 MHz | 54.35 | 54.00 | -0.35 | CISPR15ED9 | Failed |
| 169.466 MHz | 54.44 | 54.00 | -0.44 | CISPR15ED9 | Failed |
| 170.136 MHz | 54.50 | 54.00 | -0.50 | CISPR15ED9 | Failed |
| 170.805 MHz | 54.59 | 54.00 | -0.59 | CISPR15ED9 | Failed |
| 171.356 MHz | 54.61 | 54.00 | -0.61 | CISPR15ED9 | Failed |
| 171.829 MHz | 54.71 | 54.00 | -0.71 | CISPR15ED9 | Failed |
| 172.301 MHz | 54.62 | 54.00 | -0.62 | CISPR15ED9 | Failed |
| 172.577 MHz | 54.54 | 54.00 | -0.54 | CISPR15ED9 | Failed |
| 173.089 MHz | 54.32 | 54.00 | -0.32 | CISPR15ED9 | Failed |
| 194.784 MHz | 38.97 | 54.00 | 15.03 | CISPR15ED9 |  |
| 282.236 MHz | 34.11 | 51.45 | 17.34 | CISPR15ED9 |  |