TI chip：dra725

TI  sdk version：tisdk-rootfs-image-dra7xx-evm_vsdk_3_5

1、Switch machine test repeatedly, boot (14 v 1A 30 seconds) shutdown (0 v 10 seconds)

2、Phenomenon of the problem: After the test, The screen does not display, read the relevant register values, found DISPC_GFX_ATTRIBUTES values change
Relating to register:

| Register | Normal value | fault value |
| --- | --- | --- |
| DISPC_CONFIG1 | 0x00000C04 | NULL |
| DISPC_CONFIG2 | 0x00000000 | NULL |
| DISPC_CONFIG3 | 0x00000000 | NULL |
| DISPC_GLOBAL_ALPHA | 0xFFFFFFFF | NULL |
| DISPC_TRANS_COLOR0 | 0x00000000 | NULL |
| DISPC_TRANS_COLOR1 | 0x00000000 | NULL |
| DISPC_TRANS_COLOR2 | 0x00000000 | NULL |
| DISPC_TRANS_COLOR3 | 0x00000000 | NULL |
| DISPC_GFX_ATTRIBUTES | 0x0E000891 | 0x0E0000A0 |
| DISPC_VID1_ATTRIBUTES | 0x02288873 | NULL |
| DISPC_VID2_ATTRIBUTES | 0x06008400 | NULL |
| DISPC_VID3_ATTRIBUTES | 0x0A288A60 | NULL |