# JIANGMEN GLORY FAITH PCB COMPANY LIMITED

## Information Clarification Sheet

Tel:0750-3868888              Fax：0750-3868800                  CR-PE(MI)-008

Cust. name：PST791        Cust. P/N：70661340R2F         GF P/N： 6791139A0

To:     MKT                Date:  2011-9-13   Page:     1    of   2

From: GF PE Dept.          Prepared By:  	YYM 	  Reviewed by：  WZH

Question (1)

Question description: Please refer to the enclosed picture, We can’t control impedance 50 ohm +/-10%, because the space of trace is too small (only 4mil). If we will insure impedance value , so the space of trace have only 1.6mil after compensated .

GF Suggestion : In order to meet our company production, please allow to increase impedance value to build, L1/L6 :55 ohm+/-10%, L3: 60 ohm+/-10%.

Customer Reply:    □ Accept             □ Further  advice

Remark:

Question (2)

Question description: If customer accept suggestion of question(1), so we will control trace width tolerance as +/-30% to insure 4mil impedance value.

GF Suggestion : please kindly confirm.

Customer Reply:    □ Accept             □ Further  advice

Remark:

Question (3)

Question description: Please refer to the enclosed picture, we don’t have this material in warehouse per your drawing provided last time .

GF Suggestion : Ignore it, we would like to use common KB material as FR-4 to build. Please kindly confirm.

Customer Reply:    □ Accept             □ Further  advice

Remark:

Question (4)

Question description: Please refer to the enclosed picture, for customer layer stack-up, we can’t insure finished thickness 1.2+/-0.12mm, and we also haven’t core of 3mil in warehouse.

GF Suggestion : Ignore them, we would like to follow below “A” picture to build, please kindly confirm.

Customer Reply:    □ Accept             □ Further  advice

Remark:

A

Thanks and best regards!