线性汇编问题：

基本的C程序入下，实现的功能为读取length长度的value数据，寻找最大值，将其存为value数据的第一个数。为冒泡排序的一部分程序。

void Sorting(UInt8 value[], int length)

{

int cntr;

UInt8	maxValue;

maxValue = value[0];

for (cntr = 1; cntr < length; cntr++)

{

if (value[cntr] > maxValue)

{

maxValue = value[cntr];

}

}

value[0] = maxValue;

}

为了便于分析给出了此方案循环中的相关图：

编写线性汇编函数如下：

.sect     ".text:_sortingASMNew"

.global	_sortingASMNew

_sortingASMNew:	.cproc  value, length

.no_mdep

.reg tempvalue, absvalue, maxvalue

.reg	cntr, ifvalue, addr, tempmax

length, cntr

MV			value, addr

LDB		*addr++, maxvalue

ABS		maxvalue, maxvalue

MV			maxvalue, tempmax

LOOP:	.trip		100

LDB.D1		*addr++, tempvalue

CMPGT.L1	maxvalue, tempvalue , ifvalue

NOP

[ifvalue] MV.L1 	tempvalue, maxvalue

[cntr]	ADD.L2	-1, cntr, cntr

[cntr]	B.S2		LOOP

STB		maxvalue, *value++

.endproc

获得的.asm文件中软件流水的部分结果如下：

;******************************************************************************

;*                                                                            *

;* Using -g (debug) with optimization (-o3) may disable key optimizations!    *

;*                                                                            *

;******************************************************************************

_sortingASMNew:

.map	value/A6

.map	value'/A4

.map	maxvalue/A5

.map	ifvalue/A0

.map	addr/A4

.map	length/B4

.map	tempvalue/A3

.map	cntr/B4

;*----------------------------------------------------------------------------*

;*   SOFTWARE PIPELINE INFORMATION

;*

;*      Loop label : LOOP

;*      Loop source line                 : 224

;*      Loop closing brace source line   : 229

;*      Known Minimum Trip Count         : 100

;*      Known Max Trip Count Factor      : 1

;*      Loop Carried Dependency Bound(^) : 2

;*      Unpartitioned Resource Bound     : 1

;*      Partitioned Resource Bound(*)    : 1

;*      Resource Partition:

;*                                A-side   B-side

;*      .L units                     1*       0

;*      .S units                     0        0

;*      .D units                     1*       0

;*      .M units                     0        0

;*      .X cross paths               0        0

;*      .T address paths             1*       0

;*      Long read paths              0        0

;*      Long write paths             0        0

;*      Logical  ops (.LS)           0        0     (.L or .S unit)

;*      Addition ops (.LSD)          1        0     (.L or .S or .D unit)

;*      Bound(.L .S .LS)             1*       0

;*      Bound(.L .S .D .LS .LSD)     1*       0

;*

;*      Searching for software pipeline schedule at ...

;*         ii = 2  Schedule found with 4 iterations in parallel

;*      Done

;*

;*      Loop will be splooped

;*      Collapsed epilog stages       : 0

;*      Collapsed prolog stages       : 0

;*      Minimum required memory pad   : 0 bytes

;*

;*      Minimum safe trip count       : 1

;*----------------------------------------------------------------------------*

$C$L23:    ; PIPED LOOP PROLOG

SPLOOPD 2       ;8                ; (P)

||         MVC     .S2     B4,ILC

;** --------------------------------------------------------------------------*

$C$L24:    ; PIPED LOOP KERNEL

$C$DW$L$_sortingASMNew$3$B:

LDB     .D1T1   *addr++,tempvalue ; |../test.sa:224| (P) <0,0>

NOP             4

CMPGT   .L1     maxvalue,tempvalue,ifvalue ; |../test.sa:225| (P) <0,5>  ^

SPKERNEL 2,1

|| [ ifvalue] MV   .L1     tempvalue,maxvalue ; |../test.sa:227| <0,6>  ^

$C$DW$L$_sortingASMNew$3$E:

可以看出最小迭代间隔为2，由相关图可以看出最小迭代间隔是由于.L1运算单元调用了两次。

下面是我碰到的问题：

- MV运算指令也可使用.S或.D运算单元，将线性汇编中[ifvalue] MV.L1 	tempvalue, maxvalue语句变为[ifvalue] MV.S1 	tempvalue, maxvalue希望能使用.S1运算单元，使最小迭代间隔变为1。但是查看相关的.asm文件，获得的流水信息和loop kernel的信息此语句并没有使用.S1运算单元，不知道为什么？

;*----------------------------------------------------------------------------*

;*   SOFTWARE PIPELINE INFORMATION

;*

;*      Loop label : LOOP

;*      Loop source line                 : 224

;*      Loop closing brace source line   : 229

;*      Known Minimum Trip Count         : 100

;*      Known Max Trip Count Factor      : 1

;*      Loop Carried Dependency Bound(^) : 2

;*      Unpartitioned Resource Bound     : 1

;*      Partitioned Resource Bound(*)    : 1

;*      Resource Partition:

;*                                A-side   B-side

;*      .L units                     1*       0

;*      .S units                     0        0

;*      .D units                     1*       0

;*      .M units                     0        0

;*      .X cross paths               0        0

;*      .T address paths             1*       0

;*      Long read paths              0        0

;*      Long write paths             0        0

;*      Logical  ops (.LS)           0        0     (.L or .S unit)

;*      Addition ops (.LSD)          1        0     (.L or .S or .D unit)

;*      Bound(.L .S .LS)             1*       0

;*      Bound(.L .S .D .LS .LSD)     1*       0

;*

;*      Searching for software pipeline schedule at ...

;*         ii = 2  Schedule found with 4 iterations in parallel

;*      Done

;*

;*      Loop will be splooped

;*      Collapsed epilog stages       : 0

;*      Collapsed prolog stages       : 0

;*      Minimum required memory pad   : 0 bytes

;*

;*      Minimum safe trip count       : 1

;*----------------------------------------------------------------------------*

$C$L23:    ; PIPED LOOP PROLOG

SPLOOPD 2       ;8                ; (P)

||         MVC     .S2     B4,ILC

;** --------------------------------------------------------------------------*

$C$L24:    ; PIPED LOOP KERNEL

$C$DW$L$_sortingASMNew$3$B:

LDB     .D1T1   *addr++,tempvalue ; |../test.sa:224| (P) <0,0>

NOP             4

CMPGT   .L1     maxvalue,tempvalue,ifvalue ; |../test.sa:225| (P) <0,5>  ^

SPKERNEL 2,1

|| [ ifvalue] MV   .L1     tempvalue,maxvalue ; |../test.sa:227| <0,6>  ^

$C$DW$L$_sortingASMNew$3$E:

- 使用MPY指令代替MV运算指令，使用.M1，将线性汇编中[ifvalue] MV.L1 	tempvalue, maxvalue语句变为[ifvalue] MPY.M1 	tempvalue, 1, maxvalue希望能使用.M1运算单元，使最小迭代间隔变为1。但是查看相关的.asm文件，A side使用了2个.X交叉通路，最小迭代间隔反而变成了3，不知道这里的交叉通路是怎么造成的？

;*----------------------------------------------------------------------------*

;*   SOFTWARE PIPELINE INFORMATION

;*

;*      Loop label : LOOP

;*      Loop source line                 : 224

;*      Loop closing brace source line   : 229

;*      Known Minimum Trip Count         : 100

;*      Known Max Trip Count Factor      : 1

;*      Loop Carried Dependency Bound(^) : 3

;*      Unpartitioned Resource Bound     : 1

;*      Partitioned Resource Bound(*)    : 1

;*      Resource Partition:

;*                                A-side   B-side

;*      .L units                     1*       0

;*      .S units                     0        0

;*      .D units                     1*       0

;*      .M units                     1*       0

;*      .X cross paths               2*       0

;*      .T address paths             0        1*

;*      Long read paths              0        0

;*      Long write paths             0        0

;*      Logical  ops (.LS)           0        0     (.L or .S unit)

;*      Addition ops (.LSD)          0        0     (.L or .S or .D unit)

;*      Bound(.L .S .LS)             1*       0

;*      Bound(.L .S .D .LS .LSD)     1*       0

;*

;*      Searching for software pipeline schedule at ...

;*         ii = 3  Schedule found with 3 iterations in parallel

;*      Done

;*

;*      Loop will be splooped

;*      Collapsed epilog stages       : 0

;*      Collapsed prolog stages       : 0

;*      Minimum required memory pad   : 0 bytes

;*

;*      Minimum safe trip count       : 1

;*----------------------------------------------------------------------------*

$C$L23:    ; PIPED LOOP PROLOG

SPLOOPD 3       ;9                ; (P)

||         MVC     .S2     B4,ILC

;** --------------------------------------------------------------------------*

$C$L24:    ; PIPED LOOP KERNEL

$C$DW$L$_sortingASMNew$3$B:

LDB     .D1T1   *addr++,tempvalue ; |../test.sa:224| (P) <0,0>

NOP             4

CMPGT   .L1     maxvalue,tempvalue,ifvalue ; |../test.sa:225| (P) <0,5>  ^

SPKERNEL 1,1

|| [ ifvalue] MPY  .M1     0x1,tempvalue,maxvalue ; |../test.sa:227| <0,6>  ^

$C$DW$L$_sortingASMNew$3$E: