KeyStone 2

GE Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial release |

Contents

1	Overview	3

2	Test Methods	4

2.1	MAC loopback Test	4

2.2	SGMII loopback test	5

2.3	SerDes loopback test	5

2.4	External loopback test	5

2.5	device 0 to device 1 test	6

3	Test Coverage and test cases	7

3.1	Test with varying packet size	7

3.2	Test with single/multi port transmission	7

4	Test code/project	8

4.1	CCS project	8

4.2	To run the test program on KeyStone 2 EVM	10

4.3	Test configuration	11

4.4	Migrate the test to custom board	12

References	12

Appendix: Typical test output	13

Figures

Figure 1.	KeyStone II Gigabit Ethernet Hardware Structure	3

Figure 2.	KeyStone II Ethernet Internal Test	4

Figure 3.	KeyStone II Ethernet External Test	4

Figure 4.	Directory structure of test project	9

Figure 5.	define symbols in CCS compiler options to select device and board	11

Tables

Table 1.	Source files of the test codes	9

Table 2.	Debug functionality	10

- Overview

Keystone II device provide up to 9 port Gigabit Ethernet switch, among of them, port 1~8 can be used to connect to external network, working in 10/100/1000M mode; port 0 is used to connect to CPUs. Figure 1 show the hardware structure.

- KeyStone II Gigabit Ethernet Hardware Structure

Gigabit Ethernet provide MAC/SGMII/SerDes/FIFO loopback functionality for debug purpose. With these features, Gigabit Ethernet self tests are designed to verify whether Ethernet working in correct status, error information will be collected in case wrong status detected. For above purpose, the tests are implemented to cover:

- All loopback functionality inside SOC

- All external data transmission

- Ethernet features supported by hardware

- Packet size from 64 bytes to 10K bytes

- Different data type

- Packet source/destination location in LL2/MSRAM (Multicore Shared RAM)/DDR memory

- Single/Multi port transmission

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test Methods

Five test methods are introduced in this section, and the purpose of these test methods is discussed. The algorithms are divided into two classes: internal test and external test. Figure 2 and Figure 3 show different test path.

- KeyStone II Ethernet Internal Test

- KeyStone II Ethernet External Test

  - MAC loopback Test

The MAC loopback is the basic internal test to verify:

. Input packet push to proper TX queue

. Packet DMA working correctly to move data between cores/switch

. ALE engine inside switch working correctly to route the packet to right port

. MAC port working in correct status to loopback packet

. RX flow/queue setting properly to receive loopbacked packet.

To enable MAC loopback test on a port, user need to configure test mode in ge_port_cfg[] in “GE_test.c”, following is an example:

{ETHERNET_1000M_FULLDUPLEX, GE_PORT_NO_CONNECT, ETHERNET_MAC_LOOPBACK}//SGMII port 7

  - SGMII loopback test

The SGMII loopback is the internal test to verify:

. Input packet push to proper TX queue

. Packet DMA working correctly to move data between cores/switch

. ALE engine inside switch working correctly to route the packet to right port

. MAC port working in correct status to transmit/receive packet

. SGMII working correctly to loopback packet

. RX flow/queue setting properly to receive loopbacked packet.

To enable SGMII loopback test on a port, user need to configure test mode in ge_port_cfg[] in “GE_test.c”, following is an example:

{ETHERNET_1000M_FULLDUPLEX, GE_PORT_NO_CONNECT, ETHERNET_SGMII_LOOPBACK}//SGMII port 1

  - SerDes loopback test

The SerDes loopback is the internal test to verify:

. Input packet push to proper TX queue

. Packet DMA working correctly to move data between cores/switch

. ALE engine inside switch working correctly to route the packet to right port

. MAC port working in correct status to transmit/receive packet

. SGMII working correctly to do 8b/10b encoding and decoding

. SerDes configured correctly and loopback packet correctly

. RX flow/queue setting properly to receive loopbacked packet.

To enable SerDes loopback test on a port, user need to configure test mode in ge_port_cfg[] in “GE_test.c”, following is an example:

{ETHERNET_1000M_FULLDUPLEX, GE_PORT_NO_CONNECT, ETHERNET_SERDES_INTERNAL_LOOPBACK},

  - External loopback test

Two devices used for external loopback test, the test path is SCO 0->SOC 1->SOC 0.  The data may be loopback on SerDes or FIFO of the SOC1.

To simplify the STK usage, same test program are used on two Devices test. The trick is that user must load the program into core 0 of Device 0, and load the same program into core 1 of Device 1. The program will detect the core number at run time, if it is core 0, then it executes the configuration and functions for Device 0; if it is core 1, then it executes the configuration and functions for Device 1.

The external loopback is the external test to verify:

. Input packet push to proper TX queue in device 0

. Packet DMA working correctly to move data between cores/switch in device 0

. ALE engine inside switch working correctly to route the packet to right port in device 0

. MAC port working in correct status to transmit/receive packet in device 0

. SGMII working correctly to do 8b/10b encoding and decoding in device 0

. SerDes configured correctly and transmit/receive packet correctly in device 0

.The signals transmission on PCB are good.

. SerDes configured correctly and transmit/receive packet correctly in device 1

. RX flow/queue setting properly to receive loopbacked packet in device 0.

To enable external loopback test on a port, user need to configure test mode in ge_port_cfg[] in “GE_test.c”, following is an example:

{ ETHERNET_AUTO_NEGOTIAT_SLAVE, GE_PORT_CABLE_CONNECT, ETHERNET_EXTERNAL_LINE_LOOPBACK},

In external test case, two devices may be connected through SGMII directly, or connected through Ethernet PHY and cable. For SGMII direct connection, one device should be configured as ETHERNET_AUTO_NEGOTIAT_SLAVE, and the other device should be configured as ETHERNET_AUTO_NEGOTIAT_MASTER. If two devices are connected through Ethernet PHY and cable, both device should be configured as ETHERNET_AUTO_NEGOTIAT_SLAVE because the PHY is master in this case.

  - device 0 to device 1 test

The device 0 to device 1 test is the external test to verify:

. Input packet push to proper TX queue in device 0

. Packet DMA working correctly to move data between cores/switch in device 0

. MAC port working in correct status to transmit packet in device 0

. SGMII working correctly to do 8b/10b encoding and decoding in device 0

. SerDes configured correctly and transmit/receive packet correctly in device 0

. SerDes configured correctly and transmit/receive packet correctly in device 1

. SGMII working correctly to do 8b/10b encoding and decoding in device 1

. MAC port working in correct status to receive packet in device 1

. ALE engine inside switch working correctly to route the packet to right port in device 1

. Packet DMA working correctly to move data between cores/switch in device 1

. RX flow/queue setting properly to receive packet in device 1.

If there are no two devices to do this test, a PC can be used instead of device 1. In this case, an Ethernet cable is used to connect K2 board and PC. On the PC, an Ethernet packet capturing tool should be used to receive and verify the packets from device 0. Please note, in this case, device 0 may receive many broadcasting packets from PC.

To enable external two device test on a port, user need to configure test mode in ge_port_cfg[] in “GE_test.c”, following is an example:

{ ETHERNET_AUTO_NEGOTIAT_SLAVE, GE_PORT_CABLE_CONNECT, ETHERNET_LOOPBACK_DISABLE},

- Test Coverage and test cases

To fully cover the use cases in practice, Gigabit Ethernet tests should be implemented to cover:

- Packet size from 64 bytes to 10K bytes

- Single/Multi port transmission

  - Test with varying packet size

The packet size can be configured by user in following macros in “GE_test.c”.

#define TEST_PACKET_SIZE_MIN 			128

#define TEST_PACKET_SIZE_MAX 			8192

  - Test with single/multi port transmission

The STK supports single port or any combination of multiple ports transmission.

Following configuration structure are used to control the ports used for the test. If a port is not used for the test, then “port_connect” should be set as “GE_PORT_NOT_USED”; if a port is used for test but no external cable or SGMII connection, it should be set as “GE_PORT_NO_CONNECT”, and this port will only be used for internal loopback test.

//The port connection state for the test

GE_Port_Config ge_port_cfg[GE_NUM_ETHERNET_PORT]=

{

/*	ethernet_mode 	                   port_connect                            loopback_mode*/

#if 0

{ETHERNET_AUTO_NEGOTIAT_SLAVE, GE_PORT_CABLE_CONNECT, ETHERNET_LOOPBACK_DISABLE}, //SGMII port 0

{ETHERNET_AUTO_NEGOTIAT_SLAVE, GE_PORT_CABLE_CONNECT, ETHERNET_LOOPBACK_DISABLE}//SGMII port 1

#else

{ETHERNET_1000M_FULLDUPLEX, GE_PORT_NO_CONNECT, ETHERNET_SERDES_INTERNAL_LOOPBACK}, //SGMII port 0

{ETHERNET_1000M_FULLDUPLEX, GE_PORT_NO_CONNECT, ETHERNET_SGMII_LOOPBACK}//SGMII port 1

#endif

#if (GE_NUM_ETHERNET_PORT>2)

,{ETHERNET_1000M_FULLDUPLEX, GE_PORT_NO_CONNECT, ETHERNET_SERDES_INTERNAL_LOOPBACK}, //SGMII port 2

{ETHERNET_1000M_FULLDUPLEX, GE_PORT_NO_CONNECT, ETHERNET_SGMII_LOOPBACK}//SGMII port 3

#endif

#if (GE_NUM_ETHERNET_PORT>4)

,{ETHERNET_1000M_FULLDUPLEX, GE_PORT_NO_CONNECT, ETHERNET_SERDES_INTERNAL_LOOPBACK}, //SGMII port 4

{ETHERNET_1000M_FULLDUPLEX, GE_PORT_NO_CONNECT, ETHERNET_SGMII_LOOPBACK}, //SGMII port 5

{ETHERNET_1000M_FULLDUPLEX, GE_PORT_NO_CONNECT, ETHERNET_SGMII_LOOPBACK}, //SGMII port 6

{ETHERNET_1000M_FULLDUPLEX, GE_PORT_NOT_USED, ETHERNET_MAC_LOOPBACK}//SGMII port 7

#endif

};

- Test code/project

This section introduces the implementation of Gigabit Ethernet tests on KeyStone 2 SOC.

  - CCS project

Following figure shows the directory structure of the test project

- Directory structure of test project

The project files are in “GE” folder. The main source code files are in the “GE\src” subfolder. There is some commonly used initialization and driver code for GE, DDR, and PLL… in “common” folder. Below table describes these source files.

- Source files of the test codes

| Source files | Descriptions |
| --- | --- |
| K2_common | Initialization for PLL. The key API include: void KeyStone_main_PLL_init (float ref_clock_MHz, unsigned int multiplier, unsigned int divisor); |
| K2_DDR_Init | Low level DDR initialization. The key API is: void K2_DDR_Init(DDR3_Config * DDR_cfg); |
| K2_GE_Init_drv | Low level GE initialization and driver. The key API is: void K2_GE_Init(GE_Config * ge_cfg); void K2_GE_Accumulate_Statistics(); void Fill_EMAC_header(Uint8 *buffer, Ethernet_Packet_Type type, unsigned long long sourceMAC, unsigned long long destMAC); |
| GE_packet_process | Implementation of Gigabit Ethernet packet transmit and receive. |
| GE_Interrupts | Implementation of interrupt initialization and interrupt handling. The key APIs are: void GE_Interrupts_Init(void) ; interrupt void GE_Message_ISR() |
| GE_PktDMA_Init | Implementation of QMSS PktDMA initialization. The key APIs are: void GE_PktDMA_init(); |
| GE_Test | Implementation of Gigabit Ethernet test main function. |

For test convenience, self test code provide debug functionality in “GE_PktDMA_Init.c” and “GE_debug.c”, which cover following items

- Debug functionality

| Debug code | Description |
| --- | --- |
| GE_Check_Free_Queues() | Monitor number of descriptors change in FDQ |
| GE_Check_TxRx_Queues() | Monitor number of descriptors change in TX/RX queue |
| print_GE_statistics() | Record number of packets TX/RX in 5 Ethernet ports |
| print_GE_Ethernet_ports_status() | Record Ethernet port status |
| print_GE_MDIO_status() | Record PHY status |

  - To run the test program on KeyStone 2 EVM

The test program can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- Extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- For loopback test, load the program to ARM core0 of a device. For test between two devices, load the program to core0 or first device, and load the program to core1 of the second device.

- For loopback test, run core0; for test between two devices, run the core1 of the second device firstly and then run the core0 of the first device.

- See output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

There are several configurations can be modified to run different test cases in “GE_test.c”:

Refer to Chapter 2 and 3 for more details about the definition of these test cases.

  - Migrate the test to custom board

These tests are implemented based on TI’s EVM boards. In your real system, the DDR configuration may be changed according to your hardware design in the function K2_DDR3A_config() and K2_DDR3A_config() in “K2_Board_Init.c”.

The core speed and PASS speed may also be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define PASS_PLL_REF_CLK_MHZ    122.88

#define PASS_PLL_DIVISOR        24

#define PASS_PLL_MULTIPLIER     205

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

The GE Serdes reference clock speed may be changed through following macro in “K2_Board_Init.h”:

#define SGMII_SERDES_REF_CLK    CSL_SERDES_REF_CLOCK_156p25M

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone Gigabit Ethernet (GbE) Switch Subsystem User's Guide (SPRUGV9C)

- KeyStone Multicore Navigator User's Guide (SPRUGR9F)

- KeyStone Network Coprocessor (NETCP) User's Guide (SPRUGZ6)

Appendix: Typical test output

Below is the loopback test output on K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

DDR3A initialization

Initialize DDR data rate = 100.000/1*20/6*4= 1333.3 MTS, bus width = 64 bits.

Ethernet port 0 test setup: no connection  , forced 1000M fullduplex  , SerDes internal loopback

Ethernet port 1 test setup: no connection  , forced 1000M fullduplex  , SGMII loopback

Ethernet port 2 test setup: no connection  , forced 1000M fullduplex  , SerDes internal loopback

Ethernet port 3 test setup: no connection  , forced 1000M fullduplex  , SGMII loopback

Initialize PASS PLL clock = 122.88MHz/24x205 = 1049MHz

STATSA.RxGoodFrames     =4

STATSA.RxOctets         =584

STATSA.TxGoodFrames     =2

STATSA.TxOctets         =292

STATSA.Frame128t255     =6

STATSA.NetOctets        =876

STATSB.RxGoodFrames     =2

STATSB.RxOctets         =292

STATSB.TxGoodFrames     =2

STATSB.TxOctets         =292

STATSB.Frame128t255     =4

STATSB.NetOctets        =584

STATSC.TxGoodFrames     =2

STATSC.TxOctets         =292

STATSC.Frame128t255     =2

STATSC.NetOctets        =292

STATSD.RxGoodFrames     =2

STATSD.RxOctets         =292

STATSD.TxGoodFrames     =2

STATSD.TxOctets         =292

STATSD.Frame128t255     =4

STATSD.NetOctets        =584

Ethernet port 0 TX  1 packets x  128 bytes, RX  1 packets,     5686 cycles,   215 Mbps

Ethernet port 1 TX  1 packets x  128 bytes, RX  1 packets,     5842 cycles,   210 Mbps

Ethernet port 2 TX  1 packets x  128 bytes, RX  1 packets,     6143 cycles,   199 Mbps

Ethernet port 3 TX  1 packets x  128 bytes, RX  1 packets,     5631 cycles,   217 Mbps

STATSA.RxGoodFrames     =8

STATSA.RxOctets         =4752

STATSA.TxGoodFrames     =4

STATSA.TxOctets         =2376

STATSA.Frame1024tUp     =6

STATSA.NetOctets        =7128

STATSB.RxGoodFrames     =4

STATSB.RxOctets         =2376

STATSB.TxGoodFrames     =4

STATSB.TxOctets         =2376

STATSB.Frame1024tUp     =4

STATSB.NetOctets        =4752

STATSC.TxGoodFrames     =4

STATSC.TxOctets         =2376

STATSC.Frame1024tUp     =2

STATSC.NetOctets        =2376

STATSD.RxGoodFrames     =4

STATSD.RxOctets         =2376

STATSD.TxGoodFrames     =4

STATSD.TxOctets         =2376

STATSD.Frame1024tUp     =4

STATSD.NetOctets        =4752

Ethernet port 0 TX  1 packets x 1024 bytes, RX  1 packets,    21719 cycles,   451 Mbps

Ethernet port 1 TX  1 packets x 1024 bytes, RX  1 packets,    24036 cycles,   408 Mbps

Ethernet port 2 TX  1 packets x 1024 bytes, RX  1 packets,    26773 cycles,   366 Mbps

Ethernet port 3 TX  1 packets x 1024 bytes, RX  1 packets,    28530 cycles,   344 Mbps

STATSA.RxGoodFrames     =12

STATSA.RxOctets         =37592

STATSA.TxGoodFrames     =6

STATSA.TxOctets         =18796

STATSA.Frame1024tUp     =12

STATSA.NetOctets        =56388

STATSB.RxGoodFrames     =6

STATSB.RxOctets         =18796

STATSB.TxGoodFrames     =6

STATSB.TxOctets         =18796

STATSB.Frame1024tUp     =8

STATSB.NetOctets        =37592

STATSC.TxGoodFrames     =6

STATSC.TxOctets         =18796

STATSC.Frame1024tUp     =4

STATSC.NetOctets        =18796

STATSD.RxGoodFrames     =6

STATSD.RxOctets         =18796

STATSD.TxGoodFrames     =6

STATSD.TxOctets         =18796

STATSD.Frame1024tUp     =8

STATSD.NetOctets        =37592

Ethernet port 0 TX  1 packets x 8192 bytes, RX  1 packets,   145797 cycles,   538 Mbps

Ethernet port 1 TX  1 packets x 8192 bytes, RX  1 packets,   173438 cycles,   452 Mbps

Ethernet port 2 TX  1 packets x 8192 bytes, RX  1 packets,   201269 cycles,   390 Mbps

Ethernet port 3 TX  1 packets x 8192 bytes, RX  1 packets,   227601 cycles,   344 Mbps

STATSA.RxGoodFrames     =28

STATSA.RxOctets         =39928

STATSA.TxGoodFrames     =14

STATSA.TxOctets         =19964

STATSA.Frame128t255     =30

STATSA.NetOctets        =59892

STATSB.RxGoodFrames     =14

STATSB.RxOctets         =19964

STATSB.TxGoodFrames     =14

STATSB.TxOctets         =19964

STATSB.Frame128t255     =20

STATSB.NetOctets        =39928

STATSC.TxGoodFrames     =14

STATSC.TxOctets         =19964

STATSC.Frame128t255     =10

STATSC.NetOctets        =19964

STATSD.RxGoodFrames     =14

STATSD.RxOctets         =19964

STATSD.TxGoodFrames     =14

STATSD.TxOctets         =19964

STATSD.Frame128t255     =20

STATSD.NetOctets        =39928

Ethernet port 0 TX  4 packets x  128 bytes, RX  4 packets,    40530 cycles,   121 Mbps

Ethernet port 1 TX  4 packets x  128 bytes, RX  4 packets,    40055 cycles,   122 Mbps

Ethernet port 2 TX  4 packets x  128 bytes, RX  4 packets,    40823 cycles,   120 Mbps

Ethernet port 3 TX  4 packets x  128 bytes, RX  4 packets,    40291 cycles,   121 Mbps

STATSA.RxGoodFrames     =44

STATSA.RxOctets         =56600

STATSA.TxGoodFrames     =22

STATSA.TxOctets         =28300

STATSA.Frame1024tUp     =36

STATSA.NetOctets        =84900

STATSB.RxGoodFrames     =22

STATSB.RxOctets         =28300

STATSB.TxGoodFrames     =22

STATSB.TxOctets         =28300

STATSB.Frame1024tUp     =24

STATSB.NetOctets        =56600

STATSC.TxGoodFrames     =22

STATSC.TxOctets         =28300

STATSC.Frame1024tUp     =12

STATSC.NetOctets        =28300

STATSD.RxGoodFrames     =22

STATSD.RxOctets         =28300

STATSD.TxGoodFrames     =22

STATSD.TxOctets         =28300

STATSD.Frame1024tUp     =24

STATSD.NetOctets        =56600

Ethernet port 0 TX  4 packets x 1024 bytes, RX  4 packets,    68464 cycles,   573 Mbps

Ethernet port 1 TX  4 packets x 1024 bytes, RX  4 packets,    70960 cycles,   553 Mbps

Ethernet port 2 TX  4 packets x 1024 bytes, RX  4 packets,    74348 cycles,   528 Mbps

Ethernet port 3 TX  4 packets x 1024 bytes, RX  4 packets,    76579 cycles,   512 Mbps

STATSA.RxGoodFrames     =60

STATSA.RxOctets         =187960

STATSA.TxGoodFrames     =30

STATSA.TxOctets         =93980

STATSA.Frame1024tUp     =60

STATSA.NetOctets        =281940

STATSB.RxGoodFrames     =30

STATSB.RxOctets         =93980

STATSB.TxGoodFrames     =30

STATSB.TxOctets         =93980

STATSB.Frame1024tUp     =40

STATSB.NetOctets        =187960

STATSC.TxGoodFrames     =30

STATSC.TxOctets         =93980

STATSC.Frame1024tUp     =20

STATSC.NetOctets        =93980

STATSD.RxGoodFrames     =30

STATSD.RxOctets         =93980

STATSD.TxGoodFrames     =30

STATSD.TxOctets         =93980

STATSD.Frame1024tUp     =40

STATSD.NetOctets        =187960

Ethernet port 0 TX  4 packets x 8192 bytes, RX  4 packets,   489223 cycles,   641 Mbps

Ethernet port 1 TX  4 packets x 8192 bytes, RX  4 packets,   516242 cycles,   608 Mbps

Ethernet port 2 TX  4 packets x 8192 bytes, RX  4 packets,   543167 cycles,   578 Mbps

Ethernet port 3 TX  4 packets x 8192 bytes, RX  4 packets,   570799 cycles,   550 Mbps

STATSA.RxGoodFrames     =124

STATSA.RxOctets         =197304

STATSA.TxGoodFrames     =62

STATSA.TxOctets         =98652

STATSA.Frame128t255     =126

STATSA.NetOctets        =295956

STATSB.RxGoodFrames     =62

STATSB.RxOctets         =98652

STATSB.TxGoodFrames     =62

STATSB.TxOctets         =98652

STATSB.Frame128t255     =84

STATSB.NetOctets        =197304

STATSC.TxGoodFrames     =62

STATSC.TxOctets         =98652

STATSC.Frame128t255     =42

STATSC.NetOctets        =98652

STATSD.RxGoodFrames     =62

STATSD.RxOctets         =98652

STATSD.TxGoodFrames     =62

STATSD.TxOctets         =98652

STATSD.Frame128t255     =84

STATSD.NetOctets        =197304

Ethernet port 0 TX 16 packets x  128 bytes, RX 16 packets,   171301 cycles,   114 Mbps

Ethernet port 1 TX 16 packets x  128 bytes, RX 16 packets,   171207 cycles,   114 Mbps

Ethernet port 2 TX 16 packets x  128 bytes, RX 16 packets,   171984 cycles,   114 Mbps

Ethernet port 3 TX 16 packets x  128 bytes, RX 16 packets,   171410 cycles,   114 Mbps

STATSA.RxGoodFrames     =188

STATSA.RxOctets         =263992

STATSA.TxGoodFrames     =94

STATSA.TxOctets         =131996

STATSA.Frame1024tUp     =156

STATSA.NetOctets        =395988

STATSB.RxGoodFrames     =94

STATSB.RxOctets         =131996

STATSB.TxGoodFrames     =94

STATSB.TxOctets         =131996

STATSB.Frame1024tUp     =104

STATSB.NetOctets        =263992

STATSC.TxGoodFrames     =94

STATSC.TxOctets         =131996

STATSC.Frame1024tUp     =52

STATSC.NetOctets        =131996

STATSD.RxGoodFrames     =94

STATSD.RxOctets         =131996

STATSD.TxGoodFrames     =94

STATSD.TxOctets         =131996

STATSD.Frame1024tUp     =104

STATSD.NetOctets        =263992

Ethernet port 0 TX 16 packets x 1024 bytes, RX 16 packets,   262158 cycles,   599 Mbps

Ethernet port 1 TX 16 packets x 1024 bytes, RX 16 packets,   264539 cycles,   593 Mbps

Ethernet port 2 TX 16 packets x 1024 bytes, RX 16 packets,   267266 cycles,   587 Mbps

Ethernet port 3 TX 16 packets x 1024 bytes, RX 16 packets,   270607 cycles,   580 Mbps

STATSA.RxGoodFrames     =252

STATSA.RxOctets         =789432

STATSA.TxGoodFrames     =126

STATSA.TxOctets         =394716

STATSA.Frame1024tUp     =252

STATSA.NetOctets        =1184148

STATSB.RxGoodFrames     =126

STATSB.RxOctets         =394716

STATSB.TxGoodFrames     =126

STATSB.TxOctets         =394716

STATSB.Frame1024tUp     =168

STATSB.NetOctets        =789432

STATSC.TxGoodFrames     =126

STATSC.TxOctets         =394716

STATSC.Frame1024tUp     =84

STATSC.NetOctets        =394716

STATSD.RxGoodFrames     =126

STATSD.RxOctets         =394716

STATSD.TxGoodFrames     =126

STATSD.TxOctets         =394716

STATSD.Frame1024tUp     =168

STATSD.NetOctets        =789432

Ethernet port 0 TX 16 packets x 8192 bytes, RX 16 packets,  1861705 cycles,   674 Mbps

Ethernet port 1 TX 16 packets x 8192 bytes, RX 16 packets,  1888960 cycles,   665 Mbps

Ethernet port 2 TX 16 packets x 8192 bytes, RX 16 packets,  1916320 cycles,   655 Mbps

Ethernet port 3 TX 16 packets x 8192 bytes, RX 16 packets,  1943294 cycles,   646 Mbps

GE test complete.