KeyStone 2

HyperLink Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial Release |

Contents

1	Introduction	3

2	Test Methods	3

2.1	Integrity Test	3

2.1.1	Data filling Test	3

2.1.2	Addressing test	4

2.2	HyperLink Performance Test	5

2.2.1	HyperLink Latency Test	5

2.2.2	HyperLink Throughput Test	6

2.3	HyperLink error reporting and checking	6

3	Test cases	6

3.1	Test by different masters	6

3.2	Test through different data path	6

3.3	Test different memories in the system	7

4	Test code/project	8

4.1	CCS project	9

4.2	To run the test program on KeyStone 2 EVM	10

4.3	Test configuration	10

4.4	Migrate the test to custom board	12

References	12

Appendix: Typical test output	13

Figures

Figure 1.	Example of HyperLink mapping different memories	8

Figure 2.	Directory structure of test project	9

Figure 3.	define symbols in CCS compiler options to select device and board	11

Tables

Table 1.	Source files of the test codes	9

- Introduction

HyperLink provides a highest-speed, low-latency, and low-pin-count communication interface between two Keystone devices.

The speed of HyperLink was designed for 12.5Gbps, on most Keystone SOCs, the speed is limited to 10Gbps because of Serdes or Layout limitation. The HyperLink is a TI-specific peripheral, compared to the traditional 8b10b encoding scheme for high speed Serdes interfaces, HyperLink reduces the encoding overhead; the efficient encoding scheme in HyperLink is equivalent to 8b9b.

The HyperLink uses similar memory map scheme like PCIE, but it provides more flexible features designed for multicore SOC. The HyperLink STK (Self Test Kit) is designed to cover following usage cases:

- Access HyperLink with ARM core or EDMA

- Access different memories through HyperLink

- Generate interrupt through HyperLink

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test Methods

  - Integrity Test

Two memory test algorithms are used to test the memory access through HyperLink, and the purpose of these test algorithms is discussed below.

    - Data filling Test

The pseudocode for data pattern filling test is:

for(memory range under test)

fill the memory with a value;

for(memory range under test)

read back the memory and compare the readback value to the written value

Normally, this test is done several times with different test data value. The common values used for test include 0, 0xFFFFFFFF, 0x55555555, 0xAAAAAAAA.

This test can detect stuck data bit, for example, if

written value = 0, readback value = 0x8,

It indicates bit 3 sticks to 1. If

written value = 0xFFFFFFFF, readback value = 0xFFFFFFFE,

It indicates bit 0 sticks to 0.

If this test fails, the possible reasons include HyperLink physical layer error, the HyperLink buffer error or target memory error. Trying internal loopback test against test between two devices may isolate the reason.

    - Addressing test

The pseudocode for addressing test is:

for(memory range under test)

fill each memory unit with its address value;

for(memory range under test)

read back the memory and compare the readback value to the written value

This test can detect stuck bit on address bus, for example, if

written value = 0 at address 0

written value = 1 at address 1

written value = 2 at address 2

written value = 3 at address 3

……

readback value = 2 at address 0

readback value = 3 at address 1

readback value = 2 at address 2

readback value = 3 at address 3

……

For this case, it indicates bit 1 of address bus sticks. So, the contents in address 0 and 2 are same, and the contents in address 1 and 3 are same.

If the data filling test passes, while this test fails, the possible reasons include HyperLink address translation error, the HyperLink buffer error or target memory error. Trying internal loopback test against test between two devices may isolate the reason.

  - HyperLink Performance Test

The performance of HyperLink is also measured. If the test shows performance degradation, normally we should check clock speed related configuration.

    - HyperLink Latency Test

The pseudo codes for measuring the latency of ARM core access a memory unit through HyperLink are like following:

flushCache();

startCycle= getCycleCount();

for(i=0; i< accessTimes; i++)

{

Access Memory at address;

address+= stride;

}

cycles = getCycleCount()-startCycle;

cycles/Access= cycles/accessTimes;

The latency of the interrupt generated by HyperLink is also measured with following pseudo code:

……

startCCNT= GetCycleCount();

// manually trigger the hardware event, which will generate interrupt packet to remote side

hyperLinkRegs->SW_INT= HW_EVENT_FOR_INT_TEST;

wait for the queue pending interrupt ;

delay= intCCNT - startCCNT;

……

void HyperLinkISR()	//HyperLink Interrupt Service Routine

{

intCCNT= GetCycleCount(); 	//save the Cycle Count when the interrupt happens

……

}

This test is done with internal loopback mode only.

    - HyperLink Throughput Test

The throughput is measured for different memory copy cases with ARM core or EDMA. The throughput is measured by taking total bytes copied and dividing it by the time it used.

  - HyperLink error reporting and checking

HyperLink detects error during transfer and reports them through some error/status registers. After the test, all the error/status registers are checked and error/status information is printed/reported.

The HyperLink error can also trigger interrupt to ARM core. Interrupt Service Routine is also implemented to print/report the error information.

- Test cases

HyperLink Tests should be implemented to cover:

- Internal loopback and test between two devices

- Access HyperLink with ARM core or EDMA

- Access different memories through HyperLink

  - Test by different masters

The test cases executed with ARM core including:

- Throughput test

- Latency of Memory access through HyperLink

- HyperLink Interrupt latency test

The test cases executed with EDMA including:

- Throughput test

- overhead of EDMA transfer through HyperLink

  - Test through different data path

Internal loopback test is done with the TX data loopback in Serdes module. The internal loopback test can be used to check the function of HyperLink modules inside the KeyStone SOC. If internal loopback test failed, normally, we should check the HyperLink related powers and clocks.

For the test between two devices, device1's memory are mapped to device0 through HyperLink, so, device0 accesses device 1 through the HyperLink memory window just like access it’s own memories.

To simplify the STK usage, same test program are used on two Devices test. The trick is that user must load the program into core 0 of Device0, and load same program into core 1 of Device1. The program will detect the core number at run time, if it is core 0, then it executes the configuration and functions for Device0; if it is core 1, then it executes the configuration and functions for Device1.

If internal loopback test passes, while the test between two devices fails, normally, we should check the signal integrity on board, and measure the eye-diagram.

Please note, the HyperLink supports automatic power down of Serdes lane, that is, if there is no data transfer over the HyperLink, one or more of the 4 serdes lanes may be powered down, and then we can not capture the eye-diagram on the Serdes lane. Two solutions may be used to overcome this behavior:

- Continuously transfer data on the HyperLink during the eye-diagram test.

- Disable the Serdes Lane power down feature for eye-diagram test.

  - Test different memories in the system

All memories of SOC can be accessed through HyperLink. The HyperLink memory map is very flexible. Following figure shows the configuration for the tests.

- Example of HyperLink mapping different memories

In this test, device1’s memory are mapped to device0’s memory window from 0x40000000 to 0x50000000, the device0 can access all memory types in device1 including DSP core LL2 (Local L2), MSRAM (Multicore Shared RAM) and DDR just like access device0’s local memory. In device0, all masters access same outbound window at 0x40000000, but they may actually access different memory space in device1, because the PrivID of the masters in device0 is carried with address, and the inbound memory segment configuration of device1 is set up separately for different PrivID.

For internal loopback test, the memory map configuration is same, the only difference is that, device0 actually access its own local memory through HyperLink memory window.

Above configuration is for HyperLink 0, for the SOC has two HyperLinks, the configuration for the second HyperLink is similar, the key difference is the memory window for the second HyperLink is from 0x28000000 to 0x30000000.

- Test code/project

This section introduces the implementation of these HyperLink tests on KeyStone 2 SOC.

  - CCS project

Following figure shows the directory structure of the test project.

- Directory structure of test project

The project files are in “HyperLink” folder. There is some commonly used initialization and driver code for HyperLink, DDR, EDMA, Serdes and PLL… in “common” folder. The main source code files are in the “HyperLink\src” subfolder. Below table describes these source files.

- Source files of the test codes

| Source files | Descriptions |
| --- | --- |
| K2_common | Initialization for EDMA and PLL. The key API include: void KeyStone_main_PLL_init (float ref_clock_MHz, unsigned int multiplier, unsigned int divisor); void EDMA_init (); |
| K2_DDR_Init | Low level DDR initialization. The key API is: void K2_DDR_Init(DDR3_Config * DDR_cfg); |
| K2_HyperLink_Init | Low level HyperLink initialization. The key API is: void K2_HyperLink_Init(HyperLink_Config * hyperLink_cfg); |
| SerDes | Low level Serdes initialization. They are copied from pdk_keystone2_3_01_01_04. |
| HyperLink_Test | main function of the tests, HyperLink configuration, integrity test and interrupt latency test. |
| HyperLink_CPU_performance | HyperLink throughput test with ARM core; test latency of memory access through HyperLink with ARM core. |
| HyperLink_EDMA_performance | HyperLink throughput test with EDMA; test overhead for EDMA transfer through HyperLink |
| HyperLink_Intc | Interrupts configuration and ISR for HyperLink |
| HyperLink_debug | print HyperLink status and error information for debug |

  - To run the test program on KeyStone 2 EVM

The test program can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- For loopback test, Load the program to ARM core 0 of a SOC. For test between two devices, load the program to core 0 of first device, and load the program to core 1 of the second device.

- For loopback test, run core 0; for test between two devices, run the core 1 of the second device firstly and then run the core 0 of the first device.

- See output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

There are multiple macros defined in the source code to configure the test.

Each HyperLink can be configured to be tested or not through following macros in “HyperLink_Test.c”. The value of “1” means enable the test, and “0” means disable the test. For device does not have Hyperlink 1, “TEST_HYPERLINK_1” must be 0.

#define TEST_HYPERLINK_0 		1

#define TEST_HYPERLINK_1 		1

Following configuration structure in the “HyperLink_Test.c” is used to switch between the internal loopback test and the two devices test.

HyperLink_Loopback_Mode loopbackMode[]=

{

HYPERLINK_INTERNAL_DIGITAL_LOOPBACK, 	//for link 0

HYPERLINK_LOOPBACK_DISABLED 	//for link 1

};

Following macro in the “HyperLink_Test.c” is used to set the test speed (at the unit of GHz).

#define HYPERLINK_SPEED_GHZ 	5.f

The base address of the DDR be accessed through Hyperlink are defined with following macros in “HyperLink_Test.c”. For device does not have DDR3B, comment off the “DDRB_SPACE_ACCESSED_BY_HYPERLINK”.

#define DDRA_SPACE_ACCESSED_BY_HYPERLINK 	0x88000000

#define DDRB_SPACE_ACCESSED_BY_HYPERLINK 	0x68000000

  - Migrate the test to custom board

These tests are implemented based on TI’s EVM boards. In your real system, the DDR configuration may be changed according to your hardware design in the function K2_DDR3A_config() and K2_DDR3A_config() in “K2_Board_Init.c”.

The core speed may be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

The HyperLink Serdes reference clock speed may be changed through following macro in “K2_Board_Init.h”:

#define HYPERLINK_SERDES_REF_CLK    CSL_SERDES_REF_CLOCK_312p5M

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone Architecture HyperLink User Guide (SPRUGW8)

- KeyStone Architecture Enhanced Direct Memory Access (EDMA3) Controller User Guide (SPRUGS5)

- KeyStone II Architecture ARM CorePac User's Guide (SPRUHJ4)

Appendix: Typical test output

Below is the test output on K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

DDR3A initialization

Initialize DDR data rate = 100.000/1*20/6*4= 1333.3 MTS, bus width = 64 bits.

DDR3B initialization

Initialize DDR data rate = 100.000/1*16/4*4= 1600.0 MTS, bus width = 64 bits.

HyperLink 0 internal digital loopback test at 5.000GHz...

HyperLink 1 internal digital loopback test at 5.000GHz...

Waiting for HyperLink0 up...

Waiting for HyperLink1 up...

HyperLink memory test passed at address 0x40040000

HyperLink memory test passed at address 0x44300000

HyperLink memory test passed at address 0x47300000

HyperLink memory test passed at address 0x46140000

Hyperlink 0 interrupt latency is 1240 cycles

800 MB/s, copy  65536 bytes from 0x0c080000( local MSRAM) to 0x40040000(  remote LL2), consumes 98248 cycles

54 MB/s, copy  65536 bytes from 0x40040000(  remote LL2) to 0x0c080000( local MSRAM), consumes 1450177 cycles

50 MB/s, copy  65536 bytes from 0x40040000(  remote LL2) to 0x40050000(  remote LL2), consumes 1561887 cycles

575 MB/s, copy  65536 bytes from 0x0c080000( local MSRAM) to 0x46140000(remote MSRAM), consumes 136769 cycles

53 MB/s, copy  65536 bytes from 0x46140000(remote MSRAM) to 0x0c080000( local MSRAM), consumes 1463791 cycles

49 MB/s, copy  65536 bytes from 0x46140000(remote MSRAM) to 0x46150000(remote MSRAM), consumes 1585600 cycles

800 MB/s, copy  65536 bytes from 0x0c080000( local MSRAM) to 0x44300000(remote DDR3A), consumes 98240 cycles

49 MB/s, copy  65536 bytes from 0x44300000(remote DDR3A) to 0x0c080000( local MSRAM), consumes 1600034 cycles

45 MB/s, copy  65536 bytes from 0x44300000(remote DDR3A) to 0x44310000(remote DDR3A), consumes 1725006 cycles

800 MB/s, copy  65536 bytes from 0x0c080000( local MSRAM) to 0x47300000(remote DDR3B), consumes 98240 cycles

48 MB/s, copy  65536 bytes from 0x47300000(remote DDR3B) to 0x0c080000( local MSRAM), consumes 1612997 cycles

45 MB/s, copy  65536 bytes from 0x47300000(remote DDR3B) to 0x47310000(remote DDR3B), consumes 1718725 cycles

Memory load performance test at 0x40040000(  remote LL2)

Index=   0  Cycles/LDRD= 104.22

Index=   1  Cycles/LDRD= 104.03

Index=   2  Cycles/LDRD= 104.05

Index=   3  Cycles/LDRD= 103.96

Index=   4  Cycles/LDRD= 103.93

Index=   5  Cycles/LDRD= 103.82

Index=   6  Cycles/LDRD= 103.97

Index=   7  Cycles/LDRD= 103.87

Index=   8  Cycles/LDRD= 103.97

Index=  10  Cycles/LDRD= 104.04

Index=  12  Cycles/LDRD= 104.02

Index=  14  Cycles/LDRD= 104.00

Index=  16  Cycles/LDRD= 103.80

Index=  18  Cycles/LDRD= 103.97

Index=  20  Cycles/LDRD= 103.98

Index=  22  Cycles/LDRD= 103.94

Index=  24  Cycles/LDRD= 104.04

Index=  26  Cycles/LDRD= 103.95

Index=  28  Cycles/LDRD= 103.94

Index=  30  Cycles/LDRD= 104.07

Index=  32  Cycles/LDRD= 104.00

Index=  40  Cycles/LDRD= 103.95

Index=  48  Cycles/LDRD= 103.94

Index=  56  Cycles/LDRD= 103.99

Memory store performance test at 0x40040000(  remote LL2)

Index=   0  Cycles/STRD= 24.39

Index=   1  Cycles/STRD= 24.39

Index=   2  Cycles/STRD= 24.39

Index=   3  Cycles/STRD= 24.39

Index=   4  Cycles/STRD= 24.39

Index=   5  Cycles/STRD= 24.39

Index=   6  Cycles/STRD= 24.39

Index=   7  Cycles/STRD= 24.39

Index=   8  Cycles/STRD= 24.39

Index=  10  Cycles/STRD= 24.39

Index=  12  Cycles/STRD= 24.39

Index=  14  Cycles/STRD= 24.39

Index=  16  Cycles/STRD= 24.39

Index=  18  Cycles/STRD= 24.39

Index=  20  Cycles/STRD= 24.39

Index=  22  Cycles/STRD= 24.39

Index=  24  Cycles/STRD= 24.39

Index=  26  Cycles/STRD= 24.39

Index=  28  Cycles/STRD= 24.39

Index=  30  Cycles/STRD= 24.39

Index=  32  Cycles/STRD= 24.39

Index=  40  Cycles/STRD= 24.39

Index=  48  Cycles/STRD= 24.39

Index=  56  Cycles/STRD= 24.39

Memory load performance test at 0x46140000(remote MSRAM)

Index=   0  Cycles/LDRD= 105.59

Index=   1  Cycles/LDRD= 105.66

Index=   2  Cycles/LDRD= 105.52

Index=   3  Cycles/LDRD= 105.48

Index=   4  Cycles/LDRD= 105.66

Index=   5  Cycles/LDRD= 105.16

Index=   6  Cycles/LDRD= 105.37

Index=   7  Cycles/LDRD= 105.06

Index=   8  Cycles/LDRD= 104.65

Index=  10  Cycles/LDRD= 104.90

Index=  12  Cycles/LDRD= 105.24

Index=  14  Cycles/LDRD= 105.01

Index=  16  Cycles/LDRD= 104.34

Index=  18  Cycles/LDRD= 105.02

Index=  20  Cycles/LDRD= 105.32

Index=  22  Cycles/LDRD= 105.05

Index=  24  Cycles/LDRD= 104.54

Index=  26  Cycles/LDRD= 105.02

Index=  28  Cycles/LDRD= 105.17

Index=  30  Cycles/LDRD= 104.95

Index=  32  Cycles/LDRD= 104.71

Index=  40  Cycles/LDRD= 104.63

Index=  48  Cycles/LDRD= 104.57

Index=  56  Cycles/LDRD= 104.64

Index=  64  Cycles/LDRD= 104.77

Index=  96  Cycles/LDRD= 104.76

Index= 128  Cycles/LDRD= 105.07

Index= 192  Cycles/LDRD= 104.96

Memory store performance test at 0x46140000(remote MSRAM)

Index=   0  Cycles/STRD= 35.53

Index=   1  Cycles/STRD= 32.09

Index=   2  Cycles/STRD= 29.13

Index=   3  Cycles/STRD= 27.96

Index=   4  Cycles/STRD= 27.55

Index=   5  Cycles/STRD= 24.49

Index=   6  Cycles/STRD= 24.39

Index=   7  Cycles/STRD= 24.39

Index=   8  Cycles/STRD= 24.39

Index=  10  Cycles/STRD= 24.39

Index=  12  Cycles/STRD= 24.39

Index=  14  Cycles/STRD= 24.39

Index=  16  Cycles/STRD= 24.39

Index=  18  Cycles/STRD= 24.39

Index=  20  Cycles/STRD= 24.39

Index=  22  Cycles/STRD= 24.39

Index=  24  Cycles/STRD= 24.39

Index=  26  Cycles/STRD= 24.39

Index=  28  Cycles/STRD= 24.39

Index=  30  Cycles/STRD= 24.39

Index=  32  Cycles/STRD= 24.39

Index=  40  Cycles/STRD= 24.38

Index=  48  Cycles/STRD= 24.39

Index=  56  Cycles/STRD= 24.39

Index=  64  Cycles/STRD= 24.39

Index=  96  Cycles/STRD= 24.39

Index= 128  Cycles/STRD= 34.72

Index= 192  Cycles/STRD= 24.39

Memory load performance test at 0x44300000(remote DDR3A)

Index=   0  Cycles/LDRD= 117.66

Index=   1  Cycles/LDRD= 115.95

Index=   2  Cycles/LDRD= 115.49

Index=   3  Cycles/LDRD= 115.37

Index=   4  Cycles/LDRD= 115.21

Index=   5  Cycles/LDRD= 115.52

Index=   6  Cycles/LDRD= 116.09

Index=   7  Cycles/LDRD= 115.41

Index=   8  Cycles/LDRD= 114.66

Index=  10  Cycles/LDRD= 115.30

Index=  12  Cycles/LDRD= 116.89

Index=  14  Cycles/LDRD= 115.21

Index=  16  Cycles/LDRD= 116.67

Index=  18  Cycles/LDRD= 115.48

Index=  20  Cycles/LDRD= 117.16

Index=  22  Cycles/LDRD= 117.04

Index=  24  Cycles/LDRD= 116.44

Index=  26  Cycles/LDRD= 114.73

Index=  28  Cycles/LDRD= 114.62

Index=  30  Cycles/LDRD= 114.76

Index=  32  Cycles/LDRD= 115.75

Index=  40  Cycles/LDRD= 115.54

Index=  48  Cycles/LDRD= 114.99

Index=  56  Cycles/LDRD= 117.43

Index=  64  Cycles/LDRD= 115.61

Index=  96  Cycles/LDRD= 115.48

Index= 128  Cycles/LDRD= 116.91

Index= 192  Cycles/LDRD= 116.92

Index= 256  Cycles/LDRD= 120.01

Index= 384  Cycles/LDRD= 119.28

Index= 512  Cycles/LDRD= 124.78

Index= 768  Cycles/LDRD= 125.33

Index=1024  Cycles/LDRD= 122.44

Index=1536  Cycles/LDRD= 122.27

Index=2048  Cycles/LDRD= 124.22

Index=3072  Cycles/LDRD= 122.40

Memory store performance test at 0x44300000(remote DDR3A)

Index=   0  Cycles/STRD= 24.39

Index=   1  Cycles/STRD= 24.39

Index=   2  Cycles/STRD= 24.39

Index=   3  Cycles/STRD= 24.39

Index=   4  Cycles/STRD= 24.39

Index=   5  Cycles/STRD= 24.39

Index=   6  Cycles/STRD= 24.39

Index=   7  Cycles/STRD= 24.39

Index=   8  Cycles/STRD= 24.39

Index=  10  Cycles/STRD= 24.39

Index=  12  Cycles/STRD= 24.39

Index=  14  Cycles/STRD= 24.39

Index=  16  Cycles/STRD= 24.39

Index=  18  Cycles/STRD= 24.39

Index=  20  Cycles/STRD= 24.39

Index=  22  Cycles/STRD= 24.39

Index=  24  Cycles/STRD= 24.39

Index=  26  Cycles/STRD= 24.39

Index=  28  Cycles/STRD= 24.39

Index=  30  Cycles/STRD= 24.39

Index=  32  Cycles/STRD= 24.39

Index=  40  Cycles/STRD= 24.39

Index=  48  Cycles/STRD= 24.39

Index=  56  Cycles/STRD= 24.39

Index=  64  Cycles/STRD= 24.39

Index=  96  Cycles/STRD= 24.39

Index= 128  Cycles/STRD= 24.39

Index= 192  Cycles/STRD= 24.39

Index= 256  Cycles/STRD= 24.39

Index= 384  Cycles/STRD= 24.39

Index= 512  Cycles/STRD= 24.39

Index= 768  Cycles/STRD= 24.39

Index=1024  Cycles/STRD= 24.39

Index=1536  Cycles/STRD= 24.39

Index=2048  Cycles/STRD= 24.39

Index=3072  Cycles/STRD= 24.40

Memory load performance test at 0x47300000(remote DDR3B)

Index=   0  Cycles/LDRD= 114.64

Index=   1  Cycles/LDRD= 115.00

Index=   2  Cycles/LDRD= 115.26

Index=   3  Cycles/LDRD= 115.33

Index=   4  Cycles/LDRD= 114.51

Index=   5  Cycles/LDRD= 115.77

Index=   6  Cycles/LDRD= 114.15

Index=   7  Cycles/LDRD= 113.47

Index=   8  Cycles/LDRD= 115.04

Index=  10  Cycles/LDRD= 116.18

Index=  12  Cycles/LDRD= 113.79

Index=  14  Cycles/LDRD= 115.03

Index=  16  Cycles/LDRD= 114.19

Index=  18  Cycles/LDRD= 114.41

Index=  20  Cycles/LDRD= 116.03

Index=  22  Cycles/LDRD= 114.35

Index=  24  Cycles/LDRD= 112.81

Index=  26  Cycles/LDRD= 116.17

Index=  28  Cycles/LDRD= 116.56

Index=  30  Cycles/LDRD= 114.96

Index=  32  Cycles/LDRD= 114.29

Index=  40  Cycles/LDRD= 114.51

Index=  48  Cycles/LDRD= 113.75

Index=  56  Cycles/LDRD= 114.53

Index=  64  Cycles/LDRD= 114.87

Index=  96  Cycles/LDRD= 113.65

Index= 128  Cycles/LDRD= 116.08

Index= 192  Cycles/LDRD= 115.38

Index= 256  Cycles/LDRD= 118.84

Index= 384  Cycles/LDRD= 117.31

Index= 512  Cycles/LDRD= 121.11

Index= 768  Cycles/LDRD= 120.89

Index=1024  Cycles/LDRD= 122.93

Index=1536  Cycles/LDRD= 121.89

Index=2048  Cycles/LDRD= 120.21

Index=3072  Cycles/LDRD= 120.91

Memory store performance test at 0x47300000(remote DDR3B)

Index=   0  Cycles/STRD= 24.39

Index=   1  Cycles/STRD= 24.39

Index=   2  Cycles/STRD= 24.39

Index=   3  Cycles/STRD= 24.39

Index=   4  Cycles/STRD= 24.39

Index=   5  Cycles/STRD= 24.39

Index=   6  Cycles/STRD= 24.39

Index=   7  Cycles/STRD= 24.39

Index=   8  Cycles/STRD= 24.39

Index=  10  Cycles/STRD= 24.39

Index=  12  Cycles/STRD= 24.39

Index=  14  Cycles/STRD= 24.39

Index=  16  Cycles/STRD= 24.39

Index=  18  Cycles/STRD= 24.39

Index=  20  Cycles/STRD= 24.39

Index=  22  Cycles/STRD= 24.39

Index=  24  Cycles/STRD= 24.39

Index=  26  Cycles/STRD= 24.39

Index=  28  Cycles/STRD= 24.39

Index=  30  Cycles/STRD= 24.39

Index=  32  Cycles/STRD= 24.39

Index=  40  Cycles/STRD= 24.39

Index=  48  Cycles/STRD= 24.39

Index=  56  Cycles/STRD= 24.39

Index=  64  Cycles/STRD= 24.39

Index=  96  Cycles/STRD= 24.39

Index= 128  Cycles/STRD= 24.39

Index= 192  Cycles/STRD= 24.39

Index= 256  Cycles/STRD= 24.39

Index= 384  Cycles/STRD= 24.39

Index= 512  Cycles/STRD= 24.39

Index= 768  Cycles/STRD= 24.39

Index=1024  Cycles/STRD= 24.39

Index=1536  Cycles/STRD= 24.39

Index=2048  Cycles/STRD= 24.39

Index=3072  Cycles/STRD= 24.39

Overhead test with EDMA CC0 TC0

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes    479 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes    483 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes    480 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1037 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1148 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes    484 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes    482 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes    480 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1037 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes    593 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1030 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1036 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1142 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes   1037 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x46140000(local MSRAM ), consumes   1036 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes    480 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes   1036 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes    483 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes   1037 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes   1149 cycles, achieve throughput     8 MB/s

Throughput test with EDMA CC0 TC0

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes  39292 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes  39200 cycles, achieve throughput  2006 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes  39293 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes  76541 cycles, achieve throughput  1027 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes  60476 cycles, achieve throughput  1300 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes  63528 cycles, achieve throughput  1237 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes  39183 cycles, achieve throughput  2007 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes  39217 cycles, achieve throughput  2005 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes  39181 cycles, achieve throughput  2007 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes  76541 cycles, achieve throughput  1027 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes  60117 cycles, achieve throughput  1308 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes  62091 cycles, achieve throughput  1266 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes  39279 cycles, achieve throughput  2002 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes  39307 cycles, achieve throughput  2000 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes  39295 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes  76532 cycles, achieve throughput  1027 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes  60336 cycles, achieve throughput  1303 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes  71205 cycles, achieve throughput  1104 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes 115908 cycles, achieve throughput   678 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x46140000(remote MSRAM), consumes  98091 cycles, achieve throughput   801 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes  98892 cycles, achieve throughput   795 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes  39291 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes  59523 cycles, achieve throughput  1321 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes  39181 cycles, achieve throughput  2007 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes  61082 cycles, achieve throughput  1287 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes  39283 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes  59964 cycles, achieve throughput  1311 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes  93218 cycles, achieve throughput   843 MB/s

Overhead test with EDMA CC0 TC1

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes    480 cycles, achieve throughput    20 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes    482 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes    483 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1148 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes    483 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes    481 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes    483 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1037 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1037 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes   1367 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1031 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1145 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes   1040 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x46140000(local MSRAM ), consumes   1036 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes   1036 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes    481 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes    482 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes   1040 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes   1038 cycles, achieve throughput     9 MB/s

Throughput test with EDMA CC0 TC1

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes  39289 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes  39211 cycles, achieve throughput  2005 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes  39294 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes  76539 cycles, achieve throughput  1027 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes  60344 cycles, achieve throughput  1303 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes  64431 cycles, achieve throughput  1220 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes  39183 cycles, achieve throughput  2007 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes  39197 cycles, achieve throughput  2006 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes  39186 cycles, achieve throughput  2006 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes  76544 cycles, achieve throughput  1027 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes  60449 cycles, achieve throughput  1300 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes  63536 cycles, achieve throughput  1237 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes  39290 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes  39301 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes  39290 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes  76546 cycles, achieve throughput  1027 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes  60124 cycles, achieve throughput  1308 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes  69198 cycles, achieve throughput  1136 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes 115904 cycles, achieve throughput   678 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x46140000(remote MSRAM), consumes  98020 cycles, achieve throughput   802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes  98673 cycles, achieve throughput   797 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes  39293 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes  60203 cycles, achieve throughput  1306 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes  39180 cycles, achieve throughput  2007 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes  60305 cycles, achieve throughput  1304 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes  39300 cycles, achieve throughput  2001 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes  59751 cycles, achieve throughput  1316 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes  92218 cycles, achieve throughput   852 MB/s

Overhead test with EDMA CC1 TC0

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes    601 cycles, achieve throughput    15 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes    593 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1036 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1036 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes    595 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes    594 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes    591 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1149 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes    697 cycles, achieve throughput    13 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes    589 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1144 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1040 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1256 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x46140000(local MSRAM ), consumes   1036 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes   1147 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes    594 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes    594 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes   1151 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes    588 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes   1144 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes   1148 cycles, achieve throughput     8 MB/s

Throughput test with EDMA CC1 TC0

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes  43625 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes  43660 cycles, achieve throughput  1801 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes  43630 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes  83655 cycles, achieve throughput   940 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes  82704 cycles, achieve throughput   950 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes  90995 cycles, achieve throughput   864 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes  43626 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes  43699 cycles, achieve throughput  1799 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes  43626 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes  87987 cycles, achieve throughput   893 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes  82941 cycles, achieve throughput   948 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes  91110 cycles, achieve throughput   863 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes  43732 cycles, achieve throughput  1798 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes  43775 cycles, achieve throughput  1796 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes  43738 cycles, achieve throughput  1798 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes  83097 cycles, achieve throughput   946 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes  83023 cycles, achieve throughput   947 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes  98557 cycles, achieve throughput   797 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes 120348 cycles, achieve throughput   653 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x46140000(remote MSRAM), consumes 102440 cycles, achieve throughput   767 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes 106341 cycles, achieve throughput   739 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes  43628 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes  86427 cycles, achieve throughput   909 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes  43626 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes  85653 cycles, achieve throughput   918 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes  43745 cycles, achieve throughput  1797 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes  86316 cycles, achieve throughput   911 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes  98561 cycles, achieve throughput   797 MB/s

Overhead test with EDMA CC1 TC1

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes    591 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes    485 cycles, achieve throughput    19 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes    591 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1037 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1146 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes    595 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes    595 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1927 cycles, achieve throughput     4 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes    698 cycles, achieve throughput    13 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes    589 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1148 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1261 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes   1037 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x46140000(local MSRAM ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes   1148 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes    594 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes   1040 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes    595 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes   1148 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes    587 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes   1144 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes   1151 cycles, achieve throughput     8 MB/s

Throughput test with EDMA CC1 TC1

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes  43630 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes  43653 cycles, achieve throughput  1801 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes  43636 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes  83874 cycles, achieve throughput   937 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes  82704 cycles, achieve throughput   950 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes  93560 cycles, achieve throughput   840 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes  43634 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes  43650 cycles, achieve throughput  1801 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes  43631 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes  85766 cycles, achieve throughput   916 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes  82786 cycles, achieve throughput   949 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes  91659 cycles, achieve throughput   857 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes  43734 cycles, achieve throughput  1798 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes  44656 cycles, achieve throughput  1761 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes  43740 cycles, achieve throughput  1797 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes  84317 cycles, achieve throughput   932 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes  82786 cycles, achieve throughput   949 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes  98554 cycles, achieve throughput   797 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes 120343 cycles, achieve throughput   653 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x46140000(remote MSRAM), consumes 102440 cycles, achieve throughput   767 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes 104995 cycles, achieve throughput   749 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes  43627 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes  87765 cycles, achieve throughput   896 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes  43630 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes  86102 cycles, achieve throughput   913 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes  44073 cycles, achieve throughput  1784 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes  87547 cycles, achieve throughput   898 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes 100009 cycles, achieve throughput   786 MB/s

Overhead test with EDMA CC1 TC2

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes    593 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes    591 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1037 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1149 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes    595 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes    593 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes    593 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1152 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes    698 cycles, achieve throughput    13 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes    595 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes    593 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1032 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1260 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes   1037 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x46140000(local MSRAM ), consumes   1038 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes   1149 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes    594 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes   1371 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes    594 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes   1150 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes   1148 cycles, achieve throughput     8 MB/s

Throughput test with EDMA CC1 TC2

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes  43634 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes  43662 cycles, achieve throughput  1801 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes  43624 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes  82657 cycles, achieve throughput   951 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes  82673 cycles, achieve throughput   951 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes  90327 cycles, achieve throughput   870 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes  43630 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes  43656 cycles, achieve throughput  1801 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes  43631 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes  82219 cycles, achieve throughput   956 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes  82892 cycles, achieve throughput   948 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes  93881 cycles, achieve throughput   837 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes  43736 cycles, achieve throughput  1798 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes  43774 cycles, achieve throughput  1796 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes  43727 cycles, achieve throughput  1798 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes  82200 cycles, achieve throughput   956 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes  82807 cycles, achieve throughput   949 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes 100102 cycles, achieve throughput   785 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes 120121 cycles, achieve throughput   654 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x46140000(remote MSRAM), consumes 102441 cycles, achieve throughput   767 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes 105111 cycles, achieve throughput   748 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes  43623 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes  86876 cycles, achieve throughput   905 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes  43632 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes  86988 cycles, achieve throughput   904 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes  43740 cycles, achieve throughput  1797 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes  85539 cycles, achieve throughput   919 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes  98342 cycles, achieve throughput   799 MB/s

Overhead test with EDMA CC1 TC3

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes    594 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes    593 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes   1035 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes   1036 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes    594 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes    595 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes    594 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes   1040 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes   1040 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes   1149 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes    698 cycles, achieve throughput    13 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes    589 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes    593 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes   1033 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes   1032 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes   1256 cycles, achieve throughput     7 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes   1039 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x46100000(remote MSRAM) to 0x46140000(local MSRAM ), consumes   1037 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes   1147 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes   1036 cycles, achieve throughput     9 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes    592 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes   1150 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes    589 cycles, achieve throughput    16 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes   1149 cycles, achieve throughput     8 MB/s

transfer    1 *     8 Bytes with index=    8 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes   1149 cycles, achieve throughput     8 MB/s

Throughput test with EDMA CC1 TC3

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x40040000(remote LL2  ), consumes  43627 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x46140000(remote MSRAM), consumes  43662 cycles, achieve throughput  1801 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x44300000(remote DDRA ), consumes  43635 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x10840000(local LL2   ), consumes  82318 cycles, achieve throughput   955 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x10840000(local LL2   ), consumes  82682 cycles, achieve throughput   951 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x10840000(local LL2   ), consumes  90876 cycles, achieve throughput   865 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x40040000(remote LL2  ), consumes  43630 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x46140000(remote MSRAM), consumes  43690 cycles, achieve throughput  1800 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x44300000(remote DDRA ), consumes  43631 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x c0c0000(local MSRAM ), consumes  82323 cycles, achieve throughput   955 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x c0c0000(local MSRAM ), consumes  82928 cycles, achieve throughput   948 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x c0c0000(local MSRAM ), consumes  90890 cycles, achieve throughput   865 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x40040000(remote LL2  ), consumes  43954 cycles, achieve throughput  1789 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x46140000(remote MSRAM), consumes  44106 cycles, achieve throughput  1783 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x44300000(remote DDRA ), consumes  43741 cycles, achieve throughput  1797 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x88100000(local DDR   ), consumes  84332 cycles, achieve throughput   932 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x88100000(local DDR   ), consumes  83006 cycles, achieve throughput   947 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x88100000(local DDR   ), consumes  99109 cycles, achieve throughput   793 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x40020000(remote LL2  ) to 0x40040000(remote LL2  ), consumes 120346 cycles, achieve throughput   653 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x46100000(remote MSRAM) to 0x46140000(remote MSRAM), consumes 102504 cycles, achieve throughput   767 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x44200000(remote DDRA ) to 0x44300000(remote DDRA ), consumes 105336 cycles, achieve throughput   746 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x10820000(local LL2   ) to 0x47300000(remote DDRB ), consumes  43629 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x10840000(local LL2   ), consumes  85879 cycles, achieve throughput   915 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x c080000(local MSRAM ) to 0x47300000(remote DDRB ), consumes  43634 cycles, achieve throughput  1802 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x c0c0000(local MSRAM ), consumes  86437 cycles, achieve throughput   909 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x88000000(local DDR   ) to 0x47300000(remote DDRB ), consumes  43734 cycles, achieve throughput  1798 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x88100000(local DDR   ), consumes  85642 cycles, achieve throughput   918 MB/s

transfer    4 * 16384 Bytes with index=16384 from 0x47200000(remote DDRB ) to 0x47300000(remote DDRB ), consumes 100673 cycles, achieve throughput   781 MB/s

EDMA test complete

-------------HyperLink 0 status--------------

a request has been detected on the Tx VBUSM slave interface.

the Tx PLS layer has linked to the remote device.

lane zero has been identified during training

TX Serdes lane 0 is enabled.

RX Serdes lane 0 is enabled.

TX Serdes lane 1 is enabled.

RX Serdes lane 1 is enabled.

TX Serdes lane 2 is enabled.

RX Serdes lane 2 is enabled.

TX Serdes lane 3 is enabled.

RX Serdes lane 3 is enabled.