KeyStone 2

Self Test Kit on ARM

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial Release |

Contents

1	Overview	3

2	Framework	3

3	Test methods	7

4	Test coverage	8

5	To run the test program on KeyStone 2 EVM	8

6	Test configuration	11

7	Migrate the test to custom board	12

References	13

Figures

Figure 1.	Two layers of STK	4

Figure 2.	Directory structure of test projects	5

Figure 3.	Default include path	9

Figure 4.	Setup path variable for workspace	10

Figure 5.	define symbols in CCS compiler options to select device and board	11

Tables

Table 1.	Source files of  low level common codes	6

- Overview

The main purpose of the STK is to help hardware debug including new board bring-up and failure analysis. The STK is implemented based on TI’s EVM, but it is designed in the way that it is easy to be modified to test other custom boards.

The STK also measures the performance of most modules in the KeyStone device. The performance data measured on TI’s EVM can be used for solution evaluation and analysis; it can also be used as reference when user measures the performance on other custom boards.

Please note, performance/throughput test is also useful for hardware debug. For example, lower performance than expectation may indicate:

- Improper speed (PLL) configuration

- Hardware interface signal integrity issue which result in retransmission

The STK code can also be used as reference code for user to develop their own driver code.

To achieve good Readability and portability, in STK:

- No directly register access through its raw address

- No directly register write with raw value

Most STK modules are developed based on register layer of CSL (Chip Support Library). Register overlay pointer is used to access register; Register values are composed with SHIFT/MASK macros defined in CSL header files. For some registers not defined in CSL, such as PLL and Serdes, the STK defines the SHIFT/MASK macros in the style of CSL.

CSL is lowest level of PDK (Platform Development Kit), and PDK is part of MCSDK (MultiCore Software Development Kit).

To make STK simple and highly efficient, BIOS and LLD of MCSDK is not used in STK.

STK is designed to not rely on GEL initialization because normally GEL is not available on other custom boards. All modules used or tested by STK are initialized by STK.

- Framework

The STK includes test project for each interface or hardware module, the test program executes multiple test cases to cover most usage cases for each module.

The STK programs are implemented in two layers, Figure 1 shows it.

- Two layers of STK

Low level hardware configuration and driver is implemented based on CSL.

Typical configuration API are defined as a configuration structure and a configuration function like below:

typedef struct {

Parameter 1;

Parameter 2;

…

} KeyStone_xxx_Config;

KeyStone_xxx_Init(KeyStone_xxx_Config * xxx_cfg);

For hardware debug purpose, users only need modify the parameters of higher level API to execute different test cases on different boards.

Low level code only need be modified to implement very rare/special test case. Low level code may be used directly in user’s driver, or used as reference code for user’s driver development.

Figure 2 shows the directory structure of test projects.

- Directory structure of test projects

All documents of the STK are in the “docs” folder.

“auto_test” folder includes JavaScripts to run multiple STK programs automatically, and reference test results of all test programs are also included in the folder for reference.

The low level hardware configuration and driver code are in the “common” folder, they may be linked into multiple test projects.

Each of all other folders includes one test project, the high level test codes are in the “src” subfolder of each test project.

Table 1 describes some commonly used low level code; other modules specific low level code and high level test code are described in the user’s guide of each test module.

- Source files of  low level common codes

| Source files | Descriptions |
| --- | --- |
| startup_A15.s | ARM Cortex A15 core startup code, and Interrupt vector table. |
| ARMv7_CPU.s/h | ARM core IRQ/FIQ enable/disable functions. |
| CP15.s/h | Functions based on ARM CP15 including MMU, cache, PMN… |
| K2_common.c/h | Implementation of APIs for: - PLL configuration; - timer configuration and watch dog service function; - CCNT (cycle counter) utility functions; - MMU configuration; - ARM GIC (Generic Interrupt Controller) initialization and common interrupt handler. - Exception configuration and handlers for CPU internal exception, memory protection error, bus error, EDMA error… - CIC (Chip Interrupt Controller) initialization; - PSC(Power Sleep controller) functions; - EDMA initialization and EDMA copy/fill functions; - MSRAM (Multicore Shared RAM) and DDR MPAX setup and Peripherals MPU setup; - MSRAM EDC setup and error handler; - memory test functions including address, data pattern and bus tests. - Functions to detect and print device information including device type and revision, required core voltage, speed grade, boot mode… |
| common_test.c/h | Common test functions including memory test, memory copy, EDMA copy… |
| K2_Navigator_init_drv.c/h KeyStone_Packet_Descriptor.h | Implementation of APIs for: - QMSS linking RAM initialization; - QMSS descriptor region initialization; - Queue initialization and operations; - QMSS PDSP firmware setup, accumulation and reclamation control; - Packet DMA configuration and channel control; |
| K2_Board_Init.c/h | Definition/configuration board specific parameters (such as input clock speed, external memory size, DDR configuration parameters…). These parameters may be modified when user migrates the STK to custom board. |
| K2_DDR_Init.c/h | DDR3 initialization |
| SerDes folders | Source code for SerDes initialization, they are copied from pdk_keystone2_3_01_01_04. |

- Test methods

The test methods or algorithms for each module may be different, refer to the user’s guide of each module for details. This section introduces some common test methods:

The basic test methods include: write/readback test, loopback test, output test and input test.

The basic steps for Write/readback test are:

- STK code writes special data patterns (such as 0, 0xFFFFFFFF, 0x55555555, address…) to destination

- STK code reads back data from the destination and compares with the known pattern.

Write/readback test is normally used for memories including internal memories, external DDR3, NOR FLASH and NAND flash through EMIF16, NOR FLASH through SPI, EEPROM through I2C. Write/readback test is also used for modules can be accessed like memory including PCIE, HyperLink.

The basic steps for loopback test are:

- Setup the module under test in loopback mode. For internal loopback, the STK code setup the module in loopback mode; for external loopback test, it may be implemented with an external cable or wires setup by user manually, or it may need run test program on the  device on the other end to let it send data back.

- The STK code generates test data (normally, with special pattern); and then sends the data to the module under test;

- The data is looped back in the module under test.

- The STK code receives the loop backed data, and verifies the data.

The modules support both internal and external loopback test include HyperLink, PCIE, Ethernet, UART. The STK modules support internal loopback include Packet DMA, SPI, I2C.

The output only test is normally used to output data for hardware signal measurement or for testing the other device. For example, continuously transferring 0x55555555 on UART or SPI will generate square wave on the TX pin, this facilitates hardware signal measurement. For SerDes based interfaces, to measure the eye-diagram, normally, we also need transfer data continuously.

The input only test is normally used to detect data or signal generated from another device, for example, the GPIO interrupt triggered by external device; the ATR sequence from a SIM card.

For tests of most interfaces, both internal loopback and two devices tests are implemented. To simplify the STK usage, same test program are used on two Devices test. The trick is that user must load the program into core 0 of Device0, and load same program into core 1 of Device1. The program will detect the core number at run time, if it is core 0, then it executes the configuration and functions for Device0; if it is core 1, then it executes the configuration and functions for Device1.

If internal loopback test passes, while external test fails, then, it normally means the internal modules work well, the problem should on external signals or the other device connected to the interface. Normally, the signal integrity on PCB should be checked to analyze the root cause.

- Test coverage

The tests should cover most usage cases of the module. For example, both CPU and DMA are used to test memories, PCIE, HyperLink, UART…

The test cases are designed to match real usage cases as much as possible. The data receiving of most cases is triggered by interrupt though polling is also supported.

Most modules can be tested continuously, not simple one-shot test. For example, for packet DMA based module, limited number of descriptor/packets are used to test the module continuously, the descriptor/packets recycling are fully implemented like real application.

The performance tests of a module are designed to fully stretch the capabilities of the module. Highest speed is tested; all channels/links/ports (if available) may be tested simultaneously.

- To run the test program on KeyStone 2 EVM

The test programs can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder);

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- To run the test on one core, load the program to ARM core 0 of a device; to run the test on all cores, load the program to all ARM cores; to run the test on two devices, normally, the program is loaded into ARM core 0 of device 0 and ARM core 1 of device 1.

- run the core or cores. For tests between two devices, some test may require run one device before the other device. Refer to the user’s guide of the specific module for details.

The default include path setup of most projects looks like following figure.

- Default include path

Please note that:

- The path variable “PDK_INSTALL_PATH” is not defined by default.

- the "${PDK_INSTALL_PATH}\packages" and "${PDK_INSTALL_PATH}\packages\ti\csl" are in front of the "C:\ti\pdk_keystone2_3_01_01_04\packages" and "C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl"

For user who uses PDK in default path, no modification is required for the setup. The CCS/compiler will generates a warning message about the invalid path based on variable “PDK_INSTALL_PATH” (the warning message may be ignored), and then the CCS/compiler finds the header files in the absolute path.

For other users who install PDK in different path, the CCS/compiler will generate error that it can not find header files.

For user who only want to modify the include path for one project, the straightforward way is to modify the absolute path directly in the Dircotries Options dialog. Please note, both two absolute paths "C:\ti\pdk_keystone2_3_01_01_04\packages" and "C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl" need be modified.

For user who wants to modify the include path for whole workspace (valid for all projects in the workspace), the most efficient way is to setup the “PDK_INSTALL_PATH” path variable through Window menu -> Preferences dialog as shown in following figures.

- Setup path variable for workspace

As long as the “PDK_INSTALL_PATH” path variable is defined and valid, the CCS/compiler will find the header file in it.

If both the path based on the “PDK_INSTALL_PATH” variable and the absolute path are valid, the CCS/compiler will use the header files in the path based on the variable if it is in front of the absolute path.

To run all these programs manually in CCS may take about one hour, to run all these programs on multiple boards is not only time consuming but also tedious. So, Automation test script based on DSS (Debug Server Script) is provided in “auto_test” subfolder to improve test efficiency. Refer to “KeyStone_2_STK_Automation_Test_User's_Guide.doc” for more details.

- Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

For each module of STK, there may be multiple macros defined in the source code to configure the test. Refer to the user’s guide of the specific module for details.

- Migrate the test to custom board

These tests are implemented based on TI’s EVM boards.

Most board specific parameters can be modified or added in the “K2_Board_Init.c” and “K2_Board_Init.h”. The parameters of all TI’s EVM have been setup properly. Below is example for K2H and K2K EVM.

#elif defined(K2H_EVM) || defined(K2K_EVM)

/*Digital PLLs reference clock in MHz,

target speed= xxx_REF_CLK_MHZ/xxx_DIVIDER*xxx_multiplier*/

/*122.88/4*39= 1198.08MHz*/

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

/*122.88/24*205= 1049.6MHz.

please note, there is additional 1/3 divider between PASSPLL and PASS*/

#define PASS_PLL_REF_CLK_MHZ    122.88

#define PASS_PLL_DIVISOR        24

#define PASS_PLL_MULTIPLIER     205

/*125/5*48=1200MHz*/

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

/*Serdes reference clock speeed, must be set with CSL enum value*/

#define SRIOSGMII_SERDES_REF_CLK    CSL_SERDES_REF_CLOCK_156p25M

#define SGMII_SERDES_REF_CLK    SRIOSGMII_SERDES_REF_CLK

#define PCIE_SERDES_REF_CLK         CSL_SERDES_REF_CLOCK_100M

#define HYPERLINK_SERDES_REF_CLK    CSL_SERDES_REF_CLOCK_312p5M

/*DDR3 memory size in bytes*/

#define DDR3A_SIZE_BYTES    (1*1024*1024*1024UL)

#if defined(K2K_EVM_REV2) || defined(K2H_EVM_REV2)

//the DDR3 memory on EVM rev 2.0 is K4B4G1646B

#define DDR3B_SIZE_BYTES    (2*1024*1024*1024)

#else

//the DDR3 memory on EVM rev 1.0 is K4B2G1646E

#define DDR3B_SIZE_BYTES    (1*1024*1024*1024)

#endif

/*FLASH, EEPROM size in bytes, speed in KHz*/

#define SPI_FLASH_SIZE_BYTES 	(16*1024*1024)

#define SPI_FLASH_SPEED_KHZ 	54000

#define I2C_EEPROM_SIZE_BYTES 	(64*1024)

#define I2C_EEPROM_ADDRESS 	0x51

#else

#error Board type is NOT defined.

#endif

To run the test programs on different EVM, user only need select different macro switch as introduced in above section.

To migrate the test program to custom board, user may define new macro switch with different parameter values. For example, for a board named K2H_PROCESSING_BOARD, which uses 100MHz reference clock, user should add following macro switch:

#elif defined(K2H_PROCESSING_BOARD)

/*Digital PLLs reference clock in MHz,

target speed= xxx_REF_CLK_MHZ/xxx_DIVIDER*xxx_multiplier*/

/*100/1*12= 1200MHz*/

#define MAIN_PLL_REF_CLK_MHZ    100

#define MAIN_PLL_DIVISOR        1

#define MAIN_PLL_MULTIPLIER     12

……

……

……

#else

To run the program on this board, in the compiler option as shown in Figure 5, the user should change K2H_EVM to K2H_PROCESSING_BOARD.

In real system, the DDR configuration may also be changed according to user’s hardware design in the function K2_DDR3A_config() and K2_DDR3B_config() in “K2_Board_Init.c”.

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone II Architecture ARM CorePac User's Guide (SPRUHJ4)

- KeyStone Multicore Shared Memory Controller (MSMC) User's Guide (SRPUGW7)

- KeyStone II DDR3 Memory Controller User's Guide (SPRUHN7)

- KeyStone Architecture Enhanced Direct Memory Access (EDMA3) Controller User Guide (SPRUGS5)