KeyStone 2

I2C Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial release |

Contents

1	Overview	3

2	Test Method	3

2.1	Internal loopback test	3

2.2	I2C EEPROM test	3

2.2.1	Data pattern filling Test	4

2.2.2	Addressing test	4

2.3	I2C temperature sensor test	5

3	Test code/project	5

3.1	CCS project	5

3.2	To run the test program on KeyStone 2 EVM	7

3.3	Test configuration	7

3.4	Migrate the test to custom board	9

References	9

Appendix: Typical test output	10

Figures

Figure 1.	Directory structure of test projects	6

Figure 2.	define symbols in CCS compiler options to select device and board	8

Tables

Table 1.	Source files of the test codes	6

- Overview

I2C is a commonly used control bus, most KeyStone devices integrate one to three I2C interfaces.

The I2C Self Test Kit (STK) is implemented to test:

- All I2C interfaces in the device;

- Internal loopback

- I2C EEPROM (if available)

- I2C temperature sensor (if available)

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test Method

  - Internal loopback test

With Internal loopback test, the data in TX buffer is transmitted to I2C TX module, and loopback to I2C RX module, and then received to RX data buffer. The data in TX and RX buffer are compared, the throughput are calculated.

The key purposes of the internal loopback test include:

- Verify the functions of I2C controller and related internal modules. If internal loopback test passes, while external test fails, then, it normally means the I2C controller and related internal modules work well, the problem should on external signals or the other device on the I2C bus.

- Verify the throughput. Normally, the throughput in bps should be about (clock speed)*8/9. If the measured throughput is not as expected, it normally indicates I2C clock divider or ARM core PLL is not configured properly.

Please note, the I2C clock speed = (ARM core speed)/(divider factor), and the divider factor is integer, so, the effective I2C clock speed may not exactly meet standard clock speed. For example, 400KHz is standard speed, but the divider can only be configured to generate 399KHz clock. This should be OK in most cases.

  - I2C EEPROM test

The STK is designed to test AT24C512B or AT24C1024B on EVM, which should be compatible with most of the I2C EEPROM.

Two algorithms are used to test I2C EEPROM.

    - Data pattern filling Test

The pseudocode for data pattern filling test is:

for(memory range under test)

fill the memory with a value;

for(memory range under test)

read back the memory and compare the readback value to the written value

Normally, this test is done several times with different test data value. The common values used for test include 0, 0xFFFFFFFF, 0x55555555, 0xAAAAAAAA.

This test can detect stuck data bit, for example, if

written value = 0, readback value = 0x8,

It indicates bit 3 sticks to 1. If

written value = 0xFFFFFFFF, readback value = 0xFFFFFFFE,

It indicates bit 0 sticks to 0.

The test case is also used to generate specific waveforms on hardware signals, which is helpful for hardware signal integrity test. For example, transfer of 0x55555555 or 0xAAAAAAAA will generate square wave on I2C data bus.

    - Addressing test

The pseudocode for addressing test is:

for(memory range under test)

fill each memory unit with its address value;

for(memory range under test)

read back the memory and compare the readback value to the written value

This test can detect stuck bit on address bus inside EEPROM, for example, if

written value = 0 at address 0

written value = 1 at address 1

written value = 2 at address 2

written value = 3 at address 3

……

readback value = 2 at address 0

readback value = 3 at address 1

readback value = 2 at address 2

readback value = 3 at address 3

……

For this case, it indicates bit 1 of address bus sticks. So, the contents in address 0 and 2 are same, and the contents in address 1 and 3 are same.

  - I2C temperature sensor test

I2C temperature sensor is one of the most popular usages of I2C interface. I2C temperature sensor test supports commonly used temperature sensor including TMP100, TMP101, ADT75.

The test code initializes the temperature sensor firstly, and then, reads the temperature from the sensor and prints it out.

The test code can not judge pass or fail automatically. The user should estimate the temperature and compare it to the test result.

- Test code/project

This section introduces the implementation of these tests on KeyStone 2 devices.

  - CCS project

Following figure shows the directory structure of the test projects:

- Directory structure of test projects

The project files are in “I2C” folder. There is some commonly used initialization and driver code for PLL and interrupt… in “common” folder. The main source code files are in the “I2C\src” subfolder. Below table describes these source files.

- Source files of the test codes

| Function | Source file | Description |
| --- | --- | --- |
| K2_I2C_Master_Init | \common\ K2_I2C_Init_drv.c | Initialize I2C as master |
| K2_I2C_read K2_I2C_write |  | Implementation of low level operations on I2C |
| I2C_loopback_test | \I2C\src\I2C_loopback_Test.c | Loopback data and verify them, measure the throughput. |
| I2C_EEPROM_Test | \I2C\src\I2C_EEPROM_Test.c | Fill I2C EEPROM with fixed pattern or address, and then verify them. |
| I2C_EEPROM_read | \I2C\src\I2C_EEPROM_drv.c | Implementation of low level operations on I2C EEPROM. |
| I2C_EEPROM_write |  |  |
| GetTemperature | \I2C\src\I2C_Temp_Sensor_drv.c | Read temperature from the I2C temperature sensor |

  - To run the test program on KeyStone 2 EVM

The test program can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- Load the program to ARM core 0 of a device.

- run the core, then, you should see output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

There are multiple macros defined in the source code to configure the test.

Each test case can be configured to be tested or not through following macros in “I2C_main.c”. The value of “1” means enable the test, and “0” means disable the test.

#define I2C_LOOPBACK_TEST 		1

#define I2C_EEPROM_TEST 		1

#define I2C_TEMP_SENSOR_TEST 	0

The data pattern used for data filling test is defined in “I2C_EEPROM_test.c” as below. You can add, remove or modify the values.

unsigned int uiDataPatternTable[] = { 0x00000000, 0xffffffff, 0xaaaaaaaa, 0x55555555 };

The two memory test algorithms introduced in above section can be enabled or disabled separately through following macros in “I2C_EEPROM_test.c”. “1” means enable, “0” means disable.

#define BIT_PATTERN_FILLING_TEST 		1

#define ADDRESS_TEST 			1

  - Migrate the test to custom board

These tests are implemented based on TI’s EVM boards. In your real system, the I2C EERPOM configuration may be changed according to your hardware design in “K2_Board_Init.h”.

#define I2C_EEPROM_SIZE_BYTES 	(64*1024)

#define I2C_EEPROM_ADDRESS 		0x51

The core speed may be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

In your real system, the I2C EERPOM driver may need be changed in “I2C_EEPROM_drv.c”.

If the temperature sensor in real system is different from TMP100, TMP101, ADT75, then the temperature sensor diver code may be modified in “I2C_Temp_Sensor_drv.c”.

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone Architecture Inter-IC Control Bus (I2C) User Guide (SRPUGV3)

- KeyStone II Architecture ARM CorePac User's Guide (SPRUHJ4)

Appendix: Typical test output

Below is the test output on K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

DDR3A initialization

Initialize DDR data rate = 100.000/1*20/6*4= 1333.3 MTS, bus width = 64 bits.

I2C 0 expected speed = 400KHz, effective speed = 399KHz

I2C 0 loopback test passed with data pattern 0x0. Throughput= 363Kbps

I2C 0 loopback test passed with data pattern 0xff. Throughput= 363Kbps

I2C 0 loopback test passed with data pattern 0x55. Throughput= 363Kbps

I2C 1 expected speed = 400KHz, effective speed = 399KHz

I2C 1 loopback test passed with data pattern 0x0. Throughput= 363Kbps

I2C 1 loopback test passed with data pattern 0xff. Throughput= 363Kbps

I2C 1 loopback test passed with data pattern 0x55. Throughput= 363Kbps

I2C 2 expected speed = 400KHz, effective speed = 399KHz

I2C 2 loopback test passed with data pattern 0x0. Throughput= 363Kbps

I2C 2 loopback test passed with data pattern 0xff. Throughput= 363Kbps

I2C 2 loopback test passed with data pattern 0x55. Throughput= 363Kbps

I2C 0 expected speed = 400KHz, effective speed = 399KHz

I2C EEPROM test start...

Save data from 0x0f000 to 0x0f400.

Passed I2C_EEPROM Fill Test from 0x f000 to 0x f400 with pattern 0x       0

Passed I2C_EEPROM Fill Test from 0x f000 to 0x f400 with pattern 0xffffffff

Passed I2C_EEPROM Fill Test from 0x f000 to 0x f400 with pattern 0xaaaaaaaa

Passed I2C_EEPROM Fill Test from 0x f000 to 0x f400 with pattern 0x55555555

Passed I2C_EEPROM Address Test from 0x f000 to 0x f400

Restore data from 0x0f000 to 0x0f400.

Save data from 0x0f400 to 0x0f800.

Passed I2C_EEPROM Fill Test from 0x f400 to 0x f800 with pattern 0x       0

Passed I2C_EEPROM Fill Test from 0x f400 to 0x f800 with pattern 0xffffffff

Passed I2C_EEPROM Fill Test from 0x f400 to 0x f800 with pattern 0xaaaaaaaa

Passed I2C_EEPROM Fill Test from 0x f400 to 0x f800 with pattern 0x55555555

Passed I2C_EEPROM Address Test from 0x f400 to 0x f800

Restore data from 0x0f400 to 0x0f800.

Save data from 0x0f800 to 0x0fc00.

Passed I2C_EEPROM Fill Test from 0x f800 to 0x fc00 with pattern 0x       0

Passed I2C_EEPROM Fill Test from 0x f800 to 0x fc00 with pattern 0xffffffff

Passed I2C_EEPROM Fill Test from 0x f800 to 0x fc00 with pattern 0xaaaaaaaa

Passed I2C_EEPROM Fill Test from 0x f800 to 0x fc00 with pattern 0x55555555

Passed I2C_EEPROM Address Test from 0x f800 to 0x fc00

Restore data from 0x0f800 to 0x0fc00.

Save data from 0x0fc00 to 0x10000.

Passed I2C_EEPROM Fill Test from 0x fc00 to 0x10000 with pattern 0x       0

Passed I2C_EEPROM Fill Test from 0x fc00 to 0x10000 with pattern 0xffffffff

Passed I2C_EEPROM Fill Test from 0x fc00 to 0x10000 with pattern 0xaaaaaaaa

Passed I2C_EEPROM Fill Test from 0x fc00 to 0x10000 with pattern 0x55555555

Passed I2C_EEPROM Address Test from 0x fc00 to 0x10000

Restore data from 0x0fc00 to 0x10000.

I2C test complete.