KeyStone 2

Memory Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial release |

Contents

1	Overview of KeyStone 2 memory System	3

2	Test Algorithms	4

2.1	Data Test	4

2.2	Addressing test	5

2.3	Bit walking test	6

2.3.1	Data bit walking test	6

2.3.2	Address bit walking test	7

2.4	Floating bus problem	8

2.5	Further investigation of test result	8

3	Test Coverage	8

3.1	Test by different masters	9

3.2	Test all memories in the system	9

3.2.1	Internal Multicore Shared RAM (MSRAM)	9

3.2.2	External DDR SDRAM	10

3.3	Test through different paths	10

4	Test code/project	10

4.1	CCS project	10

4.2	To run the test program on KeyStone 2 EVM	12

4.3	Test configuration	12

4.4	Test time	15

4.5	Migrate the test to custom board	15

References	16

Appendix: Typical test output	17

Figures

Figure 1.	KeyStone 2 SOC memory architecture	3

Figure 2.	Directory structure of test projects	11

Figure 3.	define symbols in CCS compiler options to select device and board	13

Tables

Table 1.	KeyStone 2 Memory system comparison	4

Table 2.	Source files of the test codes	11

- Overview of KeyStone 2 memory System

Figure 1 shows the common KeyStone 2 SOC memory architecture.

- KeyStone 2 SOC memory architecture

For different SOC, the size of memory are different, the number of ARM core and the number of EDMA transfer controller are also different. Table 1 summarizes these differences of KeyStone 2 SOCs.

- KeyStone 2 Memory system comparison

|  | K2H/K2K | K2L | K2E |
| --- | --- | --- | --- |
| L1D(KB/core) | 32 | 32 | 32 |
| L1P(KB/core) | 32 | 32 | 32 |
| L2 Cache(KB) | 4*1024 | 1024 | 4*1024 |
| Shared RAM (KB) | 6*1024 | 2*1024 | 2*1024 |
| ARM core number | 4 | 2 | 4 |
| EDMA transfer controller number | 14 | 10 | 14 |

Memory tests are implemented to cover:

- All memories in the system;

- All ARM cores and DMA transfer controllers which can access memory

- All data paths for accessing memory

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test Algorithms

Three memory test algorithms are introduced in this section, and the purpose of these test algorithms is discussed.

  - Data Test

The pseudocode for data test is:

for(memory range under test)

fill the memory with a value;

for(memory range under test)

read back the memory and compare the readback value to the written value

Normally, this test is done several times with different test data value. The common values used for test include 0x55555555, 0xAAAAAAAA, 0x33333333, 0xCCCCCCCC, 0x0F0F0F0F, 0xF0F0F0F0, 0x00FF00FF, 0xFF00FF00FF00, 0xFFFFFFFF, 0…

This test can detect stuck data bit, for example, if

written value = 0, readback value = 0x8,

It indicates bit 3 sticks to 1. If

written value = 0xFFFFFFFF, readback value = 0xFFFFFFFE,

It indicates bit 0 sticks to 0.

If 0x55555555 (or 0xAAAAAAAA) can be correctly written and read back, it indicates two adjacent bits do not stick to each other; if 0x33333333 (or 0xCCCCCCCC) can be correctly written and read back, it indicates four adjacent bits do not stick to each other; if 0x0F0F0F0F (or 0xF0F0F0F0) can be correctly written and read back, it indicates eight adjacent bits do not stick to each other…

This algorithm can be used to test data bus connection, and can also be used to test memory cells. If this algorithm is used to test memory cells, all memory cells need be written and read with these data patterns, which will consume a lot of time depends on the size of memory; if this algorithm is used to test data bus connection, only one address (any valid address) need be written and read with these patterns.

  - Addressing test

The pseudocode for addressing test is:

for(memory range under test)

fill each memory unit with its address value;

for(memory range under test)

read back the memory and compare the readback value to the written value

This test can detect stuck bit on address bus, for example, if

written value = 0 at address 0

written value = 1 at address 1

written value = 2 at address 2

written value = 3 at address 3

……

readback value = 2 at address 0

readback value = 3 at address 1

readback value = 2 at address 2

readback value = 3 at address 3

……

For this case, it indicates bit 1 of address bus sticks. So, the contents in address 0 and 2 are same, and the contents in address 1 and 3 are same.

The main target of this test is to test the address bus or address decoding unit in memory, but it actually writes and reads all memory cells, so to some extent, it also tests the data bus and memory cells. If the test time is limit, and only allow test the whole memory space for one time, then, the addressing test introduced in this section is the best choice.

  - Bit walking test

Walking “1” is to set only one bit as “1” in the data or address, while all other bits are “0”, and the position of the bit “1” shifts during consecutive accesses, which looks like the “1” walks on the bus. Walking “0” just uses inverted value, which looks like a “0” walks on the bus.

    - Data bit walking test

The pseudocode for data bit walking test is:

bit_mask_value = 1;

for(number of valid data bits)

{

write bit_mask_value to memory;

readback the data and compared to written value;

inversed_bit_mask_value = bit inverse of bit_mask_value;

write inversed_bit_mask_value to memory;

readback the data and compared to written value;

bit_mask_value = bit_mask_value<<1; 	//bit walking

}

This test can detect stuck bit problem, more importantly, this can be used to identify cross-talk issue. The test intentionally make one bit on the bus or memory cells different from all other bits, thus the negative cross-talk from all other bits is strongest. If there is cross-talk issue, it should be exposed by this test.

For example, if

written value = 0x00000010, readback value = 0

written value = 0xFFFFFFEF, readback value = 0xFFFFFFFF

It indicates the bit 4 is affected by cross-talk issue.

This algorithm can be used to test data bus connection and memory cells. To identify cross-talk issue on a data bus, this test only need be executed at one address (any valid address); to identify cross-talk of memory bit cells, this test need be executed for whole memory range under test, which will be the most time consuming test depends on the size of memory.

    - Address bit walking test

The pseudocode for address bit walking test is:

Write 0 to first address;

Write 0xFFFFFFFF to last address;

bit_mask_value = 1;

for(number of valid address bits)

{

write bit_mask_value to address of bit_mask_value;

readback the data at first address and compare to 0;

inversed_bit_mask_value = bit inverse of bit_mask_value;

write inversed_bit_mask_value to address of inversed_bit_mask_value;

readback the data at last address and compare to 0xFFFFFFFF;

bit_mask_value = bit_mask_value<<1; 	//bit walking

}

In a similar way as data bit walking, this test can be used to detect address bit-stuck problem and cross talk issue.

For example, if

written value = 16 at address 16

readback value = 16 at address 0

written value = 0xFFFFFFEF at address 0xFFFFFFEF

readback value = 0xFFFFFFEF at address 0xFFFFFFFF

In this test, the data for address 16 is actually written to address 0; while the data for address 0xFFFFFFEF is actually written to the last address. Normally, this indicates bit 4 of address bus is affected.

  - Floating bus problem

If test program writes and read back from same address immediately, and if there is capacitance on bus, the write operation may charge the capacitor, and the bus status may keep for a short time because of the capacitance, thus immediate read back may get correct (original) value even the correct value is not actually written into the memory cells. This is the so-called floating bus problem.

Floating bus may “cheat” the test program. To avoid floating bus, a dummy write to another address with inverted value is inserted into original write and read operations. For example,

write A to address X;

write inversion of A to address Y;

read value from address X;

In this way, the floating bus is avoided.

  - Further investigation of test result

If above algorithms detects any error, we need analyze further to find the root cause. Normally, if the error symptom is random, we should double check DDR configuration firstly.

If we identify bit-stuck or cross talk, normally, there are three possible causes:

- For external memory, PCB issue is the most common issue. For example, soldering issue, one bus bit is connected to power or ground. We can use multimeter to measure the resistance between the bus signal and the power or ground. If the resistance to power or ground closes to 0, it normally indicates soldering issue. Cross-talk issue is relative complex, oscilloscope may be used to check the signal integrity of all related signals to find the source of cross-talk, and then PCB re-layout may be necessary to solve it.

- Memory cells failure. For external memory, we can use oscilloscope to monitor the bus, if the bit only sticks when read back, it normally indicates external memory failure.

- memory controller failure. If we exclude above possibilities, then we can swap the devices on a good board and the problematic board, if the problem goes with the K2 SOC, then it normally indicates it is memory controller failure.

- Test Coverage

Memory tests should be implemented to cover:

- All memories in the system;

- All ARM cores and DMA transfer controllers which can access memory

- All paths for memory access

  - Test by different masters

As shown in figure 1, there are mainly two types of masters can be used to access memory:

- ARM core, can access all memories

- EDMA, can access all memories

Please note, there are DSP cores in most of KeyStone 2 devices, but this document only discuss the tests run on the ARM core. Separate test program is needed to test memory through DSP core.

All masters should be covered by memory test. However, if all masters are used to test all available memories, there are too much combinations, the test time may be very long. So, memory test should be optimized to use proper master to test proper memories and make sure all of them are covered.

Normally, DMA is much faster than CPU for memory access. So, for big external memory test, EDMA is preferred.

To use DMA to test memory, we need an extra DMA buffer for the test, the test flow is:

ARM core wirtes test data to the DMA buffer

DMA copy the contents of the DMA buffer to the memory under test

DMA copy the contents of the memory under test back to the DMA buffer

ARM core read the contents in DMA buffer and compare to expected value.

Normally, the DMA buffer is a small buffer allocated in internal memory, the memory under test may be much larger than the DMA buffer, so, above operation need be repeated for multiple times to complete the test.

EDMA has multiple transfer controller, they should be used alternately to make sure all of them are covered. For example, we can use EDMA TC1 to executed data filling test with the first data pattern, and then we use EDMA TC2 to executed data filling test with the second data pattern, and so on…

Additionally, on multi-core ARM, the test program may run on multicores simultaneously to stretch the loading. In this case, the MSRAM (Multicore Shared RAM) and DDR should be equally divided for each core’s test.

  - Test all memories in the system

As shown in figure 1, there are mainly four types of memories in the system, all of them should be covered by memory test.

    - Internal Multicore Shared RAM (MSRAM)

Each ARM core and DMA has separated bus to access MSRAM memory, so each ARM core and DMA should be used to test MSRAM memory.

Normally, test code and test data buffer are allocated at the beginning of MSRAM. So, the test program should determine how much memory is occupied by test code and test data buffer, only the remainder part of the MSRAM can be tested by normal memory test algorithms.

Though the part of memory for test code can not be covered by normal memory test algorithms, it is actually used by the test program, if it has problem, the test program will fail somehow. So, to some extent, this part of memory is actually covered by the test. If you really want to cover this part of memory by normal memory test algorithms, you need implement two test programs. One program has the test code in the first half of the memory and test the other part of the memory; and the other program has the test code in the second half of the memory and test the first half of the memory.

    - External DDR SDRAM

External DDR SDRAM should be configured properly before test. If DDR test failed on some boards but passed on some other boards, you may try to relax some timing configuration, if it makes some failed board passes, then it may indicate timing issue on DDR interface.

Both ARM cores and EDMA should be used to test DDR SDRAM. For ARM core test, DDR SDRAM should be test with both cache enabled and disabled.

DDR size may be up to 8GB, DDR test consumes most of the test time. So, we should use EDMA to test full space of DDR, since EDMA is the fastest master to access DDR. And them, for other masters, such as ARM core, they do not need test the full space of DDR again, they only need test a relative small part of the DDR, the purpose is just to cover the data path from those masters to DDR memory since the DDR memory itself has been fully tested by EDMA. For example, to test ARM core access DDR through L2 cache, we only need test a DDR space larger than L2 cache, thus cover the data path and L2 cache.

  - Test through different paths

Since each master has its own bus, same memory may be accessed by different masters from different data paths. For example, ARM core and EDMA access Local L2 through different path.

Same memory may be accessed thought different data path by same master. For example, DDR SDRAM can be accessed by ARM core through cache or directly without cache.

All these different paths should be covered by memory test.

- Test code/project

This section introduces the implementation of these memory tests on KeyStone 2 SOC.

  - CCS project

Following figure shows the directory structure of the test project.

- Directory structure of test projects

The project files are in “Memory_Test” folder. There is some commonly used initialization and driver code for DDR, EDMA and PLL… in “common” folder. The main source code files are in the “Memory_Test\src” subfolder. Below table describes these source files.

- Source files of the test codes

| Source files | Descriptions |
| --- | --- |
| Common_test | Functions for memory bus test, the key API include: unsigned int Memory_Data_Bus_Test(unsigned int uiBaseAddress, unsigned int uiBusWidth) unsigned int Memory_Address_Bus_Test(unsigned int uiBaseAddress, unsigned int uiNumBytes, unsigned int uiMAU_bytes) |
| K2_DDR_Init | DDR initialization. |
| mem_test_DMA | Implementation of memory test algorithms by EDMA, the key API include: int EDMA_MEM_Test(unsigned int uiStartAddress, unsigned int uiStopAddress, unsigned int uiDmaBufAddress, unsigned int uiDmaBufSize); |
| mem_test_CPU | Implementation of memory test algorithms by ARM core. The key API is: int CPU_MEM_Test(unsigned int uiStartAddress, unsigned int uiStopAddress, unsigned int uiStep); |

The test flow is controlled by the code in “mem_test_main.c”. The basic test flow is:

Disable all caches

Test MSRAM bus

Test DDR bus

Enable Instruction cache

Test MSRAM with ARM core

Test 1KB of external memory with ARM core (just cover the data path)

Enable Data cache

Test MSRAM with EDMA

Test MSRAM with ARM core

Test external memory with EDMA (cover full external memory space)

Test 4MB external memory with ARM core (just cover the data path and L1, L2 caches)

  - To run the test program on KeyStone 2 EVM

The test program can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- To run the test on one core, load the program to ARM core 0 of the device; to run the test on all cores, load the program to all cores.

- run the core or cores, then, you should see output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

There are multiple macros defined in the source code to configure the test.

Each memory can be configured to be tested or not through following macros in “Mem_test_main.c”. The value of “1” means enable the test, and “0” means disable the test.

#define MSMC_RAM_MEM_TEST 	1

#define OSR_MEM_TEST		1

#define DDR3A_MEM_TEST 		1

#define DDR3B_MEM_TEST 		1

Whether ARM core or DMA be used for test can be configured through following macros in “Mem_test_main.c”

#define TEST_BY_CPU 			1

#define TEST_BY_DMA 			1

The test may only run on one core or on multiple cores. To run the test on single core, the program should be loaded and run on core 0 only. If only run the test on single core, the maximum DDR3A test size is 2GB, and maximum DDR3B size be tested is 512MB. To run the test on multiple cores, the program should be loaded and run on all cores simultaneously. Following macros in “Mem_test_main.c” can switch between these two tests.

#define SINGLE_CORE_TEST 		1

To test the reliability of memories, user can set following macro to 1 in “Mem_test_main.c”, which will let the tests run infinitely until it is stopped by user manually.

#define TEST_INFINITELY 	0

The data pattern used for data filling test is defined in “Mem_test_CPU.c” as below. You can add, remove or modify the values.

unsigned long long ulDataPatternTable[] = {

0x0000000000000000,

0xffffffffffffffff,

0xaaaaaaaaaaaaaaaa,

0x5555555555555555,

0xcccccccccccccccc,

0xf0f0f0f0f0f0f0f0,

0xff00ff00ff00ff00,

0xffff0000ffff0000,

0xffffffff00000000

};

The three memory test algorithms introduced in above section can be enabled or disabled separately through following macros in “Mem_test_CPU.c” and “Mem_test_DMA.c”. “1” means enable, “0” means disable.

#define BIT_PATTERN_FILLING_TEST 		1

#define ADDRESS_TEST 			1

#define BIT_WALKING_TEST 			1

Normally, disable bit walking test or only executing bit walking test on several memory address is a good option to save test time.

To reduce test time with ARM core, the ARM core memory test function has a parameter “uiStep”:

int CPU_MEM_Test(unsigned int uiStartAddress, unsigned int uiStopAddress, unsigned int uiStep)

Normally, it is called with uiStep= 1 for full coverage like below:

CPU_MEM_Test(DDR_TEST_START_ARRD, DDR_TEST_STOP_ADDR, 1)

uiStep= 1 means to test all DDR memory sequentially. This parameter can be set to a bigger value which means to test one memory unit for every “uiStep” units, thus save time.

The EDMA memory test function does not have this parameter because the efficiency of EDMA index transfer is not good.

  - Test time

The test time depends on memory size, ARM and memory speed. The typical test time on K2 EVM is about several minutes.

Bus tests take hundreds of milliseconds, while full memory cells test takes long time, and DDR test takes more than 90% of the total time.

The proportion of test time between data filling test, addressing test and bit walking test is about 10:1:64, because the data filling test is done for about ten patterns; the addressing test is done for one time; the bit walking test is done on 32 bits for two times.

User can configure the test cases according to acceptable testing time. Normally, there are three combinations:

- Simple and quick buses tests in hundreds of milliseconds.

- Full memory space basic test in several seconds, test cases include buses tests and addressing tests.

- Full memory space thorough test in several to tens of minutes, executes all test cases.

  - Migrate the test to custom board

In your real system, the DDR configuration may be changed according to your hardware design in the function K2_DDR3A_config() and K2_DDR3B_config() in “K2_Board_Init.c”.

The size of DDR3 may be changed through following macros in “K2_Board_Init.h”.

#define DDR3A_SIZE_BYTES    (1*1024*1024*1024UL)

#define DDR3B_SIZE_BYTES    (1*1024*1024*1024)

The core speed may be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone II Architecture ARM CorePac User's Guide (SPRUHJ4)

- KeyStone Multicore Shared Memory Controller (MSMC) User's Guide (SRPUGW7)

- KeyStone II DDR3 Memory Controller User's Guide (SPRUHN7)

- KeyStone Architecture Enhanced Direct Memory Access (EDMA3) Controller User Guide (SPRUGS5)

Appendix: Typical test output

Below is the test output on K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

DDR3A initialization

Initialize DDR data rate = 100.000/1*20/6*4= 1333.3 MTS, bus width = 64 bits.

DDR3B initialization

Initialize DDR data rate = 100.000/1*16/4*4= 1600.0 MTS, bus width = 64 bits.

Memory Test Start at 11322237 cycle

Disable instruction and data cache.

MSMC RAM bus test at 15692531 cycle

Passed Times:        1 		Failed Times:        0

DDR3A bus test at 16642553 cycle

Passed Times:        2 		Failed Times:        0

DDR3B bus test at 16886142 cycle

Passed Times:        3 		Failed Times:        0

Enable Instruction cache only.

MSMC RAM memory test at 17164371 cycle

Memory Fill Test from 0x c03a680 to 0x c580000 with patterns:

0x0000000000000000

0xffffffffffffffff

0xaaaaaaaaaaaaaaaa

0x5555555555555555

0xcccccccccccccccc

0xf0f0f0f0f0f0f0f0

0xff00ff00ff00ff00

0xffff0000ffff0000

0xffffffff00000000

Passed Memory Address Test from 0x c03a680 to 0x c580000

Memory Bit Walking from 0x c03a680 to 0x c580000

0x0000000100000001 		 0xfffffffefffffffe

0x0000000200000002 		 0xfffffffdfffffffd

0x0000000400000004 		 0xfffffffbfffffffb

0x0000000800000008 		 0xfffffff7fffffff7

0x0000001000000010 		 0xffffffefffffffef

0x0000002000000020 		 0xffffffdfffffffdf

0x0000004000000040 		 0xffffffbfffffffbf

0x0000008000000080 		 0xffffff7fffffff7f

0x0000010000000100 		 0xfffffefffffffeff

0x0000020000000200 		 0xfffffdfffffffdff

0x0000040000000400 		 0xfffffbfffffffbff

0x0000080000000800 		 0xfffff7fffffff7ff

0x0000100000001000 		 0xffffefffffffefff

0x0000200000002000 		 0xffffdfffffffdfff

0x0000400000004000 		 0xffffbfffffffbfff

0x0000800000008000 		 0xffff7fffffff7fff

0x0001000000010000 		 0xfffefffffffeffff

0x0002000000020000 		 0xfffdfffffffdffff

0x0004000000040000 		 0xfffbfffffffbffff

0x0008000000080000 		 0xfff7fffffff7ffff

0x0010000000100000 		 0xffefffffffefffff

0x0020000000200000 		 0xffdfffffffdfffff

0x0040000000400000 		 0xffbfffffffbfffff

0x0080000000800000 		 0xff7fffffff7fffff

0x0100000001000000 		 0xfefffffffeffffff

0x0200000002000000 		 0xfdfffffffdffffff

0x0400000004000000 		 0xfbfffffffbffffff

0x0800000008000000 		 0xf7fffffff7ffffff

0x1000000010000000 		 0xefffffffefffffff

0x2000000020000000 		 0xdfffffffdfffffff

0x4000000040000000 		 0xbfffffffbfffffff

0x8000000080000000 		 0x7fffffff7fffffff

Passed Times:        4 		Failed Times:        0

DDR3B memory test at 10024014929 cycle

Memory Fill Test from 0x60000000 to 0x60000400 with patterns:

0x0000000000000000

0xffffffffffffffff

0xaaaaaaaaaaaaaaaa

0x5555555555555555

0xcccccccccccccccc

0xf0f0f0f0f0f0f0f0

0xff00ff00ff00ff00

0xffff0000ffff0000

0xffffffff00000000

Passed Memory Address Test from 0x60000000 to 0x60000400

Memory Bit Walking from 0x60000000 to 0x60000400

Passed Times:        5 		Failed Times:        0

DDR3A memory test at 10027591647 cycle

Memory Fill Test from 0x80000000 to 0x80000400 with patterns:

0x0000000000000000

0xffffffffffffffff

0xaaaaaaaaaaaaaaaa

0x5555555555555555

0xcccccccccccccccc

0xf0f0f0f0f0f0f0f0

0xff00ff00ff00ff00

0xffff0000ffff0000

0xffffffff00000000

Passed Memory Address Test from 0x80000000 to 0x80000400

Memory Bit Walking from 0x80000000 to 0x80000400

Passed Times:        6 		Failed Times:        0

Enable both data and instruction cache.

MSMC RAM memory test with EDMA at 10030861880 cycle

Memory Fill Test from 0x c03a680 to 0x c580000 with patterns:

0x0000000000000000 by EDMA CC0 TC0

0xffffffffffffffff by EDMA CC1 TC2

0xaaaaaaaaaaaaaaaa by EDMA CC2 TC2

0x5555555555555555 by EDMA CC4 TC0

Passed Memory Address Test from 0x c03a680 to 0x c580000 with EDMA CC0 TC0

Memory Bit Walking from 0x c03a680 to 0x c580000 with EDMA

0x0000000100000001 with CC1 TC2

0xfffffffefffffffe with CC2 TC2

0x0000000200000002 with CC4 TC0

0xfffffffdfffffffd with CC0 TC0

0x0000000400000004 with CC1 TC2

0xfffffffbfffffffb with CC2 TC2

0x0000000800000008 with CC4 TC0

0xfffffff7fffffff7 with CC0 TC0

0x0000001000000010 with CC1 TC2

0xffffffefffffffef with CC2 TC2

0x0000002000000020 with CC4 TC0

0xffffffdfffffffdf with CC0 TC0

0x0000004000000040 with CC1 TC2

0xffffffbfffffffbf with CC2 TC2

0x0000008000000080 with CC4 TC0

0xffffff7fffffff7f with CC0 TC0

0x0000010000000100 with CC1 TC2

0xfffffefffffffeff with CC2 TC2

0x0000020000000200 with CC4 TC0

0xfffffdfffffffdff with CC0 TC0

0x0000040000000400 with CC1 TC2

0xfffffbfffffffbff with CC2 TC2

0x0000080000000800 with CC4 TC0

0xfffff7fffffff7ff with CC0 TC0

0x0000100000001000 with CC1 TC2

0xffffefffffffefff with CC2 TC2

0x0000200000002000 with CC4 TC0

0xffffdfffffffdfff with CC0 TC0

0x0000400000004000 with CC1 TC2

0xffffbfffffffbfff with CC2 TC2

0x0000800000008000 with CC4 TC0

0xffff7fffffff7fff with CC0 TC0

0x0001000000010000 with CC1 TC2

0xfffefffffffeffff with CC2 TC2

0x0002000000020000 with CC4 TC0

0xfffdfffffffdffff with CC0 TC0

0x0004000000040000 with CC1 TC2

0xfffbfffffffbffff with CC2 TC2

0x0008000000080000 with CC4 TC0

0xfff7fffffff7ffff with CC0 TC0

0x0010000000100000 with CC1 TC2

0xffefffffffefffff with CC2 TC2

0x0020000000200000 with CC4 TC0

0xffdfffffffdfffff with CC0 TC0

0x0040000000400000 with CC1 TC2

0xffbfffffffbfffff with CC2 TC2

0x0080000000800000 with CC4 TC0

0xff7fffffff7fffff with CC0 TC0

0x0100000001000000 with CC1 TC2

0xfefffffffeffffff with CC2 TC2

0x0200000002000000 with CC4 TC0

0xfdfffffffdffffff with CC0 TC0

0x0400000004000000 with CC1 TC2

0xfbfffffffbffffff with CC2 TC2

0x0800000008000000 with CC4 TC0

0xf7fffffff7ffffff with CC0 TC0

0x1000000010000000 with CC1 TC2

0xefffffffefffffff with CC2 TC2

0x2000000020000000 with CC4 TC0

0xdfffffffdfffffff with CC0 TC0

0x4000000040000000 with CC1 TC2

0xbfffffffbfffffff with CC2 TC2

0x8000000080000000 with CC4 TC0

0x7fffffff7fffffff with CC0 TC0

Passed Times:        7 		Failed Times:        0

MSMC RAM memory test at 10491315350 cycle

Memory Fill Test from 0x c03a680 to 0x c580000 with patterns:

0x0000000000000000

0xffffffffffffffff

0xaaaaaaaaaaaaaaaa

0x5555555555555555

0xcccccccccccccccc

0xf0f0f0f0f0f0f0f0

0xff00ff00ff00ff00

0xffff0000ffff0000

0xffffffff00000000

Passed Memory Address Test from 0x c03a680 to 0x c580000

Memory Bit Walking from 0x c03a680 to 0x c580000

0x0000000100000001 		 0xfffffffefffffffe

0x0000000200000002 		 0xfffffffdfffffffd

0x0000000400000004 		 0xfffffffbfffffffb

0x0000000800000008 		 0xfffffff7fffffff7

0x0000001000000010 		 0xffffffefffffffef

0x0000002000000020 		 0xffffffdfffffffdf

0x0000004000000040 		 0xffffffbfffffffbf

0x0000008000000080 		 0xffffff7fffffff7f

0x0000010000000100 		 0xfffffefffffffeff

0x0000020000000200 		 0xfffffdfffffffdff

0x0000040000000400 		 0xfffffbfffffffbff

0x0000080000000800 		 0xfffff7fffffff7ff

0x0000100000001000 		 0xffffefffffffefff

0x0000200000002000 		 0xffffdfffffffdfff

0x0000400000004000 		 0xffffbfffffffbfff

0x0000800000008000 		 0xffff7fffffff7fff

0x0001000000010000 		 0xfffefffffffeffff

0x0002000000020000 		 0xfffdfffffffdffff

0x0004000000040000 		 0xfffbfffffffbffff

0x0008000000080000 		 0xfff7fffffff7ffff

0x0010000000100000 		 0xffefffffffefffff

0x0020000000200000 		 0xffdfffffffdfffff

0x0040000000400000 		 0xffbfffffffbfffff

0x0080000000800000 		 0xff7fffffff7fffff

0x0100000001000000 		 0xfefffffffeffffff

0x0200000002000000 		 0xfdfffffffdffffff

0x0400000004000000 		 0xfbfffffffbffffff

0x0800000008000000 		 0xf7fffffff7ffffff

0x1000000010000000 		 0xefffffffefffffff

0x2000000020000000 		 0xdfffffffdfffffff

0x4000000040000000 		 0xbfffffffbfffffff

0x8000000080000000 		 0x7fffffff7fffffff

Passed Times:        8 		Failed Times:        0

DDR3B memory test with EDMA at 10939565063 cycle

Memory Fill Test from 0x60000000 to 0x70000000 with patterns:

0x0000000000000000 by EDMA CC1 TC2

0xffffffffffffffff by EDMA CC2 TC2

0xaaaaaaaaaaaaaaaa by EDMA CC4 TC0

0x5555555555555555 by EDMA CC0 TC0

Passed Memory Address Test from 0x60000000 to 0x70000000 with EDMA CC1 TC2

Memory Bit Walking from 0x60000000 to 0x70000000 with EDMA

0x0000000100000001 with CC2 TC2

0xfffffffefffffffe with CC4 TC0

0x0000000200000002 with CC0 TC0

0xfffffffdfffffffd with CC1 TC2

0x0000000400000004 with CC2 TC2

0xfffffffbfffffffb with CC4 TC0

0x0000000800000008 with CC0 TC0

0xfffffff7fffffff7 with CC1 TC2

0x0000001000000010 with CC2 TC2

0xffffffefffffffef with CC4 TC0

0x0000002000000020 with CC0 TC0

0xffffffdfffffffdf with CC1 TC2

0x0000004000000040 with CC2 TC2

0xffffffbfffffffbf with CC4 TC0

0x0000008000000080 with CC0 TC0

0xffffff7fffffff7f with CC1 TC2

0x0000010000000100 with CC2 TC2

0xfffffefffffffeff with CC4 TC0

0x0000020000000200 with CC0 TC0

0xfffffdfffffffdff with CC1 TC2

0x0000040000000400 with CC2 TC2

0xfffffbfffffffbff with CC4 TC0

0x0000080000000800 with CC0 TC0

0xfffff7fffffff7ff with CC1 TC2

0x0000100000001000 with CC2 TC2

0xffffefffffffefff with CC4 TC0

0x0000200000002000 with CC0 TC0

0xffffdfffffffdfff with CC1 TC2

0x0000400000004000 with CC2 TC2

0xffffbfffffffbfff with CC4 TC0

0x0000800000008000 with CC0 TC0

0xffff7fffffff7fff with CC1 TC2

0x0001000000010000 with CC2 TC2

0xfffefffffffeffff with CC4 TC0

0x0002000000020000 with CC0 TC0

0xfffdfffffffdffff with CC1 TC2

0x0004000000040000 with CC2 TC2

0xfffbfffffffbffff with CC4 TC0

0x0008000000080000 with CC0 TC0

0xfff7fffffff7ffff with CC1 TC2

0x0010000000100000 with CC2 TC2

0xffefffffffefffff with CC4 TC0

0x0020000000200000 with CC0 TC0

0xffdfffffffdfffff with CC1 TC2

0x0040000000400000 with CC2 TC2

0xffbfffffffbfffff with CC4 TC0

0x0080000000800000 with CC0 TC0

0xff7fffffff7fffff with CC1 TC2

0x0100000001000000 with CC2 TC2

0xfefffffffeffffff with CC4 TC0

0x0200000002000000 with CC0 TC0

0xfdfffffffdffffff with CC1 TC2

0x0400000004000000 with CC2 TC2

0xfbfffffffbffffff with CC4 TC0

0x0800000008000000 with CC0 TC0

0xf7fffffff7ffffff with CC1 TC2

0x1000000010000000 with CC2 TC2

0xefffffffefffffff with CC4 TC0

0x2000000020000000 with CC0 TC0

0xdfffffffdfffffff with CC1 TC2

0x4000000040000000 with CC2 TC2

0xbfffffffbfffffff with CC4 TC0

0x8000000080000000 with CC0 TC0

0x7fffffff7fffffff with CC1 TC2

Passed Times:        9 		Failed Times:        0

DDR3A memory test with EDMA at 32884186065 cycle

Memory Fill Test from 0x80000000 to 0x90000000 with patterns:

0x0000000000000000 by EDMA CC2 TC2

0xffffffffffffffff by EDMA CC4 TC0

0xaaaaaaaaaaaaaaaa by EDMA CC0 TC0

0x5555555555555555 by EDMA CC1 TC2

Passed Memory Address Test from 0x80000000 to 0x90000000 with EDMA CC2 TC2

Memory Bit Walking from 0x80000000 to 0x90000000 with EDMA

0x0000000100000001 with CC4 TC0

0xfffffffefffffffe with CC0 TC0

0x0000000200000002 with CC1 TC2

0xfffffffdfffffffd with CC2 TC2

0x0000000400000004 with CC4 TC0

0xfffffffbfffffffb with CC0 TC0

0x0000000800000008 with CC1 TC2

0xfffffff7fffffff7 with CC2 TC2

0x0000001000000010 with CC4 TC0

0xffffffefffffffef with CC0 TC0

0x0000002000000020 with CC1 TC2

0xffffffdfffffffdf with CC2 TC2

0x0000004000000040 with CC4 TC0

0xffffffbfffffffbf with CC0 TC0

0x0000008000000080 with CC1 TC2

0xffffff7fffffff7f with CC2 TC2

0x0000010000000100 with CC4 TC0

0xfffffefffffffeff with CC0 TC0

0x0000020000000200 with CC1 TC2

0xfffffdfffffffdff with CC2 TC2

0x0000040000000400 with CC4 TC0

0xfffffbfffffffbff with CC0 TC0

0x0000080000000800 with CC1 TC2

0xfffff7fffffff7ff with CC2 TC2

0x0000100000001000 with CC4 TC0

0xffffefffffffefff with CC0 TC0

0x0000200000002000 with CC1 TC2

0xffffdfffffffdfff with CC2 TC2

0x0000400000004000 with CC4 TC0

0xffffbfffffffbfff with CC0 TC0

0x0000800000008000 with CC1 TC2

0xffff7fffffff7fff with CC2 TC2

0x0001000000010000 with CC4 TC0

0xfffefffffffeffff with CC0 TC0

0x0002000000020000 with CC1 TC2

0xfffdfffffffdffff with CC2 TC2

0x0004000000040000 with CC4 TC0

0xfffbfffffffbffff with CC0 TC0

0x0008000000080000 with CC1 TC2

0xfff7fffffff7ffff with CC2 TC2

0x0010000000100000 with CC4 TC0

0xffefffffffefffff with CC0 TC0

0x0020000000200000 with CC1 TC2

0xffdfffffffdfffff with CC2 TC2

0x0040000000400000 with CC4 TC0

0xffbfffffffbfffff with CC0 TC0

0x0080000000800000 with CC1 TC2

0xff7fffffff7fffff with CC2 TC2

0x0100000001000000 with CC4 TC0

0xfefffffffeffffff with CC0 TC0

0x0200000002000000 with CC1 TC2

0xfdfffffffdffffff with CC2 TC2

0x0400000004000000 with CC4 TC0

0xfbfffffffbffffff with CC0 TC0

0x0800000008000000 with CC1 TC2

0xf7fffffff7ffffff with CC2 TC2

0x1000000010000000 with CC4 TC0

0xefffffffefffffff with CC0 TC0

0x2000000020000000 with CC1 TC2

0xdfffffffdfffffff with CC2 TC2

0x4000000040000000 with CC4 TC0

0xbfffffffbfffffff with CC0 TC0

0x8000000080000000 with CC1 TC2

0x7fffffff7fffffff with CC2 TC2

Passed Times:       10 		Failed Times:        0

DDR3B memory test at 56012018346 cycle

Memory Fill Test from 0x60000000 to 0x60400000 with patterns:

0x0000000000000000

0xffffffffffffffff

0xaaaaaaaaaaaaaaaa

0x5555555555555555

0xcccccccccccccccc

0xf0f0f0f0f0f0f0f0

0xff00ff00ff00ff00

0xffff0000ffff0000

0xffffffff00000000

Passed Memory Address Test from 0x60000000 to 0x60400000

Memory Bit Walking from 0x60000000 to 0x60400000

0x0000000100000001 		 0xfffffffefffffffe

0x0000000200000002 		 0xfffffffdfffffffd

0x0000000400000004 		 0xfffffffbfffffffb

0x0000000800000008 		 0xfffffff7fffffff7

0x0000001000000010 		 0xffffffefffffffef

0x0000002000000020 		 0xffffffdfffffffdf

0x0000004000000040 		 0xffffffbfffffffbf

0x0000008000000080 		 0xffffff7fffffff7f

0x0000010000000100 		 0xfffffefffffffeff

0x0000020000000200 		 0xfffffdfffffffdff

0x0000040000000400 		 0xfffffbfffffffbff

0x0000080000000800 		 0xfffff7fffffff7ff

0x0000100000001000 		 0xffffefffffffefff

0x0000200000002000 		 0xffffdfffffffdfff

0x0000400000004000 		 0xffffbfffffffbfff

0x0000800000008000 		 0xffff7fffffff7fff

0x0001000000010000 		 0xfffefffffffeffff

0x0002000000020000 		 0xfffdfffffffdffff

0x0004000000040000 		 0xfffbfffffffbffff

0x0008000000080000 		 0xfff7fffffff7ffff

0x0010000000100000 		 0xffefffffffefffff

0x0020000000200000 		 0xffdfffffffdfffff

0x0040000000400000 		 0xffbfffffffbfffff

0x0080000000800000 		 0xff7fffffff7fffff

0x0100000001000000 		 0xfefffffffeffffff

0x0200000002000000 		 0xfdfffffffdffffff

0x0400000004000000 		 0xfbfffffffbffffff

0x0800000008000000 		 0xf7fffffff7ffffff

0x1000000010000000 		 0xefffffffefffffff

0x2000000020000000 		 0xdfffffffdfffffff

0x4000000040000000 		 0xbfffffffbfffffff

0x8000000080000000 		 0x7fffffff7fffffff

Passed Times:       11 		Failed Times:        0

DDR3A memory test at 56346765832 cycle

Memory Fill Test from 0x80000000 to 0x80400000 with patterns:

0x0000000000000000

0xffffffffffffffff

0xaaaaaaaaaaaaaaaa

0x5555555555555555

0xcccccccccccccccc

0xf0f0f0f0f0f0f0f0

0xff00ff00ff00ff00

0xffff0000ffff0000

0xffffffff00000000

Passed Memory Address Test from 0x80000000 to 0x80400000

Memory Bit Walking from 0x80000000 to 0x80400000

0x0000000100000001 		 0xfffffffefffffffe

0x0000000200000002 		 0xfffffffdfffffffd

0x0000000400000004 		 0xfffffffbfffffffb

0x0000000800000008 		 0xfffffff7fffffff7

0x0000001000000010 		 0xffffffefffffffef

0x0000002000000020 		 0xffffffdfffffffdf

0x0000004000000040 		 0xffffffbfffffffbf

0x0000008000000080 		 0xffffff7fffffff7f

0x0000010000000100 		 0xfffffefffffffeff

0x0000020000000200 		 0xfffffdfffffffdff

0x0000040000000400 		 0xfffffbfffffffbff

0x0000080000000800 		 0xfffff7fffffff7ff

0x0000100000001000 		 0xffffefffffffefff

0x0000200000002000 		 0xffffdfffffffdfff

0x0000400000004000 		 0xffffbfffffffbfff

0x0000800000008000 		 0xffff7fffffff7fff

0x0001000000010000 		 0xfffefffffffeffff

0x0002000000020000 		 0xfffdfffffffdffff

0x0004000000040000 		 0xfffbfffffffbffff

0x0008000000080000 		 0xfff7fffffff7ffff

0x0010000000100000 		 0xffefffffffefffff

0x0020000000200000 		 0xffdfffffffdfffff

0x0040000000400000 		 0xffbfffffffbfffff

0x0080000000800000 		 0xff7fffffff7fffff

0x0100000001000000 		 0xfefffffffeffffff

0x0200000002000000 		 0xfdfffffffdffffff

0x0400000004000000 		 0xfbfffffffbffffff

0x0800000008000000 		 0xf7fffffff7ffffff

0x1000000010000000 		 0xefffffffefffffff

0x2000000020000000 		 0xdfffffffdfffffff

0x4000000040000000 		 0xbfffffffbfffffff

0x8000000080000000 		 0x7fffffff7fffffff

Passed Times:       12 		Failed Times:        0

Memory test complete at 56677120643 cycle