KeyStone 2

Robust Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial Version |

Contents

1	Introduction	3

2	Test Method	4

2.1	Memory Protection	4

2.2	Error Detection and Correction (EDC/ECC)	4

2.3	Other Robust Features	4

2.4	Exception location	4

3	Test cases	5

3.1	Memory Protection	5

3.2	EDC/ECC	6

4	Test code/project	6

4.1	CCS project	6

4.2	To run the test program on KeyStone 2 EVM	8

4.3	Test configuration	8

4.4	Migrate the test to custom board	10

References	10

Appendix: Typical test output	11

Figures

Figure 1.	Robust system on Keystone devices	3

Figure 2.	Directory structure of test project	7

Figure 3.	define symbols in CCS compiler options to select device and board	9

Tables

Table 1.	Different memory protection modules	5

Table 2.	EDC/ECC mechanism implement in keystone memory	6

Table 3.	Source files of the example codes	7

- Introduction

The KeyStone devices provide many features, which can support users to build robust application on it. Following figure shows those features.

- Robust system on Keystone devices

With ARM core, and EDMA as test master respectively, the STK verifies below features:

1. Memory protection for MSRAM (Multicore Shared RAM), DDR3, reserved space and peripherals;

2. EDC MSRAM, DDR3 ECC;

3. EDMA error handling;

4. Watch-dog error;

5. Interrupt drop detection.

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test Method

  - Memory Protection

Configure the memory access permission, when a master access the memory, its permission is checked and exception will happen when it is denied. The STK manually lets one of the ARM core or EDMA master to access the memory to trigger exception.

The pseudocode for MSRAM/DDR3 memory protection is:

Modify the MPAX configuration;

Access the protected memory to trigger error;

The pseudocode for peripherals configuration space protection is:

Modify the related MPU MPPA access permission according to the address;

Access the protected memory to trigger error;

  - Error Detection and Correction (EDC/ECC)

MSRAM and DDR support error detection and correction. STK manually generates bit error to trigger exception with ARM core or DMA as master.

The pseudocode for EDC test is:

Pend the EDC;

Inject error;

Enable EDC;

Access data to trigger the error;

  - Other Robust Features

Besides the memory protection and EDC/ECC, the watch-dog, EDMA error are verified.

Timer 16 + N is dedicated to ARM core N as a watchdog. The STK configures the Timer 16+N in watch-dog mode, and stop servicing the dog to generate exception.

To generate EDMA error, a NULL transfer and an EDMA transfer with destination address 0 are configured and triggered.

  - Exception location

Exception may be captured about several cycles after the access violation, which depends on many factors including the operation type, the target module which generates exception event... For example, for a write operation to a register protected by MPU, the delay includes: the delay of the write instruction from ARM core to the register; and the delay for the error event routing from the MPU to GIC and then to CPU exception module. So if the test function is written like below:

test_func()

{

illegal access;  //to trigger the exception

return;

}

The exception address captured by the exception module will be in the location of parent function which call this test function.

To make the exception address correlates to the function which triggers the exception more obviously, in STK, the function is implemented like below:

test_func()

{

illegal access;  //to trigger the exception

for(dummyLoopIndex=0; dummyLoopIndex<N; dummyLoopIndex++)

__asm__(" ISB");

return;

}

- Test cases

  - Memory Protection

As shown in figure 1, there are many memory protection modules, following table summarizes their difference.

- Different memory protection modules

| Memory protection module | Position in System | page/segment/range |  | Comments |
| --- | --- | --- | --- | --- |
|  |  | Number | Size |  |
| SMS MPAX | Master port | 8 | Programmable | Each Privilege ID has a pair of SMS and SES. |
| SES MPAX | Master port | 8 | Programmable |  |
| MPU | Slave port | 1~16 | Programmable | Refer to device specific data manual for number of ranges and programmable size for each MPU. |

All the memory protection features in table1 are tested with manually generated error.

For SMS MPAX, ARM core is used as master to access the protected MSRAM and DDR memory to trigger exception.

For SES/SMS MPAX, and EDMA are used as master to access the protected MSRAM/DDR to trigger exception.

The MPU is used to protect the configure space of slave peripherals. The STK configures MPU11 to deny all the access to PLL registers, and exception is generated when ARM core access the one of the PLL registers. The STK also configures the MPU1 and MPU2 to prevent some queues from being accessed by ARM cores, and then triggers expectation by ARM core accessing.

  - EDC/ECC

EDC (Error Detection and Correction) or ECC (Error Checking and Correcting) are designed for soft error. EDC or ECC mechanism are implemented through keystone all levels of memory, following table compares these mechanism in different memory modules:

- EDC/ECC mechanism implement in keystone memory

| memory | error detection | Error correction | Segment line size |
| --- | --- | --- | --- |
| MSRAM | 2 bit | 1bit | 256bit |
| DDR | 2 bit | 1bit | 64bit |

One bit error is injected into memory, EDMA and ARM core are used to trigger the excpetion.

- Test code/project

This section introduces the implementation of these robust features tests on KeyStone 2 SOC.

  - CCS project

Following figure shows the directory structure of the test project.

- Directory structure of test project

The project files are in “Robust” folder. There is some commonly used initialization and driver code for DDR, EDMA and PLL… in “common” folder. The main source code files are in the “Robust\src” subfolder. Below table describes these source files.

- Source files of the example codes

| Source files | Functions | Descriptions |
| --- | --- | --- |
| K2_common.c/h | KeyStone_MSRAM_EDC_enable KeyStone_MSMC_MP_interrupt_en KeyStone_MPU_interrupts_enable KeyStone_Exception_cfg | Initialization for EDC, ECC and interrupt controller mapping. |
| Robust_xxx.c/h | SMS_MSRAM_MP_test SES_DDR_MP_test peripheral_MP_test Reserved_Space_Test | Implementation of below memory protection features. SMS_MSRAM, SES_DDR, Slave MPU, reserve space protection |
|  | MSRAM_EDC_test DDR_ECC_test | Implementation of EDC/ECC features. MSRAM EDC, DDR ECC |
|  | watch_dog_test EDMA_ERROR_test | Implementation of other robust features. Watch-dog error, EDMA error handling, |

  - To run the test program on KeyStone 2 EVM

The test program can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- Load the program to ARM core 0 of a device.

- run the core, then you should see output in console window.

Refer to appendix of this document to see the typical test output.

  - Test configuration

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

There are multiple macros defined in the source code to configure the test.

Each robust feature can be configured to be tested or not through following macros in “Robust_System.c”. The value of “1” means enable the test, and “0” means disable the test.

#define SMS_MSMC_RAM_MP_TEST 	1

#define SES_DDR_MP_TEST 	1

#define PERIPHERAL_MP_TEST 	1

#define MSMC_RAM_EDC_TEST 	1

#define DDR_ECC_TEST 		1 /*only for EVM with ECC memory, NOT supported by K2H, K2K rev 1.x*/

#define EDMA_ERROR_TEST 	1

#define WATCH_DOG_TEST 	1

#define RESERVED_SPACE_TEST 	1

  - Migrate the test to custom board

These tests are implemented based on TI’s EVM boards. In your real system, the DDR configuration may be changed according to your hardware design in the function K2_DDR3A_config() and K2_DDR3A_config() in “K2_Board_Init.c”.

The core speed may be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- Memory Protection On Keystone Devices (SPRWIKI9012)

- KeyStone II Architecture ARM CorePac User's Guide (SPRUHJ4)

- KeyStone Architecture Multicore Shared Memory Controller User Guide (SPRUGW7)

- KeyStone Architecture Memory Protection Unit (MPU) User Guide (SPRUGW5)

- KeyStone Architecture Enhanced Direct Memory Access (EDMA3) Controller User Guide (SPRUGS5)

- KeyStone Architecture Timer64 User Guide (SPRUGV5)

- KeyStone Architecture Chip Interrupt Controller (CIC) User Guide (SPRUGW4)

- KeyStone Architecture DDR3 Memory Controller’s user guider (SPRUGV8)

- “Interrupts”, “MPU” and “Memory Map Summary” sections in Device specific Data Manuals

Appendix: Typical test output

Below is the test output on K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

DDR3A initialization

DDR ECC is enabled.

Initialize DDR data rate = 100.000/1*20/6*4= 1333.3 MTS, bus width = 64 bits.

Enable Exception handling...

===================-SMS MSMC_RAM memory protection test-==============

EDMA copy from 0xc034000 (read-only) to 0xc030000 (write-only)...

!!!EDMA copy from 0xc032000 (write-only) to 0xc035000 (read-only)...

External exception happened at program address 0xc00b548!

EDMA module 0 error

TC 0 error happened

EDMA3TC has detected an error at source or destination address.

read error (2). TCC= 0. TCINTEN= 1. TCCHEN= 0

memory protection exception caused by master 25 (PrivID= 8) accessing address 0xc032000

===================-SES DDR memory protection test-==============

EDMA copy from 0x80004000 (read-only) to 0x80000000 (write-only)...

!!!EDMA copy from 0x80002000 (write-only) to 0x80005000 (read-only)...

External exception happened at program address 0xc00a9a0!

EDMA module 0 error

TC 0 error happened

EDMA3TC has detected an error at source or destination address.

read error (1). TCC= 0. TCINTEN= 1. TCCHEN= 0

memory protection exception caused by master 25 (PrivID= 8) accessing address 0x80002000

DDR3_A error happened.

unaligned write to ECC range.

VBUSM command or address error.

===================-peripherals protection test-=================

!!!Write to PLL register at 0x02310110 which is protected by MPU11...

External exception happened at program address 0xc00b6c0!

MPU 11 protection violation error

The MSTID 8 (PRIVID 8) triggered MPU error at 0x2310110 with secure access

Supervisor write fault!

MPU 1 setup 5 of 16 ranges.

MPU 2 setup 10 of 16 ranges.

CPU Pop descriptor from queue 2049 at 0x2a8801c in a read-only range...

!!!CPU Push descriptor to queue 2049 at 0x2a8801c in a read-only range...

External exception happened at program address 0xc00b768!

MPU 2 protection violation error

The MSTID 8 (PRIVID 8) triggered MPU error at 0x2a8801c with secure access

Supervisor write fault!

CPU Push descriptor to queue 2049 at 0x23a8801c in a writeable range...

===================-MSMC_RAM EDC test-================================

!!!manually generate one bit error data at 0xc02f000, and then read it...

External exception happened at program address 0xc00add0!

MSMC_RAM Correctable error occurred at bit 8 of address 0xc02f000 by PrivID 8 (from CorePacs)

===================-EDMA Error test-===========================

!!!start a NULL transfer with EDMA CC0 Channel 0, to trigger event miss error...

External exception happened at program address 0xc00aa6c!

EDMA module 0 error

EDMA Channel 0 event missed.

!!!start a transfer to address 0 with EDMA CC1 Channel 2, to trigger EDMA bus error...

External exception happened at program address 0xc00aac0!

EDMA module 1 error

TC 2 error happened

EDMA3TC has detected an error at source or destination address.

write error (10). TCC= 2. TCINTEN= 1. TCCHEN= 0

===================-watch dog timer test-========================

start watch-dog timer...

service watch-dog 1 times, at time counter = 20

service watch-dog 2 times, at time counter = 41438584

service watch-dog 3 times, at time counter = 41523872

service watch-dog 4 times, at time counter = 39483256

service watch-dog 5 times, at time counter = 40616527

service watch-dog 6 times, at time counter = 41480429

service watch-dog 7 times, at time counter = 41536855

service watch-dog 8 times, at time counter = 41473892

service watch-dog 9 times, at time counter = 39649072

service watch-dog 10 times, at time counter = 41204755

!!!stop servicing watch-dog, it will timeout and trigger exception after 3000 ms...

Watch dog timer for ARM core 0 expired at program address 0xc00baf0!

==================-Reserved space access test-=================

--------------------access zero address------------------------

!!!write to reserved address 0x0...

Data abort exception happened at program address 0xc00b86c!

Permission fault on second level translation happened at address 0x0.

--------------------access unmapped space----------------------

!!!read from reserved address 0x1c000000...

Data abort exception happened at program address 0xc00b8a0!

Translation fault on second level translation happened at address 0x1c000000.

!!!write to reserved address 0x1c000000...

Data abort exception happened at program address 0xc00b8c8!

Translation fault on second level translation happened at address 0x1c000000.

!!!execute at reserved address 0x1c000000...

Instruction prefetch abort happened at program address 0x1c000000!

Translation fault on second level translation happened at address 0x1c000000.