KeyStone 2

UART Self Test Kit

User’s Guide

Revision history

| Version | Date | Author | Description of Change |
| --- | --- | --- | --- |
| 1.0 | March 13, 2015 | Brighton Feng | Initial release |

Contents

1	Overview	3

2	Test Methods and test cases	4

2.1	UART Loopback & Throughput Test	4

2.2	UART Transmit Increasing Sequence Test	5

2.3	UART Transmit Continuous Data Pattern Test	5

2.4	UART Interact with PC Test	5

2.5	Test with different masters	5

2.5.1	Transmit with ARM core Directly	6

2.5.2	Transmit with EDMA	6

2.5.3	Receive through UART Interrupt to a ARM core	6

2.6	UART Baud Rate Configuration Accuracy	6

3	Test Project	7

3.1	CCS Project	7

3.2	Hardware Connections and Drivers Preparation	8

3.2.1	Install K2 EVM USB to COM Driver for Normal Tests	8

3.2.2	Hardware Connection for External Loopback Tests	11

3.3	To Run the Test Program on KeyStone 2 EVM	11

3.4	Test Configurations	12

3.5	Migrate the Project to Customs’ Boards	14

References	14

Appendix: Typical test output	15

Figures

Figure 1.	KeyStone2 UART functional diagram	3

Figure 2.	UART internal loopback	4

Figure 3.	UART external line loopback	5

Figure 4.	Directory structure of test projects	7

Figure 5.	define symbols in CCS compiler options to select device and board	12

Tables

Table 1.	Source files of the test codes	7

- Overview

Figure 1 shows the KeyStone2 UART functional diagram.

- KeyStone2 UART functional diagram

Most of KeyStone2 devices have 2 to 4 UART modules, Each module can work separately. The major features supported by UART in KeyStone2 are:

- 1 START bit + data bits (5, 6, 7, 8) + 1 PARITY bit (optional) + STOP bit (1, 1.5, 2) for UART transmitting format.

- 1 START bit + data bits (5, 6, 7, 8) + 1 PARITY bit (optional) + 1 STOP bit for UART receiving format.

- Configurable baud rate.

- The receiver and transmitter FIFOs store up to 16 bytes.

Refer to “KeyStone_2_Self_Test_Kit_User's_Guide (READ ME FIRST).doc” for some common information about K2 STK.

- Test Methods and test cases

Several test methods are used to cover multiple usages cases.

  - UART Loopback & Throughput Test

Two loopback tests are implemented: internal loopback and external line loopback.

- UART internal loopback

- UART external line loopback

The purpose of the internal loopback test is mainly to check the KeyStone2 SOC itself; the purpose of the external line loopback test is mainly to check the components and connections for UART on the board.

If the internal loopback test fails, the problem should be in the KeyStone2 SOC, you may need check the clock, power or configurations for the SOC; if the internal loopback test passes while the external line loopback test fails, then the problem is on the other components or connection on the board.

The throughput is also measured in loopback test to provide the basic benchmark of the throughput in different baud rate configurations, ranging from 115200 to 3686400 bps.

  - UART Transmit Increasing Sequence Test

The purpose of this test method is mainly to check the UART configurations in Tx side, by outputting increasing sequence to PC terminal, it’s easy to check whether the UART configuration is correct or not.

The ASCII code value of the increasing sequence is from 0 – 127 which covers the alphabet, number and daily used punctuations in ASCII, by checking the output on PC terminal, we can basically know whether the UART configuration is correct or not.

  - UART Transmit Continuous Data Pattern Test

The purpose of this test method is mainly to generate waveform on TX pin to make it easy for hardware signal checking with oscilloscope. We can use the continuous data pattern such as 0x55 or 0xAA to output square wave, and then check the signal from the hardware circuit to check the signal quality or the hardware connections.

  - UART Interact with PC Test

The purpose of this test method is mainly to check both of the Tx and Rx sides on hardware. For Tx side, the data sent by UART will be received and printed on PC terminal. For Rx side, the data received from PC will be retransmitted from UART back to PC which will show the echo of the keyboard input.

In this test scenario, the UART configurations will be printed in CCS output window to show which parameters it’s using now, PC terminal (such as Putty or TeraTerm) should be configured with the same parameters, or the interaction will not be successful.

  - Test with different masters

UART tests are implemented with different masters:

- Transmit with ARM core directly

- Transmit with EDMA

- Receive through UART interrupt to an ARM core

    - Transmit with ARM core Directly

In the testing code, user can choose to use ARM core to transmit data through UART, in this case, ARM core will be responsible to move all required data to the UART transmit register for transmission, ARM need to wait until all required data transfer complete.

    - Transmit with EDMA

In the testing code, user can choose to use EDMA to transmit data through UART, in this case, ARM only need to setup EDMA parameters and trigger the EDMA, EDMA will be responsible to move all required data to the UART transmit register for transmission, there is no need for ARM to wait until all required data transfer complete.

The EDMA usage designed here is using the A-Sync, ARM will only trigger the initial transmit, the rest EDMA transmit events are generated by UART Tx event, due to the hardware connections in SoC, different UART will generate different EDMA CC’s trigger events, for more information, please refer the relative Data Manual of the corresponding KeyStone2 devices.

    - Receive through UART Interrupt to a ARM core

Due to UART FIFO can generate interrupt from Rx side, we can free the ARM core from polling the UART receive status register to check whether there is received data or not. In UART FIFO mode, the interrupt will be generated in case of FIFO full or in case of FIFO not full but reaches the time out limitation. In the receive interrupt service routine, ARM core will read the UART register to get the received data.

  - UART Baud Rate Configuration Accuracy

UART board rate is generated by a divider, which derives UART clock from main core’s clock. For a given ARM core clock, the divider may not generate exact baud rate as required.

In the test project, we calculated the divisor value with following formula:

Divisor = round_off(main_Core_Speed/6/Over_Sampling_Factor/baudRate)

The rounding off is used to minimize the accuracy error, but normally, there is still error.

Since UART resynchronizes at the start edge of each byte, so the error only accumulates in one byte. In theory, if the error is 5%, when it accumulate to the 10th bit, the error will be 50%, which will make the sampling fail. So, in theory, the clock error must be less than 5%; in reality, the clock error should be more accurate because of signal distortion or delay.

- Test Project

This section introduces the implementation of the UART tests on KeyStone 2 devices.

  - CCS Project

Following figure shows the directory structure of the test projects

- Directory structure of test projects

The project files are in UART folder. There is some commonly used initialization and driver code for UART, EDMA and PLL… in common folder. The main source code files are in the UART->src subfolder. Below table describes these source files.

- Source files of the test codes

| Source files | Descriptions |
| --- | --- |
| UART_main.c | Implementation of main UART test methods, the key APIs include: K2_UART_config(Uint32 baudRate, Bool bLoopBackEnable, UART_Tx_Master txMaster) K2_UART_loopback_with_throughput(Bool bInternalLoopback) UART_TX_Increase_Sequence() UART_continue_TX_data_pattern(Uint8 dataPattern) UART_Interact_with_PC() |
| K2_UART_Init_drv.c | Implementation of UART initialization and driver level APIs. The key APIs are: KeyStone_UART_init (UART_Config *pUARTCfg, Uint32 uartNum) K2_UART_write(unsigned char *buffer, unsigned int byteLen, Uint32 uartNum) K2_UART_TX_wait(Uint32 uartNum) K2_UART_read(unsigned char *buffer, unsigned int buffByteLen, Uint32 uartNum) K2_UART_Error_Handler(Uint32 uartNum) |
| UART_Interrupt.c | Implementation of UART interrupt initialization, configuration and ISR: UART_Echo_back(Uint32 rx_cnt, Uint32 uartNum) K2_UART_Rx_ISR() K2_UART_Error_ISR() UART_EDMA_complete_handler(Uint32 tpccNum) K2_UART_EDMA_ISR() K2_UART_Interrupts_Init() |

  - Hardware Connections and Drivers Preparation

    - Install K2 EVM USB to COM Driver for Normal Tests

Due to the K2 EVM have the FTDI chip to convert the COM to USB interface, we need to download and install the FTDI drivers on the PC side to use the USB as COM port. You should download and install the driver from the following link:

http://www.ftdichip.com/Drivers/D2XX.htm

Choose the latest Windows version to download and install, e.g: “CDM 2.08.28 WHQL Certified.7z”

Connect the USB cables before power on the EVM board:

There are 2 mini-USB ports, the one on the mezzanine board is the USB XDS200 Emulator cable, the one on the main EVM board is the USB-COM cable. Make sure connect 2 USB cables on your PC.

Here we use K2H EVM as example, the connectors on different EVM are different; refer to the EVM user guide for more information.

Before we power on the EVM, please make sure the boot DIP pins on EVM in the position like:

This makes the board in “no boot” mode for running program through CCS.

Power on the EVM and the driver of the Emulator and USB-COM will be installed on your PC.

If the USB-COM drivers are correctly installed, you should found 2 more items were showed in your device manager:

Then open the hyper-terminal tool (we use TeraTerm as example) to create 2 sessions on your PC:

After we created 2 sessions, you will see the terminals as below:

The one which starts with “BMC>” is the control port to configure the boot mode pins of K2 EVM, refer to following webpage for more information about the configuration:

http://processors.wiki.ti.com/index.php/EVMK2H_Hardware_Setup

The one without BMC> prompt is the UART0 output through USB2COM port.

    - Hardware Connection for External Loopback Tests

On K2H EVM, the UART0 is connected with both FTDI USB2COM port and the 4-Pin on-board connectors, we can use the USB2COM port for normal tests, we need to use the 4-Pin on-board connectors for the external loopback tests due to some extra manual connections is required.

According the test method for external loopback, we need to short the UART Tx and Rx pins on the connector side, so the 2 extra jumper connector need to be placed on the EVM UART on-board connectors as the following picture:

The jumper 1 is connected to the UART0 “Detect” and “GND” pins which let the UART0 connect to the  4-pin connector instead of the USB2COM port, the jumper 2 connects UART0 “Tx” and “Rx” pins which implements the external loopback. Please also noted that the FTDI USB2COM cable should not be connected when testing for external loopback with the 4-pin connector.

  - To Run the Test Program on KeyStone 2 EVM

The test program can be run on all KeyStone 2 EVM.

The steps to run the test cases on EVM board are:

- extract (or install) the package, and switch CCS workspace to the extracted folder (or installation folder)

- Import the project to CCS6.1 or higher;

- Build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_keystone2_3_01_01_04\packages\ti\csl;

- Set the boot mode of the device on EVM to no boot and make ARM as boot master.

- Connect the Com cable( or USB to Com cable) and setup the PC terminal with the same configuration which UART uses in the CCS projects

- Load the program to ARM core 0 of a device.

- Run the core 0, then, you should see output in console window.

Refer to appendix of this document to see the typical test output.

  - Test Configurations

These tests are implemented based on TI’s EVM boards. One of following symbols can be defined in the CCS compiler options to select between different devices:

- DEVICE_K2H

- DEVICE_K2K

- DEVICE_K2E

- DEVICE_K2L

- define symbols in CCS compiler options to select device and board

One of following symbols can be defined in the CCS compiler options to select between different boards:

- K2H_EVM

- K2K_EVM

- K2E_EVM

- K2L_EVM

There are multiple macros defined in the source code to configure the test.

To specify which UART for initialization and testing, use the following macro defined in “UART_main.c”.

#define TEST_UART_NUM 	0

Following macros in “UART_main.c” can be used to select test cases. Defining the macros means enable the tests, and undefining (or commenting) the macros means disable the test.

#define UART_INTERNAL_LOOPBACK_TEST

//#define UART_EXTERNAL_LINK_LOOPBACK_TEST

#define UART_TX_INCREASE_SEQUENCE

#define UART_CONTINUE_TX_DATA_PATTERN 	0x55

#define	UART_INTERACT_WITH_PC

Please note, if UART_EXTERNAL_LINK_LOOPBACK_TEST is defined, the INCREASE_SEQUENCE,  TX_DATA_PATTERN and INTERACT_WITH_PC tests will not be executed.

The UART configurations are configured in function “K2_UART_config” which defined in “UART_main.c” as below. You can modify the parameters with values or corresponding enums defined in “K2_UART_Init_drv.h”.

gUARTCfg.baudRate = baudRate;

gUARTCfg. Main_Core_Speed_Hz = gMain_Core_Speed_Hz;

gUARTCfg.dataLen = DATA_LEN_8BIT;

gUARTCfg.parityMode = PARITY_DISABLE;

gUARTCfg.stopMode = ONE_STOP_BIT;

gUARTCfg.autoFlow = AUTO_FLOW_DIS;

gUARTCfg.osmSel = OVER_SAMPLING_16X;

gUARTCfg.fifoRxTriBytes = TRIGGER_LEVEL_14BYTE;

gUARTCfg.txMaster = txMaster;

gUARTCfg.bLoopBackEnable = bLoopBackEnable;

  - Migrate the Project to Customs’ Boards

These tests are implemented based on TI’s EVM boards. In your real system, the core speed may be changed through following macros in “K2_Board_Init.h”:

#define MAIN_PLL_REF_CLK_MHZ    122.88

#define MAIN_PLL_DIVISOR        4

#define MAIN_PLL_MULTIPLIER     39

#define ARM_PLL_REF_CLK_MHZ     125

#define ARM_PLL_DIVISOR         5

#define ARM_PLL_MULTIPLIER      48

To make your own configurations take effect, you must rebuild the project. Since CSL (Chip Support Library) header files are used by these projects, you may need change CSL including path in your system before you rebuild the project.

References

- KeyStone Architecture Universal Asynchronous Receiver_Transmitter (UART) User Guide (sprugp1)

- KeyStone Architecture Enhanced Direct Memory Access (EDMA3) Controller User Guide (SPRUGS5)

Appendix: Typical test output

Below is the test output on K2H EVM.

JTAG ID= 0x1b98102f. This is a K2H/K2K device, version variant = 1

DEVSTAT= 0x02060ee1. little endian, no boot or I2C slave boot, boot master is ARM core, PLL configuration implies the input clock for core is 122.88MHz

SmartReflex VID= 46, required core voltage= 0.995V.

Die ID= 0x0c011004, 0x04010235, 0x00000000, 0x60ba0001

DSP speed grade = 1200MHz, ARM speed grade= 1200MHz

Enable IRQ and FIQ.

Enable Exception handling...

Initialize main core clock = 122.88MHz/4x39 = 1198MHz

Initialize ARM clock = 125.00MHz/5x48 = 1200MHz

Init UART0 at 115200bps in loopback mode...

Required baud rate 115200, effective baud rate 115555!

EDMA CC2 for UART transfer complete.

UART0: Tx 1024 bytes, Rx 1024 bytes, 1024 correct bytes. Consumes 106160742 cycles, throughput is 92Kbps

Init UART0 at 230400bps in loopback mode...

Required baud rate 230400, effective baud rate 231111!

EDMA CC2 for UART transfer complete.

UART0: Tx 1024 bytes, Rx 1024 bytes, 1024 correct bytes. Consumes 53093967 cycles, throughput is 184Kbps

Init UART0 at 460800bps in loopback mode...

Required baud rate 460800, effective baud rate 462222!

EDMA CC2 for UART transfer complete.

UART0: Tx 1024 bytes, Rx 1024 bytes, 1024 correct bytes. Consumes 26560616 cycles, throughput is 369Kbps

Init UART0 at 921600bps in loopback mode...

Required baud rate 921600, effective baud rate 891428!

EDMA CC2 for UART transfer complete.

UART0: Tx 1024 bytes, Rx 1024 bytes, 1024 correct bytes. Consumes 13785191 cycles, throughput is 711Kbps

Init UART0 at 1843200bps in loopback mode...

Required baud rate 1843200, effective baud rate 1782857!

EDMA CC2 for UART transfer complete.

UART0: Tx 1024 bytes, Rx 1024 bytes, 1024 correct bytes. Consumes 6907468 cycles, throughput is 1420Kbps

Init UART0 at 3686400bps in loopback mode...

Required baud rate 3686400, effective baud rate 4160000!

EDMA CC2 for UART transfer complete.

UART0: Tx 1024 bytes, Rx 1024 bytes, 1024 correct bytes. Consumes 2975573 cycles, throughput is 3298Kbps

Init UART0 at 115200bps...

Required baud rate 115200, effective baud rate 115555!

Start print character code from 0 to 255 over UART0...

EDMA CC2 for UART transfer complete.

Below is the print info on PC terminal when UART_INTERACT_WITH_PC case is enabled: