- Revision history

| Revision | Date | Comments |
| --- | --- | --- |
| 0.9 | September 2007 | ECP (MSP430) API documentation for beta boards with firmware v5. |

ECP I2C Access:

The ECP is mapped to I2C address 0x70.  The ECP uses 7 bit addressing.  The ECP also utilizes the built-in not ready mechanism of the I2C bus – holding the clock low to extend access times.  It is crucial that the I2C master access the ECP properly.

An ECP write cycle consists of writing the ECP I2C address (0x70), the desired ECP offset, and the data to be written to that offset.  Auto-increment/bursting is not supported.

The ECP I2C WRITE cycle is as follows:

| S | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | A | y | y | y | y | y | y | y | y | A | w | w | w | w | w | w | w | w | A | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Where yyyyyyyy is the desired offset, and wwwwwwww is the write data.

The ECP READ cycle is similar.  It consists of writing the ECP I2C address (0x70), the desired ECP offset, and then issuing a read via a repeated start sequence:

| S | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | A | y | y | y | y | y | y | y | y | A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| R | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | A | r | r | r | r | r | r | r | r | N | P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Where yyyyyyyy is the desired offset, and rrrrrrrr is the read data.

(S = start condition, R = repeated start condition, A = acknowledge, N = nack, P = stop condition.  Acknowledge comes from destination.)

Each ECP read and write is a single I2C transaction.  It is very important that the I2C master honor the I2C ready mechanism.  When the ECP is not-ready it will hold the clock line low.  At the end of the not-ready condition, the ECP will let the clock line go, allowing the master to continue clocking.

To test that the ECP has completed a WRITE, one could issue the WRITE followed by a READ of any ECP offset.  When the READ completes, the user is guaranteed the ECP operation caused by the WRITE has completed.

Alternatively, if two sequential WRITEs occur, the user is guaranteed the ECP operation caused by the first WRITE is complete when the second WRITE occurs.

Note:  The ECP uses the built-in ready mechanism of I2C.  The I2C HW on the ECP is also buffered.   The I2C HW on the ECP will accept one ECP write transaction with no intervention by the MSP430.  This write transaction will not be acted upon until the MSP430 responds to the interrupt from the I2C hardware.  Thus, if it is necessary for the ECP to finish a write operation (i.e. setting up a mux), then the DM648 code should follow the write by another write or read to ensure the previous operation has completed.

It is very important that the DM648 I2C driver honor the I2C ready mechanism which consists of the ECP slave holding down the SCL line when not ready.

ECP Register Definitions:

ECP Command Register (Offset 0x00)

7                                                                                                                                            0

COMMAND CODE

R/W-0

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | COMMAND CODE | 0xnn | Command code for MSP430 internal function |

Valid COMMAND CODEs  are:

| 0x00 | Null command |
| --- | --- |
| 0x01 | Cold Reset POR – The EVM is powered off, back on, and reset using POR. Command ignored when installed in PCI slot. |
| 0x02 | Warm Reset RST – The EVM is reset. The DM648 recieves a reset via RST and the peripherals are reset. Command ignored when installed in PCI slot. |
| 0x03 | Warm Reset POR – The EVM is reset. The DM648 recieves a reset via POR and the peripherals are reset. Command ignored when installed in PCI slot. |
| 0x04 | ECP Reset POR – Forces reset of ECP processor. This will cause a cold reset and POR the DM648. Command ignored when installed in PCI slot |
| 0x05 | Power Off – The EVM is powered off and will not turn back on until SW1 is pressed, the MSP430 is reset via JP1, or the 5V power is removed and reapplied. Not valid while installed in PCI slot |
| 0x06 | 74.25 MHz HD Video Clock Mode (see reg 0x24). |
| 0x07 | Reserved |
| 0x08 | 27.00 MHz HD Video Clock Mode (see reg 0x24). |

ECP Status Register (Offset 0x01)

7                                                                                                                                            0

STATUS CODE

R/W

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | STATUS CODE | 0xnn | Status code for MSP430 internal function |

| 0x01 | ECP Busy |
| --- | --- |
| 0x02 | ECP I2C Error – Invalid I2C transaction. i.e. a read was attempted without first writing an offset |
| 0x04 | ECP Invalid Offset – A read or write was attempted from an invalid offset address |
| 0x08 | ECP Port Error – A read or write was attempted on an invalid ECP H/W IO Port |
| 0x10 | ECP Invalid Command – A write command code to command register was invalid command code. |
| 0x20 | Reserved |
| 0x40 | Reserved |
| 0x80 | Reserved |

ECP Status Register is not accurate.

Reset Input Register (Offset 0x02)

7                                                                                    2              1                           0

| Reserved | TGTRSET# | USERRSET# |
| --- | --- | --- |

R0                                                       R                         R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-2 | Reserved | 0x00 | Reserved |
| 1 | TGTRSET# | 0 | Target reset asserted by 60 pin emulator |
| 0 | USERRESET# | 0 | User reset asserted by reset button |

Reset Output Register (Offset 0x03)

7                   6                   5            4            3          2                      1                  0

| DSPPOR | DSPRST | DSPNMI | Reserved | PERRST | GRNSTAT | REDSTAT |
| --- | --- | --- | --- | --- | --- | --- |

R                   R               R/W-0            R0             R/W-0               R                  R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7 | DSPPOR | 1 | DSP POR Asserted |
| 6 | DSPRST | 1 | DSP Reset Asserted |
| 5 | DSPNMI | 1 | DSP NMI Asserted |
| 4-3 | Reserved | 0 | Reserved |
| 2 | PERRST | 0 1 | Peripherals not in reset Peripherals in reset |
| 1 | GRNSTAT | 0 1 | Green Status LED extinguished Green Status LED lit |
| 0 | REDSTAT | 0 1 | Red Status LED extinguished Red Status LED lit |

PCI Bridge Configuration Register (Offset 0x04)*

7                         6                        5                       4                  3-1                      0

| S66MHZ | SASYNCCLK | SASYNCSEL | SRATE | Reserved | PCI_PLUG# |
| --- | --- | --- | --- | --- | --- |

R-?                 R0                       R0                  R/W-1              R0                     R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7 | S66MHZ | 0 1 | Secondary PCI Bus reference clk is 33MHz Secondary PCI Bus reference clk is 66MHz |
| 6 | SASYNCCLK | 0 | Not used, forced to 0 |
| 5 | SASYNCSEL | 0 | Not supported, forced to 0 |
| 4 | SRATE* | 0 1 | Secondary PCI clock rate ½ primary Secondary PCI clock rate same as primary |
| 3-1 | Reserved | 0 | Reserved |
| 0 | PCI_PLUG# | 0 1 | EVM is installed in PCI slot EVM is not installed in PCI slot |

*On write these bits reflect the PCI configuration that will be applied at the next DM648 reset.  On read, these bits indicate the actual state of the PCI configuration from the last reset.

Firmware/PCB Revision Register (Offset 0x05)

7                                                                   3   2                                                                  0

| FIRMREV | PCBREV |
| --- | --- |

R                                                                        R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-3 | FIRMREV |  | PCB Firmware Revision Number |
| 2-0 | PCBREV |  | PCB Hardware Revision Number |

Video Port 0, 2, 3, and 4 Mux Control (Offset 0x06)

7                                          5   4                                          2   1                                          0

| VP34MUX | VP02MUX | Reserved |
| --- | --- | --- |

R/W-001                                 R/W-000                                          R0

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-5 | VP34MUX | 0x0 0x1 0x2 0x3 0x4 0x5 0x6 0x7 | Disconnect EMIF connected to EVM Flash/Bootmode VP3 and VP4 connected to EVM TVP5154 Video Inputs Ch5 – Ch8 Reserved Reserved VP3VP4/EMIF connected to daughter card Reserved Reserved |
| 4-2 | VP02MUX | 0x0 0x1 0x2 0x3 0x4 0x5 0x6 0x7 | Disconnect VP0 and VP2 connected to EVM TVP5154 Video Inputs Ch1 – Ch4 VP0 connected to TVP7000 and VP2 connected to daughter card Reserved Reserved Reserved Reserved Reserved |
| 1-0 | Reserved | 0 | Reserved |

User LEDs and Switches (Offset 0x07)

7                6                 5                4                 3               2                 1                 0

| LEDDS5 | LEDDS4 | LEDDS3 | LEDDS2 | USW4 | USW3 | USW2 | USW1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

R/W-0       R/W-0        R/W-0        R/W-0            R               R                R               R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7 | LEDDS5 | 0 1 | User LED DS5 extinguished User LED DS5 lit |
| 6 | LEDDS4 | 0 1 | User LED DS4 extinguished User LED DS4 lit |
| 5 | LEDDS3 | 0 1 | User LED DS3 extinguished User LED DS3 lit |
| 4 | LEDDS2 | 0 1 | User LED DS2 extinguished User LED DS2 lit |
| 3 | USW4 | 0 1 | User Switch SW3-4 off User Switch SW3-4 on |
| 2 | USW3 | 0 1 | User Switch SW3-3 off User Swtich SW3-3 on |
| 1 | USW2 | 0 1 | User Switch SW3-2 off User Switch SW3-2 on |
| 0 | USW1 | 0 1 | User Switch SW3-1 off User Switch SW3-1 on |

Daughter Card General Purpose IO Control (Offset 0x08)

7      6             5                    4                   3                     2                     1                     0

| Res | DCGPIO5M | DCGPIO4M | DCGPIO3M | DCGPIO2M | DCGPIO1M | DCGPIO0M |
| --- | --- | --- | --- | --- | --- | --- |

R0          R/W-0            R/W-0           R/W-0            R/W-0            R/W-0          R/W-0

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-6 | Reserved | 0 | Reserved |
| 5 | DCGPIO5M | 0 1 | DCGPIO5 pin is input DCGPIO5 pin is output |
| 4 | DCGPIO4M | 0 1 | DCGPIO4 pin is input DCGPIO4 pin is output |
| 3 | DCGPIO3M | 0 1 | DCGPIO3 pin is input DCGPIO3 pin is output |
| 2 | DCGPIO2M | 0 1 | DCGPIO2 pin is input DCGPIO2 pin is output |
| 1 | DCGPIO1M | 0 1 | DCGPIO1 pin is input DCGPIO1 pin is output |
| 0 | DCGPIO0M | 0 1 | DCGPIO0 pin is input DCGPIO0 pin is output |

Daughter Card General Purpose IO State (Offset 0x09)

7             6            5                    4                   3                  2                    1                   0

| Reserved | DCGPIO5 | DCGPIO4 | DCGPIO3 | DCGPIO2 | DCGPIO1 | DCGPIO0 |
| --- | --- | --- | --- | --- | --- | --- |

R0                R/W              R/W             R/W             R/W              R/W              R/W

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-6 | Reserved | 0 | Reserved |
| 5 | DCGPIO5 | 0 1 | DCGPIO5 pin is low or driven low DCGPIO5 pin is high or driven high |
| 4 | DCGPIO4 | 0 1 | DCGPIO4 pin is low or driven low DCGPIO4 pin is high or driven high |
| 3 | DCGPIO3 | 0 1 | DCGPIO3 pin is low or driven low DCGPIO3 pin is high or driven high |
| 2 | DCGPIO2 | 0 1 | DCGPIO2 pin is low or driven low DCGPIO2 pin is high or driven high |
| 1 | DCGPIO1 | 0 1 | DCGPIO1 pin is low or driven low DCGPIO1 pin is high or driven high |
| 0 | DCGPIO0 | 0 1 | DCGPIO0 pin is low or driven low DCGPIO0 pin is high or driven high |

DSP Timer Interface (Offset 0x0A)

7                                    5             4                           3                2                                       0

| Reserved | DSPTOUT1 | DSPTIN1 | Reserved |
| --- | --- | --- | --- |

R0                                 R                      R/W-0                               R0

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-5 | Reserved | 0 | Reserved |
| 4 | DSPTOUT1 | 0 1 | DSP TOUT1 is low DSP TOUT1 is high |
| 3 | DSPTIN1 | 0 1 | DSP TIN1 is driven low DSP TIN1 is driven high |
| 2-0 | Reserved | 0 | Reserved |

Power Supply Control (Offset 0x0B)

7                                                                                                                                            0

| Reserved | 3V3EN | 2V5EN | 1V8EN | SGMIIVEN | VCOREEN |
| --- | --- | --- | --- | --- | --- |

R0                       R                     R                      R                      R                      R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-5 | Reserved | 0 | Reserved |
| 4 | 3V3EN | 0 1 | 3.3V power supply off 3.3V power supply on |
| 3 | 2V5EN | 0 1 | 2.5V power supply off 2.5V power supply on |
| 2 | 1V8EN | 0 1 | 1.8V power supply off 1.8V power supply on |
| 1 | SGMIIVEN | 0 1 | SGMII power supply on SGMII power supply off |
| 0 | VCOREEN | 0 1 | Core power supply off Core power supply on |

Last Bootmode Switch Settings Low (read live)  (Offset 0x0C)

7                        6                     5                          4              3                                     0

| EMIFBWIDTH | PCI66 | HPIWIDTH | LENDIAN | BOOTMODE |
| --- | --- | --- | --- | --- |

R                      R                    R                           R                              R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | Boot Config Switch Settings |  | State of DM648 boot config at last reset. See DM648 data sheet for definitions. |

Warning: to get correct reading here, VP34MUX should be set to avoid incorrect reading.

Last Bootmode Switch Settings High (read live) (Offset 0x0D)

7                                                 6                                               5

| USERSWINT | BOOTSWINT | ALTBOOT |
| --- | --- | --- |

R                                               R                                              R

4                            3                            2                            1                           0

| DVREN | PLLBYPASS2 | AECLKINSEL | HPIEN | FASTBOOT |
| --- | --- | --- | --- | --- |

R                           R                            R                           R                          R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7 | USERSWINT | 0 1 | User switch GPIO U63 not requesting interrupt User switch GPIO U63 requesting interrupt |
| 6 | BOOTSWINT | 0 1 | Forced bootmode GPIO U64 not requesting interrupt Forced bootmode GPIO U64 requesting interrupt |
| 5 | ALTBOOT | 0 1 | ALTBOOT bootmode switch off – boot mode switches will be used at next reset. ALTBOOT bootmode switch on override boot config will be used at next reset. |
| 4-0 | Boot Config Switch Settings |  | State of DM648 boot config at last reset. See DM648 data sheet for definitions. |

Warning: to get correct reading here, VP34MUX should be set to avoid incorrect reading.

Bootmode Switch Overide Low (Offset 0x0E)

7                        6                     5                          4              3                                     0

| EMIFBWIDTH | PCI66 | HPIWIDTH | LENDIAN | BOOTMODE |
| --- | --- | --- | --- | --- |

R/W-0             R/W-0             R/W-0                 R/W-0                        R/W-0

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | Override Boot Config Switch Settings |  | State of EVM ALT boot configuration. See DM648 data sheet for definitions. Note: ALTBOOT must be 1 to have an effect. |

Bootmode Switch Override High (Offset 0x0F)

7                                                                                                                                            5

Reserved

R0

4                            3                            2                            1                           0

| DVREN | PLLBYPASS2 | AECLKINSEL | HPIEN | FASTBOOT |
| --- | --- | --- | --- | --- |

R/W-0                   R/W-0                  R/W-0                    R/W-0                 R/W-0

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-5 | Reserved |  | Reserved |
| 4-0 | Boot Config Switch Settings |  | State of EVM ALT boot configuration. See DM648 data sheet for definitions. Note: ALTBOOT must be 1 to have an effect. |

UART/SPI Mode Control (Offset 0x10)

7                                                                                                                1               0

| Reserved | UART_SPIn |
| --- | --- |

R0                                                                R/W-0

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-1 | Reserved | 0 | Reserved |
| 0 | UART_SPIn | 0 1 | DSP UART/SPI set to SPI mode and connected to SPI EEPROM and DCC DSP UART/SPI set to UART mode and connected to 9 pin EVM serial port connector |

Video Status (Offset 0x11)

7                                                                                           2          1                       0

| Reserved | TVD | NTSC_PAL |
| --- | --- | --- |

R0                                                         R                       R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-2 | Reserved | 0 | Reserved |
| 1 | TVD | 0 1 | TV Detect low TV Detect high |
| 0 | NTSC_PAL | 0 | JP4 set to PAL mode (Pins 2 & 3) JP4 set to NTSC mode (Pins 1 & 2) |

Clock Control (Offset 0x12)

7                                                                                       2              1                         0

| Reserved | PLLBYPASS | STOP |
| --- | --- | --- |

R0                                                     R/W-1                 R/W-1

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-2 | Reserved | 0 | Reserved |
| 1 | PLLBYPASSn | 0 | Drives CDCE906 S0 low Drives CDCE906 S0 high |
| 0 | STOPn | 0 | Drives CDCE906 S1 low Drives CDCE906 S1 high |

McASP/TSIP and Uart Mux Control (Offset 0x13)

7                                       5               4                       3           2                                            0

| Reserved | SPDIFO | SPDIFC | MCASPMUX |
| --- | --- | --- | --- |

R0                              R/W-0                R/W-0                        R/W-000 (DVR)

R/W-010 (CIV)

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-5 | Reserved | 0 | Reserved |
| 4 | SPDIFO | 0 1 | SPDIF optical output disabled SPDIF optical output enabled |
| 3 | SPDIFC | 0 1 | SPDIF coax output disabled SPDIF coax output enabled |
| 2-0 | MCASPMUX | 0x0 0x1 0x2 0x3 0x4 0x5 0x6 0x7 | McASP/TSIP port disconnected McASP/TSIP port connected to AIC33s McASP/TSIP port connected to DCC Reserved Reserved Reserved Reserved Reserved |

Peripheral Resets (Offset 0x14)*

7                6                 5                4                 3               2                 1                 0

| FLASH | VID5-8 | VID1-4 | SDOUT | HDOUT | PHY | AUDIO | HDIN |
| --- | --- | --- | --- | --- | --- | --- | --- |

R/W-0       R/W-0        R/W-0        R/W-0        R/W-0       R/W-0       R/W-0        R/W-0

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7 | FLASH | 0 1 | FLASH memory out of reset FLASH memory in reset |
| 6 | VID5-8 | 0 1 | TVP5154#2 out of reset TVP5154#2 in reset |
| 5 | VID1-4 | 0 1 | TVP5154#1 out of reset TVP5154#1 in reset |
| 4 | SDOUT | 0 1 | SAA7105 out of reset SAA7105 in reset |
| 3 | HDOUT | 0 1 | THS8200 out of reset THS8200 in reset |
| 2 | PHY | 0 1 | PHY out of reset PHY in reset |
| 1 | AUDIO | 0 1 | AUDIO out of reset AUDIO in reset |
| 0 | HDIN | 0 1 | HDIN out of reset HDIN in reset |

*Invalid for Alpha boards, always reads 0

Power Supply Status (Offset 0x15)*

7                6              5               4                 3                   2                     1               0

| PWRGD | 3.3VGD | 2.5VGD | 1.8VGD | SGMIIGD | COREGD | ECPGD | 5VGD |
| --- | --- | --- | --- | --- | --- | --- | --- |

R             R              R                R                R                  R                    R                R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7 | PWRGD | 0 1 | A power supply has failed All power supplies good |
| 6 | 3.3VGD | 0 1 | 3.3V power supply fail 3.3V power supply within +/-3% |
| 5 | 2.5VGD | 0 1 | 2.5V power supply fail 2.5V power supply within +/-3% |
| 4 | 1.8VGD | 0 1 | 1.8V power supply fail 1.8V power supply within +/-3% |
| 3 | SGMIIGD | 0 1 | SGMII power supply fail SGMII power supply within 1.2V +/-3% |
| 2 | COREGD | 0 1 | Core power supply fail Core power supply within 1.2V +/-3% |
| 1 | ECPGD | 0 1 | MSP430 power supply fail MSP430 power supply within 3.3V +/- 3% |
| 0 | 5VGD | 0 1 | 5V power supply fail 5V power supply within +/-10% |

*Reading from this register will initiate a voltage reading on all the power supplies.  Power good status as well as the supply voltage readings available from offset 0x16 through 0x21 will be updated.  Reading from the voltage readings only will give the values at the time of the last voltage reading taken by the last read from offset 0x15.

ECP Voltage Low and High (Offsets 0x16 and 0x17)

7                                                                                                                                            0

ECP Voltage Low

R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | ECP Voltage Low |  | Lower 8 bits of measured ECP voltage |

7                                                                  4    3                                                                  0

| Reserved | ECP Voltage High |
| --- | --- |

R-0                                                                     R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | Reserved |  | Reserved |
| 3-0 | ECP Voltage High |  | Upper 4 bits of measured ECP voltage |

ECP voltage (v) = ECP Voltage Count/819

VCC Core Voltage Low and High (Offsets 0x18 and 0x19)

7                                                                                                                                            0

Core Voltage Low

R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | Core Voltage Low |  | Lower 8 bits of measured core voltage |

7                                                                  4    3                                                                  0

| Reserved | Core Voltage High |
| --- | --- |

R-0                                                                     R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | Reserved |  | Reserved |
| 3-0 | Core Voltage High |  | Upper 4 bits of measured core voltage |

Core voltage (v) = Core Voltage Count/1638

SGMII Voltage Low and High (Offsets 0x1A and 0x1B)

7                                                                                                                                            0

SGMII Voltage Low

R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | SGMII Voltage Low |  | Lower 8 bits of measured SGMII voltage |

7                                                                  4    3                                                                  0

| Reserved | SGMII Voltage High |
| --- | --- |

R-0                                                                     R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | Reserved |  | Reserved |
| 3-0 | SGMII Voltage High |  | Upper 4 bits of measured SGMII voltage |

SGMII voltage (v) = SGMII Voltage Count/1638

1.8V Voltage Low and High (Offsets 0x1C and 0x1D)

7                                                                                                                                            0

1.8V Voltage Low

R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | 1.8V Voltage Low |  | Lower 8 bits of measured 1.8V voltage |

7                                                                  4    3                                                                  0

| Reserved | 1.8V Voltage High |
| --- | --- |

R-0                                                                     R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | Reserved |  | Reserved |
| 3-0 | 1.8V Voltage High |  | Upper 4 bits of measured 1.8V voltage |

1.8V voltage (v) = 1.8V Voltage Count/1638

2.5V Voltage Low and High (Offsets 0x1E and 0x1F)

7                                                                                                                                            0

2.5V Voltage Low

R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | 2.5V Voltage Low |  | Lower 8 bits of measured 2.5V voltage |

7                                                                  4    3                                                                  0

| Reserved | 2.5V Voltage High |
| --- | --- |

R-0                                                                     R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | Reserved |  | Reserved |
| 3-0 | 2.5V Voltage High |  | Upper 4 bits of measured 2.5V voltage |

2.5V voltage (v) = 2.5V Voltage Count/1290.744

3.3V Voltage Low and High (Offsets 0x20 and 0x21)

7                                                                                                                                            0

3.3V Voltage Low

R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | 3.3V Voltage Low |  | Lower 8 bits of measured 3.3V voltage |

7                                                                  4    3                                                                  0

| Reserved | 3.3V Voltage High |
| --- | --- |

R-0                                                                     R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | Reserved |  | Reserved |
| 3-0 | 3.3V Voltage High |  | Upper 4 bits of measured 3.3V voltage |

3.3V voltage (v) = 3.3V Voltage Count/972.872

5V Voltage Low and High (Offsets 0x22 and 0x23)

7                                                                                                                                            0

5V Voltage Low

R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | 5V Voltage Low |  | Lower 8 bits of measured 5V voltage |

7                                                                  4    3                                                                  0

| Reserved | 5V Voltage High |
| --- | --- |

R-0                                                                     R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | Reserved |  | Reserved |
| 3-0 | 5V Voltage High |  | Upper 4 bits of measured 5V voltage |

5V voltage (v) = 5V Voltage Count/655.2

ECP Register Definitions:

HD video Clock Control (Offset 0x24)

7                                                                                                                                            0

VIDCLK CODE

R/W-0

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | VIDCLK CODE | 0xnn | Code used for setting HD Video Clock supplied to TVP7000 and THS8200 |

Valid VIDCLK CODEs  are:

| 0x00 | Default – 74.250 MHz |
| --- | --- |
| 0x01 | 25.125 MHz |
| 0x02 | 27.000 MHz |
| 0x03 | 31.500 MHz |
| 0x04 | 36.000 MHz |
| 0x05 | 40.000 MHz |
| 0x06 | 49.500 MHz |
| 0x07 | 50.000 MHz |
| 0x08 | 56.250 MHz |
| 0x09 | 65.000 MHz |
| 0x0A | 74.250 MHz |
| 0x0B | 75.000 MHz |
| 0x0C | 108.000 MHz |

Last Bootmode Switch Settings Low (Offset 0x25)

7                        6                     5                          4              3                                     0

| EMIFBWIDTH | PCI66 | HPIWIDTH | LENDIAN | BOOTMODE |
| --- | --- | --- | --- | --- |

R                      R                    R                           R                              R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7-0 | Boot Config Switch Settings |  | State of DM648 boot config at last reset. See DM648 data sheet for definitions. |

ECP Register Definitions:

Last Bootmode Switch Settings High (Offset 0x26)

7                                                 6                                               5

| Reserved | Reserved | Reserved |
| --- | --- | --- |

R                                               R                                              R

4                            3                            2                            1                           0

| DVREN | PLLBYPASS2 | AECLKINSEL | HPIEN | FASTBOOT |
| --- | --- | --- | --- | --- |

R                           R                            R                           R                          R

| Bit | Field | Value | Description |
| --- | --- | --- | --- |
| 7 | Reserved | 0 |  |
| 6 | Reserved | 0 |  |
| 5 | Reserved | 0 |  |
| 4-0 | Boot Config Switch Settings |  | State of DM648 boot config at last reset. See DM648 data sheet for definitions. |