Introduce the UART test

These test case verify the basic UART functions:

- Send data to the UART by polling the UART transmit flag

- Receive the data from the UART by EDMA

- UART error interrupt processing

Contents in the test project

The folders of the test project as below:

The main code are described as below:

| Function | Positions | Description |
| --- | --- | --- |
| KeyStone_main_PLL_init | \common\KeyStone_common.c | DSP core PLL initialization |
| KeyStone_UART_calc | \common\KeyStone_UART_Init_drv.c | UART parameters cfg |
| KeyStone_UART_init |  | UART peripheral initialization |
| KeyStone_uartWrite_Busy |  | UART send char with polling busy bit |
| UART_ISR | \UART\src\uart_test.c | UART error interrupt routine |
| UART_EDMA_init | \UART\src\uart_edma_setup.c | UART EDMA initialization |
| UART_Interrupt_Init | \UART\src\uart_intc_setup.c | UART interrupt initialization |

To Run the Test on TCI6608 EVM

These code has been verified on TCI6608 EVM, the step to run the test on the EVM is as following:

- Extra the package to your workspace

- Import the project to the CCS

- Build the project, please pay attention that the CSL path must be modified according to your system environment

- Connect the UART to the PC, and open the serial port tools to watch the data

- Load the code to core 0

- Run the code, and you will find the following messages

[C66xx_0] UART test start

[C66xx_0] Initialize main PLL = x10/1

[C66xx_0] Initialize DDR PLL = x20/1

[C66xx_0] configure DDR at 666 MHz

[C66xx_0] UART setup started.

[C66xx_0] Hello from your Keystone board!

[C66xx_0] UART0 sending a few characters one at a time...

[C66xx_0] UART0 sending a string all at once...

And you will find the character string “HELLO” in the PC, when you send char from PC to the target, the target will send the received string back to the PC, noted, the UART on the test case worked on FIFO mode, and the FIFO length configuration is 8 bytes, so if the character string length is higher than 8 bytes, the receive may be incorrect.