### Introduction to the test cases

The program includes test cases for:

- I2C EEPROM test, which supports AT24C512B, AT24C1024B and M24M01 on Shannon, Appleton and Nyquist EVM. The basic test method is to fill destination memory with fixed pattern (0, 0xFF, 0x55, 0xAA) or address, and then verify it.

- Read temperature from I2C temperature sensor (TMP100, TMP101 or ADT75).

### Contents in the test project

The folders of the test project are as below:

The main code are described as below:

| Function | Source file | Description |
| --- | --- | --- |
| I2C_Master_Init | \common\ KeyStone_I2C_Init_drv.c | Initialize I2C as master |
| I2C_read I2C_write |  | Implementation of low level operations on I2C |
| GetTemperature | \I2C\src\I2C_Temp_Sensor_drv.c | Read temperature from the I2C temperatue sensor |
| I2C_EEPROM_Test | \I2C\src\I2C_EEPROM_Test.c | Fill I2C EEPROM with fixed pattern or address, and then verify them. |
| I2C_EEPROM_read | \I2C\src\I2C_EEPROM_drv.c | Implementation of low level operations on I2C EEPROM. |
| I2C_EEPROM_write |  |  |

### To run the test program on EVM

The project is built with CCS5.1, it can be run on Shannon (TCI6608/C6678) EVM, Nyquist (TCI6618/C6670) EVM, Appleton (TCI6614 EVM) or Dual Turbo Nyquist (TCI6616/TCI6618) EVM without code modification.

The steps to run the test cases on EVM are:

- extract the package to your workspace

- import the project to CCS

- build the project. You may need to change the CSL including path, by default the project use CSL header files in: C:\ti\pdk_tci6614_1_00_00_06\packages\ti\csl

- load the project to any core of a DSP.

- run the core, you should see output in console window like below:

Initialize main PLL = x236/29

I2C EEPROM test start...

Passed I2C_EEPROM Fill Test from 0x    0 to 0x 4000 with pattern 0x       0

Passed I2C_EEPROM Fill Test from 0x    0 to 0x 4000 with pattern 0xffffffff

Passed I2C_EEPROM Fill Test from 0x    0 to 0x 4000 with pattern 0xaaaaaaaa

Passed I2C_EEPROM Fill Test from 0x    0 to 0x 4000 with pattern 0x55555555

Passed I2C_EEPROM Address Test from 0x    0 to 0x 4000

Passed I2C_EEPROM Fill Test from 0x 4000 to 0x 8000 with pattern 0x       0

Passed I2C_EEPROM Fill Test from 0x 4000 to 0x 8000 with pattern 0xffffffff

Passed I2C_EEPROM Fill Test from 0x 4000 to 0x 8000 with pattern 0xaaaaaaaa

Passed I2C_EEPROM Fill Test from 0x 4000 to 0x 8000 with pattern 0x55555555

Passed I2C_EEPROM Address Test from 0x 4000 to 0x 8000

Passed I2C_EEPROM Fill Test from 0x 8000 to 0x c000 with pattern 0x       0

Passed I2C_EEPROM Fill Test from 0x 8000 to 0x c000 with pattern 0xffffffff

Passed I2C_EEPROM Fill Test from 0x 8000 to 0x c000 with pattern 0xaaaaaaaa

Passed I2C_EEPROM Fill Test from 0x 8000 to 0x c000 with pattern 0x55555555

Passed I2C_EEPROM Address Test from 0x 8000 to 0x c000

Passed I2C_EEPROM Fill Test from 0x c000 to 0x10000 with pattern 0x       0

Passed I2C_EEPROM Fill Test from 0x c000 to 0x10000 with pattern 0xffffffff

Passed I2C_EEPROM Fill Test from 0x c000 to 0x10000 with pattern 0xaaaaaaaa

Passed I2C_EEPROM Fill Test from 0x c000 to 0x10000 with pattern 0x55555555

Passed I2C_EEPROM Address Test from 0x c000 to 0x10000

I2C test complete.

### To modify the test cases

You can enable/disable the test cases with following macro at the beginning of “I2C_main.c”.

#define I2C_EEPROM_TEST 		1

#define I2C_TEMP_SENSOR_TEST 	0

For other I2C devices, you may need change some of the I2C configurations in “I2C_main.c”:

#define I2C_EEPROM_SIZE_KB 	64

Uint32 I2C_EEPROM_address= 0x50;

I2C_Master_Init(400); /*I2C output clock <= 400 KHz*/