# PMP4394 24Vin 1W Bias Supply with

# ±5V/100mA

# Output

# Texas Instruments

# PMP4394 Test Procedure

# China Power Reference Design

# REV A

GENERAL

  - PURPOSE

To provide detailed data for evaluating and verifying the PMP4394.

  - REFERENCE DOCUMENTATION

Schematic: PMP4394_SCH_RevA

Assembly: PMP4394_PCB_RevA

BOM

  - TEST EQUIPMENTS

Multi-meter(voltage): Fluke 287

DC Source: TDK-Lambda GEN100-33

Load: Chroma 63110A module

- INPUT CHARACTERISTICS

  - Full load Efficiency

| Vin (V) | Iin(A) | Vo1(V) | Vo2(V) | Io1(mA) | Io2(mA) | Effi.(%) |
| --- | --- | --- | --- | --- | --- | --- |
| 21.6 | 53.00 | 4.83 | -4.78 | 100 | 100 | 84.1 |
| 23.0 | 50.00 | 4.87 | -4.81 | 100 | 100 | 84.2 |
| 24.0 | 48.25 | 4.89 | -4.83 | 100 | 100 | 84.0 |
| 25.0 | 46.60 | 4.91 | -4.85 | 100 | 100 | 83.9 |
| 26.4 | 44.40 | 4.92 | -4.85 | 100 | 100 | 83.4 |

  - Efficiency versus output current

| Io1/Io2(mA) | Vin (V) | Iin(mA) | Vo1(V) | Vo2(V) | Effi.(%) |
| --- | --- | --- | --- | --- | --- |
| 10 | 24.07 | 8.56 | 5.12 | -5.05 | 49.4 |
| 20 | 24.06 | 12.95 | 5.05 | -4.97 | 64.3 |
| 30 | 24.05 | 17.30 | 5.03 | -4.96 | 72.0 |
| 40 | 24.04 | 21.69 | 5.00 | -4.92 | 76.1 |
| 50 | 24.03 | 25.95 | 4.97 | -4.90 | 79.1 |
| 60 | 24.02 | 30.51 | 4.96 | -4.88 | 80.6 |
| 70 | 24.01 | 35.04 | 4.93 | -4.86 | 81.5 |
| 80 | 24.00 | 39.52 | 4.92 | -4.86 | 82.5 |
| 90 | 23.99 | 44.00 | 4.91 | -4.84 | 83.1 |
| 100 | 23.98 | 48.60 | 4.89 | -4.83 | 83.4 |

- OUTPUT CHARACTERISTICS

  - Line and load Regulation

| Vin (V) | Io1=Io2=10mA |  | Io1=Io2=30mA |  | Io1=Io2=50mA |  | Io1=Io2=70mA |  | Io1=Io2=100mA |  | % |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Vo1 (V) | Vo2 (V) | Vo1 (V) | Vo2 (V) | Vo1 (V) | Vo2 (V) | Vo1 (V) | Vo2 (V) | Vo1 (V) | Vo2 (V) |  |
| 21.6 | 5.06 | -5.00 | 4.96 | -4.90 | 4.92 | -4.84 | 4.87 | -4.81 | 4.81 | -4.76 | 4.8 |
| 24.0 | 5.10 | -5.03 | 5.01 | -4.94 | 4.96 | -4.89 | 4.92 | -4.85 | 4.86 | -4.81 | 3.8 |
| 26.4 | 5.15 | -5.07 | 5.05 | -4.98 | 5.00 | -4.93 | 4.96 | -4.90 | 4.92 | -4.85 | 3.0 |

  - Ripple and noise

| Vin (V) | Io1=Io2=10mA |  | Io1=Io2=100mA |  |
| --- | --- | --- | --- | --- |
|  | Vo1 (mV) | Vo2 (mV) | Vo1 (mV) | Vo2 (mV) |
| 21.6 | 4.8 | 6.0 | 15.2 | 16.0 |
| 24 | 4.8 | 5.6 | 13.6 | 14.8 |
| 26.4 | 4.8 | 6.0 | 13.6 | 14.8 |

| Vin=21.6V Io=10mA Ch1: Vo1 Ripple | Vin=21.6V Io=100mA Ch1: Vo1 Ripple |
| --- | --- |
| Vin=24V Io=10mA Ch1: Vo1 Ripple | Vin=24V Io=100mA Ch1: Vo1 Ripple |
| Vin=26.4V Io=10mA Ch1: Vo1 Ripple | Vin=26.4V Io=100mA Ch1: Vo1 Ripple |
| Vin=21.6V Io=10mA Ch1: Vo2 Ripple | Vin=21.6V Io=100mA Ch1: Vo2 Ripple |
| Vin=24V Io=10mA Ch1: Vo2 Ripple | Vin=24V Io=100mA Ch1: Vo2 Ripple |
| Vin=26.4V Io=10mA Ch1: Vo2 Ripple | Vin=26.4V Io=100mA Ch1: Vo2 Ripple |

  - Start up and shut down

| Vin=24V Io=100mA Ch1: Vo1 Start up | Vin=24V Io=100mA Ch1: Vo1 shut down |
| --- | --- |
| Vin=24V Io=100mA Ch1: Vo2 Start up | Vin=24V Io=100mA Ch1: Vo2 shut down |

  - Output short protection

Vin=24V Io=100mA Vo1 from full load to short Ch1: Vo1 Ch2: U12 FB