# b

# Texas Instruments

# PMP4390 Test Procedure

# China Power Reference Design

# REV A

# 12/09/2013

- GENERAL

  - PURPOSE

To provide detailed data for evaluating and verifying the PMP4390, which uses TI new Primary Side Controller UCC28713 and synchronous rectified controller UCC24610 for 5.5V3A adapter with size 44mmx35mmx15mm and can be placed into standard 10W England version shell. The below photo shows this demo board.

  - REFERENCE DOCUMENTATION

Schematic PMP4390_SCH.PDF

Assembly PMP4390_PCB.PDF

BOM

Gerber files

Promotion tools

  - TEST EQUIPMENTS

Power-meter: YOKOGAWA WT210

Multi-meter(current): Fluke 8845A

Multi-meter(voltage): Fluke 187

AC Source: Chroma 61530

Electronic load: Chroma 63105A module

Testing demoboard

- INPUT CHARACTERISTICS

Efficiency is tested on USB-end

Otherwise Specified, the test is under the condition with 100cm cable

  - STANDBY POWER

  - EFFICIENCY DATA

Notes: efficiency test is based on USB port

| 85v |  |  |  |  |  | 115v |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pi(w) | Io(A) |  | Vo(V) | efficiency |  | Pi(w) | Io(A) | Vo(V) | efficiency |
| 1.025 | 0.15 |  | 5.197 | 0.761 |  | 1.038 | 0.15 | 5.195 | 0.751 |
| 1.902 | 0.3 |  | 5.206 | 0.821 |  | 1.925 | 0.3 | 5.203 | 0.811 |
| 2.339 | 0.375 |  | 5.211 | 0.835 |  | 2.361 | 0.375 | 5.209 | 0.827 |
| 4.604 | 0.75 |  | 5.246 | 0.855 |  | 4.598 | 0.75 | 5.243 | 0.855 |
| 6.9 | 1.125 |  | 5.276 | 0.86 |  | 6.882 | 1.125 | 5.279 | 0.863 |
| 9.28 | 1.5 |  | 5.311 | 0.858 |  | 9.219 | 1.5 | 5.31 | 0.864 |
| 11.74 | 1.875 |  | 5.35 | 0.854 |  | 11.62 | 1.875 | 5.353 | 0.864 |
| 14.25 | 2.25 |  | 5.402 | 0.853 |  | 14.05 | 2.25 | 5.394 | 0.864 |
| 16.81 | 2.625 |  | 5.442 | 0.85 |  | 16.53 | 2.625 | 5.436 | 0.863 |
| 19.35 | 3 |  | 5.476 | 0.849 |  | 18.99 | 3 | 5.461 | 0.863 |
| 132v |  |  |  |  |  | 150v |  |  |  |
| Pi(w) | Io(A) |  | Vo(V) | efficiency |  | Pi(w) | Io(A) | Vo(V) | efficiency |
| 1.052 | 0.15 |  | 5.2 | 0.741 |  | 1.085 | 0.15 | 5.205 | 0.72 |
| 1.946 | 0.3 |  | 5.202 | 0.802 |  | 1.976 | 0.3 | 5.203 | 0.79 |
| 2.378 | 0.375 |  | 5.21 | 0.822 |  | 2.406 | 0.375 | 5.209 | 0.812 |
| 4.602 | 0.75 |  | 5.244 | 0.855 |  | 4.627 | 0.75 | 5.242 | 0.85 |
| 6.87 | 1.125 |  | 5.281 | 0.865 |  | 6.899 | 1.125 | 5.279 | 0.861 |
| 9.2 | 1.5 |  | 5.312 | 0.866 |  | 9.213 | 1.5 | 5.312 | 0.865 |
| 11.58 | 1.875 |  | 5.347 | 0.866 |  | 11.58 | 1.875 | 5.346 | 0.866 |
| 14 | 2.25 |  | 5.394 | 0.867 |  | 13.99 | 2.25 | 5.394 | 0.868 |
| 16.44 | 2.625 |  | 5.438 | 0.868 |  | 16.43 | 2.625 | 5.434 | 0.868 |
| 18.86 | 3 |  | 5.463 | 0.869 |  | 18.88 | 3 | 5.465 | 0.868 |
| 180v |  |  |  |  |  |  |  |  |  |
| Pi(w) |  | Io(A) | Vo(V) | efficiency |  |  |  |  |  |
| 1.133 |  | 0.15 | 5.203 | 0.689 |  |  |  |  |  |
| 2.026 |  | 0.3 | 5.202 | 0.77 |  |  |  |  |  |
| 2.455 |  | 0.375 | 5.209 | 0.796 |  |  |  |  |  |
| 4.665 |  | 0.75 | 5.242 | 0.843 |  |  |  |  |  |
| 6.916 |  | 1.125 | 5.28 | 0.859 |  |  |  |  |  |
| 9.25 |  | 1.5 | 5.315 | 0.862 |  |  |  |  |  |
| 11.62 |  | 1.875 | 5.348 | 0.863 |  |  |  |  |  |
| 13.99 |  | 2.25 | 5.395 | 0.868 |  |  |  |  |  |
| 16.46 |  | 2.625 | 5.434 | 0.867 |  |  |  |  |  |
| 18.93 |  | 3 | 5.467 | 0.866 |  |  |  |  |  |
| 230v |  |  |  |  |  | 264v |  |  |  |
| Pi(w) | Io(A) |  | Vo(V) | efficiency |  | Pi(w) | Io(A) | Vo(V) | efficiency |
| 1.27 | 0.15 |  | 5.193 | 0.613 |  | 1.345 | 0.15 | 5.192 | 0.579 |
| 2.14 | 0.3 |  | 5.2 | 0.729 |  | 2.219 | 0.3 | 5.2 | 0.703 |
| 2.57 | 0.375 |  | 5.206 | 0.76 |  | 2.654 | 0.375 | 5.207 | 0.736 |
| 4.766 | 0.75 |  | 5.236 | 0.824 |  | 4.846 | 0.75 | 5.237 | 0.811 |
| 7.027 | 1.125 |  | 5.275 | 0.845 |  | 7.099 | 1.125 | 5.274 | 0.836 |
| 9.328 | 1.5 |  | 5.312 | 0.854 |  | 9.388 | 1.5 | 5.312 | 0.849 |
| 11.66 | 1.875 |  | 5.348 | 0.86 |  | 11.73 | 1.875 | 5.348 | 0.855 |
| 14.05 | 2.25 |  | 5.383 | 0.862 |  | 14.14 | 2.25 | 5.383 | 0.857 |
| 16.54 | 2.625 |  | 5.433 | 0.862 |  | 16.62 | 2.625 | 5.432 | 0.858 |
| 19 | 3 |  | 5.448 | 0.86 |  | 19.1 | 3 | 5.469 | 0.859 |

2.2.1 Load and input voltage Vs efficiency curve tested at USB-end

2.2.2 efficiency curve at 5% and 10% of full load

2.2.3 Average efficiency curve at 0.75A,1.5A,2.25A,3A

2.2.5 Efficiency Vs load curve

  - INPUT CURRENT

| Vin(Vac) | Freq(Hz) | Iin(Arms) | Pass/Fail |
| --- | --- | --- | --- |
| 85 | 60 | 0.36 |  |

  - INPUT INRUSH CURRENT

Vin:264Vac Io: 3A Ch1: Input bulk CAP voltage 100V/div Ch4: Input inrush current 1A/div

- OUTPUT CHARACTERISTICS

  - STARTUP TIME

| Input voltage | Output current | Startup time | Pass/Fail |
| --- | --- | --- | --- |
| 115Vac | 3A | 584mS |  |
| 230Vac | 3A | 569mS |  |
| Vin:115Vac Io: 3A Ch1: MOSFET’s drain voltage, 100V/div Ch3: output voltage, 2V/div |  |  |  |
| Vin:230Vac Io: 3A Ch1: MOSFET’s drain voltage, 100V/div Ch3: output voltage, 2V/div |  |  |  |

  - OUTPUT VOLTAGE RISE TIME

| Input voltage | Output current | Startup time | Pass/Fail |
| --- | --- | --- | --- |
| 115Vac | 3A | 31mS |  |
| 230Vac | 3A | 27.3mS |  |

Vin:115Vac Io: 3A Ch1: MOSFET’s drain voltage, 100V/div Ch3: output voltage, 2V/div

Vin:230Vac Io: 3A Ch1: MOSFET’s drain voltage, 100V/div Ch3: output voltage, 2V/div

  - OUTPUT VOLTAGE OVERSHOOT

| Input voltage | Output current | overshoot voltage | Pass/Fail |
| --- | --- | --- | --- |
| 115Vac | 0A | <1% |  |
| 230Vac | 0A | <1% |  |
| Vin:115Vac Io: 0A Ch1: MOSFET’s drain voltage, 100V/div Ch3: output voltage, 2V/div |  |  |  |
| Vin:230Vac Io: 0A Ch1: MOSFET’s drain voltage, 100V/div Ch3: output voltage, 2V/div |  |  |  |

  - RIPPLE VOLTAGE

| Input voltage | Output current | Ripple voltage | Pass/Fail |
| --- | --- | --- | --- |
| 115Vac | 0.75A | 54mV |  |
| 115Vac | 1.5A | 52mV |  |
| 115Vac | 2.25A | 70mV |  |
| 115Vac | 3A | 62mV |  |
| 230Vac | 0.75A | 36mV |  |
| 230Vac | 1.5A | 58mV |  |
| 230Vac | 2.25A | 58mV |  |
| 230Vac | 3A | 66mV |  |

Vin:115Vac Io: 0.75A Ch3: output ripple voltage, 50mV/div

Vin:115Vac Io: 1.5A Ch3: output ripple voltage, 50mV/div

Vin:115Vac Io: 2.25A Ch3: output ripple voltage, 50mV/div

Vin:115Vac Io: 3A Ch3: output ripple voltage, 50mV/div

Vin:230Vac Io: 0.75A Ch3: output ripple voltage, 50mV/div

Vin:230Vac Io: 1.5A Ch3: output ripple voltage, 50mV/div

Vin:230Vac Io: 2.25A Ch3: output ripple voltage, 50mV/div

Vin:230Vac Io: 3A Ch3: output ripple voltage, 50mV/div

  - DYNAMIC RESPONSE

| Input voltage | Output current | Max voltage | Min voltage |
| --- | --- | --- | --- |
| 115Vac | 5%-95% of full load | 5.58V | 4.92V |
| 230Vac | 5%-95% of full load | 5.6V | 4.92V |

Vin:115Vac test condition: 5%-95% of full load, 0.1A/us, 30ms cycle, 100cm cable Ch3: output voltage Ch4: output current

Vin:230Vac test condition: 5%-95% of full load, 0.1A/us, 30ms cycle, 100cm cable Ch3: output voltage Ch4: output current

  - OUTPUT VOLTAGE PROTECTION

| CONDITIONS | Protection voltage（V） | Pass/Fail |
| --- | --- | --- |
| Vin (Vac) |  |  |
| 115&230 | 6 |  |

Vin: 115Vac OVP: 6V CH2: Vdd voltage, 10V/div CH3: Vo, 2V/div

  - OUTPUT SHORT PROTECTION

| Input voltage | Output short protection |
| --- | --- |
| 115&230Vac | Hiccup up mode |

Vin:115Vac CH1: Vds of MOSFET, 100V/div CH2: Vdd, 10V/div CH3: Vo, 5V/div CH4: output current, 2A/div

Vin:230Vac CH1: Vds of MOSFET, 100V/div CH2: Vdd, 10V/div CH3: Vo, 5V/div CH4: output current, 2A/div

- IV CURVE

- THERMAL TEST

5.1. Thermal test with case at Ta=25ºC

- EMI Test

6.1 Conduction emission

Vin: 230Vac, Line, Io: 3A Test condition: 1.5m cable with 1.77R load resistor

Vin:230Vac, Neutral, Io: 3A Test condition: 1.5m cable with 1.77R load resistor

6.2 Radiation emission

Vin: 230Vac, Horizontal, Io: 3A Test condition: 1.5m cable with 1.77R load resistor

Vin:230Vac, Vertical, Io: 3A Test condition: 1.5m cable with 1.77R load resistor