ADC10模块

----------------------7.17

[image: image1.png]22.3 ADC10 Registers
The ADC10 registers are listed in Table 22-3.

Table 22-3. ADC10 Registers

Register Short Form Register Type Address Initial State
ADC10 input enable register 0 ADC10AEQ Read/write 04Ah Reset with POR
ADC10 input enable register 1 ADC10AE1 Readlwrite 04Bh Reset with POR
ADC10 control register 0 ADC10CTLO Read/write 01B0h Reset with POR
ADC10 control register 1 ADC10CTL1 Readlwrite 01B2h Reset with POR
ADC10 memory ADC10MEM Read 01B4h Unchanged
ADC10 data transfer control register 0 ADC10DTCO Read/write 048h Reset with POR
ADC10 data transfer control register 1 ADC10DTC1 Readlwrite 049h Reset with POR
ADC10 data transfer start address ADC10SA Read/write 01BCh 0200h with POR





[image: image2.png]22.3.1 ADC10CTLO, ADC10 Control Register 0

15 14 13 12 1 10 9 8
SREFx ADC10SHTx ADC10SR REFOUT REFBURST
w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0)
7 6 5 4 3 2 1 0
MSC REF2_5V REFON ADC100N ADC10IE ADC10IFG ENC ADC10SC
w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0)

Can be modified only when ENC = 0




[image: image3.png]22.3.2 ADC10CTL1, ADC10 Control Register 1

15 14 13 12 " 10 9 8
INCHx SHSx ADC10DF ISSH
w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0)
7 6 5 4 3 2 1 0
ADC10DIVX ADC10SSELx CONSEQx ADC10BUSY
w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) 0

Can be modified only when ENC = 0





[image: image4.png]22.3.5 ADC10MEM, Conversion-Memory Register, Binary Format

15 14 13 12 1 10 9 8
0 0 0 0 0 0 Conversion Results
0 0 0 0 0 0 r r
7 6 5 4 3 2 1 0
Conversion Results
r r r r r r r r
Conversion Bits 15-0  The 10-bit conversion results are right justified, straight-binary format. Bit 9 is the MSB. Bits 15-10 are

Results

always 0.





[image: image5.png]22.3.3 ADC10AEQ, Analog (Input) Enable Control Register 0

7 6 5 4 3 2 1 0
[ ADC10AEOX

w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0)
ADC10AEOx  Bits 7-0

AADC10 analog enable. These bits enable the corresponding pin for analog input. BITO corresponds to A0,

BIT1 corresponds to A1, etc. The analog enable bit of not implemented channels should not be programmed
to1.

0 Analog input disabled
1 Analog input enabled




[image: image6.png]22.3.4 ADC10AE1, Analog (Input) Enable Control Register 1 (MSP430F22xx only)

7 6 5 4 3

2 1 0
ADC10AE1X Reserved

w-(0) w-(0) w-(0) w-(0) w-(0) w-(0)
ADC10AE1x  Bits 7-4

w-(0) w-(0)

ADC10 analog enable. These bits enable the corresponding pin for analog input. BIT4 corresponds to A12,

BIT5 corresponds to A13, BIT6 corresponds to A14, and BIT7 corresponds to A15. The analog enable bit of
not implemented channels should not be programmed to 1.

0 Analog input disabled

1 Analog input enabled
Reserved Bits 30  Reserved





单通道单次转换。
[image: image7.jpg]A single channel selected by INCHXx is sampled and converted once. The ADC result is written to
ADC10MEM. Figure 22-5 shows the flow of the single-channel, single-conversion mode. When ADC10SC
triggers a conversion, successive conversions can be triggered by the ADC10SC bit. When any other
trigger source is used, ENC must be toggled between each conversion.




这个是说。ADC10CTL里面的ADC10SC是控制ADC10开始转换的bit，只有当ADC10SC触发转换，ADC10才开始转换。要不然，即使ADC10ON，也不转换。

同时也说了ADC10MEM里面存的是转换完成的值。

ENC必须在每次转换的时候都要触发。[image: image8.png]CCONSEQx = 00

AADC100N

x=INCHx
Wait for Enable

SHS =0
and ENC
ENC=1ord
and -
ADC10SC = & ( Wait for Trigger

(418/16/64) x ADC10CLK

Sample, Input
Channel

12 x ADC10CLK

Conversion
Completed,
Result to
ADC10
[ADC10IFG is Sel,

npredictable




关于ADC10CTL：

[image: image9.png]22.3.1 ADC10CTLO, ADC10 Control Register 0

15 14 13 12 1 10 9 8
[ SREFx [ ADC10SHTx [ ADC10SR | REFOUT | REFBURST |
™w-(0) ™w-(0) ™w-(0) ™w-(0) ™w-(0) ™w-(0) ™w-(0) ™w-(0)

7 6 5 4 3 2 1 4
[msc | REF25v | REFON | ADC10ON | ADC10IE | ADC10IFG | ENC___ | ADC10sC |
™w-(0) ™w-(0) ™w-(0) ™w-(0) ™w-(0) ™w-(0) ™w-(0) ™w-(0)

Can be modified only when ENC = 0




BIT1：

ENC必须每次都触发。

BIT0：

[image: image10.png]Start conversion. Software-controlled sample-and-conversion start. ADC10SC and ENC may be set together

ADC10SC Bit0
with one instruction. ADC10SC is reset automatically

0 No sample-and-conversion start
1 ‘Start sample-and-conversion




[image: image11.png]ADC100N Bit4 ADC10 on

0 ADC10 off
1 ADC100n
ADC10IE Bit3 ADC10 interrupt enable
0 Interrupt disabled

1 Interrupt enabled




[image: image12.png]ADC10 Registers

www.ti.com

ADC10IFG Bit 2 'ADC10 interrupt flag. This bit is set if ADC10MEM is loaded with a conversion resull

It_It g it
when the internuot request is accepled. or it.may be reset by software, When using the DTC this flag is set
when a block of transfers is completed.

0 No interrupt pending
1 Interrupt pending




单次转换：
[image: image13.png]vhile(l)

{

ADC10CTLO |=ADC10SC;
if(flag)

{
ADC10CTLO |=EN

display();
flag=0;





ADC10单次转换的初始化：
[image: image14.png]ADC10CTLO=ADC10SHT_2+ADCLOCN+ADCLOIE; // S B 45474 16xADCI0CLK 72D 7 2428
TNCH_2:// % BAE. TNCH 25201 25 EHA.
ADC102E0|=0%027





总的程序源代码：

void main( void )

{

  // Stop watchdog timer to prevent time out reset

  WDTCTL = WDTPW + WDTHOLD;

  lcdinit();

  W_str(2,1,"ADC10");

  W_str(0,2,"电压为：");

  ADC10CTL0=ADC10SHT_2+ADC10ON+ADC10IE;//设置的时钟 16xADC10CLK 开AD  开中断使能

  ADC10CTL1=INCH_2;//设置通道。INCH_2就是P1.2引脚输入。

  ADC10AE0|=0x02;

  P1DIR|=0x01;

    _EINT();

    while(1)

    {

    ADC10CTL0|=ADC10SC;

    if(flag)

       { 

         ADC10CTL0|=ENC;//转换完成一次之后蔡能进这个函数，才能转换第二次。

          display();

          flag=0;

       }

     }

}

#pragma vector=ADC10_VECTOR

__interrupt void ADC (void)

{

   ad=ADC10MEM;

   v=ad*3.56*1000/1024;//将ADC10MEM中的数据转换成电压值

   flag=1;

}

其他的模块自己看需要添加。

[image: image15.png]22.3.2 ADC10CTL1, ADC10 Control Register 1

15 14 13 12 " 10 9 8
INCHx SHSx ADC10DF ISSH
w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0)
7 6 5 4 3 2 1 0
ADC10DIVX ADC10SSELx CONSEQx ADC10BUSY
w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) w-(0) 0

Can be modified only when ENC = 0





INCH的资料见user guid手册：P568

刚才找资料，以后用到ADC10模块的话，尽量使用REF2_5V,这个参考电压相当准确。

用ADC10模块做的电压表：

REF2_5V开的话，一定要同时打开REFON。

  这句的作用是只有等一次转换完成才能第二次








