Sniffer Installation Guide

- Flash sniffer build (under sniffer_builds directory) on V3 kit.

- The zip files contain the AutomatedTestMiddleware install files and the Sniffer PLC DSP software.  Unzip these files to a temp directory to install.  Copy wireshark folder (choose wireshark_32 OR wireshark_64 depending on your system.) to C:\.

- Install the AutomatedTestMiddleware-Debug.msi (Note that  .Net 3.5 is required to run the middleware)

- Open the Unzip temp directory and COPY the Sniffer Automated.lnk to your desktop. This is an example short cut with the parameters necessary to start communicating with Wireshark. You will need to change the path in the short cut (the example is for Win 7) and the port number on the command line.  In the example the target line is "C:\Program Files (x86)\Texas Instruments\AutomatedTest\PLC_Application_Suite.exe" mainform=AutomatedTestMiddleware sniffer=snifferpipe,scib,com40 include=com40

- For XP the C:\Program Files (x86) should become C:\Program Files. The command line parameter mainform instructs the PLC_Applicate_Suite.exe to execute the Automated TestMiddleware.  The command line parameter sniffer defines the named pipe name to communicate to Wireshark and the com port the PLC sniffer is attached to.  Notice there is no space between the pipe name, comma, and serial.  Put a space there and it will not work.

- Install the Visual Studio runtime x86 or x64 depending if your machine is a 32 or 64 bit machine.  The link to the VS x86 is http://www.microsoft.com/en-us/download/details.aspx?id=5555 and the link to the VS x64 is http://www.microsoft.com/en-us/download/details.aspx?id=14632

- Open the wireshark folder (should be locataed in C:\Wireshark_32 or C:\Wireshark_64), The wireshark exe and dlls should be found in C:\wireshark_XX and not in a lower subdirectory.  The file is not an install file.  Do not overlay any current Wireshark installation. The version of Wireshark installed in this manner will only as a PLC sniffer and will not monitor regular network traffic.

- Create a short cut of Wireshark on the desktop, The command line parameters for wareshark are  –k –i \\.\pipe\snifferpipe

- Note that type the complete command line options on the target window instead of “copy and paste”. The –k tells wireshark to immediately start monitoring when activated. The –i \\.\pipe\snifferpipe defines the named pipe and is required.

- Now ready to use wireshark sniffer!

Sniffer Operation Guide

- Connect SCI-B (or SCI-A depending on your command-line option) of the Sniffer PLC Kit to your computer. Check the COM number/physical connection (SCI-A or SCI-B) and update target line options in the middleware.

- Start the AutomatedTestMiddleware.

- When you see below output (green-colored) from Automated Test Middleware, It means the sniffer works OK.

- After the Automated Test Middleware start successfully, we can then start the wireshark. The below figure shows snapshot for captured sniffer.