1.示例程序中几种状态说明

第一次看i2c_eeprom示例程序，对程序中的MsgStatus信息状态切换非常懵懂，为什么要有这几个状态？状态切换顺序如何安排？一大堆的状态，让人有些摸不着头脑。先把程序中的头文件涉及的7种状态分析一下。

// I2C  Message Commands for I2CMSG struct

#define I2C_MSGSTAT_INACTIVE          0x0000    //未激活状态：一般成功发送数据或者//接受数据后可以设置信息状态为此状态，告诉用户可进行下一次的写数据或读数据。

#define I2C_MSGSTAT_SEND_WITHSTOP     0x0010 //发送带停止位数据：这是为写数据而设///的状态，写入地址和数据之后发个停止位告诉存储器数据写入完毕。

#define I2C_MSGSTAT_WRITE_BUSY        0x0011  //写数据忙状态：在将待写的数据放入//缓存后，就可以使能IIC传输数据了，然后把信息状态设为该状态，意在告诉用户：数据//已经在传送过程中。当然是否传送完毕，还需要通过查询SCD位来判断。

#define I2C_MSGSTAT_SEND_NOSTOP       0x0020//发送无停止位数据：这个状态是为了读//取数据而设的，有查阅过AT24C1024EEPROM存储器使用手册的读者知道，在读数据之前//要发送数据的地址，发完地址不能产生停止位，这是存储器硬件设计决定的。设为这个状//态意在告诉读者，可以发送要读取的数据的地址了。

#define I2C_MSGSTAT_SEND_NOSTOP_BUSY  0x0021//发送无停止位数据忙状态：这个状态是//为了读取数据而设的,似于I2C_MSGSTAT_WRITE_BUSY，说明地址数据已经在传送过程中。//传送是否成功，还要看ARDY的状态。

#define I2C_MSGSTAT_RESTART           0x0022//重发开始位状态：这个状态也是为读取

////数据而设。我们知道，读取存储器数据主要分两个步骤：第一，发送START位+设备地址

//+数据地址+无停止位。第二，再发START位+设备地址，紧接着存储器发送数据到IIC接收

//缓存器(I2CDRR)，接收到设定好的数据数量（I2CCNT值）时输出停止位STOP.

//值得注意的是：理论上写完数据就能马上读取 数据 ，但事实上EEPROM存储器仍需要一

////定延时来存储数据，约有2ms左右。通过示波器可以观察到，写完数据后，并不能马上

//成功读取数据，也就是说读数据的第一步骤要重复好几次（总线为50K时，大约要重复

//8次）才能成功。

#define I2C_MSGSTAT_READ_BUSY         0x0023//读取数据忙状态：这个状态是为读取数

//据而设。在读数据的第二步骤中，发完START位+设备地址后，就设为这一状态。意在说//明IIC开始等待接收固定数量（I2CCNT值）的数据。可以通过查询ARDY位判断。

//头文件中的其他定义应该没什么大问题了！

2. AT24C1024 EEPROM读写数据格式

（1）AT24C1024设备地址：

| 1 | 0 | 1 | 0 | 0 | A1 | P0 | R/W |
| --- | --- | --- | --- | --- | --- | --- | --- |

（2）单字节写入：START -> 发送从设备地址(写控制码R/W=0) -> 处理Ack -> 发送字节地址 -> 处理Ack [-> 发送1字节数据 -> 处理Ack] -> STOP。如下图：

（3）按页写入： START -> 发送从设备地址(写控制码R/W=0) -> 处理Ack -> 发送字节地址 -> 处理Ack [-> 发送1字节数据 -> 处理Ack-> 发送第2字节数据 -> 处理Ack-> 发送第3字节数据 -> 处理Ack……直到发完X字节] -> STOP。如下图。注意，连续写入的数据字节数不能超过每页所能容下的总量。如果写入的数据超过一页的长度，将发生回卷，即从EEPROM的0地址处进行数据覆盖。比如，AT24C1024有512页，每页最大容纳256字节。超过这个长度，地址指针将从每页首地址重新开始。

（4）随机单字节读取：第一步：发START -> 发送从设备地址(写控制码R/W=0) -> 处理Ack -> 发送字节地址高位 -> 处理Ack -> 发送字节地址低位 -> 处理Ack -> 第二步：发START -> 发送器件地址(读控制码R/W=1) -> 处理Ack -> 接收1字节数据 -> STOP。

（5）随机连续读取：在随机单字节读取操作的STOP信号发送之前，加入若干个 [-> 发送Ack -> 接收1字节数据] 即可实现。

（6）当前位置单字节读取：START -> 发送从设备地址(读控制码) -> 处理Ack -> 发送字节地址 -> 处理Ack -> 接收1字节数据 -> STOP。当前指的是之前进行过读取操作但是没有发送STOP信号，EEPROM芯片内部指针所在的位置即为当前位置。

（7）当前位置连续读取：在当前位置单节读取操作的STOP信号发送之前，加入若干个 [-> 发送Ack -> 接收1字节数据] 即可实现。

备注：部分内容引用百度文库中的《I2C读写流程》文档

3. 主程序说明及流程图

详细的注释可以参见程序附录，，这里主要解释一下设备地址取0x50的缘由。从AT24C1024的数据手册可知，设备地址为。而在程序中设置的地址是0x50，即0B0101 0000。看似不妥，其实，这个例子中设备地址采用7位地址模式，这样只取0x50的低7位作为设备地址的高七位，设备地址的最低位R/W则由寄存器I2CMDR中的TRX位决定。这样读数据时设备地址为0B1010 0001，写数据时设备地址改为0B1010 0000。通过示波器可以验证这些数据。

问题1：每次检测到SCD中断时，也就是顺利发完指定数量的数据或者接收指定数量的数据了，理论上I2caRegs.I2CCNT==0，但是本人设了几个中断点去观察该寄存器的值，发现总是不为零。通过观察I2caRegs.I2CFFTX.bit.TXFFST和I2caRegs.I2CFFRX.bit.RXFFST，可以发现这两个寄存器为零时，I2caRegs.I2CCNT也不为零。由此可知，I2CCNT不能实时反映发送/接收存储器中数据的数量，不过可以用TXFFST/RXFFST来代替。

图1 主程序流程图,图2为写数据程序流程图，图3为读数据程序流程图，图4为中断函数流程图。

图1 主程序流程图

图2写数据程序流程图                    图3读数据程序流程图

图4中断函数流程图

4.程序附录

//可以用以下代码替换示例程序Example_280xI2c_eeprom.c中的代码，再进行调试。//

//该程序跟原始程序没多大区别，主要改变为：引入几个统计变量，改变发送数据长度为2字节//

// TI File $Revision: /main/5 $

// Checkin $Date: April 4, 2007   17:18:36 $

//###########################################################################

//

// FILE:    Example_280xI2c_eeprom.c

//

// TITLE:   DSP280x I2C EEPROM Example

//

// ASSUMPTIONS:

//

//    This program requires the DSP280x header files.

//

//    This program requires an external I2C EEPROM connected to

//    the I2C bus at address 0x50.

//

//    As supplied, this project is configured for "boot to SARAM"

//    operation.  The 280x Boot Mode table is shown below.

//    For information on configuring the boot mode of an eZdsp,

//    please refer to the documentation included with the eZdsp,

//

//       Boot      GPIO18     GPIO29    GPIO34

//       Mode      SPICLKA    SCITXDA

//                  SCITXB

//       -------------------------------------

//       Flash       1          1        1

//       SCI-A       1          1        0

//       SPI-A       1          0        1

//       I2C-A       1          0        0

//       ECAN-A      0          1        1

//       SARAM       0          1        0  <- "boot to SARAM"

//       OTP         0          0        1

//       I/0         0          0        0

//

// DESCRIPTION:

//

//    This program will write 1-14 words to EEPROM and read them back.

//    The data written and the EEPROM address written to are contained

//    in the message structure, I2cMsgOut1. The data read back will be

//    contained in the message structure I2cMsgIn1.

//    --------------------------------------------------------------

//    CODE MODIFICATIONS ARE REQUIRED FOR 60 MHZ DEVICES (In

//    DSP280x_Examples.h in the common/include/ directory, set

//    #define CPU_FRQ_60MHZ to 1, and #define CPU_FRQ_100MHZ to 0).

//    --------------------------------------------------------------

//    This program will work with the on-board I2C EEPROM supplied on

//    the F280x eZdsp.

//

//

//###########################################################################

// Original Author: D.F.

//

// $TI Release: DSP280x Header Files V1.60 $

// $Release Date: December 3, 2007 $

//###########################################################################

#include "DSP280x_Device.h"     // DSP280x Headerfile Include File

#include "DSP280x_Examples.h"   // DSP280x Examples Include File

// Note: I2C Macros used in this example can be found in the

// DSP280x_I2C_defines.h file

// Prototype statements for functions found within this file.

void   I2CA_Init(void);

Uint16 I2CA_WriteData(struct I2CMSG *msg);

Uint16 I2CA_ReadData(struct I2CMSG *msg);

interrupt void i2c_int1a_isr(void);

void pass(void);

void fail(void);

#define I2C_SLAVE_ADDR        0x50  //EEPROM地址

#define I2C_NUMBYTES          2     //为方便示波器观察，设置发送2字节的数据

#define I2C_EEPROM_HIGH_ADDR  0x11  //数据的写入地址高位

#define I2C_EEPROM_LOW_ADDR   0x0F  //数据的写入地址低位

// Global variables//全局变量

// Two bytes will be used for the outgoing address,//有2个字节是地址

// thus only setup 14 bytes maximum//最多只能设置14字节数据

struct I2CMSG I2cMsgOut1={I2C_MSGSTAT_SEND_WITHSTOP,//初始状态为：发送带停止位数据

I2C_SLAVE_ADDR,

I2C_NUMBYTES,

I2C_EEPROM_HIGH_ADDR,

I2C_EEPROM_LOW_ADDR,

0xff,                   // Msg Byte 01

0x3F,                   // Msg Byte 02

0x56,                   // Msg Byte 03

0x78,                   // Msg Byte 04

0x9A,                   // Msg Byte 05

0xBC,                   // Msg Byte 06

0xDE,                   // Msg Byte 07

0xF0,                   // Msg Byte 08

0x11,                   // Msg Byte 09

0x10,                   // Msg Byte 10

0x11,                   // Msg Byte 11

0x12,                   // Msg Byte 12

0x13,                   // Msg Byte 13

0x12,		       // Msg Byte 14

};

struct I2CMSG I2cMsgIn1={ I2C_MSGSTAT_SEND_NOSTOP,

I2C_SLAVE_ADDR,

I2C_NUMBYTES,

I2C_EEPROM_HIGH_ADDR,

I2C_EEPROM_LOW_ADDR};

struct I2CMSG *CurrentMsgPtr;				// Used in interrupts

Uint16 PassCount;

Uint16 FailCount;

Uint16 ARDY_ISRC_NACK_number=0;  //统计ARDY中断源引起的中断中NACK次数

Uint16 SCD_ISRC_number=0;       //统计SCD中断源引起的中断次数

Uint16 ARDY_ISRC_number=0;      //统计ARDY中断源引起的中断次数

Uint16 all_ISRC_number=0;       //统计所有中断源引起的中断次数

Uint16 Write_load_num=0;        //统计写数据函数调用次数

Uint16 Read_load_num1=0;        //统计读数据函数调用次数1第一步骤

Uint16 Read_load_num2=0;        //统计读数据函数调用次数2第二步骤

void main(void)

{

Uint16 Error;

Uint16 i;

CurrentMsgPtr = &I2cMsgOut1;

// Step 1. Initialize System Control:

// PLL, WatchDog, enable Peripheral Clocks

// This example function is found in the DSP280x_SysCtrl.c file.

InitSysCtrl();

// Step 2. Initalize GPIO:

// This example function is found in the DSP280x_Gpio.c file and

// illustrates how to set the GPIO to it's default state.

// InitGpio();

// Setup only the GP I/O only for I2C functionality

InitI2CGpio();

// Step 3. Clear all interrupts and initialize PIE vector table:

// Disable CPU interrupts

DINT;

// Initialize PIE control registers to their default state.

// The default state is all PIE interrupts disabled and flags

// are cleared.

// This function is found in the DSP280x_PieCtrl.c file.

InitPieCtrl();

// Disable CPU interrupts and clear all CPU interrupt flags:

IER = 0x0000;

IFR = 0x0000;

// Initialize the PIE vector table with pointers to the shell Interrupt

// Service Routines (ISR).

// This will populate the entire table, even if the interrupt

// is not used in this example.  This is useful for debug purposes.

// The shell ISR routines are found in DSP280x_DefaultIsr.c.

// This function is found in DSP280x_PieVect.c.

InitPieVectTable();

// Interrupts that are used in this example are re-mapped to

// ISR functions found within this file.

EALLOW;	// This is needed to write to EALLOW protected registers

PieVectTable.I2CINT1A = &i2c_int1a_isr;

EDIS;   // This is needed to disable write to EALLOW protected registers

// Step 4. Initialize all the Device Peripherals:

// This function is found in DSP280x_InitPeripherals.c

// InitPeripherals(); // Not required for this example

I2CA_Init();

// Step 5. User specific code

// Clear Counters

PassCount = 0;

FailCount = 0;

// Clear incoming message buffer

for (i = 0; i < I2C_MAX_BUFFER_SIZE; i++)

{

I2cMsgIn1.MsgBuffer[i] = 0x0000;

}

// Enable interrupts required for this example

// Enable I2C interrupt 1 in the PIE: Group 8 interrupt 1

PieCtrlRegs.PIEIER8.bit.INTx1 = 1;

// Enable CPU INT8 which is connected to PIE group 8

IER |= M_INT8;

EINT;

// Application loop

for(;;)

{

//////////////////////////////////

// Write data to EEPROM section //

//////////////////////////////////

// Check the outgoing message to see if it should be sent.

// In this example it is initialized to send with a stop bit.

if(I2cMsgOut1.MsgStatus == I2C_MSGSTAT_SEND_WITHSTOP)

{   Write_load_num++;

Error = I2CA_WriteData(&I2cMsgOut1);

// If communication is correctly initiated, set msg status to busy//如果通信已经正确初始化，设置msg状态为’忙‘，并

// and update CurrentMsgPtr for the interrupt service routine.    //更新作为中断服务路径的指针CurrentMsgPtr。

// Otherwise, do nothing and try again next loop. Once message is //否则，不做任何操作，等待下一次重新写数据。一旦信息正确

// initiated, the I2C interrupts will handle the rest. Search for //初始化，I2C中断将处理接下来的事情。可以在

// ICINTR1A_ISR in the i2c_eeprom_isr.c file.                     //i2c_eeprom_isr.c文件中找到ICINTR1A_ISR。

if (Error == I2C_SUCCESS)                            //数据已成功放入缓存寄存器

{

CurrentMsgPtr = &I2cMsgOut1;                      //改变指针路径

I2cMsgOut1.MsgStatus = I2C_MSGSTAT_WRITE_BUSY;    //设为发送数据忙状态

}

}  // end of write section

///////////////////////////////////

// Read data from EEPROM section //该过程包括两次发送START位，第一次发送地址

///////////////////////////////////

// Check outgoing message status. Bypass read section if status is

// not inactive.

if (I2cMsgOut1.MsgStatus == I2C_MSGSTAT_INACTIVE)

{

// Check incoming message status.

if(I2cMsgIn1.MsgStatus == I2C_MSGSTAT_SEND_NOSTOP)

{  Read_load_num1++;

// EEPROM address setup portion

while(I2CA_ReadData(&I2cMsgIn1) != I2C_SUCCESS)

{

// Maybe setup an attempt counter to break an infinite while    //这里可以设置一个计数器用于终止可能存在的无限循环。

// loop. The EEPROM will send back a NACK while it is performing//当EEPROM忙于处理写数据操作时，将返回一个NACK。

// a write operation. Even though the write communique is  //对DSP来说，此刻写操作已经完成。但是，EEPROM仍需要

// complete at this point, the EEPROM could still be busy //一定时间完成数据的烧写1.8ms左右。在EEPROM完成烧写之前，都会

// programming the data. Therefore, multiple attempts are  //返回NACK，告诉DSP它还在忙。因此，DSP需要多次尝试，直到

// necessary.                                       //EEPROM空闲了，并返回ACK确认为止。

}

// Update current message pointer and message status

CurrentMsgPtr = &I2cMsgIn1;

I2cMsgIn1.MsgStatus = I2C_MSGSTAT_SEND_NOSTOP_BUSY;

//第一次发送START成功，即地址数据已发送成功，给发无停止位数据忙状态，作为第二次发送START位标志

}

// Once message has progressed past setting up the internal address//一旦成功写入EEPROM地址，便可以再次发送START起始位

// of the EEPROM, send a restart to read the data bytes from the   //然后从EEPROM存储器中读取数据。

// EEPROM. Complete the communique with a stop bit. MsgStatus is   //DSP采可用一个停止位来结束数据接收。

// updated in the interrupt service routine.                       //msg状态在中断程序中被更新了。

else if(I2cMsgIn1.MsgStatus == I2C_MSGSTAT_RESTART)               //该状态是在中断服务程序中态被赋予

{  Read_load_num2++;

// Read data portion

while(I2CA_ReadData(&I2cMsgIn1) != I2C_SUCCESS)

{

// Maybe setup an attempt counter to break an infinite while

// loop.

}

// Update current message pointer and message status

CurrentMsgPtr = &I2cMsgIn1;

I2cMsgIn1.MsgStatus = I2C_MSGSTAT_READ_BUSY;

//第二次发送START，且数据接收成功，给下一个状态-读忙，作为中断中读取接收缓存中数据的标志

}

}  // end of read section

}   // end of for(;;)

}   // end of main

void I2CA_Init(void)

{

// Initialize I2C

I2caRegs.I2CSAR = 0x0050;		// Slave address - EEPROM control code

#if (CPU_FRQ_100MHZ)

I2caRegs.I2CPSC.all = 9;		// Prescaler - need 7-12 Mhz on module clk

#endif

#if (CPU_FRQ_60MHZ)

I2caRegs.I2CPSC.all = 6;		// Prescaler - need 7-12 Mhz on module clk

#endif

I2caRegs.I2CCLKL = 95;			// NOTE: must be non zero

I2caRegs.I2CCLKH = 95;			// NOTE: must be non zero

I2caRegs.I2CIER.all = 0x24;		// Enable SCD & ARDY interrupts

I2caRegs.I2CMDR.all = 0x0020;	// Take I2C out of reset

// Stop I2C when suspended

//0b0000 0000 0010 0000，NACKMOD=0，FREE=0中断产生时IIC停止运行，STT=0不产生开始位，

//STP=0不产生停止位,MST=0从模式,TRX=0接收模式，XA=0 七位地址模式，RM=0不重复,DLB=0无回来

//IRS=1使能I2C模块，STB=0模块未处在开始字节模式,FDF=0自由数据格式被禁止

//注意：TRX决定AT24C1024设备地址中的R/W，这里R/W=1读数据

I2caRegs.I2CFFTX.all = 0x6000;	// Enable FIFO mode and TXFIFO

I2caRegs.I2CFFRX.all = 0x2040;	// Enable RXFIFO, clear RXFFINT,

return;

}

Uint16 I2CA_WriteData(struct I2CMSG *msg)

{

Uint16 i;

// Wait until the STP bit is cleared from any previous master communication.

// Clearing of this bit by the module is delayed until after the SCD bit is

// set. If this bit is not checked prior to initiating a new message, the

// I2C could get confused.

if (I2caRegs.I2CMDR.bit.STP == 1)

{

return I2C_STP_NOT_READY_ERROR;

}

// Setup slave address

I2caRegs.I2CSAR = msg->SlaveAddress;

// Check if bus busy

if (I2caRegs.I2CSTR.bit.BB == 1)

{

return I2C_BUS_BUSY_ERROR;

}

////////判断总线空闲后进行数据发送////////

// Setup number of bytes to send

// MsgBuffer + Address

I2caRegs.I2CCNT = msg->NumOfBytes+2; //递减

// Setup data to send

I2caRegs.I2CDXR = msg->MemoryHighAddr;

I2caRegs.I2CDXR = msg->MemoryLowAddr;

// for (i=0; i<msg->NumOfBytes-2; i++)

for (i=0; i<msg->NumOfBytes; i++)

{

I2caRegs.I2CDXR = *(msg->MsgBuffer+i);

}

// Send start as master transmitter

I2caRegs.I2CMDR.all = 0x6E20; // 按以下参数配置IIC并使能IIC

//【0b0110 1110 0010 0000，NACKMOD=0，FREE=1中断产生时IIC能继续运行，STT=1产生开始位】

//【STP=1产生停止位,MST=1主模式,TRX=1发送，XA=0 七位地址模式，RM=0不重复,DLB=0无回路，IRS=1使能I2C模块，STB=0,FDF=0】

//开始发送数据，I2CCNT 递减模式统计数据到达0时产生中断，且产生停止位

//注意：TRX决定AT24C1024设备地址中的R/W，这里R/W=0写数据模式

return I2C_SUCCESS;

}

Uint16 I2CA_ReadData(struct I2CMSG *msg)

{  //要完成一次读数据任务：要产生两个START位，产生第1个START位之后发送设备地址和数据地址；

//产生第2个START位之后写入设备地址，并开始接收存储器提供的数据。

// Wait until the STP bit is cleared from any previous master communication.

// Clearing of this bit by the module is delayed until after the SCD bit is

// set. If this bit is not checked prior to initiating a new message, the

// I2C could get confused.

if (I2caRegs.I2CMDR.bit.STP == 1)

{

return I2C_STP_NOT_READY_ERROR;

}

I2caRegs.I2CSAR = msg->SlaveAddress;

if(msg->MsgStatus == I2C_MSGSTAT_SEND_NOSTOP) //产生第一个START位之后发送设备地址和数据地址////

{

// Check if bus busy

if (I2caRegs.I2CSTR.bit.BB == 1)

{

return I2C_BUS_BUSY_ERROR;

}

I2caRegs.I2CCNT = 2;                  // 设置发送数据字节数-递减模式-等0时可以允许产生stop位（若使能STP）

I2caRegs.I2CDXR = msg->MemoryHighAddr;//发送要读取数据的开始地址

I2caRegs.I2CDXR = (msg->MemoryLowAddr);

I2caRegs.I2CMDR.all = 0x2620;			// 按以下参数配置IIC并使能IIC

//【STT =1发送起始位，STP=0不产生停止位，MST=1主模式,TRX=1发送,IRS=1使能I2C模块】

}                                        // 开始发送数据I2CCNT 递减模式统计数据到达0时产生中断，但是没有产生停止位

//注意：TRX决定AT24C1024设备地址中的R/W，这里R/W=0写数据

else if(msg->MsgStatus == I2C_MSGSTAT_RESTART)//产生第2个START位之后发送设备地址，然后开始接收存储器提供的数据//

{

I2caRegs.I2CCNT = msg->NumOfBytes;	// 设置数据接收字节数量

I2caRegs.I2CMDR.all = 0x2C20;			// 按以下参数配置IIC并使能IIC

// 【STT =1发送起始位,STP=1产生停止位,MST=1主模式,TRX=0接收,IRS=1使能I2C模块】

}                                        //开始接收数据，I2CCNT递减模式统计数据到达0时产生中断，有产生停止位

//注意-TRX决定AT24C1024设备地址中的R/W，这里R/W=1读数据

return I2C_SUCCESS;                      //到此说明发送地址/读取数据成功

}

interrupt void i2c_int1a_isr(void)          // I2C-A

{

Uint16 IntSource, i;

all_ISRC_number++;//统计所有中断次数

// Read interrupt source

IntSource = I2caRegs.I2CISRC.all;

// Interrupt source = stop condition detected

if(IntSource == I2C_SCD_ISRC)

{

SCD_ISRC_number++;

// If completed message was writing data, reset msg to inactive state

if (CurrentMsgPtr->MsgStatus == I2C_MSGSTAT_WRITE_BUSY)

{

CurrentMsgPtr->MsgStatus = I2C_MSGSTAT_INACTIVE;//发送完成，便换为未启动状态；说明可以进行下一次写数据

}

else

{

// If a message receives a NACK during the address setup portion of the

// EEPROM read, the code further below included in the register access ready

// interrupt source code will generate a stop condition. After the stop

// condition is received (here), set the message status to try again.

// User may want to limit the number of retries before generating an error.

if(CurrentMsgPtr->MsgStatus == I2C_MSGSTAT_SEND_NOSTOP_BUSY)//发送无停止位地址忙

{

CurrentMsgPtr->MsgStatus = I2C_MSGSTAT_SEND_NOSTOP;//更新为发送停止位状态，可允许下一发送地址

}

// If completed message was reading EEPROM data, reset msg to inactive state

// and read data from FIFO.

else if (CurrentMsgPtr->MsgStatus == I2C_MSGSTAT_READ_BUSY) //读数据忙状态，说明可对接收缓存器I2CDRR进行接收数据读取了

{

CurrentMsgPtr->MsgStatus = I2C_MSGSTAT_INACTIVE;         //设为未启状态动，再读数据，这样允许下一次写数据到EEPROM

for(i=0; i < I2C_NUMBYTES; i++)

{

CurrentMsgPtr->MsgBuffer[i] = I2caRegs.I2CDRR;

}

// Check recieved data     //读完接收到的数据，接下来进行判断数据是否准确

for(i=0; i < I2C_NUMBYTES; i++)

{

if(I2cMsgIn1.MsgBuffer[i] == I2cMsgOut1.MsgBuffer[i])

{

PassCount++;

}

else

{

FailCount++;

}

}

if(PassCount == I2C_NUMBYTES)

{

pass();

}

else

{

fail();

}

//检查完毕

}

}

}  // end of stop condition detected

// Interrupt source = Register Access Ready

// This interrupt is used to determine when the EEPROM address setup portion of the

//该中断用于判断读数据通讯中何时完成EEPROM数据地址的设置；

// read data communication is complete. Since no stop bit is commanded, this flag

//由于不产生停止位，这个标志（ARDY）告诉我们地址数据是否发送完成。

// tells us when the message has been sent instead of the SCD flag. If a NACK is

//如果收到NACK，清除NACK位，并设置产生一个停止位。

// received, clear the NACK bit and command a stop. Otherwise, move on to the read

//否则，继续读取通讯数据。

// data portion of the communication.   else if(IntSource == I2C_ARDY_ISRC)           //IIC模块准备好中断：包括【复位，发完一个数据单元（1~8bits的数据）

{                                             //却没收到ACK/STOP或者出现counter=0】都可引发该中断

ARDY_ISRC_number++;

if(I2caRegs.I2CSTR.bit.NACK == 1)          //因为EEPROM处理数据忙而回复的NACK

{

I2caRegs.I2CMDR.bit.STP = 1;            //产生一个STOP停止位，从而引发一次SCD中断。

I2caRegs.I2CSTR.all = I2C_CLR_NACK_BIT; //清除NACK位

ARDY_ISRC_NACK_number++;

}

else if(CurrentMsgPtr->MsgStatus == I2C_MSGSTAT_SEND_NOSTOP_BUSY)

{

CurrentMsgPtr->MsgStatus = I2C_MSGSTAT_RESTART;//更新为重发START位状态，为接收数据准备

}

}  // end of register access ready

else

{

// Generate some error due to invalid interrupt source

asm("   ESTOP0");

}

// Enable future I2C (PIE Group 8) interrupts

PieCtrlRegs.PIEACK.all = PIEACK_GROUP8;  //重启中断允许

}

void pass()

{

asm("   ESTOP0");

for(;;);

}

void fail()

{

asm("   ESTOP0");

for(;;);

}

//===========================================================================

// No more.

//===========================================================================