tvp5158图像质量问题描述

使用SONY的摄像头模块（输出PAL制式），用tvp5158进行视频采集之后，发现图像质量出现以下问题：

在反差明显的竖直物体（特别是比较细的物体）边缘上容易出现彩色的偏蓝色的条纹，并且抖动厉害。


修改tvp5158的寄存器，发现：（1）降低视频的色彩饱和度可以减轻彩色条纹，如果直接将色彩信息去除，画面只有亮度信息（即灰度图）时，彩色条纹抖动消失；（2）Luminance Processing Control 2（19h）寄存器中滤波器的选择对彩色条纹有较大影响，如果使能梳状滤波器（Luminance adaptive comb enable），或者使能陷波器（Luminance adaptive comb disable），彩色条纹比较明显，如果将梳状滤波器和陷波器都bypass（Luma comb/trap filter bypassed），则彩色条纹反而不明显；（3）对其他寄存器的修改有：Video Standard Select（0Dh）中将制式选择固定为(B, D, G, H, I, N) PAL，OFM Mode Control（B2h）中将Out_CLK_P_En和Video_Port_En使能，Noise Reduction Control（5Dh）中将NR_Bypass关闭，剩下的寄存器保持默认值。

根据上述现象，可以判断问题出现在色度和亮色分离上，有以下几个问题：
（1）为什么使用滤波器反而令彩色条纹更加严重？


（2）Luminance Processing Control 2（19h）中所描述的梳状滤波器，是否只要使能便可以作用，还是需要设置滤波器的参数，才可以起作用？参数在哪里设置？

下面的图中有通过vbus设置comb filter ram的描述，但是没有具体的说明如何设置

[image: image1.png]Host
Processor

’c Registers
00h

EOh

E1h
E8h

EAh
FFh

VBUS
Data

VBUS
Address

VBUS Registers

Comb
Filter RAM

VBUS [23:0

Figure 3-20. VBUS Access

00 0000h

40 3E50h

40 3EEFh

A0 3FFFh




另外在tvp5150中有以下设置comb filter参数的寄存器，而在tvp5158中没有。

[image: image2.png]3.22.26 Chrominance Control #2 Register

Address 18h

7 6

5

4 3 2 1 []

Chrominance comb filter mode [3:0]

Reserved WCF Chrominance fiter select

Chrominance comb filter mode [3:0] (CM[3:0]): Chrominance control #2 register 1Bh, bits 7-4

ACE | cE | cm3 | cmiz | oM | cMiop COMB FILTER SELECTION
[ 0 X X X X__| Comb filter disabled
[ 1 0 [ [ X_| Fixed 3-ine comb fiter with (1, 2, 1)/4 coefiicients
[ 1 [ [ 1 X__| Fixed 3-ine comb filter with (1, 0, 1)/2 coefiicients
[ 1 1 [ X X_| Fixed 2-ine comb filter
[ 1 [ 1 [ X__| Fixed 4-ine (1, 1, 1, 1/4 comb filter
[ 1 0 1 1 X_| Fixed 4-ine (1, 3, 3, 1)/8 comb filer
[ 1 1 1 X X_| Fixed 2-ine comb filter
1 X X [ [ 0| Adaptive between 3-ine (1, 2, 1)/4 and 2-ine comb fiter
at X X of of 1T__| Adaptive between 31ine (1, 2, 1)/4 comb fiter and no comb filter
1 X X [ 1 0| Adaptive between 3-ine (1, 0, 1)/2 comb filter and 2 ine comb fiter
1 X X [ 1 1__| Adaptive between 3-iine (1, 0, 1)22 comb fiter and no comb fiter
1 X X [ 0| Adaptive between 4-line (1, 1, 1, 1)/4 and 2 ine comb fiter
1% X X 11 of 1% | Adaptive between 4-line (1, 1, 1, 1)/4 comb fiiter and no comb filter
1 X X 1 1 0__| Adaptive between 4-line (1, 3, 3, 1)/8 comb filler and 2-ine comb fiter
1 X X 1 1 1| Adaptive between 4-line (1, 3. 3. 18 comb fiter and no comb fiter

T Indicates default settings for NTSC
T Indicates default settings for PAL mode





