//*****************************************************************************

//初始化EPI

//HB8模式,ALE+双片选

//*****************************************************************************

void M4InitEPI(void)

{

//Enable and provide a clock to the EPI module in Run mode

SysCtlPeripheralEnable(SYSCTL_PERIPH_EPI0);//RCGCEPI

//Enable the clock to the appropriate GPIO module

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOA);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOB);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOC);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOG);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOK);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOL);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOM);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPION);

SysCtlPeripheralEnable(SYSCTL_PERIPH_GPIOQ);

//Set the GPIO AFSEL bits for the appropriate pins.

//Set the GPIO current 8mA , slew rate, push-pull

GPIOPinTypeEPI(GPIO_PORTA_BASE, GPIO_PIN_6 | GPIO_PIN_7); //AFSEL

GPIOPinTypeEPI(GPIO_PORTB_BASE, GPIO_PIN_2 | GPIO_PIN_3);

GPIOPinTypeEPI(GPIO_PORTC_BASE, GPIO_PIN_4 | GPIO_PIN_5 | GPIO_PIN_6 | GPIO_PIN_7);

GPIOPinTypeEPI(GPIO_PORTG_BASE, GPIO_PIN_0 | GPIO_PIN_1);

GPIOPinTypeEPI(GPIO_PORTK_BASE, GPIO_PIN_0 | GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3

| GPIO_PIN_6 | GPIO_PIN_7);

GPIOPinTypeEPI(GPIO_PORTL_BASE, GPIO_PIN_0 | GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3

| GPIO_PIN_4);

GPIOPinTypeEPI(GPIO_PORTM_BASE, GPIO_PIN_0 | GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3);

GPIOPinTypeEPI(GPIO_PORTN_BASE, GPIO_PIN_2 | GPIO_PIN_3);

GPIOPinTypeEPI(GPIO_PORTQ_BASE, GPIO_PIN_0 | GPIO_PIN_1 | GPIO_PIN_2 | GPIO_PIN_3);

//Assign the EPI signals to the appropriate pins

GPIOPinConfigure(GPIO_PA6_EPI0S8);

GPIOPinConfigure(GPIO_PA7_EPI0S9);

GPIOPinConfigure(GPIO_PB2_EPI0S27);

GPIOPinConfigure(GPIO_PB3_EPI0S28);

GPIOPinConfigure(GPIO_PC4_EPI0S7);

GPIOPinConfigure(GPIO_PC5_EPI0S6);

GPIOPinConfigure(GPIO_PC6_EPI0S5);

GPIOPinConfigure(GPIO_PC7_EPI0S4);

GPIOPinConfigure(GPIO_PG0_EPI0S11);

GPIOPinConfigure(GPIO_PG1_EPI0S10);

GPIOPinConfigure(GPIO_PK0_EPI0S0);

GPIOPinConfigure(GPIO_PK1_EPI0S1);

GPIOPinConfigure(GPIO_PK2_EPI0S2);

GPIOPinConfigure(GPIO_PK3_EPI0S3);

GPIOPinConfigure(GPIO_PK6_EPI0S25);

GPIOPinConfigure(GPIO_PK7_EPI0S24);

//

GPIOPinConfigure(GPIO_PL0_EPI0S16);

GPIOPinConfigure(GPIO_PL1_EPI0S17);

GPIOPinConfigure(GPIO_PL2_EPI0S18);

GPIOPinConfigure(GPIO_PL3_EPI0S19);

GPIOPinConfigure(GPIO_PL4_EPI0S26);

GPIOPinConfigure(GPIO_PM0_EPI0S15);

GPIOPinConfigure(GPIO_PM1_EPI0S14);

GPIOPinConfigure(GPIO_PM2_EPI0S13);

GPIOPinConfigure(GPIO_PM3_EPI0S12);

GPIOPinConfigure(GPIO_PN2_EPI0S29);

GPIOPinConfigure(GPIO_PN3_EPI0S30);

GPIOPinConfigure(GPIO_PQ0_EPI0S20);

GPIOPinConfigure(GPIO_PQ1_EPI0S21);

GPIOPinConfigure(GPIO_PQ2_EPI0S22);

GPIOPinConfigure(GPIO_PQ3_EPI0S23);

//EPI mode select EPI_O_CFG

EPIModeSet(EPI0_BASE, EPI_MODE_HB8);

//HB8CFG HB8CFG22

//ALE+双CSn:EPI30用作ALE,EPI27用作CS1n,EPI26用作CS0n,

//CS波特率相同,

//CS信号及读写选通信号:低电平有效

//最长等待时间1,读/写等待态1个EPI周期

//字访问模式

EPIConfigHB8Set(EPI0_BASE,

EPI_HB8_MODE_ADMUX | EPI_HB8_WRWAIT_3 | EPI_HB8_RDWAIT_3  | EPI_HB8_WORD_ACCESS | EPI_HB8_CSCFG_ALE_DUAL_CS | EPI_HB8_ALE_HIGH, 1);

//CS Bauld divider

EPIDividerSet(EPI0_BASE, 0x0);//不分频，CS1n CS0n使用相同频率

//EPI Address mapping

//外设基地址0xA000 0000

//RAM基地址0x6000 0000

EPIAddressMapSet(EPI0_BASE, EPI_ADDR_PER_SIZE_256B | EPI_ADDR_PER_BASE_A | EPI_ADDR_RAM_SIZE_16MB

|EPI_ADDR_RAM_BASE_6);

}

void M3InitEPI(void)

{

SYSCTL_RCGC1_R |=  0x40000000;

SYSCTL_RCGC2_R |=	 0x1FC;

if(SYSCTL_GPIOHBCTL_R & 0x080)

GPIO_PORTH_AHB_PCTL_R = 0x88888888;

else

GPIO_PORTH_PCTL_R = 0x88888888;

if(SYSCTL_GPIOHBCTL_R & 0x04)

{

GPIO_PORTC_AHB_PCTL_R = GPIO_PORTC_PCTL_R & 0x0000ffff;

GPIO_PORTC_AHB_PCTL_R = GPIO_PORTC_PCTL_R | 0x88880000;

}

else

{

GPIO_PORTC_PCTL_R = GPIO_PORTC_PCTL_R & 0x0000ffff;

GPIO_PORTC_PCTL_R = GPIO_PORTC_PCTL_R | 0x88880000;

}

if(SYSCTL_GPIOHBCTL_R & 0x10)

{

GPIO_PORTE_AHB_PCTL_R = GPIO_PORTE_PCTL_R & 0xffffff00;

GPIO_PORTE_AHB_PCTL_R = GPIO_PORTE_PCTL_R | 0x00000088;

}

else

{

GPIO_PORTE_PCTL_R = GPIO_PORTE_PCTL_R & 0xffffff00;

GPIO_PORTE_PCTL_R = GPIO_PORTE_PCTL_R | 0x00000088;

}

if(SYSCTL_GPIOHBCTL_R & 0x20)

{

GPIO_PORTF_AHB_PCTL_R = GPIO_PORTF_PCTL_R & 0xff00ffff;

GPIO_PORTF_AHB_PCTL_R = GPIO_PORTF_PCTL_R | 0x00880000;

}

else

{

GPIO_PORTF_PCTL_R = GPIO_PORTF_PCTL_R & 0xff00ffff;

GPIO_PORTF_PCTL_R = GPIO_PORTF_PCTL_R | 0x00880000;

}

if(SYSCTL_GPIOHBCTL_R & 0x40)

{

GPIO_PORTG_AHB_PCTL_R = GPIO_PORTG_PCTL_R & 0xffffff00;

GPIO_PORTG_AHB_PCTL_R = GPIO_PORTG_PCTL_R | 0x00000088;

}

else

{

GPIO_PORTG_PCTL_R = GPIO_PORTG_PCTL_R & 0xffffff00;

GPIO_PORTG_PCTL_R = GPIO_PORTG_PCTL_R | 0x00000088;

}

if(SYSCTL_GPIOHBCTL_R & 0x100)

{

GPIO_PORTJ_AHB_PCTL_R = GPIO_PORTJ_PCTL_R & 0xffffff00;

GPIO_PORTJ_AHB_PCTL_R = GPIO_PORTJ_PCTL_R | 0x00000088;

}

else

{

GPIO_PORTJ_PCTL_R = GPIO_PORTJ_PCTL_R & 0xffffff00;

GPIO_PORTJ_PCTL_R = GPIO_PORTJ_PCTL_R | 0x00000088;

}

if(SYSCTL_GPIOHBCTL_R & 0x08)

{

GPIO_PORTD_AHB_PCTL_R = GPIO_PORTD_PCTL_R & 0x000fffff;

GPIO_PORTD_AHB_PCTL_R = GPIO_PORTD_PCTL_R | 0xaaa00000;

}

else

{

GPIO_PORTD_PCTL_R = GPIO_PORTD_PCTL_R & 0x000fffff;

GPIO_PORTD_PCTL_R = GPIO_PORTD_PCTL_R | 0xaaa00000;

}

GPIO_PORTC_DIR_R = GPIO_PORTC_DIR_R & (~(EPI_PORTC_PINS));

GPIO_PORTD_DIR_R = GPIO_PORTD_DIR_R & (~(EPI_PORTD_PINS));

GPIO_PORTE_DIR_R = GPIO_PORTE_DIR_R & (~(EPI_PORTE_PINS));

GPIO_PORTF_DIR_R = GPIO_PORTF_DIR_R & (~(EPI_PORTF_PINS));

GPIO_PORTG_DIR_R = GPIO_PORTG_DIR_R & (~(EPI_PORTG_PINS));

GPIO_PORTH_DIR_R = GPIO_PORTH_DIR_R & (~(EPI_PORTH_PINS));

GPIO_PORTJ_DIR_R = GPIO_PORTJ_DIR_R & (~(EPI_PORTJ_PINS));

GPIO_PORTC_AFSEL_R = GPIO_PORTC_AFSEL_R | EPI_PORTC_PINS;

GPIO_PORTD_AFSEL_R = GPIO_PORTD_AFSEL_R | EPI_PORTD_PINS;

GPIO_PORTE_AFSEL_R = GPIO_PORTE_AFSEL_R | EPI_PORTE_PINS;

GPIO_PORTF_AFSEL_R = GPIO_PORTF_AFSEL_R | EPI_PORTF_PINS;

GPIO_PORTG_AFSEL_R = GPIO_PORTG_AFSEL_R | EPI_PORTG_PINS;

GPIO_PORTH_AFSEL_R = GPIO_PORTH_AFSEL_R | EPI_PORTH_PINS;

GPIO_PORTJ_AFSEL_R = GPIO_PORTJ_AFSEL_R | EPI_PORTJ_PINS;

GPIO_PORTC_DR2R_R = GPIO_PORTC_DR2R_R & (~(EPI_PORTC_PINS));

GPIO_PORTC_DR4R_R = GPIO_PORTC_DR4R_R & (~(EPI_PORTC_PINS));

GPIO_PORTC_DR8R_R = GPIO_PORTC_DR8R_R | (EPI_PORTC_PINS);

GPIO_PORTC_SLR_R = GPIO_PORTC_SLR_R & (~(EPI_PORTC_PINS));

GPIO_PORTD_DR2R_R = GPIO_PORTD_DR2R_R & (~(EPI_PORTD_PINS));

GPIO_PORTD_DR4R_R = GPIO_PORTD_DR4R_R & (~(EPI_PORTD_PINS));

GPIO_PORTD_DR8R_R = GPIO_PORTD_DR8R_R | (EPI_PORTD_PINS);

GPIO_PORTD_SLR_R = GPIO_PORTD_SLR_R & (~(EPI_PORTD_PINS));

GPIO_PORTE_DR2R_R = GPIO_PORTE_DR2R_R & (~(EPI_PORTE_PINS));

GPIO_PORTE_DR4R_R = GPIO_PORTE_DR4R_R & (~(EPI_PORTE_PINS));

GPIO_PORTE_DR8R_R = GPIO_PORTE_DR8R_R | (EPI_PORTE_PINS);

GPIO_PORTE_SLR_R = GPIO_PORTE_SLR_R & (~(EPI_PORTE_PINS));

GPIO_PORTF_DR2R_R = GPIO_PORTF_DR2R_R & (~(EPI_PORTF_PINS));

GPIO_PORTF_DR4R_R = GPIO_PORTF_DR4R_R & (~(EPI_PORTF_PINS));

GPIO_PORTF_DR8R_R = GPIO_PORTF_DR8R_R | (EPI_PORTF_PINS);

GPIO_PORTF_SLR_R = GPIO_PORTF_SLR_R & (~(EPI_PORTF_PINS));

GPIO_PORTG_DR2R_R = GPIO_PORTG_DR2R_R & (~(EPI_PORTG_PINS));

GPIO_PORTG_DR4R_R = GPIO_PORTG_DR4R_R & (~(EPI_PORTG_PINS));

GPIO_PORTG_DR8R_R = GPIO_PORTG_DR8R_R | (EPI_PORTG_PINS);

GPIO_PORTG_SLR_R = GPIO_PORTG_SLR_R & (~(EPI_PORTG_PINS));

GPIO_PORTH_DR2R_R = GPIO_PORTH_DR2R_R & (~(EPI_PORTH_PINS));

GPIO_PORTH_DR4R_R = GPIO_PORTH_DR4R_R & (~(EPI_PORTH_PINS));

GPIO_PORTH_DR8R_R = GPIO_PORTH_DR8R_R | (EPI_PORTH_PINS);

GPIO_PORTH_SLR_R = GPIO_PORTH_SLR_R & (~(EPI_PORTH_PINS));

GPIO_PORTJ_DR2R_R = GPIO_PORTJ_DR2R_R & (~(EPI_PORTJ_PINS));

GPIO_PORTJ_DR4R_R = GPIO_PORTJ_DR4R_R & (~(EPI_PORTJ_PINS));

GPIO_PORTJ_DR8R_R = GPIO_PORTJ_DR8R_R | (EPI_PORTJ_PINS);//8mA

GPIO_PORTJ_SLR_R = GPIO_PORTJ_SLR_R & (~(EPI_PORTJ_PINS));//禁用斜率控制

GPIO_PORTC_ODR_R = GPIO_PORTC_ODR_R & (~(EPI_PORTC_PINS));//禁止开漏

GPIO_PORTC_PUR_R = GPIO_PORTC_PUR_R & (~(EPI_PORTC_PINS));//不启用弱上拉电阻

GPIO_PORTC_PDR_R = GPIO_PORTC_PDR_R & (~(EPI_PORTC_PINS));//不启用弱下拉电阻

GPIO_PORTC_DEN_R = GPIO_PORTC_DEN_R | EPI_PORTC_PINS;     //启用数字功能

GPIO_PORTD_ODR_R = GPIO_PORTD_ODR_R & (~(EPI_PORTD_PINS));

GPIO_PORTD_PUR_R = GPIO_PORTD_PUR_R & (~(EPI_PORTD_PINS));

GPIO_PORTD_PDR_R = GPIO_PORTD_PDR_R & (~(EPI_PORTD_PINS));

GPIO_PORTD_DEN_R = GPIO_PORTD_DEN_R | EPI_PORTD_PINS;

GPIO_PORTE_ODR_R = GPIO_PORTE_ODR_R & (~(EPI_PORTE_PINS));

GPIO_PORTE_PUR_R = GPIO_PORTE_PUR_R & (~(EPI_PORTE_PINS));

GPIO_PORTE_PDR_R = GPIO_PORTE_PDR_R & (~(EPI_PORTE_PINS));

GPIO_PORTE_DEN_R = GPIO_PORTE_DEN_R | EPI_PORTE_PINS;

GPIO_PORTF_ODR_R = GPIO_PORTF_ODR_R & (~(EPI_PORTF_PINS));

GPIO_PORTF_PUR_R = GPIO_PORTF_PUR_R & (~(EPI_PORTF_PINS));

GPIO_PORTF_PDR_R = GPIO_PORTF_PDR_R & (~(EPI_PORTF_PINS));

GPIO_PORTF_DEN_R = GPIO_PORTF_DEN_R | EPI_PORTF_PINS;

GPIO_PORTG_ODR_R = GPIO_PORTG_ODR_R & (~(EPI_PORTG_PINS));

GPIO_PORTG_PUR_R = GPIO_PORTG_PUR_R & (~(EPI_PORTG_PINS));

GPIO_PORTG_PDR_R = GPIO_PORTG_PDR_R & (~(EPI_PORTG_PINS));

GPIO_PORTG_DEN_R = GPIO_PORTG_DEN_R | EPI_PORTG_PINS;

GPIO_PORTH_ODR_R = GPIO_PORTH_ODR_R & (~(EPI_PORTH_PINS));

GPIO_PORTH_PUR_R = GPIO_PORTH_PUR_R & (~(EPI_PORTH_PINS));

GPIO_PORTH_PDR_R = GPIO_PORTH_PDR_R & (~(EPI_PORTH_PINS));

GPIO_PORTH_DEN_R = GPIO_PORTH_DEN_R | EPI_PORTH_PINS;

GPIO_PORTJ_ODR_R = GPIO_PORTJ_ODR_R & (~(EPI_PORTJ_PINS));

GPIO_PORTJ_PUR_R = GPIO_PORTJ_PUR_R & (~(EPI_PORTJ_PINS));

GPIO_PORTJ_PDR_R = GPIO_PORTJ_PDR_R & (~(EPI_PORTJ_PINS));

GPIO_PORTJ_DEN_R = GPIO_PORTJ_DEN_R | EPI_PORTJ_PINS;

GPIO_PORTC_AMSEL_R = GPIO_PORTC_AMSEL_R & (~(EPI_PORTC_PINS));//禁用模拟功能

GPIO_PORTD_AMSEL_R = GPIO_PORTD_AMSEL_R & (~(EPI_PORTD_PINS));

GPIO_PORTE_AMSEL_R = GPIO_PORTE_AMSEL_R & (~(EPI_PORTE_PINS));

GPIO_PORTF_AMSEL_R = GPIO_PORTF_AMSEL_R & (~(EPI_PORTF_PINS));

GPIO_PORTG_AMSEL_R = GPIO_PORTG_AMSEL_R & (~(EPI_PORTG_PINS));

GPIO_PORTH_AMSEL_R = GPIO_PORTH_AMSEL_R & (~(EPI_PORTH_PINS));

GPIO_PORTJ_AMSEL_R = GPIO_PORTJ_AMSEL_R & (~(EPI_PORTJ_PINS));

//EPI0_BAUD_R = 0;//不分频，使用系统时钟

EPI0_BAUD_R = 1;

EPI0_CFG_R = EPI_MODE_HB8;

//EPI0_HB8CFG2_R = 0x83000000;//启用字访问模式 CS0n与CS1n 使用的波特率 COUNT0寄存器的EPIBAUD位提供 ALE+双CSn

EPI0_HB8CFG2_R = 0x83000000;

//EPI0_HB8CFG_R = 0x00000150; //地址数据复用模式  读写选通低电平有效 读写1个等待状态 最长等待时间1

EPI0_HB8CFG_R = 0x00000100;

EPI0_ADDRMAP_R = (EPI_ADDR_PER_SIZE_256B | EPI_ADDR_PER_BASE_A |

EPI_ADDR_RAM_SIZE_16MB | EPI_ADDR_RAM_BASE_6);

}